3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250197

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3682 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 8.1.1.1a.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 8.1.1.1a.1 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.1.1a.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1a.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	7

3 Branches executed	9

4	Execution Log Files	9

4.1	MediaTek MT6986D UE	9

5	References	9

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.1.1a.1.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6986D

Verification Status:	      PASS

# Corrections required

## Change 1

| Function name | f_NR_Paging_Trigger_DCI_2_7 |
| --- | --- |
| Reason for change | This function calculates the time at which the PEI is supposed to be transmitted to the UE. However, the function also needs to consider the pei-FrameOffset to calculate the Paging Occasion to transmit the actual Paging to the UE. Secondly, the function also needs to consider the different paths for a RRC Resume procedure and a RRC Setup Request procedure |
| Summary of change | Modified the function to consider pei-FrameOffset to send Paging for a RRC Setup Request or a RRC Resume procedure. |
| TTCN module | NR5GC_SysinfoPaging.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently. See MCC160 implementation below. |

Before Change:

function f_NR_Paging_Trigger_DCI_2_7(NR_CellId_Type p_NR_CellId, template (omit) integer p_SlotOffsetPaging := omit, bitstring p_PagingIndication, integer p_PayloadSizeDCI_2_7 ) runs on NR5GC_PTC { var SubFrameTiming_Type v_CurrentTiming; var B32_Type v_NG_TMSI := f_NR5GC_CellInfo_GetNG_TMSI(p_NR_CellId); var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var SubFrameTiming_Type v_PagingOccasion; var integer v_SlotOffsetPaging := f_NR_GetSlotOffsetForPaging(p_NR_CellId); if (isvalue(p_SlotOffsetPaging)) { v_SlotOffsetPaging := valueof(p_SlotOffsetPaging); } f_Delay(tsc_NR_DelayForCellSelection); //Get current SFN and subframe v_CurrentTiming := f_NR_GetCurrentTiming(p_NR_CellId); //Calculate next Paging Occasion v_PagingOccasion := f_NR_Calculate_PF_PO_DCI_2_7(v_CurrentTiming, v_NG_TMSI, v_SlotsNumberPerSubframe, v_SCS, v_SlotOffsetPaging); SYS.send(cads_NR_DciTrigger_PEI_REQ(p_NR_CellId, cs_TimingInfo_NR(v_PagingOccasion), p_PagingIndication, p_PayloadSizeDCI_2_7)); }

After Change:

function f_NR_Paging_Trigger_DCI_2_7(NR_CellId_Type p_NR_CellId, template (omit) integer p_SlotOffsetPaging := omit, bitstring p_PagingIndication, integer p_PayloadSizeDCI_2_7, NG_5G_S_TMSI p_NG_5G_S_TMSI, I_RNTI_Value p_FullI_RNTI, boolean p_ResumeProc := false, boolean p_peiPaging := true ) runs on NR5GC_PTC { var SubFrameTiming_Type v_CurrentTiming; var B32_Type v_NG_TMSI := f_NR5GC_CellInfo_GetNG_TMSI(p_NR_CellId); var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var SubFrameTiming_Type v_PagingOccasion; var integer v_SlotOffsetPaging := f_NR_GetSlotOffsetForPaging(p_NR_CellId); if (isvalue(p_SlotOffsetPaging)) { v_SlotOffsetPaging := valueof(p_SlotOffsetPaging); } f_Delay(tsc_NR_DelayForCellSelection); //Get current SFN and subframe v_CurrentTiming := f_NR_GetCurrentTiming(p_NR_CellId); //Calculate next Paging Occasion v_PagingOccasion := f_NR_Calculate_PF_PO_DCI_2_7(v_CurrentTiming, v_NG_TMSI, v_SlotsNumberPerSubframe, v_SCS, v_SlotOffsetPaging); SYS.send(cads_NR_DciTrigger_PEI_REQ(p_NR_CellId, cs_TimingInfo_NR(v_PagingOccasion), p_PagingIndication, p_PayloadSizeDCI_2_7)); v_PagingOccasion.SFN.Number := v_PagingOccasion.SFN.Number + 3; //pei-FrameOffset v_PagingOccasion.Subframe.Number := v_SlotOffsetPaging / v_SlotsNumberPerSubframe; v_PagingOccasion := f_NR_SubFrameTiming_SetSlot(v_PagingOccasion, v_SlotOffsetPaging mod v_SlotsNumberPerSubframe, v_SCS); if (p_peiPaging) { if (p_ResumeProc){ SYS.send(cas_NR_CellConfig_Paging_REQ(p_NR_CellId, v_PagingOccasion, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(p_FullI_RNTI)),omit)); } else { SYS.send(cas_NR_CellConfig_Paging_REQ(p_NR_CellId, v_PagingOccasion, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(p_NG_5G_S_TMSI)),omit)); } } }

MCC160 Implementation:

function f_NR_Paging_Trigger_DCI_2_7(NR_CellId_Type p_NR_CellId, template (omit) integer p_SlotOffsetPaging := omit, bitstring p_PagingIndication, integer p_PayloadSizeDCI_2_7, boolean p_ResumeProc := false, boolean p_PeiPaging := true) runs on NR5GC_PTC { //@sic R5s250197 Ch. 1 sic@ var SubFrameTiming_Type v_CurrentTiming; var B32_Type v_NG_TMSI := f_NR5GC_CellInfo_GetNG_TMSI(p_NR_CellId); var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var NG_NAS_GutiParameters_Type v_GutiParameters; var NG_5G_S_TMSI v_NG_5G_S_TMSI; var I_RNTI_Value v_FullI_RNTI := tsc_NR_I_RNTI_Value1; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var SubFrameTiming_Type v_PagingOccasion; var integer v_SlotOffsetPaging := f_NR_GetSlotOffsetForPaging(p_NR_CellId); v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(p_NR_CellId); v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters); if (isvalue(p_SlotOffsetPaging)) { v_SlotOffsetPaging := valueof(p_SlotOffsetPaging); } f_Delay(tsc_NR_DelayForCellSelection); //Get current SFN and subframe v_CurrentTiming := f_NR_GetCurrentTiming(p_NR_CellId); //Calculate next Paging Occasion v_PagingOccasion := f_NR_Calculate_PF_PO_DCI_2_7(v_CurrentTiming, v_NG_TMSI, v_SCS ); SYS.send(cads_NR_DciTrigger_PEI_REQ(p_NR_CellId, cs_TimingInfo_NR(v_PagingOccasion), p_PagingIndication, p_PayloadSizeDCI_2_7)); v_PagingOccasion.SFN.Number := v_PagingOccasion.SFN.Number + 3; //pei-FrameOffset v_PagingOccasion.Subframe.Number := v_SlotOffsetPaging / v_SlotsNumberPerSubframe; v_PagingOccasion := f_NR_SubFrameTiming_SetSlot(v_PagingOccasion, v_SlotOffsetPaging mod v_SlotsNumberPerSubframe, v_SCS); if (p_PeiPaging){ if (p_ResumeProc){ SYS.send(cas_NR_CellConfig_Paging_REQ(p_NR_CellId, v_PagingOccasion, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(v_FullI_RNTI)))); } else { SYS.send(cas_NR_CellConfig_Paging_REQ(p_NR_CellId, v_PagingOccasion, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI)))); } } }

## Change 2

| Function name | f_NR_Calculate_PF_PO_DCI_2_7 |
| --- | --- |
| Reason for change | The UE calculates the reception of PEI frame (DCI 2_7) based on the NW configuration: PF_offset =0, T = 128, N = half T ,Ns = 1, UE_ID % N = 632 % 64 = 56 5G - s_tmsi = 0x0041c2345678 = 282431084152 UE_ID = 282431084152 %1024 = 632 (SFN+0) % 128 = (T/N)* (UE_ID % N) = 2 * 56 =112 SFN = 112 + 128n Which results in the PF for PEI = 240, 368, 496, 624, 752.... Hence, the PF_offset should be 0. Secondly, the calculation of the FirstPagingFrame needs to consider the pei-FrameOffset to calculate the first Monitoring Occasion as per TS 38.304 sec 7.2.1: “The UE determines the first PDCCH MO for PEI-O based on peiFrameOffset and firstPDCCH-MonitoringOccasionOfPEI-O, as for the case with SearchSpaceId > 0 configured.” Thirdly, the value of firstPDCCH-MonitoringOccasionOfPEI-O is 0 and the UE as per the spec description: “firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/poNumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.” Starts a reception window for PEI DCI 2_7 which is 1 SF long. |
| Summary of change | - PF Offset is set to 0 - Calculation of the FirstPagingFrame for the transmission of DCI 2_7 factors in pei-FrameOffset - Calculation of Subframe and Slot for the transmission of DCI 2_7 factors in firstPDCCH-MonitoringOccasionOfPEI-O |
| TTCN module | NR5GC_SysinfoPaging.ttcn |
| MCC160 Comment | 1.- Postponed, pending agreement at RAN5#108 2.- OK 3.- OK Also removed p_SlotsNumberPerSubframe and p_SlotOffset from header as per Change 1. |

Before Change:

function f_NR_Calculate_PF_PO_DCI_2_7 (SubFrameTiming_Type p_CurrentTiming, B32_Type p_NG_TMSI, integer p_SlotsNumberPerSubframe, SubcarrierSpacing p_SCS, integer p_SlotOffset) return SubFrameTiming_Type { // @sic R5-233824 sic@ var integer v_UE_ID := bit2int(p_NG_TMSI) mod 1024; // 5G-S-TMSI mod 1024 var integer v_PF_Offset := 1; //Ns:= 2 - number of paging occasions for a PF var integer v_NextSfn := p_CurrentTiming.SFN.Number + tsc_NR_SfnDelay; //Absolute value - no mod 1024 var integer v_SfnPlusOffsetModT; var integer v_FirstPagingFrame; var integer v_T := 128; var integer v_N := 64; // N: number of total paging frames in T var SubFrameTiming_Type v_PagingOccasion; //Calculate the first paging Frame //SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) //Absolute value - no mod 1024 v_SfnPlusOffsetModT := (v_T / v_N) * (v_UE_ID mod v_N); if (((v_NextSfn + v_PF_Offset) mod v_T) < v_SfnPlusOffsetModT) { v_FirstPagingFrame := (((v_NextSfn + v_PF_Offset) - ((v_NextSfn + v_PF_Offset) mod v_T)) + v_SfnPlusOffsetModT) - v_PF_Offset; } else { v_FirstPagingFrame := ((v_NextSfn + v_PF_Offset) - ((v_NextSfn + v_PF_Offset) mod v_T)) + v_SfnPlusOffsetModT + v_T - v_PF_Offset; } // PDCCH monitoring occasions according to TS 38.523-1 cl. 7.3.4.4. // Values taken from TS38.508-2 Table 4.6.3-53: DownlinkConfigCommonSIB condition PEI; TS 38.508-1 Table 4.6.3-162: SearchSpace condition PEI and TS 38.508-1 Table 4.6.3-96 firstPDCCH-MonitoringOccasionOfPEI-O = [0,1] // SearchSpaceExt_v1700[2] acc to 38.508-1 Table 4.6.3-96: PDCCH-ConfigCommon with condition PEI monitoringSlotPeriodicityAndOffset_v1710 is omit // For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF. v_PagingOccasion.HSFN.Number := (p_CurrentTiming.HSFN.Number + (v_FirstPagingFrame)/1024 ) mod 1024; v_PagingOccasion.SFN.Number := v_FirstPagingFrame mod 1024; v_PagingOccasion.Subframe.Number := p_SlotOffset / p_SlotsNumberPerSubframe; v_PagingOccasion.Symbol := valueof(cs_SymbolTimingInfo_FirstSlot); v_PagingOccasion := f_NR_SubFrameTiming_SetSlot(v_PagingOccasion, p_SlotOffset mod p_SlotsNumberPerSubframe, p_SCS); return v_PagingOccasion; }

After Change:

function f_NR_Calculate_PF_PO_DCI_2_7 (SubFrameTiming_Type p_CurrentTiming, B32_Type p_NG_TMSI, integer p_SlotsNumberPerSubframe, SubcarrierSpacing p_SCS, integer p_SlotOffset) return SubFrameTiming_Type { // @sic R5-233824 sic@ var integer v_UE_ID := bit2int(p_NG_TMSI) mod 1024; // 5G-S-TMSI mod 1024 var integer v_PF_Offset := 0; //WA# //Ns:= 2 - number of paging occasions for a PF var integer v_NextSfn := p_CurrentTiming.SFN.Number + tsc_NR_SfnDelay; //Absolute value - no mod 1024 var integer v_SfnPlusOffsetModT; var integer v_FirstPagingFrame; var integer v_T := 128; var integer v_N := 64; // N: number of total paging frames in T var SubFrameTiming_Type v_PagingOccasion; //Calculate the first paging Frame //SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) //Absolute value - no mod 1024 v_SfnPlusOffsetModT := (v_T / v_N) * (v_UE_ID mod v_N); if (((v_NextSfn + v_PF_Offset) mod v_T) < v_SfnPlusOffsetModT) { v_FirstPagingFrame := (((v_NextSfn + v_PF_Offset) - ((v_NextSfn + v_PF_Offset) mod v_T)) + v_SfnPlusOffsetModT) - v_PF_Offset; } else { v_FirstPagingFrame := ((v_NextSfn + v_PF_Offset) - ((v_NextSfn + v_PF_Offset) mod v_T)) + v_SfnPlusOffsetModT + v_T - v_PF_Offset; } v_FirstPagingFrame := v_FirstPagingFrame - 3; //WA#(pei-FrameOffset) // PDCCH monitoring occasions according to TS 38.523-1 cl. 7.3.4.4. // Values taken from TS38.508-2 Table 4.6.3-53: DownlinkConfigCommonSIB condition PEI; TS 38.508-1 Table 4.6.3-162: SearchSpace condition PEI and TS 38.508-1 Table 4.6.3-96 firstPDCCH-MonitoringOccasionOfPEI-O = [0,1] // SearchSpaceExt_v1700[2] acc to 38.508-1 Table 4.6.3-96: PDCCH-ConfigCommon with condition PEI monitoringSlotPeriodicityAndOffset_v1710 is omit // For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF. v_PagingOccasion.HSFN.Number := (p_CurrentTiming.HSFN.Number + (v_FirstPagingFrame)/1024 ) mod 1024; v_PagingOccasion.SFN.Number := v_FirstPagingFrame mod 1024; v_PagingOccasion.Subframe.Number := 0; //WA#(firstPDCCH-MonitoringOccasionOfPEI-O) v_PagingOccasion.Symbol := valueof(cs_SymbolTimingInfo_FirstSlot); v_PagingOccasion := f_NR_SubFrameTiming_SetSlot(v_PagingOccasion, 0,//WA#(firstPDCCH-MonitoringOccasionOfPEI-O) p_SCS); return v_PagingOccasion; }

## Change 3

| Function name | fl_TC_8_1_1_1a_1_TestBody |
| --- | --- |
| Reason for change | The function f_NR_Paging_Trigger_DCI_2_7 transmitting the DCI 2_7 at Steps 3, 14 and 16 need to be modified to consider the RRC Resume procedure and RRC Setup Request procedure |
| Summary of change | Function f_NR_Paging_Trigger_DCI_2_7 at Steps 3,14 and 16 is modified to accept additional parameters p_NG_5G_S_TMSI and p_FullI_RNTI |
| TTCN module | RRC_PEI_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle but implemented in Change 1 as per MCC160 proposed changes. |

Before Change:

function fl_TC_8_1_1_1a_1_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParameters; var float v_Wait10s := 10.0; var NG_5G_S_TMSI v_NG_5G_S_TMSI; var NG_5G_S_TMSI v_NG_5G_S_TMSI_diff; var PCCH_Config v_PcchConfig; //@siclog "Step 1" siclog@ //The SS transmits a PEI and the Paging message including an un-matched identity (incorrect ng-5G-S-TMSI) on the associated PO only. v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters); v_NG_5G_S_TMSI_diff := f_CreateNewS_TMSING_5G_S_TMSI(v_NG_5G_S_TMSI); v_PcchConfig := f_NR_GetSIB1_PcchConfig(nr_Cell1); f_NR_Paging(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI_diff)), v_PcchConfig.defaultPagingCycle); <<SKIPPED CODE>> //@siclog "Step 3" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(nr_Cell1, -,'11'B, 22 ); <<SKIPPED CODE>> //@siclog "Step 14" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only on NR Cell 1. f_NR_Paging_Trigger_DCI_2_7(nr_Cell1, -,'11'B, 22 ); <<SKIPPED CODE>> //@siclog "Step 16" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only.. f_NR_Paging_Trigger_DCI_2_7(nr_Cell2, -,'11'B, 22 ); <<SKIPPED CODE>>

After Change:

function fl_TC_8_1_1_1a_1_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParameters; var float v_Wait10s := 10.0; var NG_5G_S_TMSI v_NG_5G_S_TMSI; var NG_5G_S_TMSI v_NG_5G_S_TMSI_diff; var I_RNTI_Value v_FullI_RNTI; //WA# var PCCH_Config v_PcchConfig; //@siclog "Step 1" siclog@ //The SS transmits a PEI and the Paging message including an un-matched identity (incorrect ng-5G-S-TMSI) on the associated PO only. v_GutiParameters := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_NG_5G_S_TMSI := f_NR_Get5G_S_TMSI_FromGuti(v_GutiParameters); v_NG_5G_S_TMSI_diff := f_CreateNewS_TMSING_5G_S_TMSI(v_NG_5G_S_TMSI); v_FullI_RNTI := tsc_NR_I_RNTI_Value1; v_PcchConfig := f_NR_GetSIB1_PcchConfig(nr_Cell1); f_NR_Paging(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_5G_S_TMSI(v_NG_5G_S_TMSI_diff)), v_PcchConfig.defaultPagingCycle); <<SKIPPED CODE>> //@siclog "Step 3" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(nr_Cell1, -,'11'B, 22,v_NG_5G_S_TMSI,v_FullI_RNTI); //WA# <<SKIPPED CODE>> //@siclog "Step 14" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only on NR Cell 1. f_NR_Paging_Trigger_DCI_2_7(nr_Cell1, -,'11'B, 22,v_NG_5G_S_TMSI,v_FullI_RNTI); //WA# <<SKIPPED CODE>> //@siclog "Step 16" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only.. f_NR_Paging_Trigger_DCI_2_7(nr_Cell1, -,'11'B, 22,v_NG_5G_S_TMSI,v_FullI_RNTI); //WA# <<SKIPPED CODE>>

# 3 Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NEA1/NIA1 algorithms.

# Execution Log Files

## MediaTek MT6986D UE

The MediaTek MT6986D UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_1_1a_1_NR5GC_FR1_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250198: Supporting information for addition of NR5GC TC 8.1.1.1a.1 in FR1 |
| --- | --- |