3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250212

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3689 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Redcap testcase 7.1.1.1.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest, TEI17_Test |  |  |  |  |  | Date: |  |  | 2025-06-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A runtime exception for uninitialized nonCriticalExtension template within the SIB1-v1740-IE structure is observed while running this testcase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_NR_CellInfo_SetSIB1_SI_RequestConfigRedCap , initialized the nonCriticalExtension SIB1-v1800-IEs with omit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail due to run time exception. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.15.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_NR_CellInfo_SetSIB1_SI_RequestConfigRedCap |
| --- | --- |
| Reason for change | 1. Runtime exception for uninitialized nonCriticalExtension template within the SIB1-v1740-IE structure. |
| Summary of change | 1. Initialized the nonCriticalExtension SIB1-v1800-IEs with omit. |
| TTCN module | \Common\NR\NR_CellInfo.ttcn |
| MCC160 | Accepted, implemented sighltly differently to avoid araising any v1800 extension. See proposed implementation below. |

Before Change:

function f_NR_CellInfo_SetSIB1_SI_RequestConfigRedCap(NR_CellId_Type p_NR_CellId, template (omit) SI_RequestConfig p_SI_RequestConfig) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (isvalue(p_SI_RequestConfig)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.si_SchedulingInfo_v1740.si_RequestConfigRedCap_r17:= valueof(p_SI_RequestConfig); } else { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.si_SchedulingInfo_v1740:= omit; } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

After Change:

function f_NR_CellInfo_SetSIB1_SI_RequestConfigRedCap(NR_CellId_Type p_NR_CellId, template (omit) SI_RequestConfig p_SI_RequestConfig) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (isvalue(p_SI_RequestConfig)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.si_SchedulingInfo_v1740.si_RequestConfigRedCap_r17:= valueof(p_SI_RequestConfig); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := omit;// WA#WI=1410734 } else { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.si_SchedulingInfo_v1740:= omit; } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

MCC160 proposed implementation:

function f_NR_CellInfo_SetSIB1_SI_RequestConfigRedCap(NR_CellId_Type p_NR_CellId,

template (omit) SI_RequestConfig p_SI_RequestConfig) runs on NR_BASE_PTC

{

var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId);

if (isvalue(p_SI_RequestConfig)) {

v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.si_SchedulingInfo_v1740.si_RequestConfigRedCap_r17:= valueof(p_SI_RequestConfig);

//@sic R5s250212 sic@

if (not(isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension))) {

v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := omit;

}

} else {

v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.si_SchedulingInfo_v1740:= omit;

}

f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo);

}