3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250216

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3690 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Rel-17 NR NTN test case 7.1.2.2.5a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Modify SN of PDU at Steps 5 for UE to trigger t-reassembly timer. Required Prose CR submitted in support |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify SN of PDU at Steps 5 and 7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2.5a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.2.2.5a which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

Contact:	Narendra Kalahasti

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_2_2_5a_NR_TestBody () |
| --- | --- |
| Reason for change | Modify SN of PDU at Steps 5 for UE to trigger t-reassembly timer. Required Prose CR submitted in support |
| Summary of change | Modify SN of PDU at Steps 5 and 7. |
| TTCN module | NTN_Idle_NR5GC_Testsuite.ttcn |
| MCC160 Comment | Accepted if associated prose CR will be approved. |

Before Change:

function f_TC_7_1_2_2_5a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / UM RLC / Receive window operation and t-Reassembly expiry var integer v_RLC_Len1stSeg := 7; // Length of 1st segment var integer v_RLC_Len2ndSeg := 5; // Length of 2nd segment var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var NR_RLC_DataField_Type v_NR_RLC_Data; timer t_WatchDog; var integer v_Koffset; //Anritsu TTCN CR R5s250107 var float v_Tolerance; v_NR_RLC_DataList := fl_NR_GenerateRLC_UM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 2); // @siclog "Step 1" siclog@ //The SS transmits UMD PDU#1 containing first segment of RLC SDU#1, SN=0. f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], 0, v_RLC_Len1stSeg, tsc_NR_SI_StartOfSDU); //Anritsu TTCN CR R5s250107 v_Koffset := f_NR_CellInfo_GetKoffset (nr_Cell1); v_Tolerance := int2float(v_Koffset)/1000.0 + p_RLC_Rec.t_Reassembly; // @siclog "Step 2" siclog@ //Wait for t-Reassembly to ensure that t-Reassembly for the UMD PDU#1 expires. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, v_Tolerance)); //Anritsu TTCN CR R5s250107 // @siclog "Step 3" siclog@ //The SS transmits UMD PDU#2 containing last segment of RLC SDU#1, SN=0. p_RLC_Rec.UM_TX_Next := 0; f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], v_RLC_Len1stSeg, v_RLC_SDUsize - v_RLC_Len1stSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 4" siclog@ //Check: For 1 sec after step 3, does the UE transmit RLC SDU#1? //Anritsu TTCN CR R5s250107 v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, substr(v_NR_RLC_DataList[tsc_RLC_SDU1], 3, lengthof (v_NR_RLC_DataList[tsc_RLC_SDU1]) - 3))); t_WatchDog.start(2.0); alt { [] a_NR_RxUMD_oneSDU(p_NR_DRB_Id, v_NR_RLC_Data) { //Anritsu TTCN CR R5s250107 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_WatchDog.stop; } [] t_WatchDog.timeout {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4");} } // @siclog "Step 5" siclog@ //The SS transmits UMD PDU#3 containing first segment of RLC SDU#2, SN=1. p_RLC_Rec.UM_TX_Next := 1; f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU2], 0, v_RLC_Len1stSeg, tsc_NR_SI_StartOfSDU); v_Tolerance := int2float(v_Koffset)/1000.0 + p_RLC_Rec.t_ReassemblyExt_r17; //Anritsu TTCN CR R5s250107 // @siclog "Step 6" siclog@ //Wait for t-ReassemblyExt-r17 to ensure that t-ReassemblyExt-r17 for the UMD PDU#3 expires. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, v_Tolerance));//Anritsu TTCN CR R5s250107 // @siclog "Step 7" siclog@ //The SS transmits UMD PDU#4 containing last segment of RLC SDU#2, SN=1. f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU2], v_RLC_Len1stSeg, v_RLC_SDUsize - v_RLC_Len1stSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 8" siclog@ //Check: For 1 sec after step 7, does the UE transmit RLC SDU#2? //Anritsu TTCN CR R5s250107 v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, substr(v_NR_RLC_DataList[tsc_RLC_SDU2], 3, lengthof (v_NR_RLC_DataList[tsc_RLC_SDU2]) - 3))); t_WatchDog.start(2.0); alt { [] a_NR_RxUMD_oneSDU(p_NR_DRB_Id, v_NR_RLC_Data) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); t_WatchDog.stop; } [] t_WatchDog.timeout {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");} } }

After Change:

function f_TC_7_1_2_2_5a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / UM RLC / Receive window operation and t-Reassembly expiry var integer v_RLC_Len1stSeg := 7; // Length of 1st segment var integer v_RLC_Len2ndSeg := 5; // Length of 2nd segment var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var NR_RLC_DataField_Type v_NR_RLC_Data; timer t_WatchDog; var integer v_Koffset; //Anritsu TTCN CR R5s250107 var float v_Tolerance; v_NR_RLC_DataList := fl_NR_GenerateRLC_UM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 2); // @siclog "Step 1" siclog@ //The SS transmits UMD PDU#1 containing first segment of RLC SDU#1, SN=0. f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], 0, v_RLC_Len1stSeg, tsc_NR_SI_StartOfSDU); //Anritsu TTCN CR R5s250107 v_Koffset := f_NR_CellInfo_GetKoffset (nr_Cell1); v_Tolerance := int2float(v_Koffset)/1000.0 + p_RLC_Rec.t_Reassembly; // @siclog "Step 2" siclog@ //Wait for t-Reassembly to ensure that t-Reassembly for the UMD PDU#1 expires. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, v_Tolerance)); //Anritsu TTCN CR R5s250107 // @siclog "Step 3" siclog@ //The SS transmits UMD PDU#2 containing last segment of RLC SDU#1, SN=0. p_RLC_Rec.UM_TX_Next := 0; f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], v_RLC_Len1stSeg, v_RLC_SDUsize - v_RLC_Len1stSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 4" siclog@ //Check: For 1 sec after step 3, does the UE transmit RLC SDU#1? //Anritsu TTCN CR R5s250107 v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, substr(v_NR_RLC_DataList[tsc_RLC_SDU1], 3, lengthof (v_NR_RLC_DataList[tsc_RLC_SDU1]) - 3))); t_WatchDog.start(2.0); alt { [] a_NR_RxUMD_oneSDU(p_NR_DRB_Id, v_NR_RLC_Data) { //Anritsu TTCN CR R5s250107 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_WatchDog.stop; } [] t_WatchDog.timeout {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4");} } // @siclog "Step 5" siclog@ //The SS transmits UMD PDU#3 containing first segment of RLC SDU#2, SN=1. p_RLC_Rec.UM_TX_Next := 2; f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU2], 0, v_RLC_Len1stSeg, tsc_NR_SI_StartOfSDU); v_Tolerance := int2float(v_Koffset)/1000.0 + p_RLC_Rec.t_ReassemblyExt_r17; //Anritsu TTCN CR R5s250107 // @siclog "Step 6" siclog@ //Wait for t-ReassemblyExt-r17 to ensure that t-ReassemblyExt-r17 for the UMD PDU#3 expires. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, v_Tolerance));//Anritsu TTCN CR R5s250107 // @siclog "Step 7" siclog@ //The SS transmits UMD PDU#4 containing last segment of RLC SDU#2, SN=1. f_NR_TxUMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU2], v_RLC_Len1stSeg, v_RLC_SDUsize - v_RLC_Len1stSeg, tsc_NR_SI_EndOfSDU, true); // @siclog "Step 8" siclog@ //Check: For 1 sec after step 7, does the UE transmit RLC SDU#2? //Anritsu TTCN CR R5s250107 v_NR_RLC_Data := f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, substr(v_NR_RLC_DataList[tsc_RLC_SDU2], 3, lengthof (v_NR_RLC_DataList[tsc_RLC_SDU2]) - 3))); t_WatchDog.start(2.0); alt { [] a_NR_RxUMD_oneSDU(p_NR_DRB_Id, v_NR_RLC_Data) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); t_WatchDog.stop; } [] t_WatchDog.timeout {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");} } }