3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250225

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3695 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC eCall over IMS testcase 11.5.4 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 11.5.4 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC eCall over IMS test case 11.5.4 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | A prose CR will be submitted |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	New function f_TC_11_5_4_NR5GC_IMS1	3

2.2	Correction to the function f_TC_11_5_4_NR5GC_EUTRA	4

2.3	Correction to the function fl_TC_11_5_4_NR5GC_EUTRA_TestBody	6

2.4	Correction to the function fl_TC_11_5_4_NR5GC_TestBody	8

2.5	Correction to the function f_TC_11_5_4_IRAT_IMS2	11

2.6	Correction to testcase TC_11_5_4_NR5GC	12

2.7	Correction to function TC_11_5_4_NR5GC	14

3	Branches executed	16

4	Execution Log Files	16

4.1	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies	16

5	References	17

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC eCall over IMS testcase 11.5.4 which is part of the iwd-TTCN3-B2024-06_D25wk11 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Narendra Kalahasti

Narendra.kalahasti@anritsu.com

Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

## Verification Test Summary

Test Case: 	11.5.4.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

R&S® 5G Protocol Conformance Test platform

UE used:	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

Verification Status:	PASS

# Corrections required

## New function f_TC_11_5_4_NR5GC_IMS1

| Function name | f_TC_11_5_4_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Normal IMS registration will be expected on LTE, default IMS PTC will expect it on NR. A new function needs to be created to specify IMS registration onn LTE RAT. |
| Summary of change | New function added to expect normal IMS registration on LTE instead of NR. |
| TTCN module | ECall_IRAT_NR5GC_IMS |
| MCC160 Comment | Accepted |

New function:

function f_TC_11_5_4_NR5GC_IMS1() runs on IMS_PTC { f_IMS_PTC_Init(); f_IMS_Registration(OtherIPCAN_E); f_IMS_IpcanReleaseWithOptionalDeregistrationOtherIPCAN(); }

## Correction to the function f_TC_11_5_4_NR5GC_EUTRA

| Function name | f_TC_11_5_4_NR5GC_EUTRA |
| --- | --- |
| Reason for change | - RTP loopback needs to be enabled for the eCall established during this test case. - Test case will be initilalised in EUTRA PTC. - Test case signaling will happen mostly on EUTRA PTC calling EUTRA functions, postamble function must be aligned. Calling f_EUTRA38_MultiPDN_Postamble will provoke a runtime error. |
| Summary of change | - Added HANDLE_UM_DATA as input parameter to activate RtpRtcpLoopback. - Moved MMI for USIM configuration to EUTRA PTC. - Called f_EUTRA_Postamble implementation. |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |
| MCC160 Comment | 1, 3. Accepted 2. Not needed. Test case begins in NR PTC |

Before Change:

<< SKIPPED CODE >> function f_TC_11_5_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { /* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS */ //@sic R5s250075 sic@ f_EUTRA38_IRAT_Init(c31, -, USE_BIG_GRANTS); // E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1. // Create cell and send out system information f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_5_4_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); // take down the cell now f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); f_EUTRA_ReleaseAllCells(); } //End of f_TC_11_5_4_NR5GC_EUTRA() << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function f_TC_11_5_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { /* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS */ //@sic R5s250075 sic@ f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA, USE_BIG_GRANTS); // E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1. // Create cell and send out system information f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. f_UT_USIM_Insert(UT, "USIM configuration 24 specified in table 6.4.1-24 TS 38.508-1"); fl_TC_11_5_4_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Postamble")); f_EUTRA_Postamble (eutra_Cell1, E1_IDLE); } //End of f_TC_11_5_4_NR5GC_EUTRA() << SKIPPED CODE >>

## Correction to the function fl_TC_11_5_4_NR5GC_EUTRA_TestBody

| Function name | fl_TC_11_5_4_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - Although being a NR test case, most of signaling and messages are expected to be received in LTE. To simplify changes UE configuration and initialisation (as well as corresponding MMIs) are moved to EUTRA PTC. - Coordination message to start PRACH preamble is required after eCall is triggered to inform NR5GC PTC - Functions called for eCall establihsment on LTE are incorrrect. There is already implemented a function to cover 4.5A.27 complete procedure. - A coordination message to wait for call release is missing. |
| Summary of change | - Moved initial steps and MMIs to EUTRA PTC. - Added coordination message. - Called already implemeted function to estbliash eCall on LTE. - Added missing coordination message. |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |
| MCC160 Comment | 1, 2. Rejected. First steps take place on the NR PTC 3, 4. Accepted as an alternative solution |

Before Change:

<< SKIPPED CODE >> function fl_TC_11_5_4_NR5GC_EUTRA_TestBody()runs on EUTRA_5GS_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; // @siclog "Step 6" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Suitable cell". f_EUTRA_NR_WaitForCoOrd_Trigger(NR,"Change Power Level"); f_EUTRA38_SetCellPower(eutra_Cell1, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE_FR2); //@sic R5-241629 sic@ //@siclog "Step 7" siclog@ //Check: Does the UE sends an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? f_EUTRA_MobileInfo_SetNetworkFeatureSupport(f_GetDefaultValueNetworkFeatureSupport()); v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4 (eutra_Cell1, PREAMBLE, Combined, DEFAULT); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 7"); //@siclog "Step 8-26" siclog@ //Steps 7 to 25 of Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support specified in TS 36.508 subclause 4.5A.27 take place. f_EUTRA_InitialRegistration_Step5_16c1 (eutra_Cell1, STATE2_IDLEUPDATE, Combined, v_NAS_Ind, omit, 0); f_EUTRA_508_EstablishIMSEmergencyCall_Steps9_15(eutra_Cell1); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // @siclog "Step 27" siclog@ //Now release the rrcConnection as the IMS PTC would be releasing the eCall f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, '7'H); // Emergency default bearer hardcoded to 6 and dedicated bearer 7 f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); } // End of fl_TC_11_5_4_NR5GC_EUTRA_TestBody() << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function fl_TC_11_5_4_NR5GC_EUTRA_TestBody()runs on EUTRA_5GS_PTC { //@siclog "Step 2" siclog@ f_EUTRA_SwitchOnUEandStartIP(eutra_Cell1); // @siclog "Step 3" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state f_Delay(60.0); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("PRACH Preamble")); // @siclog "Step 4" siclog@ //An automatic eCall is initiated. (Note 1) f_UT_InitiateECall(UT, AUTOMATIC); // @siclog "Step 4a" siclog@ f_EUTRA_NR_WaitForCoOrd_Trigger(NR,"Change Power Level"); // The SS configures: E-UTRA Cell 1 as "Suitable cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE_FR2); //@sic R5-241629 sic@ //@siclog "Step 7" siclog@ //Check: Does the UE sends an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? //@siclog "Step 8-26" siclog@ f_EUTRA_508_IMSeCallOnly_Steps3_25(eutra_Cell1); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 7"); //IMS call has been established, wait for trigger from IMS PTC to release rrc connection f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "IMS Call Release Msgs Sent"); // @siclog "Step 27" siclog@ //Now release the rrcConnection as the IMS PTC would be releasing the eCall f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, '7'H); // Emergency default bearer hardcoded to 6 and dedicated bearer 7 f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); } // End of fl_TC_11_5_4_NR5GC_EUTRA_TestBody() << SKIPPED CODE >>

## Correction to the function fl_TC_11_5_4_NR5GC_TestBody

| Function name | fl_TC_11_5_4_NR5GC_TestBody |
| --- | --- |
| Reason for change | - Although being a NR test case, most of signaling and messages are expected to be received in LTE. To simplify changes UE configuration and initialisation (as well as corresponding MMIs) are moved to LTE PTC. See change 2.3. - Currently test case will be stuck for 300 seconds. This might be a misinterpretation of timer T300 mentioned in table 11.5.4.3.2-2. T300 is an RRC timer with value of f 1 second in this case, which might be not enough to provoke PRACH failure due to max attempts. Value selected will be of 5.0 seconds then PRACH failure will be assured. In addition, timer will start after first RACH preamble is received, triggered by starting the eCall throught MMI. #draft proseCR would be raised for RAN5#108 meeting - Postamble is moved and executed in LTE PTC but resources need to be released on NR PTC. |
| Summary of change | - Moved code to function f_TC_11_5_12_NR5GC_EUTRA, please check Change 2.3. - Reworked PRACHPreamble timer and implementation. A prose CR will be submitted to change timer value at table 11.5.4.3.2-2. - Released cell on NR after calling postamble in LTE. |
| TTCN module | ECall_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment | 1. Rejected. First steps take place on the NR PTC 2. Alternative solution implemented: changed timer to 1s – value of T300 in SIB2 3. Accepted |

Before Change:

<< SKIPPED CODE >> function fl_TC_11_5_4_NR5GC_TestBody() runs on NR_BASE_PTC { timer t_Wait := 300.0; //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); << MOVED CODE >> // @siclog "Step 2" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state. f_Delay(60.0); // @siclog "Step 4" siclog@ //An automatic eCall is initiated. (Note 1) f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); f_UT_InitiateECall(UT, AUTOMATIC); << MOVED CODE >> //@siclog "Step 5" siclog@ //The UE transmits preamble on PRACH. //exception:The preamble and no response below are repeated for the duration of T300. t_Wait.start; alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { repeat; } []t_Wait.timeout{} }; << MOVED CODE >> // @siclog "Step 6" siclog@ // Change the power level in EUTRA side f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); << MOVED CODE >> // Now tell EUTRA test is finished f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); } << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function fl_TC_11_5_4_NR5GC_TestBody() runs on NR_BASE_PTC { var boolean v_Transmitted_PRACH := false; timer t_Wait := 5.0; //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "PRACH Preamble"); // Enable PRACH reception indication and do not reply to it f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 5" siclog@ //The UE transmits preamble on PRACH. //exception:The preamble and no response below are repeated for the duration of T300. SYSIND.check(receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?))); t_Wait.start; alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { if(not v_Transmitted_PRACH){ v_Transmitted_PRACH := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "The UE transmits preamble on PRACH"); // Change the power level in EUTRA side f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); } repeat; } [] t_Wait.timeout{ } } f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); f_NR_ReleaseAllCells(); }<< SKIPPED CODE >>

#### MCC160 Implementation

function fl_TC_11_5_4_NR5GC_TestBody() runs on NR_BASE_PTC

{

timer t_Wait := f_NR_SetTimerToleranceMax (nr_Cell1, rrcTimer, 1.0);

//@siclog "Step 1" siclog@

//The SS configures:

// - NR Cell 1 as "Serving cell"

f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

// @siclog "Step 2" siclog@

// The UE is Switched ON

f_UT_SwitchOnUE(UT);

// @siclog "Step 3" siclog@

// Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state.

f_Delay(60.0);

// @siclog "Step 4" siclog@

//An automatic eCall is initiated. (Note 1)

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable));

f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse);

f_UT_InitiateECall(UT, AUTOMATIC);

//@siclog "Step 5" siclog@

//The UE transmits preamble on PRACH.

SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?))

// Change the power level in EUTRA side

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level"));

//exception:The preamble and no response below are repeated for the duration of T300.

t_Wait.start;

alt {

[]  SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?))

{

repeat;

}

[]t_Wait.timeout{}

};

// @siclog "Step 6" siclog@

// Now wait until EUTRA test is finished

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

}

## Correction to the function f_TC_11_5_4_IRAT_IMS2

| Function name | f_TC_11_5_4_NR5GC_IMS2 |
| --- | --- |
| Reason for change | - IMS emergency signaling will be expected over LTE. - This test case specifies an automatic eCall. - Trigger event needs to match one created in change |
| Summary of change | - Changed input parameter to expect IMS registration on LTE. - Changed input paramater to expect an automatic eCall - Added additional information to trigger event to fit change |
| TTCN module | ECall_IRAT_NR5GC_IMS |
| MCC160 Comment | Accepted |

Before Change:

<< SKIPPED CODE >> function f_TC_11_5_4_NR5GC_IMS2() runs on IMS_PTC { var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); // @siclog "Steps 8-26" siclog@ //Steps 7 to 25 of Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support specified in TS 36.508 subclause 4.5A.27 take place. f_IMS_EmergencyRegistration(); f_ECall_CallSetup_AnnexC47(MANUAL); // @siclog "Step 27" siclog@ // Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33. v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); } // End of f_TC_11_5_4_IRAT_IMS2() << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function f_TC_11_5_4_NR5GC_IMS2() runs on IMS_PTC { var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); // @siclog "Steps 8-26" siclog@ //Steps 7 to 25 of Generic Test Procedure for eCall over IMS establishment in EUTRA: eCall Only Support specified in TS 36.508 subclause 4.5A.27 take place. f_IMS_EmergencyRegistration(-, -, OtherIPCAN_E); f_ECall_CallSetup_AnnexC47(AUTOMATIC); // @siclog "Step 27" siclog@ // Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33. v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); } // End of f_TC_11_5_4_IRAT_IMS2() << SKIPPED CODE >>

## Correction to testcase TC_11_5_4_NR5GC

| Function name | All the messages of this test case will be send amd received in LTE RAT, initial IMS registration, if it happens, will occur in LTE. Being a NR5GC test case, IMS default handler will expect IMS messages over NR and not LTE. |  |
| --- | --- | --- |
| Reason for change | Created a new IMS1 PTC to specify that IMS messages will be received over LTE. |  |
| Summary of change | All the messages of this test case will be send amd received in LTE RAT, initial IMS registration, if it happens, will occur in LTE. Being a NR5GC test case, IMS default handler will expect IMS messages over NR and not LTE. |  |
| TTCN module | NR5GC_IRAT_Testsuite |  |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_11_5_4_NR5GC() runs on MTC_NR5GC_IRAT system SYSTEM_NR5GC_IRAT { // @purpose // eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS var NR5GC_PTC v_NR5GC := null; var EUTRA_Multi5GS_PTC v_EUTRA_Multi5GS := null; var UTRAN_NR5GC_PTC v_UTRAN_NR5GC := null; var GERAN_NR5GC_PTC v_GERAN_NR5GC := null; var IMS_PTC v_IMS2 := null; var WLAN_PTC v_WLAN := null; timer t_GuardTimer := int2float(600); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_EUTRA_Multi5GS := EUTRA_Multi5GS_PTC.create("EUTRA_Multi5GS") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC_IRAT(system, v_NR5GC, v_EUTRA_Multi5GS, v_UTRAN_NR5GC, v_GERAN_NR5GC, v_IMS1, v_IMS2, v_WLAN); v_NR5GC.start(f_TC_11_5_4_NR5GC()); v_EUTRA_Multi5GS.start(f_TC_11_5_4_NR5GC_EUTRA()); v_IMS2.start(f_TC_11_5_4_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC_IRAT(t_GuardTimer); } }

After Change:

testcase TC_11_5_4_NR5GC() runs on MTC_NR5GC_IRAT system SYSTEM_NR5GC_IRAT { // @purpose // eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS var NR5GC_PTC v_NR5GC := null; var EUTRA_Multi5GS_PTC v_EUTRA_Multi5GS := null; var UTRAN_NR5GC_PTC v_UTRAN_NR5GC := null; var GERAN_NR5GC_PTC v_GERAN_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; var WLAN_PTC v_WLAN := null; timer t_GuardTimer := int2float(600); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_EUTRA_Multi5GS := EUTRA_Multi5GS_PTC.create("EUTRA_Multi5GS") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC_IRAT(system, v_NR5GC, v_EUTRA_Multi5GS, v_UTRAN_NR5GC, v_GERAN_NR5GC, v_IMS1, v_IMS2, v_WLAN); v_NR5GC.start(f_TC_11_5_4_NR5GC()); v_EUTRA_Multi5GS.start(f_TC_11_5_4_NR5GC_EUTRA()); v_IMS1.start(f_TC_11_5_4_NR5GC_IMS1()); v_IMS2.start(f_TC_11_5_4_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC_IRAT(t_GuardTimer); } }

## Correction to function TC_11_5_4_NR5GC

| Function name | f_TC_11_5_4_NR5GC |
| --- | --- |
| Reason for change | Postamble has been moved and executed in EUTRA PTC, see Change 2.2. Also, resources have already been released in NR TestBody |
| Summary of change | Removed postamble function call on NR PTC |
| TTCN module | ECall_IRAT_NR5GC_NR.ttcn |
| MCC160 Comments | Accepted |

Before Change:

function f_TC_11_5_4_NR5GC() runs on NR5GC_PTC {/* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS */ f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_SetSIB1_eCallOverIMS_Support(nr_Cell1, true_); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. f_UT_USIM_Insert(UT, "USIM configuration 24 specified in table 6.4.1-24 TS 38.508-1"); // Preamble: The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_11_5_4_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_DEREGISTERED); }

After Change:

function f_TC_11_5_4_NR5GC() runs on NR5GC_PTC {/* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS */ f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_SetSIB1_eCallOverIMS_Support(nr_Cell1, true_); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. f_UT_USIM_Insert(UT, "USIM configuration 24 specified in table 6.4.1-24 TS 38.508-1"); // Preamble: The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_11_5_4_NR5GC_TestBody(); f_NR_TestBody_Set(false); // REMOVED f_NR_Postamble(nr_Cell1, STATE_DEREGISTERED); }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” and LTE Band “FDD3” using NIA1/EIA1 Integrity and NEA1/EEA1 ciphering algorithms in Anritsu Protocol Conformance Test System ME7834NR.

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” and LTE Band “FDD3” using NIA2 Integrity and NEA2 ciphering algorithms in R&S® 5G Protocol Conformance Test platform.

# Execution Log Files

## Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

Snapdragon Auto 5G Modem-RF Gen 2 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/ TC_11_5_4_LOG.xml
RnS/ tc_11_5_4_QC.log

- PICS/PIXIT parameter file:

Anritsu/ TC_11_5_4_PIXIT.txt
RnS/ tc_11_5_4_QC.log

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250226 : Supporting information for addition of NR5GC eCall testcase 11.5.4 in FR1 |
| --- | --- |