3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250232

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3698 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 NR NTN test case 8.1.5.14.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 8.1.5.14.1 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.5.14.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.14.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	9

2.4	Change 4	9

2.5	Change 5	10

2.6	Change 6	10

3	Branches executed	11

4	Execution Log Files	11

4.1	Samsung Exynos 2500	11

5	References	11

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.5.14.1 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Narendra Kalahasti

Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	8.1.5.14.1.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Exynos 2500

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | f_TC_8_1_5_14_1_NR5GC() |
| --- | --- |
| Reason for change | - Please refer to Prose CR 252348 and CR R5-252347 for sys info combination change. - With current prose 38.508 SR_Transmax is n16 and with SR periodicity 640,and after expiry of SR_TransMax UE will trigger RACH procedure at step26 due to loopback UL PDU which it received at step25 in 10.240 second while ntn-UlSyncValidityDuration-r17 timer =s60 second still running. Here SR periodicity is already set to Max value 640. So need to modify SR_Transmax to 64, and with SR periodicity 640 gives total time 40.960seconds to evaluate UL Sync status at UE side . - With value UlSyncValidityDuration-r17 =s60 UE will keep UL sync longer and UE will not able to achieve UL sync lost state with maximum SR transMax 64, So by reducing UlSyncValidityDuration-r17 to 20, UE can be tested for lost UL Sync state after 20seconds. - As per definition of epochTime and NTN-Config Field description on prose 38.331, epochTime : If this field is absent for the NTN serving cell, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when ntn-Config is provided in dedicated configuration. In this case sr-TransMax is being considered as dedicated configuration. So epochTime need to set 0 as this is mandatory to set. |
| Summary of change | - NR-28 sys info combination is replaced with NR-29 sys info combination - Set sr-TransMax = n64. - Updated UlSyncValidityDuration-r17 to s20. - Added epochTime sfn-r17 and subframeNR-r17 to 0 on Table 8.1.5.14.1.3.3-1 |
| TTCN module | RRC_Others_NTN_NR5GC.ttcn |
| MCC160 Comment | - Change 1 has been implemented in R5-252348. - Change 2-4: Accepted conditional to prose CR agreement at RAN5#108. |

Before Change:

function f_TC_8_1_5_14_1_NR5GC() runs on NR5GC_PTC { //NTN /SIB19/T430 with the timer value set to ntn-UlSyncValidityDuration / T430 Expiry f_NR5GC_Init(NR_28); //Set SIB19 acc to Table 8.1.5.14.1.3.3-1 //Set ntn-UlSyncValidityDuration-r17 f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s60); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); //The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C. //The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); f_TC_8_1_5_14_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_8_1_5_14_1_NR5GC() runs on NR5GC_PTC { //NTN /SIB19/T430 with the timer value set to ntn-UlSyncValidityDuration / T430 Expiry var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; f_NR5GC_Init(NR_29); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Set SIB19 acc to Table 8.1.5.14.1.3.3-1 //Set ntn-UlSyncValidityDuration-r17 f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s20); f_NR_CellInfo_SetSIB19_NTN_ephochTime_setTo0(nr_Cell1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); //The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C. //The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); f_TC_8_1_5_14_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

  - Change 2

| Function name | f_TC_8_1_5_14_1_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - From Change1.3, T430 value set to s20. - TTCN stops SIB19 with an activation matching with SFN=512 or SFN=0. Upon UE discovering SIB19 is stopped, UE starts T430 timer as if T430 has started from previous epochTime. This means that SS has to start the timer as if T430 is started from epochTime. For example, if TTCN sends a SIB19 STOP command to SS at SFN 232 with schedule of SIB19 STOP at SFN 512, then, TTCN will start timer value T430ss(T1) = T430 – (232*10/1000). UE will only realised that SIB19 stopped at SFN=512,and starts T430 timer with value(T430ue) = T430 – (512*10/1000). T430ss and T430ue will expire at the same instant even though these are started at different timing instances. [clause 5.2.2.6 on prose 38.331] NOTE: The exact time when UL synchronisation is obtained (after SIB19 is acquired) is left to UE implementation, which can be from the subframe indicated by epochTime and optionally before the subframe indicated by epochTime. - As this case has special requirement to just Stop other Sis (SIB19) to achieve UE gets in state of UL sync lost without updating SIB1 shceduling or SIB1. For that TTCN has to send a flag to Framework that SIs should not be transmitted at step22 as per proposed prose. - At step22 and step28 System Info modification has to be performed when UE is in RRC connected mode. Also as there is no change on SIB1 or SIB1 Scheduling, there will be no short message has to be transmitted from SS. - AT step26, There is no need of additional f_delay function as t_T430.timeout will perform same operation, its like redundant function here. - As per spec 38.321 clause5.2a NOTE: The MAC entity suspends all UL operations (e.g. stop RACH, SR, and UL HARQ operation) after receiving the indication of an uplink synchronization loss and resumes the operation when receiving an indication of uplink synchronization. So to ensure UE is in lost UL sync state conformant UE must not send SR to SS after T430 timer expiry for next 2 seconds. - At step 28, need to resume SIB19 and as per Prose CR 252348 and CR R5-252347, NR-28 sys info combination is replaced with NR-29 sys info combination. - Modified UlSyncValidityDuration as per Change 1.3. - After Step29, on postamble there is no need of continued periodic UL grants for the UE. |
| Summary of change | - Updated t_T430 =20. - Modifed step23 with epoch time calculations. Step 22 and Step 23 are switched to be accurate on timer starting instance since TTCN can take significant time to generate commands for SIB19 STOP command. - Updated function f_NR_SetSysinfoCombinationToNR_1 with and argument which can be used as flags to stop SI transmission. - Updated arguments of function f_NR_ModifySysinfo for RRC_connected state and mark false on argument for short Message flag indications. - Removed f_Delay from step26. - Added t_Watchdog (for 2 seconds) and alt statement to achieve step26A (as per proposed prose) functionality for no PUCCH SR to pass this step. - Updated function f_NR_ModifySysinfoCombinationToNR_29 instead f_NR_SetSysinfoCombinationToNR_28 - Modified argument to s20 on f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration at step28. - Added function f_NR_ULGrantConfiguration_Start for the SR based UL grants. |
| TTCN module | RRC_Others_NTN_NR5GC.ttcn |
| MCC160 Comment | - Change 7 has been implemented in R5-252348. - Other changes: Accepted conditional to prose CR agreement at RAN5#108. |

Before Change:

function f_TC_8_1_5_14_1_NR5GC_TestBody() runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var integer v_DRBId; timer t_T430 := 60.0; //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Steps 2-19" siclog@ //Steps 2 to 19a1 of the registration procedure described in TS 38.508-1 [4] table 4.5.2.2-2 with 'connected without release' and Test Loop Function (On) with UE test loop mode B, //Step 2-15 f_NR5GC_Registration(nr_Cell1,TEST_LOOPModeB_ON); //Step 19a1 if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment(nr_Cell1,-,tsc_NR_RbId_SRB1); } //@siclog "Steps 20-21" siclog@ //The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); //@siclog "Step 22" siclog@ //SS starts a timer T1 with value = T430 t_T430.start; //@siclog "Step 23" siclog@ //SS stops broadcasting SIB19 f_NR_SetSysinfoCombinationToNR_1(nr_Cell1); f_NR_ModifySysinfo(nr_Cell1); //@siclog "Step 24" siclog@ //SS stops UL periodic grant allocation f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@siclog "Step 25" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the first PDU session on NR Cell 1. v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); //@siclog "Step 26" siclog@ //Wait until expiry of T1. f_Delay(60.0); t_T430.timeout; //@siclog "Step 27" siclog@ //The SS starts the UL periodic grant transmission allowing the UE to return the IP packet as received at step 25. f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_Now, f_NR_GetDefault_ResourceAllocation(f_NR_CellInfo_GetChannelBandwidth(nr_Cell1)), 5); //@siclog "Step 28" siclog@ //SS resumes broadcasting SIB19. f_NR_SetSysinfoCombinationToNR_28(nr_Cell1); f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s60); f_NR_ModifySysinfo(nr_Cell1, -, -, true); //@siclog "Step 29" siclog@ //Check: Does the UE loop back the IP packet received at step 25 in NR Cell 1? DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 29: UE loops back the IP packet received at step 25 "); }

After Change:

function f_TC_8_1_5_14_1_NR5GC_TestBody() runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var integer v_DRBId; timer t_T430 := 20.0; timer t_Watchdog := 2.0; var SubFrameTiming_Type v_Timing; var float f1; //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Steps 2-19" siclog@ //Steps 2 to 19a1 of the registration procedure described in TS 38.508-1 [4] table 4.5.2.2-2 with 'connected without release' and Test Loop Function (On) with UE test loop mode B, //Step 2-15 f_NR5GC_Registration(nr_Cell1,TEST_LOOPModeB_ON); //Step 19a1 if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment(nr_Cell1,-,tsc_NR_RbId_SRB1); } //@siclog "Steps 20-21" siclog@ //The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); //@siclog "Step 22" siclog@ //SS starts a timer T1 with value = T430 //t_T430.start; //@siclog "Step 23" siclog@ //SS stops broadcasting SIB19 f_NR_SetSysinfoCombinationToNR_1(nr_Cell1, true); //SIs empty second argument true f_NR_ModifySysinfo(nr_Cell1,false, RRC_CONNECTED); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22: SIB19 disable "); v_Timing := f_NR_GetCurrentTiming(nr_Cell1); f1 := 20.0 - (int2float(v_Timing.SFN.Number * 10)) / 1000.0; //considering epoch time from SFN=0 and SIB modifiction t_T430.start(f1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23: timer T430 start "); //@siclog "Step 24" siclog@ //SS stops UL periodic grant allocation f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@siclog "Step 25" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the first PDU session on NR Cell 1. v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); //@siclog "Step 26" siclog@ //Wait until expiry of T1. t_T430.timeout; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 26: Timer T430 expired "); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); t_Watchdog.start (2.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE still sending SR while UE suppos to be in UL Sync Lost state "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "UE stops SR on UL Sync Lost "); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //@siclog "Step 27" siclog@ //The SS starts the UL periodic grant transmission allowing the UE to return the IP packet as received at step 25. f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_Now, f_NR_GetDefault_ResourceAllocation(f_NR_CellInfo_GetChannelBandwidth(nr_Cell1)), 5); //@siclog "Step 28" siclog@ //SS resumes broadcasting SIB19. f_NR_ModifySysinfoCombinationToNR_29(nr_Cell1); f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s20); f_NR_ModifySysinfo(nr_Cell1, false, RRC_CONNECTED); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28: Modify Sysinfo for SIB19 "); //@siclog "Step 29" siclog@ //Check: Does the UE loop back the IP packet received at step 25 in NR Cell 1? DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 29: UE loops back the IP packet received at step 25 "); f_NR_ULGrantConfiguration_Start(nr_Cell1,-, -, -, cs_NR_UplinkTimeAlignment_Stop); }

  - Change 3

| Function name | f_NR_SetSysinfoCombinationToNR_1 () |
| --- | --- |
| Reason for change | - As per change 2.3, to stop SIs transmission, called a new function f_NR_CellInfo_SetSIs_stop inside the funciton f_NR_SetSysinfoCombinationToNR_1 |
| Summary of change | - Added f_NR_CellInfo_SetSIs_stop with condition SIs_Stop |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#108. |

Before Change:

function f_NR_SetSysinfoCombinationToNR_1(NR_CellId_Type p_NR_CellId ) runs on NR_BASE_PTC { f_NR_CellInfo_SetSIs (p_NR_CellId, omit); f_NR_CellInfo_SetSIB1_SiSchedulingInfo(p_NR_CellId, omit); //no SI in NR_1 @sic R5s201011 sic@ f_NR_CellInfo_SetSysinfoCombination(p_NR_CellId, NR_1); }

After Change:

function f_NR_SetSysinfoCombinationToNR_1(NR_CellId_Type p_NR_CellId, boolean SIs_stop := false ) runs on NR_BASE_PTC { if (SIs_stop){ f_NR_CellInfo_SetSIs_stop (p_NR_CellId); } else{ f_NR_CellInfo_SetSIs (p_NR_CellId, omit); } f_NR_CellInfo_SetSIB1_SiSchedulingInfo(p_NR_CellId, omit); //no SI in NR_1 @sic R5s201011 sic@ f_NR_CellInfo_SetSysinfoCombination(p_NR_CellId, NR_1); }

  - Change 4

| Function name | f_NR_CellInfo_SetSIs_stop () |
| --- | --- |
| Reason for change | Added new function f_NR_CellInfo_SetSIs_stop definition with SIs empty template for change 2.3 |
| Summary of change | Added new function f_NR_CellInfo_SetSIs_stop definition |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#108. |

Added New Function :

function f_NR_CellInfo_SetSIs_stop (NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); v_NR_CellInfo.Sysinfo.BcchInfo.SIs := { } ; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

  - Change 5

| Function name | f_NR_CellInfo_SetSIB19_NTN_ephochTime_setTo0 () |
| --- | --- |
| Reason for change | For change 1.4, function f_NR_CellInfo_SetSIB19_NTN_ephochTime_setTo0 is being called to set epochtime values |
| Summary of change | See blow changes for details. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#108. |

Added New Function :

function f_NR_CellInfo_SetSIB19_NTN_ephochTime_setTo0(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var integer v_SI_Index := f_NR_GetSIB19_SI_Index(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination); v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib19_v1700.ntn_Config_r17.epochTime_r17 := cs_epochTime_r17_set0 ; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

  - Change 6

| Function name | cs_epochTime_r17_set0() |
| --- | --- |
| Reason for change | As per change1.4, epochTime_r17 template is required to have sfn_r17 and subFrameNR_r17 values. |
| Summary of change | Set epochTime_r17 Template to value 0 |
| TTCN module | NR_SysInfo_Templates.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#108. |

Added New Template :

template (value) EpochTime_r17 cs_epochTime_r17_set0 := {/* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ //Acc to Table 4.6.3-56C: EphemerisInfo sfn_r17 := 0, subFrameNR_r17 := 0 };

- Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n255” using NIA2 Integrity and NEA0 ciphering algorithms in GSO on Anritsu 5G Conformance Test platform.

- Execution Log Files

  - Samsung Exynos 2500

The Samsung Exynos 2500 UE  passed this test case on Anritsu 5G Protocol Conformance Test platform ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_5_14_1_LOG.html

- PICS/PIXIT parameter file:

TC_8_1_5_14_1_PIXIT.xml

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s250233: Supporting information for agreement of TC 8.1.5.14.1 |
| --- | --- |