3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250234

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3699 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 NR NTN test case 9.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-06-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 9.4.1.2 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.4.1.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 9.4.1.2	4

2.1	Change 1	5

3	Branches executed	7

4	Execution Log Files	8

4.1	Exynos 2500	8

5	References	9

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.5.14.1 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk11.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Narendra Kalahasti

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	9.4.1.2.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Exynos 2500

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_9_4_1_2_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - On Preamble step and step1, NGC CellD was not turned on, so there is no need to change power for cellD, its already turned off. - At step1, NGC CellA is Turned off already, So there is no need to turn it off again at step#3. - At step5, Prose is asking to use TS 38.508-1 [4] subclause 4.9.4, but current TTCN is calling different function f_NR_RRC_ConnEst_DefWithNas which will check again about the RRCrequest on RRC idel mode without sending paging from SS And No RRCrequest already checked at step4 mean UE not suppose to send RRCrequest by itself here a tstep5. |
| Summary of change | - Commented f_NR_SetCellPower for NGC cell D on step1. - Commented f_NR_SetCellPower for NGC cell A on step3. - Changed to correct function f_NR5GC_CheckRRCIdle which prose is asking for on TS 38.508-1 [4] subclause 4.9.4 |
| TTCN module | MobilityManagement_NTN_NR5GC.ttcn |
| MCCTF160 Comment | - Not needed, as this is as per Prose and should not impact testcase/UEbehaviour - Not needed, as this is as per Prose and should not impact testcase/UE behaviour - Accepted. |

Before Change:

function fl_TC_9_4_1_2_NR5GC_TestBody() runs on NR5GC_PTC { //var NG_MobileIdentity v_NG_MobileIdentity; //var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellD := f_NR5GC_MapNASCell(ngc_CellD, NG_AllCellsOnSamePLMN); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var TrackingAreaCode v_TAC_B := f_NR_CellInfo_GetTAC (v_NGC_NASCellB); var TrackingAreaCode v_TAC_D := f_NR_CellInfo_GetTAC (v_NGC_NASCellD); var template (value) NG_TrackingAreaIdList v_TAIList_CellB := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC_B), bit2oct(v_TAC_D)}); // main behaviour // @siclog "Step 1" siclog@ // The SS configures- NGC Cell B as "Serving Cell"- NGC Cell A and Cell D as Non-suitable "Off" cell. f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 2" siclog@ // Check: Does the UE perform on NGC Cell B the Registration procedure for mobility registration update by executing the Test procedure to check that UE is camped on a new cell belonging to a new TA as specified in TS 38.508-1 [4] subclause 4.9.5? // NOTE: During the procedure the SS assigns a TAI list containing the TAI of NGC Cell B and NGC Cell D. f_NR5GC_MobilityRegistration(v_NGC_NASCellB, -, f_LastVisited_TAI(v_NGC_NASCellA), -, v_TAIList_CellB); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2: UE perform mobility registration update and camp on NGC Cell B."); // @siclog "Step 3" siclog@ // The SS configures- NGC Cell D as "Serving Cell"- NGC Cell A and Cell B as Non-suitable "Off" cell. f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 4" siclog@ // Check: Does the UE send in the next 30 sec a request for RRC connection establishment? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellD, 30.0); // @siclog "Step 5" siclog@ // Check: Does the result of generic test procedure in TS 38.508-1 [4] subclause 4.9.4 indicate that the UE is in 5GC RRC_IDLE state on the NGC Cell D? f_NR5GC_CheckCampedOnNewCell(v_NGC_NASCellD); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5: UE is in 5GC RRC_IDLE state on the NGC Cell D"); } // end of fl_TC_9_4_1_2_NR5GC_TestBody

After Change:

function fl_TC_9_4_1_2_NR5GC_TestBody() runs on NR5GC_PTC { //var NG_MobileIdentity v_NG_MobileIdentity; //var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellB := f_NR5GC_MapNASCell(ngc_CellB, NG_AllCellsOnSamePLMN); var NR_CellId_Type v_NGC_NASCellD := f_NR5GC_MapNASCell(ngc_CellD, NG_AllCellsOnSamePLMN); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var TrackingAreaCode v_TAC_B := f_NR_CellInfo_GetTAC (v_NGC_NASCellB); var TrackingAreaCode v_TAC_D := f_NR_CellInfo_GetTAC (v_NGC_NASCellD); var template (value) NG_TrackingAreaIdList v_TAIList_CellB := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC_B), bit2oct(v_TAC_D)}); // main behaviour // @siclog "Step 1" siclog@ // The SS configures- NGC Cell B as "Serving Cell"- NGC Cell A and Cell D as Non-suitable "Off" cell. f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 2" siclog@ // Check: Does the UE perform on NGC Cell B the Registration procedure for mobility registration update by executing the Test procedure to check that UE is camped on a new cell belonging to a new TA as specified in TS 38.508-1 [4] subclause 4.9.5? // NOTE: During the procedure the SS assigns a TAI list containing the TAI of NGC Cell B and NGC Cell D. f_NR5GC_MobilityRegistration(v_NGC_NASCellB, -, f_LastVisited_TAI(v_NGC_NASCellA), -, v_TAIList_CellB); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2: UE perform mobility registration update and camp on NGC Cell B."); // @siclog "Step 3" siclog@ // The SS configures- NGC Cell D as "Serving Cell"- NGC Cell A and Cell B as Non-suitable "Off" cell. f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 4" siclog@ // Check: Does the UE send in the next 30 sec a request for RRC connection establishment? f_NR_CheckNoRRCSetupReq(v_NGC_NASCellD, 30.0); // @siclog "Step 5" siclog@ // Check: Does the result of generic test procedure in TS 38.508-1 [4] subclause 4.9.4 indicate that the UE is in 5GC RRC_IDLE state on the NGC Cell D? f_NR5GC_CheckRRCIdle(v_NGC_NASCellD); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5: UE is in 5GC RRC_IDLE state on the NGC Cell D"); } // end of fl_TC_9_4_1_2_NR5GC_TestBody

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n255” using NIA2 Integrity and NEA0 ciphering algorithms in GSO on Anritsu 5G Conformance Test platform.

# Execution Log Files

## Samsung Exynos 2500

The Samsung Exynos 2500 UE  passed this test case on Anritsu 5G Protocol Conformance Test platform ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_9_4_1_2_LOG.html

- PICS/PIXIT parameter file:

TC_9_4_1_2_PIXIT.xml

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250235: Supporting information for agreement of TC 9.4.1.2 |
| --- | --- |