3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250271

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3715 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR NTN testcase 6.7.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to 38.331 cl 6.3.2, the description for trackingAreaCode IE within PLMN-IdentityInfo states : trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN) or is an NTN cell. Hence it is proposed that for both nr_Cell1 and nr_Cell11, the IE trackingAreaCode should be set to OMIT. Please see below. According to the table 6.7.1.2.3.2-1 in 38.523-1, at T1, the power level of NR Cell 1 is “Off”. This needs to be corrected . |  |  |  |  |  |  |  |  |
| Summary of change: |  | For both of nr_Cell1 and nr_Cell11, change the trackingAreaCode to omit. Change the power level of NR Cell 1 to “Off”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	1

1	Overview	3

2	Corrections required to test case 6.7.1.2	4

2.1	Change 1	4

2.2	Change 2	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required to test case 6.7.1.2

## Change 1

| Function name | f_TC_6_7_1_2_NR5GC () |
| --- | --- |
| Reason for change | According to 38.331 cl 6.3.2, the description for trackingAreaCode IE within PLMN-IdentityInfo states : trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN) or is an NTN cell. Hence it is proposed that for both nr_Cell1 and nr_Cell11, the IE trackingAreaCode should be set to OMIT. |
| Summary of change | For both of nr_Cell1 and nr_Cell11, change the trackingAreaCode to omit. |
| TTCN module | NTN_Idle_NR5GC.ttcn |
| MCC160 Comment | Accepted, as an improvement |

Before Change:

function f_TC_6_7_1_2_NR5GC() runs on NR5GC_PTC {//NR NTN / Cell Selection / MultiTAC / trackingAreaList-r17 // System information combination NR-28 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells. f_NR5GC_Init(NR_29, -, {nr_Cell11}); //@sic R5-252351 sic@ //Configure trackingAreaList in SIB1 as per Table 9.4.1.2.3.3-1 and Table 9.4.1.2.3.3-2 f_NR_CellInfo_SetPLMNIdentityInfo_List (nr_Cell1, {cs_NR_PLMN_IdentityInfo( f_NR_CellInfo_GetPLMN (nr_Cell1), f_NR_CellInfo_GetTAC(nr_Cell1), 1, //RAN_AreaCode set to 1 as per TS 38.508 Table 4.6.3-135 f_NR_CellInfo_GetCellIdentity(nr_Cell1), {int2bit(1, 24), int2bit(2, 24)} )}); f_NR_CellInfo_SetPLMNIdentityInfo_List (nr_Cell11, {cs_NR_PLMN_IdentityInfo( f_NR_CellInfo_GetPLMN (nr_Cell11), f_NR_CellInfo_GetTAC(nr_Cell11), 1, //RAN_AreaCode set to 1 as per TS 38.508 Table 4.6.3-135 f_NR_CellInfo_GetCellIdentity(nr_Cell11), {int2bit(2, 24), int2bit(1, 24)} )}); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); // Preamble: The UE is in test state 1N-A on NR Cell 1 according to TS 38.508-1 [4]. f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_6_7_1_2_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); }

After Change:

function f_TC_6_7_1_2_NR5GC() runs on NR5GC_PTC {//NR NTN / Cell Selection / MultiTAC / trackingAreaList-r17 // System information combination NR-28 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR cells. f_NR5GC_Init(NR_29, -, {nr_Cell11}); //@sic R5-252351 sic@ //Configure trackingAreaList in SIB1 as per Table 9.4.1.2.3.3-1 and Table 9.4.1.2.3.3-2 f_NR_CellInfo_SetPLMNIdentityInfo_List (nr_Cell1, {cs_NR_PLMN_IdentityInfo( f_NR_CellInfo_GetPLMN (nr_Cell1), omit, //WI=1433271 1, //RAN_AreaCode set to 1 as per TS 38.508 Table 4.6.3-135 f_NR_CellInfo_GetCellIdentity(nr_Cell1), {int2bit(1, 24), int2bit(2, 24)} )}); f_NR_CellInfo_SetPLMNIdentityInfo_List (nr_Cell11, {cs_NR_PLMN_IdentityInfo( f_NR_CellInfo_GetPLMN (nr_Cell11), omit, //WI=1433271 1, //RAN_AreaCode set to 1 as per TS 38.508 Table 4.6.3-135 f_NR_CellInfo_GetCellIdentity(nr_Cell11), {int2bit(2, 24), int2bit(1, 24)} )}); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); // Preamble: The UE is in test state 1N-A on NR Cell 1 according to TS 38.508-1 [4]. f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_6_7_1_2_Body(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); }

## Change 2

| Function name | fl_TC_6_7_1_2_Body () |
| --- | --- |
| Reason for change | According to the table 6.7.1.2.3.2-1 in 38.523-1, at T1, the power level of NR Cell 1 is “Off”. This needs to be corrected. |
| Summary of change | Change the power level of NR Cell 1 to “Off”. |
| TTCN module | NTN_Idle_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_6_7_1_2_Body() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_NasInd; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2), //WI=1433271 cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // The SS re-adjusts the SS/PBCH EPRE levels and according to row "T1" in table 6.1.2.12.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 within 3 minutes? f_NR_CheckNoRRCSetupReq(nr_Cell11, 180.0, -, "Step 2: UE sent an RRCSetupRequest on NR Cell 11 within 3 minutes"); f_NR_ULGrantConfiguration_Start(nr_Cell11); //@sic R5s250105 sic@ // @siclog "Step 3" siclog@ // The SS transmits a Paging message on NR Cell 11. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); // @siclog "Step 4" siclog@ // Check: Does the UE transmit an RRCSetupRequest message on NR Cell 11? SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))); f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.2 Step 4"); // @siclog "Step 5-10" siclog@ // Steps 3 to 8 of the generic test procedure in TS 38.508-1 [4] subclause 4.9.4.2.2-1 are performed on NR Cell 11. f_NR_RRCSetup_Def (nr_Cell11); v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_NasInd); // @siclog "Step 11" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell11); //@sic R5s250105 sic@ }

After Change:

function fl_TC_6_7_1_2_Body() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_NasInd; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), //WI=1433271 cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // The SS re-adjusts the SS/PBCH EPRE levels and according to row "T1" in table 6.1.2.12.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 2" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 within 3 minutes? f_NR_CheckNoRRCSetupReq(nr_Cell11, 180.0, -, "Step 2: UE sent an RRCSetupRequest on NR Cell 11 within 3 minutes"); f_NR_ULGrantConfiguration_Start(nr_Cell11); //@sic R5s250105 sic@ // @siclog "Step 3" siclog@ // The SS transmits a Paging message on NR Cell 11. f_NR_Paging_Def(nr_Cell11, RRC_IDLE); // @siclog "Step 4" siclog@ // Check: Does the UE transmit an RRCSetupRequest message on NR Cell 11? SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))); f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.7.1.2 Step 4"); // @siclog "Step 5-10" siclog@ // Steps 3 to 8 of the generic test procedure in TS 38.508-1 [4] subclause 4.9.4.2.2-1 are performed on NR Cell 11. f_NR_RRCSetup_Def (nr_Cell11); v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell11, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell11, v_NasInd); // @siclog "Step 11" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell11); //@sic R5s250105 sic@ }