3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250273

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4916 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Due to the Koffset sending out the DL messages takes around 550msec, if RLC Ack and Downlink NAS Transport (CP-ACK) message at Step 16a4C are sent separately then this leads to RRC Connection release not being sent at the given activation time. In order to avoid this it is proposed to add a delay after Step 16a4C specifically for NTN test execution. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added f_Delay(2.0*f_NBIOT_NTN_Get_T_NTNoffset_InSeconds(nbiot_Cell50)) After Step 16a4C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for test case 22.1.1	2

2.1	f_TC_22_1_1_Module3	2

# Overview

This document lists all the essential changes needed to correct problems in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for test case 22.1.1

## f_TC_22_1_1_Module3

| Function name | f_TC_22_1_1_Module3 |
| --- | --- |
| Reason for change | Due to the Koffset sending out the DL messages takes around 550msec, if RLC Ack and Downlink NAS Transport (CP-ACK) message at Step 16a4C are sent separately then this leads to RRC Connection release not being sent at the given activation time. In order to avoid this it is proposed to add a delay after Step 16a4C specifically for NTN test execution. |
| Summary of change | Added f_Delay(2.0*f_NBIOT_NTN_Get_T_NTNoffset_InSeconds(nbiot_Cell50)) After Step 16a4C. |
| TTCN module | NBIOT_General_CP |
| MCC160 Comment | Accepted conditional prose CR approval |

Before Change:

function f_TC_22_1_1_Module3() runs on NBIOT_PTC { … … … //@siclog "Step 16a4C" siclog@ //The SS transmits a CP-ACK encapsulated in a Downlink NAS transport message to the UE. v_NAS_TxMessageContainer_SMS := cs_NAS_MsgContainer_SMS (f_SMS_PDU_Encvalue(cs_CP_ACK_PDU (cs_MT_TI0_FromSS))); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell50, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request (tsc_SHT_IntegrityProtected_Ciphered, cs_SMS_DL_NAS_Transport (tsc_SHT_NoSecurityProtection, v_NAS_TxMessageContainer_SMS)))); //@siclog "Step 16a5" siclog@ //The SS transmits an RRCConnectionRelease-NB message. f_NBIOT_RRC_ConnectionRelease (nbiot_Cell50);

After Change:

function f_TC_22_1_1_Module3() runs on NBIOT_PTC { … … … //@siclog "Step 16a4C" siclog@ //The SS transmits a CP-ACK encapsulated in a Downlink NAS transport message to the UE. v_NAS_TxMessageContainer_SMS := cs_NAS_MsgContainer_SMS (f_SMS_PDU_Encvalue(cs_CP_ACK_PDU (cs_MT_TI0_FromSS))); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell50, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request (tsc_SHT_IntegrityProtected_Ciphered, cs_SMS_DL_NAS_Transport (tsc_SHT_NoSecurityProtection, v_NAS_TxMessageContainer_SMS)))); //For NTN access, SS waits for timer 2* TNTN,offset f_Delay(2.0*f_NBIOT_NTN_Get_T_NTNoffset_InSeconds(nbiot_Cell50)); //for non-NTN access the delay will result in 0 /*@sic R5s240230, R5-245649, R5s240581 sic@ */ //@siclog "Step 16a5" siclog@ //The SS transmits an RRCConnectionRelease-NB message. f_NBIOT_RRC_ConnectionRelease (nbiot_Cell50);