3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250282

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0980 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC eCall over IMS testcase 11.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, Anritsu EMEA Ltd, Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add IMS_NR5GC test case 11.10 to the IWD_25wk24 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.10.NR5GC required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.10.NR5GC which is part of the IMS_NR5GC test suite in iwd-TTCN3-B2024-06_D25wk24.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

Lokesha Rajanna

lokesha.rajanna@anritsu.com

Mohit Kanchan

mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	11.10.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	R&S® 5G Protocol Conformance Test platform, Anritsu Protocol Conformance Test System ME7834NR, Keysight 5G Protocol Conformance Toolset

UE used:	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

Verification Status:	PASS

# Corrections required

## Correction in f_IMS_ECall_AbnormalCase_IMS2

| Template name | f_IMS_ECall_AbnormalCase_IMS2 |
| --- | --- |
| Reason for change | In the current TTCN implementation, test procedure sequence for eCall over IMS Emergency establishment in 5GS (eCall Only Support),is not implemented properly. Delay (of 15 second) needed to allow the UE to camp on the serving cell is not present. Due to this Ecall gets trigerred when UE is effectively in switch-off state. Please note that, as per preamble, UE was in the test state 0-A (Switched Off). |
| Summary of change | Update the TTCN to handle the above mentioned scenario. Please see below. |
| TTCN module | IMS_CC_TestcaseImplementation_eCall.ttcn |
| MCC160 Comment | This requires a prose change. Decision postponed until outcome of prose CR |

Before Change:

function f_IMS_ECall_AbnormalCase_IMS2(IMS_TestProcedure_Type p_TestProcedure, ECall_Type p_CallType, ECall_InviteRejectCause_Type p_RejectCause) runs on IMS_PTC { var SipUrl v_ContactUrl; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); f_IMS_IPCAN_StartProcedure(IPCAN, p_TestProcedure); //IPCAN_EmergencyCall_Aborted //IPCAN_ECallOnly_Aborted // @siclog "Step 1" siclog@ f_UT_InitiateECall(MMI, p_CallType); <<SKIPPED CODE>>

After Change:

function f_IMS_ECall_AbnormalCase_IMS2(IMS_TestProcedure_Type p_TestProcedure, ECall_Type p_CallType, ECall_InviteRejectCause_Type p_RejectCause) runs on IMS_PTC { var SipUrl v_ContactUrl; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); f_IMS_IPCAN_StartProcedure(IPCAN, p_TestProcedure); //IPCAN_EmergencyCall_Aborted //IPCAN_ECallOnly_Aborted if (p_TestProcedure == IPCAN_ECallOnly_Aborted) { f_Delay(15.0); // 15 seconds according to step 1 of 38.508 Table 4.9.29.2.2-1 } // @siclog "Step 1" siclog@ f_UT_InitiateECall(MMI, p_CallType); <<SKIPPED CODE>>

## New function f_NR5GC_EmergencyPDURelease

| Template name | f_NR5GC_EmergencyPDURelease |
| --- | --- |
| Reason for change | In the current TTCN implementation, for case IPCAN_ECallOnly_Aborted, release of PDU session was missing while emergency call was being released. Due to this unexpected UE behavior was observed. |
| Summary of change | Splitted function to be used in IPCAN to release emergency PDU without callimg f_IP_RtpRtcpLoopback. Call has not been established hence no RtpRtcp Loopback is started. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | This must be added to the prose. Decision postponed until outcome of prose CR |

New function:

function f_NR5GC_EmergencyPDURelease(NR_CellId_Type p_NR_CellId, boolean p_EmergencyCallEstablished := true) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var PDN_Index_Type v_PdnIndex := v_PDUSession.PdnIndex; // @sic R5s220829 sic@ f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); if(p_EmergencyCallEstablished){ f_IP_RtpRtcpLoopback(IP, v_PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363, R5s220829 sic@ } }

## Correction in f_NR5GC_EmergencyCallReleaseMT

| Template name | f_NR5GC_EmergencyCallReleaseMT |
| --- | --- |
| Reason for change | In the current TTCN implementation, for case IPCAN_ECallOnly_Aborted, release of PDU session was missing while emergency call was being released. Due to this unexpected UE behavior was observed. |
| Summary of change | Called new function f_NR5GC_EmergencyPDURelease, behaviour inside f_NR5GC_EmergencyCallReleaseMT stills the same. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | See change 2. Decision postponed until outcome of prose CR |

Before Change:

function f_NR5GC_EmergencyCallReleaseMT(NR_CellId_Type p_NR_CellId, boolean p_IMSMsgsNotYetSent:= true, EmergencyCallRelease_PDUSession p_Release_Keep := Release_PDUSession) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var PDN_Index_Type v_PdnIndex := v_PDUSession.PdnIndex; // @sic R5s220829 sic@ if ( p_IMSMsgsNotYetSent) { // @sic R5s220789, R5s221103 sic@ IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5s220039, R5-225362 sic@ sic@ } if (p_Release_Keep == Release_PDUSession) { // @sic R5-225362 sic@ f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363, R5s220829 sic@ } else { f_NR5GC_ModifyPDUSession_RemoveEmergency(p_NR_CellId); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }

After Change:

function f_NR5GC_EmergencyCallReleaseMT(NR_CellId_Type p_NR_CellId, boolean p_IMSMsgsNotYetSent:= true, EmergencyCallRelease_PDUSession p_Release_Keep := Release_PDUSession) runs on NR5GC_PTC { if ( p_IMSMsgsNotYetSent) { // @sic R5s220789, R5s221103 sic@ IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5s220039, R5-225362 sic@ sic@ } if (p_Release_Keep == Release_PDUSession) { f_NR5GC_EmergencyPDURelease(p_NR_CellId); } else { f_NR5GC_ModifyPDUSession_RemoveEmergency(p_NR_CellId); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }

## Correction in f_NR5GC_IPCAN_ProcedureEnd

| Template name | f_NR5GC_IPCAN_ProcedureEnd |
| --- | --- |
| Reason for change | In the current TTCN implementation, for case IPCAN_ECallOnly_Aborted, release of PDU session was missing while emergency call was being released. Due to this unexpected UE behavior was observed. |
| Summary of change | Update the TTCN to handle the above mentioned scenario. Please see below. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | See change 2. Decision postponed until outcome of prose CR |

Before Change:

function f_NR5GC_IPCAN_ProcedureEnd(IMS_IPCAN_CO_ORD_PORT p_Port, //FFS parameter left for future use NR_CellId_Type p_NR_CellId, IMS_TestProcedure_Type p_ProcedureType, IMS_TestConfiguration_Type p_TestConfiguration, TriggerResult_Type p_CallEstablishmentResult) runs on NR5GC_PTC { select (p_ProcedureType) { case (IPCAN_InitialRegistration) { // just release RRC connection } <<SKIPPED CODE>> case (IPCAN_CheckRRCIdle){ // @sic R5-253128 sic@ f_NR5GC_CheckRRCIdle(p_NR_CellId, noRrcConnectionRelease); // RRC is released in f_NR5GC_IPCAN_ConnectionRelease } } }

After Change:

function f_NR5GC_IPCAN_ProcedureEnd(IMS_IPCAN_CO_ORD_PORT p_Port, //FFS parameter left for future use NR_CellId_Type p_NR_CellId, IMS_TestProcedure_Type p_ProcedureType, IMS_TestConfiguration_Type p_TestConfiguration, TriggerResult_Type p_CallEstablishmentResult) runs on NR5GC_PTC { select (p_ProcedureType) { case (IPCAN_InitialRegistration) { // just release RRC connection } <<SKIPPED CODE>> case (IPCAN_CheckRRCIdle){ // @sic R5-253128 sic@ f_NR5GC_CheckRRCIdle(p_NR_CellId, noRrcConnectionRelease); // RRC is released in f_NR5GC_IPCAN_ConnectionRelease f_NR5GC_EmergencyPDURelease(p_NR_CellId, false); } } }

## 2.5 Correction in f_NR5GC_IPCAN_MainLoop

| Template name | f_NR5GC_IPCAN_MainLoop |
| --- | --- |
| Reason for change | In the current TTCN implementation, for case IPCAN_ECallOnly_Aborted, value of v_IPCAN_Registered was set as false. Due to this TTCN was not handling the rrcSetupRequest corresponding to deregistration procedure properly, causing TC failure. |
| Summary of change | Update the TTCN to handle the above mentioned scenario. Please see below. |
| TTCN module | IPCAN_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> v_Command := v_IMS_IPCAN_Coordination_MSG.IMS_IPCAN_Command; select (v_Command.Name) { case (IPCAN_INIT) { if (isvalue(v_Command.TestProcedure)) { // @sic R5s220611 sic@ if (v_Command.TestProcedure == IPCAN_UseVPLMN) { v_CellId := tsc_IMS_NR5GC_CellId_VPLMN; } } f_NR5GC_IPCAN_Init(IMS[v_PdnIndex], v_CellId); v_TestConfiguration := v_Command.TestConfiguration; } case (IPCAN_PROVIDELOCATIONINFORMATION) { f_NR5GC_IPCAN_LocationInformation(IMS[v_PdnIndex], v_CellId); } case (IPCAN_STARTPROCEDURE) { v_StartProcedureResult := f_NR5GC_IPCAN_StartProcedure(IMS[v_PdnIndex], v_CellId, v_Command.TestProcedure, v_Command.TestConfiguration, v_ConnectionEstablished); v_ConnectionEstablished := true; if ( v_Command.TestProcedure == IPCAN_InitialRegistration) { // @sic R5s211495 sic@ v_IPCAN_Registered := true; } case (IPCAN_ENDPROCEDURE) { if (v_TestConfiguration == IPCAN_SpeechAndEmergencyCall and isvalue(v_Command.TestConfiguration)) { // @sic R5-216220 sic@ // Just end the procedure as there's 2 calls ongoing, don't release the connection f_NR5GC_IPCAN_EndCall(IMS[v_PdnIndex], v_CellId, valueof(v_Command.TestConfiguration), v_Command.TestProcedure, v_StartProcedureResult); } else { f_NR5GC_IPCAN_ConnectionRelease(IMS[v_PdnIndex], v_CellId, v_TestConfiguration, v_Command.TestProcedure, v_StartProcedureResult); // @sic R5-225426 sic@ /* => RRC connection is released */ v_ConnectionEstablished := false; } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> v_Command := v_IMS_IPCAN_Coordination_MSG.IMS_IPCAN_Command; select (v_Command.Name) { case (IPCAN_INIT) { if (isvalue(v_Command.TestProcedure)) { // @sic R5s220611 sic@ if (v_Command.TestProcedure == IPCAN_UseVPLMN) { v_CellId := tsc_IMS_NR5GC_CellId_VPLMN; } } f_NR5GC_IPCAN_Init(IMS[v_PdnIndex], v_CellId); v_TestConfiguration := v_Command.TestConfiguration; } case (IPCAN_PROVIDELOCATIONINFORMATION) { f_NR5GC_IPCAN_LocationInformation(IMS[v_PdnIndex], v_CellId); } case (IPCAN_STARTPROCEDURE) { v_StartProcedureResult := f_NR5GC_IPCAN_StartProcedure(IMS[v_PdnIndex], v_CellId, v_Command.TestProcedure, v_Command.TestConfiguration, v_ConnectionEstablished); v_ConnectionEstablished := true; select(v_Command.TestProcedure){ case(IPCAN_InitialRegistration, IPCAN_ECallOnly_Aborted) {v_IPCAN_Registered := true;} case else {} } } case (IPCAN_ENDPROCEDURE) { if (v_TestConfiguration == IPCAN_SpeechAndEmergencyCall and isvalue(v_Command.TestConfiguration)) { // @sic R5-216220 sic@ // Just end the procedure as there's 2 calls ongoing, don't release the connection f_NR5GC_IPCAN_EndCall(IMS[v_PdnIndex], v_CellId, valueof(v_Command.TestConfiguration), v_Command.TestProcedure, v_StartProcedureResult); } else { f_NR5GC_IPCAN_ConnectionRelease(IMS[v_PdnIndex], v_CellId, v_TestConfiguration, v_Command.TestProcedure, v_StartProcedureResult); // @sic R5-225426 sic@ /* => RRC connection is released */ v_ConnectionEstablished := false; } } <<SKIPPED CODE>>

# Branches executed

## On Rohde & Schwarz, This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA2 Integrity and NEA2 ciphering algorithms

## On Anritsu, This TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA1 Integrity and NEA1 ciphering algorithms.

## On Keysight, This TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n78” using NIA1 Integrity and NEA1 ciphering algorithms.

# Execution Log Files

# Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

The Qualcomm Snapdragon Auto 5G Modem-RF Gen 2 UE passed this test case on R&S® 5G Protocol Conformance Test platform, Anritsu Protocol Conformance Test System ME7834NR and Keysight 5G Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Rohde & Schwarz\TC_11_10_LOG.txt

Anritsu\TC_11_10_LOG.html

Keysight\TC_11_10_LOG.html

- PICS/PIXIT parameter file:

Rohde & Schwarz\TC_11_10_LOG.txt

Anritsu\TC_11_10_PIXIT.txt

Keysight\TC_11_10_PIXIT.html

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250283: Supporting information for Addition of NR5GC eCall over IMS testcase 11.10 |
| --- | --- |