3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250289

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3720 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC eCall over IMS testcase 11.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - IP handling needs to be started in EUTRA, but Change 2.3.1 for addition of f_EUTRA_SwitchOnUEandStartIP(eutra_Cell1); got rejected in R5s250225. Instead of removing fully, it needs to be replaced by f_EUTRA_ResetIPandIMS (eutra_Cell1); - Incorrect cell being configured by f_NR_SS_RachProcedureConfig as NR Cell 2 is not used by the TC - Release of NR resources was approved in Change 2.4.3 of R5s250225, but was missing in D25wk24 TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Start IP Handling in EUTRA - Change nr_Cell2 to nr_Cell1 - Add cell release after EUTRA Postamble |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase incorrectly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Correction to the function fl_TC_11_5_4_NR5GC_EUTRA_TestBody	3

2.2	Correction to the function fl_TC_11_5_4_NR5GC_TestBody	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Correction to the function fl_TC_11_5_4_NR5GC_EUTRA_TestBody

| Function name | fl_TC_11_5_4_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | IP handling needs to be started in EUTRA, but Change 2.3.1 for addition of f_EUTRA_SwitchOnUEandStartIP(eutra_Cell1); got rejected in R5s250225. Instead of removing fully, it needs to be replaced by f_EUTRA_ResetIPandIMS (eutra_Cell1); |
| Summary of change | Start IP Handling in EUTRA |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_11_5_4_NR5GC_EUTRA_TestBody()runs on EUTRA_5GS_PTC { // @siclog "Step 6" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Suitable cell". f_EUTRA_NR_WaitForCoOrd_Trigger(NR,"Change Power Level"); f_EUTRA38_SetCellPower(eutra_Cell1, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE_FR2); //@sic R5-241629 sic@ //@siclog "Step 7" siclog@ //Check: Does the UE sends an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? //@siclog "Step 8-26" siclog@ //Anritsu TTCN CR R5s250225 MCC f_EUTRA_508_IMSeCallOnly_Steps3_25(eutra_Cell1); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 7"); //IMS call has been established, wait for trigger from IMS PTC to release rrc connection f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "IMS Call Release Msgs Sent"); // @siclog "Step 27" siclog@ //Now release the rrcConnection as the IMS PTC would be releasing the eCall f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, '7'H); // Emergency default bearer hardcoded to 6 and dedicated bearer 7 f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); } // End of fl_TC_11_5_4_NR5GC_EUTRA_TestBody()

After Change:

function fl_TC_11_5_4_NR5GC_EUTRA_TestBody()runs on EUTRA_5GS_PTC { // @siclog "Step 6" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Suitable cell". f_EUTRA_NR_WaitForCoOrd_Trigger(NR,"Change Power Level"); f_EUTRA38_SetCellPower(eutra_Cell1, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE_FR2); //@sic R5-241629 sic@ f_EUTRA_ResetIPandIMS (eutra_Cell1); // Anritsu TTCN CR R5s250225_MCC More Corrections //@siclog "Step 7" siclog@ //Check: Does the UE sends an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? //@siclog "Step 8-26" siclog@ //Anritsu TTCN CR R5s250225 MCC f_EUTRA_508_IMSeCallOnly_Steps3_25(eutra_Cell1); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 7"); //IMS call has been established, wait for trigger from IMS PTC to release rrc connection f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "IMS Call Release Msgs Sent"); // @siclog "Step 27" siclog@ //Now release the rrcConnection as the IMS PTC would be releasing the eCall f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, '7'H); // Emergency default bearer hardcoded to 6 and dedicated bearer 7 f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); } // End of fl_TC_11_5_4_NR5GC_EUTRA_TestBody()

## Correction to the function fl_TC_11_5_4_NR5GC_TestBody

| Function name | fl_TC_11_5_4_NR5GC_TestBody |
| --- | --- |
| Reason for change | - Incorrect cell being configured by f_NR_SS_RachProcedureConfig as NR Cell 2 is not used by the TC - Release of NR resources was approved in Change 2.4.3 of R5s250225, but was missing in D25wk24 TTCN. |
| Summary of change | - Change nr_Cell2 to nr_Cell1 - Add cell release after EUTRA Postamble |
| TTCN module | ECall_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment | 1. Accepted 2. Not needed. f_NR_ReleaseAllCells is called in the calling function: fl_TC_11_5_4_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_ReleaseAllCells(); // @sic R5s250225 sic@ |

Before Change:

function fl_TC_11_5_4_NR5GC_TestBody() runs on NR_BASE_PTC { timer t_Wait := f_NR_SetTimerToleranceMax (nr_Cell1, rrcTimer, 1.0); //Anritsu TTCN CR R5s250225 MCC //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 2" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state. f_Delay(60.0); // @siclog "Step 4" siclog@ //Anritsu TTCN CR R5s250225 MCC //An automatic eCall is initiated. (Note 1) f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); f_UT_InitiateECall(UT, AUTOMATIC); //@siclog "Step 5" siclog@ //The UE transmits preamble on PRACH. SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) // Change the power level in EUTRA side f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); //exception:The preamble and no response below are repeated for the duration of T300. t_Wait.start; alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { repeat; } []t_Wait.timeout{} }; // @siclog "Step 6" siclog@ // Now wait until EUTRA test is finished f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

After Change:

function fl_TC_11_5_4_NR5GC_TestBody() runs on NR_BASE_PTC { timer t_Wait := f_NR_SetTimerToleranceMax (nr_Cell1, rrcTimer, 1.0); //Anritsu TTCN CR R5s250225 MCC //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 2" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state. f_Delay(60.0); // @siclog "Step 4" siclog@ //Anritsu TTCN CR R5s250225 MCC //An automatic eCall is initiated. (Note 1) f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); // Anritsu TTCN CR R5s250225_MCC More Corrections f_UT_InitiateECall(UT, AUTOMATIC); //@siclog "Step 5" siclog@ //The UE transmits preamble on PRACH. SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) // Change the power level in EUTRA side f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Change Power Level")); //exception:The preamble and no response below are repeated for the duration of T300. t_Wait.start; alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { repeat; } []t_Wait.timeout{} }; // @siclog "Step 6" siclog@ // Now wait until EUTRA test is finished f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); f_NR_ReleaseAllCells(); // Anritsu TTCN CR R5s250225_MCC More Corrections }