3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250294

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4922 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.4.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the current Prose, in table 22.4.23.3.2-2, at Step 8C a timer of 30s is started so that the UE goes into Idle mode, and camps on Ncell 1. However, some UEs, which have multiple bands enabled take much longer than 30s to detect Ncell 1 and camp on it. In such a scenario the UE wont respond to paging in Step 9-13, and can fail the testcase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to increase the timer at Step 8C to 120s. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

- Corrections required

  - Change 1

| Template name | f_TC_22_4_23_NBIOT |
| --- | --- |
| Reason for change | As per the current Prose, in table 22.4.23.3.2-2, at Step 8C a timer of 30s is started so that the UE goes into Idle mode, and camps on Ncell 1. However, some UEs, which have multiple bands enabled take much longer than 30s to detect Ncell 1 and camp on it. In such a scenario the UE wont respond to paging in Step 9-13, and can fail the testcase. |
| Summary of change | It is proposed to increase the timer at Step 8C to 120s. A Prose CR will be raised for this. |
| TTCN module | NBIOT_RRC_ControlPlane.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 8A" siclog@ f_Delay (3.0); //@siclog "Step 8B" siclog@ //The SS changes Ncell1 according to row "T2" in Table 22.4.23.3.2-1. f_NBIOT_SetCellPower(nbiot_Cell1, -85); //@siclog "Step 8C" siclog@ f_Delay (30.0); // @sic R5-181304, R5s190173 sic@ //@siclog "Step 9-13" siclog@ //'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed f_NBIOT_508Check_CP_ResponseToPagingForMTAccess(nbiot_Cell1); // @sic R5-181304 sic@ f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9"); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 8A" siclog@ f_Delay (3.0); //@siclog "Step 8B" siclog@ //The SS changes Ncell1 according to row "T2" in Table 22.4.23.3.2-1. f_NBIOT_SetCellPower(nbiot_Cell1, -85); //@siclog "Step 8C" siclog@ f_Delay (120.0); // @sic R5-181304, R5s190173 sic@ //WA#WI1437165 WA#22_4_23 30s-->120s //@siclog "Step 9-13" siclog@ //'Generic Test Procedure NB-IoT Control Plane CIoT MT user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.2.3 are performed f_NBIOT_508Check_CP_ResponseToPagingForMTAccess(nbiot_Cell1); // @sic R5-181304 sic@ f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 9"); <<SKIPPED CODE>>