3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250296

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3722 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 9.1.14.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 9.1.14.1 to the IWD_25wk24 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 9.1.14.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.14.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes 2.1 and 2.2 from R5s250197 are needed in addition to the changes mentioned below. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3 Branches executed	10

4	Execution Log Files	10

4.1	MediaTek MT6986D UE	10

5	References	10

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

  - Verification Test Summary

Test Case: 	9.1.14.1.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6986D

Verification Status:	      PASS

- Corrections required

  - Change 1

| Function name | fl_TC_9_1_14_1_TestBody |
| --- | --- |
| Reason for change | - Step 18-23a1 of TS 38.523-1 indicates messages should be sent and received on NGC Cell A. Hence, the TAC and TAIList should based on NGC Cell A, not NGC Cell D. - At step 29A, function f_Delay(60.0) is called to enforce the T3512. However, the UE is ahead of the SS, and to accommodate this a tolerance is needed while implementing the wait for T3512 expiry at SS side. - At step 30-35a1, according to TS 24.501 ch 8.2.6.3, UE will not include the IE 5GMM Capability when it performs a Registration for periodic updating. The current TTCN checks for this IE to be present, which causes testcase failure. This needs to be corrected A Prose CR will be raised for this. - At step 41, SRB2 has been established, hence NAS message should be sent and received on RbId_SRB2. |
| Summary of change | - At step 18-23a1, update the value of v_TAC, v_TAIList according to NGC Cell A. - Called f_Delay (f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 60.0)); instead of f_Delay(60.0);. - At step 30-35a1, update f_Check_NG_RegistrationReqMsg to not check GMM_MobilityInfo from periodic registration updating procedure. - At step 41, update v_SRB to tsc_NR_RbId_SRB2 to send and receive messages. |
| TTCN module | PEIPS_NR5GC.ttcn |
| MCC160 Comments | - Accepted - Accepted - Accepted conditional to prose CR agreement at RAN5#108 - Accepted |

Before Change:

module PEIPS_NR5GC { import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from CommonDefs all; import from NR_CommonDefs all; import from NAS_CommonTemplates all; import from NG_NAS_TypeDefs all; import from NR_ASP_Templates_SRB all; import from NR_Component_Base all; import from NR_Component_NR5GC all; import from NR_ConfigurationSteps all; import from NR_CellInfo all; import from NG_NAS_Templates all; import from NR5GC_CommonFunctions all; import from NG_NAS_Common all; import from NR5GC_NASSteps all; import from NR5GC_RRCSteps all; import from NG_NAS_MsgContainers all; import from NR_RRC_Templates all; import from NG_NASTemplateFunctions all; import from NR5GC_SysinfoPaging all; import from NR_SysInfo_Templates all; ... function fl_TC_9_1_14_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellD := f_NR5GC_MapNASCell(ngc_CellD, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellD); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellD)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellD); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellD)), {bit2oct(v_TAC)}); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; //@siclog "Step 1" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); ... //@siclog "Step 18-23a1" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_NRPSSI)) { //@sic R5-240155 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, -, -, -, -, -, -, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI('5E'O, '101'B, '00001'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_PEIPS_AssistInfo('33'O, '01'O,{{'000'B,'00001'B}})); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_Registration_Complete(v_NGC_NASCellA); f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 24" siclog@ //The SS transmits a PEI with the bit corresponding for the subgroup the UE belongs to is indicated as 1 and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(v_NGC_NASCellA, -,'11'B, 22 ); //@siclog "Step 25" siclog@ //Check: Does the UE transmit an RRCSetupRequest message? f_NR_RRC_SetupRequest_Def(v_NGC_NASCellA, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 25"); //@siclog "Step 26" siclog@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (v_NGC_NASCellA); //@siclog "Step 27" siclog@ v_ReceivedMsg := f_NR_RRCSetupComplete_Def(v_NGC_NASCellA, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA)),'0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Step 28-28C" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellA, v_ReceivedMsg); //@siclog "Step 29" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 29A" siclog@ f_Delay(60.0); //@siclog "Step 30-35a1" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic, -, -, -, -, cdr_NG_GMM_Cap_NRPSSI)) { //@sic R5-240155 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(v_NGC_NASCellA, v_GMM_MobilityInfo); //@siclog "Step 36" siclog@ //The SS transmits a PEI with the bit corresponding for the subgroup the UE belongs to is indicated as 1 and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(v_NGC_NASCellA, -,'11'B, 22 ); //@siclog "Step 37" siclog@ //Check: Does the UE transmit an RRCSetupRequest message? f_NR_RRC_SetupRequest_Def(v_NGC_NASCellA, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 37"); //@siclog "Step 38" siclog@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (v_NGC_NASCellA); //@siclog "Step 39" siclog@ v_ReceivedMsg := f_NR_RRCSetupComplete_Def(v_NGC_NASCellA, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA)),'0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Step 40-40C" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellA, v_ReceivedMsg); //@siclog "Step 41" siclog@ //The SS transmits a CONFIGURATION UPDATE COMMAND containing PEIPS assistance information SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('0'B, '1'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_PEIPS_AssistInfo('1F'O, '01'O,{{'000'B,'00000'B}}))))); //@siclog "Step 42" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); ... //@siclog "Step 48-51" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellA, v_ReceivedMsg); }//End of fl_TC_9_1_14_1_Body }//End of Module

After Change:

module PEIPS_NR5GC { import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from CommonDefs all; import from NR_CommonDefs all; import from NAS_CommonTemplates all; import from NG_NAS_TypeDefs all; import from NR_ASP_Templates_SRB all; import from NR_Component_Base all; import from NR_Component_NR5GC all; import from NR_ConfigurationSteps all; import from NR_CellInfo all; import from NG_NAS_Templates all; import from NR5GC_CommonFunctions all; import from NG_NAS_Common all; import from NR5GC_NASSteps all; import from NR5GC_RRCSteps all; import from NG_NAS_MsgContainers all; import from NR_RRC_Templates all; import from NG_NASTemplateFunctions all; import from NR5GC_SysinfoPaging all; import from NR_SysInfo_Templates all; import from NR_Timing all; ... function fl_TC_9_1_14_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellD := f_NR5GC_MapNASCell(ngc_CellD, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellD); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellD)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellD); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellD)), {bit2oct(v_TAC)}); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; //@siclog "Step 1" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 2-7a1" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellD, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_NRPSSI)) { //@sic R5-240155 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_PEIPS_AssistInfo('33'O, '01'O,{{'000'B,'00000'B}})); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellD, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_Registration_Complete(v_NGC_NASCellD); f_NR_RRCRelease(v_NGC_NASCellD); //@siclog "Step 8" siclog@ //The SS transmits a PEI with the bit corresponding for the subgroup the UE belongs to is indicated as 1 and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(v_NGC_NASCellD, -,'11'B, 22 ); //@siclog "Step 9" siclog@ //Check: Does the UE transmit an RRCSetupRequest message? f_NR_RRC_SetupRequest_Def(v_NGC_NASCellD, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Step 10" siclog@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (v_NGC_NASCellD); //@siclog "Step 11" siclog@ v_ReceivedMsg := f_NR_RRCSetupComplete_Def(v_NGC_NASCellD, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellD)),'0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Step 12-15" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellD, v_ReceivedMsg); //@siclog "Step 16" siclog@ f_NR_RRCRelease(v_NGC_NASCellD); //@siclog "Step 17" siclog@ f_NR_SetCellPower (v_NGC_NASCellD, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 18-23a1" siclog@ v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, -, -, -, -, cdr_NG_GMM_Cap_NRPSSI)) { //@sic R5-240155 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, -, -, -, -, -, -, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI('5E'O, '101'B, '00001'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_PEIPS_AssistInfo('33'O, '01'O,{{'000'B,'00001'B}})); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_Registration_Complete(v_NGC_NASCellA); f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 24" siclog@ //The SS transmits a PEI with the bit corresponding for the subgroup the UE belongs to is indicated as 1 and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(v_NGC_NASCellA, -,'11'B, 22 ); //@siclog "Step 25" siclog@ //Check: Does the UE transmit an RRCSetupRequest message? f_NR_RRC_SetupRequest_Def(v_NGC_NASCellA, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 25"); //@siclog "Step 26" siclog@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (v_NGC_NASCellA); //@siclog "Step 27" siclog@ v_ReceivedMsg := f_NR_RRCSetupComplete_Def(v_NGC_NASCellA, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA)),'0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Step 28-28C" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellA, v_ReceivedMsg); //@siclog "Step 29" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 29A" siclog@ f_Delay (f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 60.0)); //@siclog "Step 30-35a1" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic)) { //@sic R5-240155 sic@, f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(v_NGC_NASCellA, v_GMM_MobilityInfo); //@siclog "Step 36" siclog@ //The SS transmits a PEI with the bit corresponding for the subgroup the UE belongs to is indicated as 1 and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(v_NGC_NASCellA, -,'11'B, 22 ); //@siclog "Step 37" siclog@ //Check: Does the UE transmit an RRCSetupRequest message? f_NR_RRC_SetupRequest_Def(v_NGC_NASCellA, mt_Access); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 37"); //@siclog "Step 38" siclog@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (v_NGC_NASCellA); //@siclog "Step 39" siclog@ v_ReceivedMsg := f_NR_RRCSetupComplete_Def(v_NGC_NASCellA, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA)),'0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Step 40-40C" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellA, v_ReceivedMsg); //@siclog "Step 41" siclog@ //The SS transmits a CONFIGURATION UPDATE COMMAND containing PEIPS assistance information v_SRB := tsc_NR_RbId_SRB2; SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('0'B, '1'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cs_PEIPS_AssistInfo('1F'O, '01'O,{{'000'B,'00000'B}}))))); //@siclog "Step 42" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); ... //@siclog "Step 48-51" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(v_NGC_NASCellA, v_ReceivedMsg); }//End of fl_TC_9_1_14_1_Body }//End of Module

3 Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41c” with NEA1 and NIA1 ciphering and integrity algorithms.

- Execution Log Files

  - MediaTek MT6986D UE

The MediaTek MT6986D UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_9_1_14_1_MediaTek.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s250297: Supporting information for addition of NR5GC testcase 9.1.14.1 in FR1 |
| --- | --- |