3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250327

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4924 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_SetSysinfoCombinationC8ToC8NoSIB10_ETWS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With the current TTCN implementations, test cases were failing at step-7 due to the assignment of non-accessible SIB1 information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the assignment of variable v_BcchInfo_BR with v_BcchInfo_BR.SIB1 before it is fully populated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1, 14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_EUTRA_SetSysinfoCombinationC8ToC8NoSIB10_ETWS |
| --- | --- |
| Reason for change | With the current TTCN implementations, test cases were failing at step-7 due to the assignment of non-accessible SIB1 information. |
| Summary of change | Removed the assignment of variable v_BcchInfo_BR with v_BcchInfo_BR.SIB1 before it is fully populated. |
| TTCN module | EUTRA_CellInfoInit.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_EUTRA_SetSysinfoCombinationC8ToC8NoSIB10_ETWS(EUTRA_CellId_Type p_CellId, template (value) SystemInformationBlockType11 p_SIB11) runs on EUTRA_PTC { var template (value) BcchInfo_Type v_CellBcchInfo; var EUTRA_CellSysInfo_Type v_EUTRA_CellSysInfo := f_EUTRA_CellInfo_GetSYSINFO(p_CellId); var template (value) BCCH_BCH_Message v_MIB := v_EUTRA_CellSysInfo.BCCH_Info.MIB; var template (value) BCCH_DL_SCH_Message v_SI_SIB2 := v_EUTRA_CellSysInfo.BCCH_Info.SIs[0]; var template (value) BCCH_DL_SCH_Message v_SI_SIB3 := v_EUTRA_CellSysInfo.BCCH_Info.SIs[1]; var template (value) BCCH_DL_SCH_Message v_SIB1 := v_EUTRA_CellSysInfo.BCCH_Info.SIB1; var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(p_CellId), f_EUTRA_CellInfo_GetUL_ChBandwidth(p_CellId)); var template (value) Bcch_BRInfo_Type v_BcchInfo_BR := v_EUTRA_CellSysInfo.BCCH_BR_Info; var template (value) BCCH_DL_SCH_Message_BR v_SIB1_BR := v_BcchInfo_BR.SIB1; var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); var BR_Si_Narrowband_Type v_BR_Si_Narrowband; v_SIB1.message_.c1.systemInformationBlockType1.schedulingInfoList := cs_SchedulingInfo_3SI(v_ChannelBandwidthDependency.SIB3_periodicity, {sibType3}, v_ChannelBandwidthDependency.SIB11_periodicity,{sibType11}); v_CellBcchInfo := cs_BCCHInfo_C1(v_MIB, v_SIB1, { v_SI_SIB2, v_SI_SIB3, cs_SystemInformation_Common(cs_SI_SIB11(p_SIB11)) }, omit); f_EUTRA_CellInfo_SetBCCH_Info(p_CellId, v_CellBcchInfo); .. .. .. v_SIB1_BR.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.systemInfoValueTagList_r13[1], v_SIB1_BR.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.systemInfoValueTagList_r13[2] }; v_BcchInfo_BR := cs_BCCHInfo_BR(omit, v_SIB1_BR, omit, omit); f_EUTRA_CellInfo_SetBCCH_BR_Info(p_CellId, v_BcchInfo_BR); } }

After Change:

function f_EUTRA_SetSysinfoCombinationC8ToC8NoSIB10_ETWS(EUTRA_CellId_Type p_CellId, template (value) SystemInformationBlockType11 p_SIB11) runs on EUTRA_PTC { var template (value) BcchInfo_Type v_CellBcchInfo; var EUTRA_CellSysInfo_Type v_EUTRA_CellSysInfo := f_EUTRA_CellInfo_GetSYSINFO(p_CellId); var template (value) BCCH_BCH_Message v_MIB := v_EUTRA_CellSysInfo.BCCH_Info.MIB; var template (value) BCCH_DL_SCH_Message v_SI_SIB2 := v_EUTRA_CellSysInfo.BCCH_Info.SIs[0]; var template (value) BCCH_DL_SCH_Message v_SI_SIB3 := v_EUTRA_CellSysInfo.BCCH_Info.SIs[1]; var template (value) BCCH_DL_SCH_Message v_SIB1 := v_EUTRA_CellSysInfo.BCCH_Info.SIB1; var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(p_CellId), f_EUTRA_CellInfo_GetUL_ChBandwidth(p_CellId)); var template (value) Bcch_BRInfo_Type v_BcchInfo_BR := v_EUTRA_CellSysInfo.BCCH_BR_Info; var template (value) BCCH_DL_SCH_Message_BR v_SIB1_BR; var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); var BR_Si_Narrowband_Type v_BR_Si_Narrowband; v_SIB1.message_.c1.systemInformationBlockType1.schedulingInfoList := cs_SchedulingInfo_3SI(v_ChannelBandwidthDependency.SIB3_periodicity, {sibType3}, v_ChannelBandwidthDependency.SIB11_periodicity,{sibType11}); v_CellBcchInfo := cs_BCCHInfo_C1(v_MIB, v_SIB1, { v_SI_SIB2, v_SI_SIB3, cs_SystemInformation_Common(cs_SI_SIB11(p_SIB11)) }, omit); f_EUTRA_CellInfo_SetBCCH_Info(p_CellId, v_CellBcchInfo); .. .. .. v_SIB1_BR.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.systemInfoValueTagList_r13[1], v_SIB1_BR.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.systemInfoValueTagList_r13[2] }; v_BcchInfo_BR := cs_BCCHInfo_BR(omit, v_SIB1_BR, omit, omit); f_EUTRA_CellInfo_SetBCCH_BR_Info(p_CellId, v_BcchInfo_BR); } }