3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250332

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4926 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC test case 8.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE vendors need to declare one set of capability per UE to the test houses for certification purposes. The current test procedure does not exclude NTN bands from checking of the reported capability vs declared capability which can cause test case to fail as UE capability differs in TN vs NTN band. And in any case, this test case is meant for checking UE capability in TN bands.There is a correponding test case 22.4.13a to check the UE capability in NTN mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Skipped the band reporting and half duplex checks for NTN bands |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-1 CR 5480 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | f_CheckUE_SupportedEUTRA_BandsExt |
| --- | --- |
| Reason for change | UE vendors need to declare one set of capability per UE to the test houses for certification purposes. The current test procedure does not exclude NTN bands from checking of the reported capability vs declared capability which can cause test case to fail as UE capability differs in TN vs NTN band. And in any case, this test case is meant for checking UE capability in TN bands.There is a correponding test case 22.4.13a to check the UE capability in NTN mode. |
| Summary of change | Skipped the NTN band verification |
| TTCN module | EUTRA_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted MCC160 implementation see below. |

Before Change:

function f_CheckUE_SupportedEUTRA_BandsExt(UE_EUTRA_Capability p_ReceivedCapabilityMsg) { // @sic R5-162652, R5-163090, R5-168397 sic@ var UE_EUTRA_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var bitstring v_SuppEutraBandsExtAccToPICS := f_DeriveSuppEutraBandsFromPics_v9e0(); var boolean v_ExtBandsSupported := false; var bitstring v_SuppEutraBandsExtAccToPICS_Chunk; var integer v_Result; var UE_EUTRA_Capability_v9a0_IEs v_Ue_EUTRA_Capability_v9a0_IEs; var bitstring v_Ue_EUTRA_Capability_v9a0_IEsContainer; var SupportedBandListEUTRA_v9e0 v_SupportedBandListEUTRA_v9e0; var SupportedBandListEUTRA v_SupportedBandListEUTRA := p_ReceivedCapabilityMsg.rf_Parameters.supportedBandListEUTRA; var integer i := 0; // PICS for new bands may be specified ... @sic R5s170603 sic@ while (i < lengthof(v_SuppEutraBandsExtAccToPICS) and (v_ExtBandsSupported == false)) { // @sic R5s170187 sic@ if (v_SuppEutraBandsExtAccToPICS[i] == '1'B) { v_ExtBandsSupported := true; } i := i + 1; } if (v_ExtBandsSupported == false) { // @sic R5s170603 sic@ return ; // ext. bands may be reported or not, does not matter as there are no PICS declared } // PICS for ext. bands have been set if (isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension)) { // @sic R5s120623 sic@ @sic R5s130863 sic@ // Rel 9 (UE-EUTRA-Capability-v9a0-IEs) IEs provided // decoding of the UE_EUTRA_Capability_v9a0_IEs CONTAINER v_Ue_EUTRA_Capability_v9a0_IEsContainer := oct2bit(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension); v_Result := decvalue(v_Ue_EUTRA_Capability_v9a0_IEsContainer, v_Ue_EUTRA_Capability_v9a0_IEs); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs cannot be decoded"); } } else { FatalError(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs not provided for UEs > Rel 8 although PICS for ext. bands are set"); // @sic R5s170298, R5s170603 sic@ } if (not isvalue (v_Ue_EUTRA_Capability_v9a0_IEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v9e0.supportedBandListEUTRA_v9e0)) { FatalError(__FILE__, __LINE__, "supportedBandListEUTRA_v9e0 not provided for UEs > Rel 8 although PICS for ext. bands are set"); // @sic R5s170298, R5s170603 sic@ } else { v_SupportedBandListEUTRA_v9e0 := v_Ue_EUTRA_Capability_v9a0_IEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v9e0.supportedBandListEUTRA_v9e0; if (lengthof(v_SupportedBandListEUTRA_v9e0) != lengthof(v_SupportedBandListEUTRA)) { f_SetVerdict(fail, __FILE__, __LINE__, " lengths of SupportedBandListEUTRA and SupportedBandListEUTRA_v9e0 differing"); // @sic R5s170446 sic@ } // It is checked that number of supported EUTRA bands match the set PICS parameters (pc_eBanda_Supp) // There may be more bands reported than there are PICS defined for bands // Check that any value B such that pc_eBandB_Supp is TRUE and different from all eutra-Band[k] where k = 1 to a - 1 // Go through the received supportedBandListEUTRA and match against the supported bands according to the PICS // Check only the bands for which there are PICS; do not check other bands which may or may not be reported. // Done for all sets of continuous PICS. v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 0, 10); // bands 65-74 [0 = 65-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 0, 10); // @sic R5-162652, R5-163090 sic@ // @sic R5-168397, R5-176304, R5-181163, R5-181164, R5-176295, R5-180920, R5-196883, R5s210295, R5-236579 sic@ // appearance of band 85 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 20, 1); // band 85 [20 = 85-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 20, 1); // @sic R5-193737 sic@ // appearance of bands 87-88 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 22, 2); // band 87 [22 = 87-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 22, 2); // @sic R5-205088, R5s210295 sic@ // appearance of band 103 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 38, 1); // band 103 [38 = 103-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 38, 1); // @sic R5-225938 sic@ // appearance of bands 255-256 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 190, 2); // band 255 [190 = 255-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 190, 2); // @sic R5-231563 sic@ } }

After Change:

function f_CheckUE_SupportedEUTRA_BandsExt(UE_EUTRA_Capability p_ReceivedCapabilityMsg) { // @sic R5-162652, R5-163090, R5-168397 sic@ var UE_EUTRA_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg; var bitstring v_SuppEutraBandsExtAccToPICS := f_DeriveSuppEutraBandsFromPics_v9e0(); var boolean v_ExtBandsSupported := false; var bitstring v_SuppEutraBandsExtAccToPICS_Chunk; var integer v_Result; var UE_EUTRA_Capability_v9a0_IEs v_Ue_EUTRA_Capability_v9a0_IEs; var bitstring v_Ue_EUTRA_Capability_v9a0_IEsContainer; var SupportedBandListEUTRA_v9e0 v_SupportedBandListEUTRA_v9e0; var SupportedBandListEUTRA v_SupportedBandListEUTRA := p_ReceivedCapabilityMsg.rf_Parameters.supportedBandListEUTRA; var integer i := 0; // PICS for new bands may be specified ... @sic R5s170603 sic@ while (i < lengthof(v_SuppEutraBandsExtAccToPICS) and (v_ExtBandsSupported == false)) { // @sic R5s170187 sic@ if (v_SuppEutraBandsExtAccToPICS[i] == '1'B) { v_ExtBandsSupported := true; } i := i + 1; } if (v_ExtBandsSupported == false) { // @sic R5s170603 sic@ return ; // ext. bands may be reported or not, does not matter as there are no PICS declared } // PICS for ext. bands have been set if (isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension)) { // @sic R5s120623 sic@ @sic R5s130863 sic@ // Rel 9 (UE-EUTRA-Capability-v9a0-IEs) IEs provided // decoding of the UE_EUTRA_Capability_v9a0_IEs CONTAINER v_Ue_EUTRA_Capability_v9a0_IEsContainer := oct2bit(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension); v_Result := decvalue(v_Ue_EUTRA_Capability_v9a0_IEsContainer, v_Ue_EUTRA_Capability_v9a0_IEs); if (v_Result != 0) { FatalError(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs cannot be decoded"); } } else { FatalError(__FILE__, __LINE__, "UE_EUTRA_Capability_v9a0_IEs not provided for UEs > Rel 8 although PICS for ext. bands are set"); // @sic R5s170298, R5s170603 sic@ } if (not isvalue (v_Ue_EUTRA_Capability_v9a0_IEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v9e0.supportedBandListEUTRA_v9e0)) { FatalError(__FILE__, __LINE__, "supportedBandListEUTRA_v9e0 not provided for UEs > Rel 8 although PICS for ext. bands are set"); // @sic R5s170298, R5s170603 sic@ } else { v_SupportedBandListEUTRA_v9e0 := v_Ue_EUTRA_Capability_v9a0_IEs.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.rf_Parameters_v9e0.supportedBandListEUTRA_v9e0; if (lengthof(v_SupportedBandListEUTRA_v9e0) != lengthof(v_SupportedBandListEUTRA)) { f_SetVerdict(fail, __FILE__, __LINE__, " lengths of SupportedBandListEUTRA and SupportedBandListEUTRA_v9e0 differing"); // @sic R5s170446 sic@ } // It is checked that number of supported EUTRA bands match the set PICS parameters (pc_eBanda_Supp) // There may be more bands reported than there are PICS defined for bands // Check that any value B such that pc_eBandB_Supp is TRUE and different from all eutra-Band[k] where k = 1 to a - 1 // Go through the received supportedBandListEUTRA and match against the supported bands according to the PICS // Check only the bands for which there are PICS; do not check other bands which may or may not be reported. // Done for all sets of continuous PICS. v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 0, 10); // bands 65-74 [0 = 65-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 0, 10); // @sic R5-162652, R5-163090 sic@ // @sic R5-168397, R5-176304, R5-181163, R5-181164, R5-176295, R5-180920, R5-196883, R5s210295, R5-236579 sic@ // appearance of band 85 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 20, 1); // band 85 [20 = 85-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 20, 1); // @sic R5-193737 sic@ // appearance of bands 87-88 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 22, 2); // band 87 [22 = 87-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 22, 2); // @sic R5-205088, R5s210295 sic@ // appearance of band 103 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 38, 1); // band 103 [38 = 103-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 38, 1); // @sic R5-225938 sic@ // appearance of bands 255-256 /*v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 190, 2); // band 255 [190 = 255-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 190, 2); // @sic R5-231563 sic@ */ // checking of 255-256 skipped } }

MCC160 Implementation:

function f_CheckUE_SupportedEUTRA_BandsExt(UE_EUTRA_Capability p_ReceivedCapabilityMsg) { … // appearance of band 103 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 38, 1); // band 103 [38 = 103-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 38, 1); // @sic R5-225938 sic@ // appearance of NTN bands 252-256 // not checked @sic R5-255055, R5s250332 sic@ } }

## Change 2

| Function name | f_CheckUE_HDX_SupportedEUTRA_Bands |
| --- | --- |
| Reason for change | UE vendors need to declare one set of capability per UE to the test houses for certification purposes. The current test procedure does not exclude NTN bands from checking of the reported capability vs declared capability which can cause test case to fail as UE capability differs in TN vs NTN band. And in any case, this test case is meant for checking UE capability in TN bands.There is a correponding test case 22.4.13a to check the UE capability in NTN mode. |
| Summary of change | Skipped the NTN bands for half-duplex check |
| TTCN module | EUTRA_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted in principle. MCC160 implementation see below. |

Before Change:

function f_CheckUE_HDX_SupportedEUTRA_Bands(UE_EUTRA_Capability p_ReceivedCapabilityMsg) { //@sic R5-193997 sic@ var integer i; var integer v_BandEUTRA; var boolean v_HalfDuplex; // @sic R5-166324 sic@ var SupportedBandListEUTRA v_SupportedBandListEUTRA := p_ReceivedCapabilityMsg.rf_Parameters.supportedBandListEUTRA; var integer v_MaxBandsReported := lengthof(v_SupportedBandListEUTRA); for (i := 0; i < v_MaxBandsReported; i := i + 1) { // check against all reported bands v_BandEUTRA := v_SupportedBandListEUTRA[i].bandEUTRA; v_HalfDuplex := v_SupportedBandListEUTRA[i].halfDuplex; // @sic R5-166324 sic@ // if the band is an extended band it is represented by maxFBI. All extended bands are FDD bands! if (f_EUTRA_FrequencyBandIsFDD(v_BandEUTRA)) { // @sic R5s160823 sic@ if ((pc_FDD_TypeB_HalfDuplex and (not v_HalfDuplex)) or ((not pc_FDD_TypeB_HalfDuplex) and v_HalfDuplex)) { // @sic R5-166324 sic@ f_SetVerdict(fail, __FILE__, __LINE__, "half duplex mode of EUTRA band No. " & int2str(v_BandEUTRA) & " inconsistent with pc_FDD_TypeB_HalfDuplex"); // half duplex mode inconsistent @sic R5s240460 add. change sic@ }; }; // TDD bands are not checked ! }; }

After Change:

function f_CheckUE_HDX_SupportedEUTRA_Bands(UE_EUTRA_Capability p_ReceivedCapabilityMsg) { //@sic R5-193997 sic@ var integer i; var integer v_BandEUTRA; var boolean v_HalfDuplex; // @sic R5-166324 sic@ var SupportedBandListEUTRA v_SupportedBandListEUTRA := p_ReceivedCapabilityMsg.rf_Parameters.supportedBandListEUTRA; var integer v_MaxBandsReported := lengthof(v_SupportedBandListEUTRA); for (i := 0; i < v_MaxBandsReported; i := i + 1) { // check against all reported bands v_BandEUTRA := v_SupportedBandListEUTRA[i].bandEUTRA; v_HalfDuplex := v_SupportedBandListEUTRA[i].halfDuplex; // @sic R5-166324 sic@ // if the band is an extended band it is represented by maxFBI. All extended bands are FDD bands! if (f_EUTRA_FrequencyBandIsFDD(v_BandEUTRA)) { // @sic R5s160823 sic@ if (v_BandEUTRA != 255 and v_BandEUTRA != 256) { // change for 8_4_5_1 if ((pc_FDD_TypeB_HalfDuplex and (not v_HalfDuplex)) or ((not pc_FDD_TypeB_HalfDuplex) and v_HalfDuplex)) { // @sic R5-166324 sic@ f_SetVerdict(fail, __FILE__, __LINE__, "half duplex mode of EUTRA band No. " & int2str(v_BandEUTRA) & " inconsistent with pc_FDD_TypeB_HalfDuplex"); // half duplex mode inconsistent @sic R5s240460 add. change sic@ }; }; }; // TDD bands are not checked ! }; }

MCC160 Implementation:

/* * @desc Function to check if a band is a TN band (i.e. not an NTN band) * @param p_BandEUTRA * @status */ function fl_EUTRA_Band_IsTN(integer p_BandEUTRA) return boolean { // @sic R5-255055, R5s250332 sic@ if ((p_BandEUTRA == 252) or (p_BandEUTRA == 253) or (p_BandEUTRA == 254) or (p_BandEUTRA == 255) or (p_BandEUTRA == 256)) {return false} else {return true}; } function f_CheckUE_HDX_SupportedEUTRA_Bands(UE_EUTRA_Capability p_ReceivedCapabilityMsg) { //@sic R5-193997 sic@ … for (i := 0; i < v_MaxBandsReported; i := i + 1) { // check against all reported bands v_BandEUTRA := v_SupportedBandListEUTRA[i].bandEUTRA; v_HalfDuplex := v_SupportedBandListEUTRA[i].halfDuplex; // @sic R5-166324 sic@ // if the band is an extended band it is represented by maxFBI. All extended bands are FDD bands! if (f_EUTRA_FrequencyBandIsFDD(v_BandEUTRA)) { // @sic R5s160823 sic@ if (fl_EUTRA_Band_IsTN (v_BandEUTRA)) { // @sic R5-255055, R5s250332 sic@ if ((pc_FDD_TypeB_HalfDuplex and (not v_HalfDuplex)) or ((not pc_FDD_TypeB_HalfDuplex) and v_HalfDuplex)) { // @sic R5-166324 sic@ f_SetVerdict(fail, __FILE__, __LINE__, "half duplex mode of EUTRA band No. " & int2str(v_BandEUTRA) & " inconsistent with pc_FDD_TypeB_HalfDuplex"); // half duplex mode inconsistent @sic R5s240460 add. change sic@ }; }; }; // TDD bands are not checked ! }; }