3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250333

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4929 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 8.1.2.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For TC 8.1.2.15 LTE branch, there are three elements in SIB1 scheduling list. However, in SS scheduling configuration, SIB29 scheduling has been removed. And SS does not configure SIB29. However, for TC 8.1.2.15 BR-r13, there are three elements in SIB1 scheduling list, but in the SS scheduling configuration, SIB29 scheduling was not removed, while at this time, SS did not configure SIB29. This will result in a mismatch between the number of SS system info and the number of scheduling info. Therefore, in the BR-R13 branch, the SIB29 scheduling is removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the BR-R13 branch, the SIB29 scheduling is removed. Please see below for detailed changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN runtime error is observed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Change 1	2

2.1 Change 2	9

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2025-06_D25wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | Correction to f_TC_8_1_2_15_EUTRA |
| --- | --- |
| Reason for change | For TC 8.1.2.15 LTE branch, there are three elements in SIB1 scheduling list. However, in SS scheduling configuration, SIB29 scheduling has been removed. And SS does not configure SIB29. For TC 8.1.2.15 BR-r13, there are three elements in SIB1 scheduling list, but in the SS scheduling configuration, SIB29 scheduling was not removed, while at this time, SS did not configure SIB29. This will result in a mismatch between the number of SS system info and the number of scheduling info. Therefore, in the BR-R13 branch, the SIB29 scheduling is removed. |
| Summary of change | In the BR-R13 branch, the SIB29 scheduling is removed. |
| TTCN module | LTE\8_1\RRC_ConnEst.ttcn |
| MCC160 Comments | Accepted, implemented differently to make more straightforward changes in the common function f_EUTRA_SS_ConfigureSysinfo. See MCC160 proposed implementation below. |

Before change

| function f_TC_8_1_2_15_EUTRA() runs on EUTRA_PTC { /* RRC connection establishment / Extended and spare fields in SI */ /* @sic R5-206433 R5-206439 R5-206054 R5s230232 R5-237431 R5-244434 sic@ */ var NAS_MSG_Indication_Type v_NasInd; var AccessStratumRelease v_Rel; var template (value) EUTRA_CellInfo_Type v_CellInfo; var SysinfoType_Type v_SysinfoType; var template (value) AllSiSchedul_Type v_SiSchedul; var SIB_Type v_SIB1_SibType1 := sibType29_v1610; // by default value given for a UE Rel-8 to 15 // Variables for case UE Rel-16 var template (value) SystemInformationBlockType1_v12j0_IEs v_SystemInformationBlockType1_v12j0; var BR_Si_Narrowband_Type v_BR_Si_Narrowband; var FreqBandIndicator_r11 v_Dummy_FreqBandIndicator_r11 := 1; f_EUTRA_Init(c1); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); // Preamble: The UE is in state Switched OFF (state 1) f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE); //check the release: Rel-8 to Rel-17 only are allowed otherwise inconclusive verdict. v_Rel := f_EUTRA_MobileInfo_GetUECapability_AccessStratumRelease(); if (not match (v_Rel, (rel8, rel9, rel10, rel11, rel13, rel14, rel15, rel16, rel17)))// NOTE: Spare field testing is not applicable for Rel-12 UE because a spare field in MIB or SIB1 does not exist in Rel-12. { f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to rel8, rel9, rel10, rel11, rel13, rel14, rel15, rel16, rel17 UE only"); } f_EUTRA_UE_Detach_SwitchOffUe(eutra_Cell1, RRC_IDLE); f_EUTRA_TestBody_Set(true); //@siclog "Step 1" siclog@ // System information messages, that include fields and values that the UE does not comprehend, are broadcasted on BCCH and BCCH-BR. v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1); v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(eutra_Cell1); // Set MIB as per Table 8.1.2.15.3.3-1: Message MasterInformationBlock v_CellInfo.Sysinfo.BCCH_Info.MIB.message_.spare := '1'B; /* @sic R5s221178 BASELINE MOVING 2022 rel-17 sic@ */ v_BR_Si_Narrowband := f_EUTRA_InitBR_SiNarrowband(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1)); // Table 8.1.2.15.3.3-2: Message SystemInformationBlockType1 if (match (v_Rel, (rel16, rel17))) { // test UEs Rel-16/Rel-17 //set schedulingInfoListExt-r12 in SIB1 with sibType33-v1800 for Message SystemInformationBlockType1 and SystemInformationBlockType1-BR-r13 v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(-, cs_SchedulingInfoListExt_1SI(rf32,{sibType33_v1800})); f_EUTRA_CellInfo_SetSysInfo_SIB1_v12j0(eutra_Cell1, v_SystemInformationBlockType1_v12j0); if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.schedulingInfoList_BR_r13[2] := cs_SchedulingInfo_BR(v_BR_Si_Narrowband); } } else { // UE is Rel-8 to Rel-15 // Set scheduling in SIB1 v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.schedulingInfoList[2].si_Periodicity := rf32; v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.schedulingInfoList[2].sib_MappingInfo[0] := v_SIB1_SibType1; // Set scheduling in SIB1-BR as per Table 8.1.2.15.3.3-3: Message SystemInformationBlockType1-BR-r13 if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.schedulingInfoList[2].si_Periodicity := rf32; v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.schedulingInfoList[2].sib_MappingInfo[0] := v_SIB1_SibType1; v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.schedulingInfoList_BR_r13[2] := cs_SchedulingInfo_BR(v_BR_Si_Narrowband); } } f_EUTRA_CellInfo_Set(eutra_Cell1, v_CellInfo); // UE is < Rel-18 => To simulate handling of a not comprehended nonCriticalExtension of a UE version < Rel-18. // Set scheduling in SIB1 as per Tables 8.1.2.15.3.3-2: Message SystemInformationBlockType1 and 8.1.2.15.3.3-3: Message SystemInformationBlockType1-BR-r13 (when UE under test is CAT M1, Table 8.1.2.15.3.2-1) f_EUTRA_CellInfo_SetSysInfo_SIB1_v1800_IEs(eutra_Cell1, cs_SystemInformationBlockType1_v1800_IEs(v_Dummy_FreqBandIndicator_r11)); // Remove SIB26 from the scheduling list for SS configuration v_SiSchedul := f_EUTRA_SS_SetupSchedulingInfo(valueof(v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1), f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1), v_SysinfoType, valueof(v_CellInfo.Frequency.DL_ChBandwidth)); v_SiSchedul.SiList := {v_SiSchedul.SiList[0], v_SiSchedul.SiList[1]}; f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul); //@siclog "Step 2" siclog@ //The UE is switched on. f_EUTRA_SwitchOnUEandStartIP(eutra_Cell1); //@siclog "Step 3" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message on Cell 1? f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell1); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.15 Step 3"); //@siclog "Step 4" siclog@ //The SS transmits an RRCConnectionSetup message on Cell 1. f_EUTRA_RRC_ConnectionSetup_Def ( eutra_Cell1); //@siclog "Step 5" siclog@ //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message? The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST. v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(eutra_Cell1, cr_508_RRCConnectionSetupComplete(tsc_RRC_TI_Def, ?), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType()), cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, f_GetPdnType()))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.15 Step 5"); //@siclog "Step 6-18" siclog@ //Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. f_EUTRA_InitialRegistration_Step5_18b1 (eutra_Cell1, STATE2_IDLEUPDATE, NORMAL, v_NasInd); f_EUTRA_TestBody_Set(false); /* Postamble */ f_EUTRA_Postamble(eutra_Cell1, E1_IDLE); } } |  |
| --- | --- |

After change

| function f_TC_8_1_2_15_EUTRA() runs on EUTRA_PTC { /* RRC connection establishment / Extended and spare fields in SI */ /* @sic R5-206433 R5-206439 R5-206054 R5s230232 R5-237431 R5-244434 sic@ */ var NAS_MSG_Indication_Type v_NasInd; var AccessStratumRelease v_Rel; var template (value) EUTRA_CellInfo_Type v_CellInfo; var SysinfoType_Type v_SysinfoType; var template (value) AllSiSchedul_Type v_SiSchedul; var SIB_Type v_SIB1_SibType1 := sibType29_v1610; // by default value given for a UE Rel-8 to 15 var template (omit) BandwidthReducedAccessRelatedInfo_Type v_BandwidthReducedAccessRelatedInfo := omit; var template (omit) SI_SubframeOffsetList_Type v_SI_SubframeOffsetList := omit; // Variables for case UE Rel-16 var template (value) SystemInformationBlockType1_v12j0_IEs v_SystemInformationBlockType1_v12j0; var BR_Si_Narrowband_Type v_BR_Si_Narrowband; var FreqBandIndicator_r11 v_Dummy_FreqBandIndicator_r11 := 1; f_EUTRA_Init(c1); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); // Preamble: The UE is in state Switched OFF (state 1) f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE); //check the release: Rel-8 to Rel-17 only are allowed otherwise inconclusive verdict. v_Rel := f_EUTRA_MobileInfo_GetUECapability_AccessStratumRelease(); if (not match (v_Rel, (rel8, rel9, rel10, rel11, rel13, rel14, rel15, rel16, rel17)))// NOTE: Spare field testing is not applicable for Rel-12 UE because a spare field in MIB or SIB1 does not exist in Rel-12. { f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to rel8, rel9, rel10, rel11, rel13, rel14, rel15, rel16, rel17 UE only"); } f_EUTRA_UE_Detach_SwitchOffUe(eutra_Cell1, RRC_IDLE); f_EUTRA_TestBody_Set(true); //@siclog "Step 1" siclog@ // System information messages, that include fields and values that the UE does not comprehend, are broadcasted on BCCH and BCCH-BR. v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1); v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(eutra_Cell1); // Set MIB as per Table 8.1.2.15.3.3-1: Message MasterInformationBlock v_CellInfo.Sysinfo.BCCH_Info.MIB.message_.spare := '1'B; /* @sic R5s221178 BASELINE MOVING 2022 rel-17 sic@ */ v_BR_Si_Narrowband := f_EUTRA_InitBR_SiNarrowband(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1)); // Table 8.1.2.15.3.3-2: Message SystemInformationBlockType1 if (match (v_Rel, (rel16, rel17))) { // test UEs Rel-16/Rel-17 //set schedulingInfoListExt-r12 in SIB1 with sibType33-v1800 for Message SystemInformationBlockType1 and SystemInformationBlockType1-BR-r13 v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(-, cs_SchedulingInfoListExt_1SI(rf32,{sibType33_v1800})); f_EUTRA_CellInfo_SetSysInfo_SIB1_v12j0(eutra_Cell1, v_SystemInformationBlockType1_v12j0); if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.schedulingInfoList_BR_r13[2] := cs_SchedulingInfo_BR(v_BR_Si_Narrowband); } } else { // UE is Rel-8 to Rel-15 // Set scheduling in SIB1 v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.schedulingInfoList[2].si_Periodicity := rf32; v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.schedulingInfoList[2].sib_MappingInfo[0] := v_SIB1_SibType1; // Set scheduling in SIB1-BR as per Table 8.1.2.15.3.3-3: Message SystemInformationBlockType1-BR-r13 if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.schedulingInfoList[2].si_Periodicity := rf32; v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.schedulingInfoList[2].sib_MappingInfo[0] := v_SIB1_SibType1; v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.schedulingInfoList_BR_r13[2] := cs_SchedulingInfo_BR(v_BR_Si_Narrowband); } } f_EUTRA_CellInfo_Set(eutra_Cell1, v_CellInfo); // UE is < Rel-18 => To simulate handling of a not comprehended nonCriticalExtension of a UE version < Rel-18. // Set scheduling in SIB1 as per Tables 8.1.2.15.3.3-2: Message SystemInformationBlockType1 and 8.1.2.15.3.3-3: Message SystemInformationBlockType1-BR-r13 (when UE under test is CAT M1, Table 8.1.2.15.3.2-1) f_EUTRA_CellInfo_SetSysInfo_SIB1_v1800_IEs(eutra_Cell1, cs_SystemInformationBlockType1_v1800_IEs(v_Dummy_FreqBandIndicator_r11)); // Remove SIB29 from the scheduling list for SS configuration v_SiSchedul := f_EUTRA_SS_SetupSchedulingInfo(valueof(v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1), f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1), v_SysinfoType, valueof(v_CellInfo.Frequency.DL_ChBandwidth)); v_SiSchedul.SiList := {v_SiSchedul.SiList[0], v_SiSchedul.SiList[1]}; if (v_SysinfoType == sysinfoAndBR) { // Remove SIB29 from the BR scheduling list for SS configuration v_BandwidthReducedAccessRelatedInfo := v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13; v_BandwidthReducedAccessRelatedInfo.schedulingInfoList_BR_r13 :={v_BandwidthReducedAccessRelatedInfo.schedulingInfoList_BR_r13[0],v_BandwidthReducedAccessRelatedInfo.schedulingInfoList_BR_r13[1]}; v_SI_SubframeOffsetList := f_EUTRA_SetSI_BRSubframeOffsetList(v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13, f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1)); v_SI_SubframeOffsetList :={v_SI_SubframeOffsetList[0],v_SI_SubframeOffsetList[1]}; } f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul,v_BandwidthReducedAccessRelatedInfo,v_SI_SubframeOffsetList); //@siclog "Step 2" siclog@ //The UE is switched on. f_EUTRA_SwitchOnUEandStartIP(eutra_Cell1); //@siclog "Step 3" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message on Cell 1? f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell1); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.15 Step 3"); //@siclog "Step 4" siclog@ //The SS transmits an RRCConnectionSetup message on Cell 1. f_EUTRA_RRC_ConnectionSetup_Def ( eutra_Cell1); //@siclog "Step 5" siclog@ //Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message? The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST. v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(eutra_Cell1, cr_508_RRCConnectionSetupComplete(tsc_RRC_TI_Def, ?), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cr_508_ATTACH_REQUEST(f_GetEAttachType(NORMAL), f_GetAdditionalUpdateType()), cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, f_GetPdnType()))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.15 Step 5"); //@siclog "Step 6-18" siclog@ //Steps 5 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. f_EUTRA_InitialRegistration_Step5_18b1 (eutra_Cell1, STATE2_IDLEUPDATE, NORMAL, v_NasInd); f_EUTRA_TestBody_Set(false); /* Postamble */ f_EUTRA_Postamble(eutra_Cell1, E1_IDLE); } } |  |
| --- | --- |

MCC160 proposed implementation

function f_TC_8_1_2_15_EUTRA() runs on EUTRA_PTC

{ /*  RRC connection establishment / Extended and spare fields in SI */

/* @sic R5-206433 R5-206439 R5-206054 R5s230232 R5-237431 R5-244434 sic@ */

var NAS_MSG_Indication_Type v_NasInd;

var AccessStratumRelease v_Rel;

var template (value) EUTRA_CellInfo_Type v_CellInfo;

var SysinfoType_Type v_SysinfoType;

var template (value) AllSiSchedul_Type v_SiSchedul;

var SIB_Type v_SIB1_SibType1 := sibType29_v1610;    // by default value given for a UE Rel-8 to 15

// Variables for case UE Rel-16

var template (value) SystemInformationBlockType1_v12j0_IEs v_SystemInformationBlockType1_v12j0;

var BR_Si_Narrowband_Type v_BR_Si_Narrowband;

var FreqBandIndicator_r11 v_Dummy_FreqBandIndicator_r11 := 1;

var template (omit) Bcch_BRConfig_Type v_Bcch_BRConfig := omit;  //@sic R5s250333 change 1 sic@

f_EUTRA_Init(c1);

//Create and configure cells

f_EUTRA_CellConfig_Def(eutra_Cell1);

// Preamble: The UE is in state Switched OFF (state 1)

f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE);

//check the release: Rel-8 to Rel-17 only are allowed otherwise inconclusive verdict.

v_Rel := f_EUTRA_MobileInfo_GetUECapability_AccessStratumRelease();

if (not match (v_Rel, (rel8, rel9, rel10, rel11, rel13, rel14, rel15, rel16, rel17)))// NOTE: Spare field testing is not applicable for Rel-12 UE because a spare field in MIB or SIB1 does not exist in Rel-12.

{

f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to rel8, rel9, rel10, rel11, rel13, rel14, rel15, rel16, rel17 UE only");

}

f_EUTRA_UE_Detach_SwitchOffUe(eutra_Cell1, RRC_IDLE);

f_EUTRA_TestBody_Set(true);

//@siclog "Step 1" siclog@

// System information messages, that include fields and values that the UE does not comprehend, are broadcasted on BCCH and BCCH-BR.

v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1);

v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(eutra_Cell1);

// Set MIB as per Table 8.1.2.15.3.3-1: Message MasterInformationBlock

v_CellInfo.Sysinfo.BCCH_Info.MIB.message_.spare := '1'B;    /* @sic R5s221178 BASELINE MOVING 2022 rel-17 sic@ */

v_BR_Si_Narrowband := f_EUTRA_InitBR_SiNarrowband(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1));

// Table 8.1.2.15.3.3-2: Message SystemInformationBlockType1

if (match (v_Rel, (rel16, rel17)))

{ // test UEs Rel-16/Rel-17

//set schedulingInfoListExt-r12 in SIB1 with sibType33-v1800 for Message SystemInformationBlockType1 and SystemInformationBlockType1-BR-r13

v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(-, cs_SchedulingInfoListExt_1SI(rf32,{sibType33_v1800}));

f_EUTRA_CellInfo_SetSysInfo_SIB1_v12j0(eutra_Cell1, v_SystemInformationBlockType1_v12j0);

if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC

v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.schedulingInfoList_BR_r13[2] := cs_SchedulingInfo_BR(v_BR_Si_Narrowband);

}

}

else

{   // UE is Rel-8 to Rel-15

// Set scheduling in SIB1

v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.schedulingInfoList[2].si_Periodicity := rf32;

v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.schedulingInfoList[2].sib_MappingInfo[0] := v_SIB1_SibType1;

// Set scheduling in SIB1-BR as per Table 8.1.2.15.3.3-3: Message SystemInformationBlockType1-BR-r13

if (v_SysinfoType == sysinfoAndBR) {//UE supports category M1 or enhanced coverage TC

v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.schedulingInfoList[2].si_Periodicity := rf32;

v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.schedulingInfoList[2].sib_MappingInfo[0] := v_SIB1_SibType1;

v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.schedulingInfoList_BR_r13[2] := cs_SchedulingInfo_BR(v_BR_Si_Narrowband);

}

}

f_EUTRA_CellInfo_Set(eutra_Cell1, v_CellInfo);

// UE is < Rel-18 => To simulate handling of a not comprehended nonCriticalExtension of a UE version < Rel-18.

// Set freqBandIndicatorAerial-r18 in SIB1 as per Tables 8.1.2.15.3.3-2: Message SystemInformationBlockType1 and 8.1.2.15.3.3-3: Message SystemInformationBlockType1-BR-r13 (when UE under test is CAT M1, Table 8.1.2.15.3.2-1)

f_EUTRA_CellInfo_SetSysInfo_SIB1_v1800_IEs(eutra_Cell1, cs_SystemInformationBlockType1_v1800_IEs(v_Dummy_FreqBandIndicator_r11));

// Remove SIB29/SIB33 from the scheduling list for SS configuration

//@sic R5s250333 sic@

v_SiSchedul := f_EUTRA_SS_SetupSchedulingInfo(valueof(v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1), f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1), v_SysinfoType, valueof(v_CellInfo.Frequency.DL_ChBandwidth));

v_SiSchedul.SiList := {v_SiSchedul.SiList[0], v_SiSchedul.SiList[1]};

if (v_SysinfoType == sysinfoAndBR) {

v_Bcch_BRConfig := f_EUTRA_SS_InitialiseBcch_BR(eutra_Cell1);

v_Bcch_BRConfig.Pdsch.SiSchedul.schedulingInfoList_BR_r13 := {  v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.schedulingInfoList_BR_r13[0],

v_CellInfo.Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.bandwidthReducedAccessRelatedInfo_r13.schedulingInfoList_BR_r13[1]

};

v_Bcch_BRConfig.Pdsch.SubframeOffsetList := {v_Bcch_BRConfig.Pdsch.SubframeOffsetList[0],v_Bcch_BRConfig.Pdsch.SubframeOffsetList[1]};

}

f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul, v_Bcch_BRConfig);

//@siclog "Step 2" siclog@

//The UE is switched on.

f_EUTRA_SwitchOnUEandStartIP(eutra_Cell1);

//@siclog "Step 3" siclog@

//Check: Does the UE transmit an RRCConnectionRequest message on Cell 1?

f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell1);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.15 Step 3");

…

}

## 2.1 Change 2

| Function name | Correction to f_EUTRA_SS_ConfigureSysinfo |
| --- | --- |
| Reason for change | For TC 8.1.2.15 LTE branch, there are three elements in SIB1 scheduling list. However, in SS scheduling configuration, SIB29 scheduling has been removed. And SS does not configure SIB29. For TC 8.1.2.15 BR-r13, there are three elements in SIB1 scheduling list, but in the SS scheduling configuration, SIB29 scheduling was not removed, while at this time, SS did not configure SIB29. This will result in a mismatch between the number of SS system info and the number of scheduling info. Therefore, add input parameters so that the scheduling can be modified in the BR-R13 branch SS scheduling configuration. |
| Summary of change | add input parameters so that the scheduling can be modified in the BR-R13 branch SS scheduling configuration. |
| TTCN module | Common/EUTRA/EUTRA_RRCSteps.ttcn |
| MCC160 Comments | Accepted, implemented differently to make more straightforward changes in the common function f_EUTRA_SS_ConfigureSysinfo. See MCC160 proposed implementation below. |

Before change

| function f_EUTRA_SS_ConfigureSysinfo(EUTRA_CellId_Type p_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, boolean p_ChangeValueTag := true, template (omit) Null_Type p_StopSib1Transmission := omit, boolean p_ModifySIB1_BROnly := false, template (omit) AllSiSchedul_Type p_SiSchedul := omit ) //@sic R5-142409 sic@ runs on EUTRA_PTC { //@sic R5-167569: added p_ModifySIB1_BROnly sic@ var template (value) AllSiSchedul_Type v_SiSchedul; var EUTRA_CellSysInfo_Type v_Sysinfo:= f_EUTRA_CellInfo_GetSYSINFO (p_CellId); // Get SYSINFO to be sent var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(p_CellId); var template (omit) Bcch_BRConfig_Type v_Bcch_BRConfig; //@sic R5-167569 R5s170180 change 3 sic@ var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); //@sic R5-167569 sic@ var template (value) BcchInfo_Type v_BcchInfo; var Dl_Bandwidth_Type v_Dl_Bandwidth := f_EUTRA_CellInfo_GetDL_ChBandwidth(p_CellId); //@sic R5s170247 sic@ // Increment the VALUE TAG if neeeded //@sic R5s130076 sic@ if (p_ChangeValueTag) { v_Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.systemInfoValueTag := (v_Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.systemInfoValueTag + 1) mod 32; if (v_Sysinfo.SysinfoType != sysinfoOnly) { //Increment ValuTag in SIB1-BR @sic R5-167569 sic@ v_Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.systemInfoValueTag := (v_Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.systemInfoValueTag + 1) mod 32; //@sic R5-171566 SIs value tag handled inside TC sic@ } f_EUTRA_SetSysinfo (p_CellId,v_Sysinfo); // Save SYSINFO with the new valueTag in the record f_EUTRA_SetSB1_ValueTag(p_CellId, false); // Set the flag to FALSE f_EUTRA_SetSIs_ValueTag(p_CellId, false); } //Initialise v_Bcch_BRConfig after SIB valueTag change @sic R5s170180 change 3 sic@ v_Bcch_BRConfig := f_EUTRA_SS_InitialiseBcch_BR(p_CellId); if (isvalue(p_SiSchedul)) { //compiler warning may be ignored due to check v_SiSchedul := p_SiSchedul; } else { v_SiSchedul := f_EUTRA_SS_SetupSchedulingInfo(v_Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1, v_EUTRA_FDD_TDD_Mode, v_SysinfoType, v_Dl_Bandwidth); /* derive SS's scheduling information from SIB1 */ } if (p_ModifySIB1_BROnly and not(v_Sysinfo.SI_BR_ValueTagChanged)) { //Modify SIB1/SIB1-BR only v_Bcch_BRConfig.BcchInfo := cs_BCCHInfo_BR(omit, v_Bcch_BRConfig.BcchInfo.SIB1, omit); v_BcchInfo := cs_BCCHInfo_C1(v_Sysinfo.BCCH_Info.MIB, v_Sysinfo.BCCH_Info.SIB1, omit); f_EUTRA_SS_CommonCellConfig(p_CellId, cas_ModifySysinfo_Def_REQ(p_CellId, p_TimingInfo, v_SiSchedul, /* scheduling of SIs */ v_BcchInfo, //SIs in BCCH info omitted f_EUTRA_CellInfo_GetDL_ChBandwidth (p_CellId), p_StopSib1Transmission, v_Bcch_BRConfig)); } else { //Modify all sysinfo f_EUTRA_SS_CommonCellConfig(p_CellId, cas_ModifySysinfo_Def_REQ(p_CellId, p_TimingInfo, v_SiSchedul, /* scheduling of SIs */ v_Sysinfo.BCCH_Info, /* system information to be scheduled */ f_EUTRA_CellInfo_GetDL_ChBandwidth (p_CellId), p_StopSib1Transmission, v_Bcch_BRConfig)); } } |  |
| --- | --- |

After change

| function f_EUTRA_SS_ConfigureSysinfo(EUTRA_CellId_Type p_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, boolean p_ChangeValueTag := true, template (omit) Null_Type p_StopSib1Transmission := omit, boolean p_ModifySIB1_BROnly := false, template (omit) AllSiSchedul_Type p_SiSchedul := omit, template (omit) BandwidthReducedAccessRelatedInfo_Type p_BandwidthReducedAccessRelatedInfo := omit, template (omit) SI_SubframeOffsetList_Type P_SiSubframeOffsetList := omit ) //@sic R5-142409 sic@ runs on EUTRA_PTC { //@sic R5-167569: added p_ModifySIB1_BROnly sic@ var template (value) AllSiSchedul_Type v_SiSchedul; var EUTRA_CellSysInfo_Type v_Sysinfo:= f_EUTRA_CellInfo_GetSYSINFO (p_CellId); // Get SYSINFO to be sent var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(p_CellId); var template (omit) Bcch_BRConfig_Type v_Bcch_BRConfig; //@sic R5-167569 R5s170180 change 3 sic@ var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); //@sic R5-167569 sic@ var template (value) BcchInfo_Type v_BcchInfo; var Dl_Bandwidth_Type v_Dl_Bandwidth := f_EUTRA_CellInfo_GetDL_ChBandwidth(p_CellId); //@sic R5s170247 sic@ // Increment the VALUE TAG if neeeded //@sic R5s130076 sic@ if (p_ChangeValueTag) { v_Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.systemInfoValueTag := (v_Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.systemInfoValueTag + 1) mod 32; if (v_Sysinfo.SysinfoType != sysinfoOnly) { //Increment ValuTag in SIB1-BR @sic R5-167569 sic@ v_Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.systemInfoValueTag := (v_Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.systemInfoValueTag + 1) mod 32; //@sic R5-171566 SIs value tag handled inside TC sic@ } f_EUTRA_SetSysinfo (p_CellId,v_Sysinfo); // Save SYSINFO with the new valueTag in the record f_EUTRA_SetSB1_ValueTag(p_CellId, false); // Set the flag to FALSE f_EUTRA_SetSIs_ValueTag(p_CellId, false); } //Initialise v_Bcch_BRConfig after SIB valueTag change @sic R5s170180 change 3 sic@ v_Bcch_BRConfig := f_EUTRA_SS_InitialiseBcch_BR(p_CellId); if (isvalue(p_BandwidthReducedAccessRelatedInfo) and isvalue(P_SiSubframeOffsetList) ) { v_Bcch_BRConfig.Pdsch.SiSchedul := p_BandwidthReducedAccessRelatedInfo; v_Bcch_BRConfig.Pdsch.SubframeOffsetList := P_SiSubframeOffsetList; } if (isvalue(p_SiSchedul)) { //compiler warning may be ignored due to check v_SiSchedul := p_SiSchedul; } else { v_SiSchedul := f_EUTRA_SS_SetupSchedulingInfo(v_Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1, v_EUTRA_FDD_TDD_Mode, v_SysinfoType, v_Dl_Bandwidth); /* derive SS's scheduling information from SIB1 */ } if (p_ModifySIB1_BROnly and not(v_Sysinfo.SI_BR_ValueTagChanged)) { //Modify SIB1/SIB1-BR only v_Bcch_BRConfig.BcchInfo := cs_BCCHInfo_BR(omit, v_Bcch_BRConfig.BcchInfo.SIB1, omit); v_BcchInfo := cs_BCCHInfo_C1(v_Sysinfo.BCCH_Info.MIB, v_Sysinfo.BCCH_Info.SIB1, omit); f_EUTRA_SS_CommonCellConfig(p_CellId, cas_ModifySysinfo_Def_REQ(p_CellId, p_TimingInfo, v_SiSchedul, /* scheduling of SIs */ v_BcchInfo, //SIs in BCCH info omitted f_EUTRA_CellInfo_GetDL_ChBandwidth (p_CellId), p_StopSib1Transmission, v_Bcch_BRConfig)); } else { //Modify all sysinfo f_EUTRA_SS_CommonCellConfig(p_CellId, cas_ModifySysinfo_Def_REQ(p_CellId, p_TimingInfo, v_SiSchedul, /* scheduling of SIs */ v_Sysinfo.BCCH_Info, /* system information to be scheduled */ f_EUTRA_CellInfo_GetDL_ChBandwidth (p_CellId), p_StopSib1Transmission, v_Bcch_BRConfig)); } } |  |
| --- | --- |

MCC160 proposed implementation

function f_EUTRA_SS_ConfigureSysinfo(EUTRA_CellId_Type p_CellId,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

boolean p_ChangeValueTag := true,

template (omit) Null_Type p_StopSib1Transmission := omit,

boolean p_ModifySIB1_BROnly := false,

template (omit) AllSiSchedul_Type p_SiSchedul := omit,

template (omit) Bcch_BRConfig_Type p_Bcch_BRConfig := omit

) //@sic R5-142409 sic@

runs on EUTRA_PTC

{ //@sic R5-167569: added p_ModifySIB1_BROnly R5s250333: added p_Bcch_BRConfig sic@

var template (value) AllSiSchedul_Type v_SiSchedul;

var EUTRA_CellSysInfo_Type  v_Sysinfo:= f_EUTRA_CellInfo_GetSYSINFO (p_CellId);  // Get SYSINFO to be sent

var EUTRA_FDD_TDD_Mode_Type  v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(p_CellId);

var template (omit) Bcch_BRConfig_Type v_Bcch_BRConfig;  //@sic R5-167569 R5s170180 change 3 sic@

var SysinfoType_Type v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(p_CellId); //@sic R5-167569 sic@

var template (value) BcchInfo_Type v_BcchInfo;

var Dl_Bandwidth_Type v_Dl_Bandwidth := f_EUTRA_CellInfo_GetDL_ChBandwidth(p_CellId); //@sic R5s170247 sic@

// Increment the VALUE TAG if neeeded

//@sic R5s130076 sic@

if (p_ChangeValueTag) {

v_Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.systemInfoValueTag :=

(v_Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.systemInfoValueTag + 1) mod 32;

if (v_Sysinfo.SysinfoType != sysinfoOnly) {

//Increment ValuTag in SIB1-BR @sic R5-167569 sic@

v_Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.systemInfoValueTag :=

(v_Sysinfo.BCCH_BR_Info.SIB1.message_.c1.systemInformationBlockType1_BR_r13.systemInfoValueTag + 1) mod 32;

//@sic R5-171566 SIs value tag handled inside TC sic@

}

f_EUTRA_SetSysinfo (p_CellId,v_Sysinfo);     // Save SYSINFO with the new valueTag in the record

f_EUTRA_SetSB1_ValueTag(p_CellId, false);   // Set the flag to FALSE

f_EUTRA_SetSIs_ValueTag(p_CellId, false);

}

//Initialise v_Bcch_BRConfig after SIB valueTag change @sic R5s170180 change 3 sic@

if (isvalue(p_Bcch_BRConfig)) {   // @sic R5s250333 change 2 sic@

v_Bcch_BRConfig := p_Bcch_BRConfig;

} else {

v_Bcch_BRConfig := f_EUTRA_SS_InitialiseBcch_BR(p_CellId);

}

if (isvalue(p_SiSchedul)) { //compiler warning may be ignored due to check

v_SiSchedul := p_SiSchedul;

} else {

v_SiSchedul := f_EUTRA_SS_SetupSchedulingInfo(v_Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1, v_EUTRA_FDD_TDD_Mode, v_SysinfoType, v_Dl_Bandwidth);      /* derive SS's scheduling information from SIB1 */

}

if (p_ModifySIB1_BROnly and not(v_Sysinfo.SI_BR_ValueTagChanged)) {

//Modify SIB1/SIB1-BR only

v_Bcch_BRConfig.BcchInfo := cs_BCCHInfo_BR(omit, v_Bcch_BRConfig.BcchInfo.SIB1, omit);

v_BcchInfo := cs_BCCHInfo_C1(v_Sysinfo.BCCH_Info.MIB, v_Sysinfo.BCCH_Info.SIB1, omit);

f_EUTRA_SS_CommonCellConfig(p_CellId,

cas_ModifySysinfo_Def_REQ(p_CellId,

p_TimingInfo,

v_SiSchedul,      /* scheduling of SIs */

v_BcchInfo,       //SIs in BCCH info omitted

f_EUTRA_CellInfo_GetDL_ChBandwidth (p_CellId),

p_StopSib1Transmission,

v_Bcch_BRConfig));

} else { //Modify all sysinfo

f_EUTRA_SS_CommonCellConfig(p_CellId,

cas_ModifySysinfo_Def_REQ(p_CellId,

p_TimingInfo,

v_SiSchedul,              /* scheduling of SIs */

v_Sysinfo.BCCH_Info,       /* system information to be scheduled */

f_EUTRA_CellInfo_GetDL_ChBandwidth (p_CellId),

p_StopSib1Transmission,

v_Bcch_BRConfig));

}

}