3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250339

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3736 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This function currently incorrectly handling the case when first EUTRA PND is Internet and second IMS. v_WaitForUserplaneSignalling is dontWaitForIMS which cause that TTCN will wait 1.2s and during this time UE can send PDN connectivity request for IMS PDN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change v_WaitForUserplaneSignalling to noDelay when first PDn is internet and second IMS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR.

Contact: 	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Template name | f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 |
| --- | --- |
| Reason for change | This function currently incorrectly handling the case when first EUTRA PND is Internet and second IMS. v_WaitForUserplaneSignalling is dontWaitForIMS which cause that TTCN will wait 1.2s and during this time UE can send PDN connectivity request for IMS PDN |
| Summary of change | Change v_WaitForUserplaneSignalling to noDelay when first PDn is internet and second IMS |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment | Alternative solution implemented (similar to sister function in NR5GC) – activated default behaviour before calling this function to receive and store any PDN connectivity request if it’s sent |

Before Change:

function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16(EUTRA_CellId_Type p_CellId, NR_TESTMODE_STATE_Type p_State, NAS_MSG_Indication_Type p_NAS_Ind, template (omit) PLMN_List p_EquivalentPlmnList := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList := omit, boolean p_FallbackSignalling := false, boolean p_PreserveIPaddress := true) runs on EUTRA_Multi5GS_PTC { var template (omit) AdditionalUpdateType v_AdditionalUpdateType; var NAS_UL_Message_Type v_PdnConnectivityRequest; var ProcedureTransactionIdentifier v_EPS_TI; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; // may be omit var template (omit) ExtdProtocolConfigOptions v_ExtdPcoFromUE := omit; var EPS_BearerIdentity v_EpsBearerId; var boolean v_IPv4AllocationViaNasFlag; var APNandPCOs_Type v_APNandPCOs; var NasCount_Type v_NasCountUL; var NR_Configuration_Type v_NR_ConfigType := f_EUTRA38_NR_Config_Get(); var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var template (omit) UERadioCapId v_UERadioCapIdToSend := omit; // @sic R5s220478 sic@ if (not ispresent (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present"); } select (v_NR_ConfigType) { case (EN_DC) { if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC"); } } case else { // other options FFS if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for EUTRA38"); } } } v_AdditionalUpdateType := p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.addUpdateType; v_PdnConnectivityRequest := p_NAS_Ind.Pdu.PiggybackedPduList[0].Msg; v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; v_PcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; v_ExtdPcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.extdProtocolConfigurationOptions; // Steps 5-10b6 v_NasCountUL := f_EUTRA_MultiPDN_InitialRegistration_Step5_8 (p_CellId, p_NAS_Ind); // @sic R5-216280, R5-222047 sic@ v_APNandPCOs := f_EUTRA38_MultiPDN_InitialRegistration_Step9_13 (p_CellId, p_State, p_NAS_Ind, v_NasCountUL); if (ispresent(v_APNandPCOs.protocolConfigurationOptions)) { v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } else if (ispresent(v_APNandPCOs.extdProtocolConfigurationOptions)) { v_ExtdPcoFromUE := v_APNandPCOs.extdProtocolConfigurationOptions; } if (isvalue(v_PcoFromUE)) { v_IPv4AllocationViaNasFlag := f_CheckPCOforIPallocationViaNas(v_PcoFromUE); } else { v_IPv4AllocationViaNasFlag := f_CheckExtdPCOforIPallocationViaNas(v_ExtdPcoFromUE); } if (pc_EPC_RACS) { // @sic R5-216280 sic@ v_UERadioCapIdToSend := f_EUTRA_MobileInfo_GetRACS_IdToSend(); // @sic R5s220478 sic@ } v_EpsBearerId := f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(p_CellId, NORMAL, v_AdditionalUpdateType, v_EPS_TI, v_IPv4AllocationViaNasFlag, v_PcoFromUE, v_APNandPCOs.accessPointName, cs_GprsTimer_v_deact, omit, omit, p_EquivalentPlmnList, 0, omit, -, v_ExtdPcoFromUE, p_PduSessionInfoList, // @sic R5s210149 sic@ v_UERadioCapIdToSend, // @sic R5-216280 sic@ -, p_PreserveIPaddress); // @sic R5s240679 sic@ // Step 10b7 if (p_FallbackSignalling) { // @sic R5-206296, R5s220177, R5s250200 sic@ /* In case of another PDN connectivity request this may be sent immediately by the UE => we cannot wait after the ATTACH_COMPLETE for any trigger from the IMS_PTC but need to do this when all PDNs are configured */ v_WaitForUserplaneSignalling := noDelay; } else { select (f_EUTRA38_MultiPDN_GetPDNType(v_APNandPCOs.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { v_WaitForUserplaneSignalling := dontWaitForIMS; } } } f_EUTRA_NAS_AttachComplete(p_CellId, v_EpsBearerId, v_WaitForUserplaneSignalling); }

After Change:

function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16(EUTRA_CellId_Type p_CellId, NR_TESTMODE_STATE_Type p_State, NAS_MSG_Indication_Type p_NAS_Ind, template (omit) PLMN_List p_EquivalentPlmnList := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList := omit, boolean p_FallbackSignalling := false, boolean p_PreserveIPaddress := true) runs on EUTRA_Multi5GS_PTC { var template (omit) AdditionalUpdateType v_AdditionalUpdateType; var NAS_UL_Message_Type v_PdnConnectivityRequest; var ProcedureTransactionIdentifier v_EPS_TI; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; // may be omit var template (omit) ExtdProtocolConfigOptions v_ExtdPcoFromUE := omit; var EPS_BearerIdentity v_EpsBearerId; var boolean v_IPv4AllocationViaNasFlag; var APNandPCOs_Type v_APNandPCOs; var NasCount_Type v_NasCountUL; var NR_Configuration_Type v_NR_ConfigType := f_EUTRA38_NR_Config_Get(); var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var template (omit) UERadioCapId v_UERadioCapIdToSend := omit; // @sic R5s220478 sic@ if (not ispresent (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present"); } select (v_NR_ConfigType) { case (EN_DC) { if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC"); } } case else { // other options FFS if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for EUTRA38"); } } } v_AdditionalUpdateType := p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.addUpdateType; v_PdnConnectivityRequest := p_NAS_Ind.Pdu.PiggybackedPduList[0].Msg; v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; v_PcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; v_ExtdPcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.extdProtocolConfigurationOptions; // Steps 5-10b6 v_NasCountUL := f_EUTRA_MultiPDN_InitialRegistration_Step5_8 (p_CellId, p_NAS_Ind); // @sic R5-216280, R5-222047 sic@ v_APNandPCOs := f_EUTRA38_MultiPDN_InitialRegistration_Step9_13 (p_CellId, p_State, p_NAS_Ind, v_NasCountUL); if (ispresent(v_APNandPCOs.protocolConfigurationOptions)) { v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } else if (ispresent(v_APNandPCOs.extdProtocolConfigurationOptions)) { v_ExtdPcoFromUE := v_APNandPCOs.extdProtocolConfigurationOptions; } if (isvalue(v_PcoFromUE)) { v_IPv4AllocationViaNasFlag := f_CheckPCOforIPallocationViaNas(v_PcoFromUE); } else { v_IPv4AllocationViaNasFlag := f_CheckExtdPCOforIPallocationViaNas(v_ExtdPcoFromUE); } if (pc_EPC_RACS) { // @sic R5-216280 sic@ v_UERadioCapIdToSend := f_EUTRA_MobileInfo_GetRACS_IdToSend(); // @sic R5s220478 sic@ } v_EpsBearerId := f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(p_CellId, NORMAL, v_AdditionalUpdateType, v_EPS_TI, v_IPv4AllocationViaNasFlag, v_PcoFromUE, v_APNandPCOs.accessPointName, cs_GprsTimer_v_deact, omit, omit, p_EquivalentPlmnList, 0, omit, -, v_ExtdPcoFromUE, p_PduSessionInfoList, // @sic R5s210149 sic@ v_UERadioCapIdToSend, // @sic R5-216280 sic@ -, p_PreserveIPaddress); // @sic R5s240679 sic@ // Step 10b7 if (p_FallbackSignalling) { // @sic R5-206296, R5s220177, R5s250200 sic@ /* In case of another PDN connectivity request this may be sent immediately by the UE => we cannot wait after the ATTACH_COMPLETE for any trigger from the IMS_PTC but need to do this when all PDNs are configured */ v_WaitForUserplaneSignalling := noDelay; } else { select (f_EUTRA38_MultiPDN_GetPDNType(v_APNandPCOs.accessPointName)) { // @sic R5-206296 sic@ case (IMS_DNN) { v_WaitForUserplaneSignalling := waitForIMS; } case (Internet_DNN) { if (tsc_EUTRA_Registration_PDN2isIMS){ v_WaitForUserplaneSignalling := noDelay; } else{ v_WaitForUserplaneSignalling := dontWaitForIMS; } } } } f_EUTRA_NAS_AttachComplete(p_CellId, v_EpsBearerId, v_WaitForUserplaneSignalling); }

#### MCC160 Implementation

function f_EUTRA38_MultiPDN_InitialRegistration_Step5_18b1(EUTRA_CellId_Type p_CellId,

NR_TESTMODE_STATE_Type p_State,

NAS_MSG_Indication_Type p_NAS_Ind,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

template (omit) PLMN_List p_EquivalentPlmnList := omit) runs on EUTRA_Multi5GS_PTC

{

var default v_MyDefaultVar := null;

// First clear store of previously received messages

f_EUTRA38_MsgInDefault_ResetStoreOfPDNConnectivityReq();

// Then activate store for any new msgs received

v_MyDefaultVar := activate(a_EUTRA38_AdditionalPDNConnectivityReq(p_CellId, cr_PDNConnReqAdditionalPDN(f_GetPdnType(), cr_AccessPointName_Any ifpresent, '001'B)));

f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 (p_CellId, p_State, p_NAS_Ind, p_EquivalentPlmnList);

deactivate(v_MyDefaultVar);

// Step 11

f_EUTRA38_MultiPDN_EstablishAdditionalPDNs (p_CellId, pc_noOf_PDNsSameConnection, -, tsc_SRB2 , -, -, -, -, -, 1);

// need to keep all PDNs active

if (pc_noOf_PDNsNewConnection > 0) {

// Step 11Aa1 - 11Aa2 @sic R5-237319 sic@

f_EUTRA_RRC_ConnectionRelease(p_CellId);

f_EUTRA38_MultiPDN_RRC_Idle_Extension (p_CellId);

}

// Step 11Ba1 - 11Ba2 @sic R5-237319 sic@

select (p_State) {

case (TESTModeA_ON, TEST_LOOPModeA_ON){

f_EUTRA_CloseUE_TestLoopModeA (p_CellId, cs_UE_TestLoopModeA_LB_Setup_NoScaling);

}

case (TESTModeB_ON, TEST_LOOPModeB_ON, PING_Or_TESTModeB_ON, PING_Or_TEST_LOOPModeB_ON){

f_EUTRA_CloseUE_TestLoopModeB (p_CellId, '00'O); // with no delay

}

}

// Step 12a1

if (p_ReleaseConnection == rrcConnectionRelease) { // @sic R5-237319 sic@

f_EUTRA_RRC_ConnectionRelease(p_CellId);

}

}

function f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(EUTRA_CellId_Type p_CellId,

integer p_ExpectedNoOfNewPDNs,

boolean p_FailWhenK_LessN := true,

integer p_SRB := tsc_SRB2,  // @sic R5s210341 sic@

template (omit) NasCount_Type p_NasCountUL := omit, // Only needed when called after TAU

B3_Type p_RequestType := '001'B,

template AccessPointName p_AccessPointName := cr_AccessPointName_Any ifpresent, // this is due to hardcoding of PDU session id in 11.1.2 @sic R5s240709 sic@

boolean p_N1disabled := false,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit ,

integer p_PDNReqAlreadyInDefault := 0

) runs on EUTRA_Multi5GS_PTC return integer

{ // Split so main function can be called after 1st PDN Conn Req has already been received @sic R5-231581 sic@

var B3_Type v_PDNType := f_GetPdnType();

var integer v_SRB;

var default v_MyDefaultVar := null;

if (p_PDNReqAlreadyInDefault == 0) {

// First clear store of previously received messages

f_EUTRA38_MsgInDefault_ResetStoreOfPDNConnectivityReq();

}

// Then activate store for any new msgs received

v_MyDefaultVar := activate(a_EUTRA38_AdditionalPDNConnectivityReq(p_CellId, cr_PDNConnReqAdditionalPDN(v_PDNType, p_AccessPointName, p_RequestType))); // @sic R5-216188 sic@

v_SRB := f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated(p_CellId,

p_ExpectedNoOfNewPDNs,

p_FailWhenK_LessN,

p_SRB,

p_NasCountUL,

p_RequestType,

p_AccessPointName,

p_N1disabled,

p_PduInfoFromNR);

// all expected msgs have been processed,  so deactivate the extra default

deactivate(v_MyDefaultVar);

return v_SRB;

}