3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250394

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3756 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_Check_MeasAndMobParametersCommon_supportedGapPattern |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currentl implementation will not take a proper format of SupportedGapPatternsPICS if config type is (NE_DC, NR_SA), because: v_SupportedGapPatternsFromPics := substr(v_SupportedGapPatternsPICS, 0, 22); is only inside the (NR_SA, NGEN_DC) “if” statement. (NR_SA, NGEN_DC) cases willl fail |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include v_SupportedGapPatternsFromPics := substr(v_SupportedGapPatternsPICS, 0, 22); Outside of if statement that both (NE_DC, NR_SA) and (NR_SA, NGEN_DC) will be cover |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1.NR5GC, 8.2.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_Check_MeasAndMobParametersCommon_supportedGapPattern |
| --- | --- |
| Reason for change | Currentl implementation will not take a proper format of SupportedGapPatternsPICS if config type is (NE_DC, NR_SA), because: v_SupportedGapPatternsFromPics := substr(v_SupportedGapPatternsPICS, 0, 22); is only inside the (NR_SA, NGEN_DC) “if” statement |
| Summary of change | Include v_SupportedGapPatternsFromPics := substr(v_SupportedGapPatternsPICS, 0, 22); Outside of if statement that both (NE_DC, NR_SA) and (NR_SA, NGEN_DC) will be cover |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5s230297 R5s230546 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var B22_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if (match(p_Configuration_Type, (NR_SA, NGEN_DC)) and f_NR_UESupportsFR2BandOnly(p_NRCapability)) // This check applies only to FR2 on NR SA or (NG)EN-DC { //The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2 if (not((pc_gp13_nr and pc_gp14_nr and pc_gp17_nr and pc_gp18_nr and pc_gp19_nr))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_gp13_nr, pc_gp14_nr, pc_gp17_nr, pc_gp18_nr or pc_gp19_nr not correctly set for a NR standalone capable UE that supports a band in FR2"); } v_SupportedGapPatternsFromPics := substr(v_SupportedGapPatternsPICS, 0, 22); if (not(match(v_SupportedGapPatternsFromPics, v_UE_supportedGapPattern))){ //safe match due to check with isvalue f_SetVerdict(inconc,__FILE__, __LINE__, "NR Measurement gap pattern PICS not set as per capabilities signalled by UE supporting FR2 Bands in a NR_SA network"); } } else if (match(p_Configuration_Type, (NE_DC, NR_SA))){ // This check applies to option 2 or option 4 if (not match(v_SupportedGapPatternsFromPics, substr(v_UE_supportedGapPattern, 0, 22))) { f_SetVerdict(inconc,__FILE__, __LINE__, "NR Measurement gap pattern PICS not set as per capabilities signalled by the UE"); } } } else if (bit2int(v_SupportedGapPatternsPICS) > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "NR Measurement gap pattern PICS are set while UE does not signal such capabilities"); } }; //end of fl_Check_MeasAndMobParametersCommon_supportedGapPattern

After Change:

function fl_Check_MeasAndMobParametersCommon_supportedGapPattern (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5s230297 R5s230546 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var B22_Type v_SupportedGapPatternsPICS := f_NR_DeriveSupportedGapsFromPics(); var B22_Type v_SupportedGapPatternsFromPics; var B22_Type v_UE_supportedGapPattern; v_Received_NRCapabilityMsg := p_NRCapability; v_SupportedGapPatternsFromPics := substr(v_SupportedGapPatternsPICS, 0, 22); if (isvalue(v_Received_NRCapabilityMsg.measAndMobParameters) and isvalue(v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern)) { // MeasAndMobParameters is OPTIONAL v_UE_supportedGapPattern := v_Received_NRCapabilityMsg.measAndMobParameters.measAndMobParametersCommon.supportedGapPattern; if (match(p_Configuration_Type, (NR_SA, NGEN_DC)) and f_NR_UESupportsFR2BandOnly(p_NRCapability)) // This check applies only to FR2 on NR SA or (NG)EN-DC { //The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2 if (not((pc_gp13_nr and pc_gp14_nr and pc_gp17_nr and pc_gp18_nr and pc_gp19_nr))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_gp13_nr, pc_gp14_nr, pc_gp17_nr, pc_gp18_nr or pc_gp19_nr not correctly set for a NR standalone capable UE that supports a band in FR2"); } if (not(match(v_SupportedGapPatternsFromPics, v_UE_supportedGapPattern))){ //safe match due to check with isvalue f_SetVerdict(inconc,__FILE__, __LINE__, "NR Measurement gap pattern PICS not set as per capabilities signalled by UE supporting FR2 Bands in a NR_SA network"); } } else if (match(p_Configuration_Type, (NE_DC, NR_SA))){ // This check applies to option 2 or option 4 if (not match(v_SupportedGapPatternsFromPics, substr(v_UE_supportedGapPattern, 0, 22))) { f_SetVerdict(inconc,__FILE__, __LINE__, "NR Measurement gap pattern PICS not set as per capabilities signalled by the UE"); } } } else if (bit2int(v_SupportedGapPatternsPICS) > 0) { f_SetVerdict(inconc,__FILE__, __LINE__, "NR Measurement gap pattern PICS are set while UE does not signal such capabilities"); } }; //end of fl_Check_MeasAndMobParametersCommon_supportedGapPattern