3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250516

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3785 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RRC NR NTN test case 8.1.5.14.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-12-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Due to the residual T1 timer (~2s) expiry at Step 26 and 2s wait at Step 26A, it can take more than 4 seconds between Step 25 and Step 29 considering the processing time of the other steps involved. So, SS RLC entity, which sent data with poll bit set to 1, will have to wait for more than 4 seconds for STATUS PDU as UL grant was stopped at Step 24 and was later to be resumed at Step 28. Since the value of t-PollRetransmit is ms80 and maxRetxThreshold is t8, SS RLC entity will wait for STATUS PDU till 640ms(80x8) only. This will cause test case to fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added TTCN code to handle the RRCReconfiguration procedure which is added in the prose to reconfigure the t-PollRetransmit to ms1000 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.14.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 CR xxxx |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Correction in f_TC_8_1_5_14_1_NR5GC_TestBody()	3

2.2	Addition of f_NR_UL_AM_RLC_8_1_5_14_1	4

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:  	Mohit Kanchan
	mohit.kanchan@keysight.com

- Corrections required

  - Correction in f_TC_8_1_5_14_1_NR5GC_TestBody()

| Template name | f_TC_8_1_5_14_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Due to the residual T1 timer (~2s) expiry at Step 26 and 2s wait at Step 26A, it can take more than 4 seconds between Step 25 and Step 29 considering the processing time of the other steps involved. So, SS RLC entity, which sent data with poll bit set to 1, will have to wait for more than 4 seconds for STATUS PDU as UL grant was stopped at Step 24 and was later to be resumed at Step 28. Since the value of t-PollRetransmit is ms80 and maxRetxThreshold is t8, SS RLC entity will wait for STATUS PDU till 640ms(80x8) only. This will cause test case to fail. |
| Summary of change | Added TTCN code to handle the RRCReconfiguration procedure which is added in the prose to reconfigure the t-PollRetransmit to ms1000 |
| TTCN module | RRC_Others_NTN_NR5GC.ttcn |
| MCCTF160 comment | Accepted conditional to prose CR agreement at RAN5#110 |

Before Change:

function f_TC_8_1_5_14_1_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-255092 R5s250232 sic@ … … //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); … … //@siclog "Step 22" siclog@ //SS stops broadcasting SIB19 f_NR_SetSysinfoCombinationToNR_1(nr_Cell1); //@sic R5s250232 sic@ f_NR_ModifySysinfo_SIB19(nr_Cell1,false); // WI = 1458048 //@siclog "Step 23" siclog@ //@sic R5-255092 R5s250232 sic@ //SS start a timer T1 with value = T430-(((Current SFN- epochsfn)*10)/1000) v_Timing := f_NR_GetCurrentTiming(nr_Cell1); v_T430fl := 20.0 - (int2float(v_Timing.SFN.Number * 10)) / 1000.0; //considering epoch time from SFN=0 and SIB modifiction // WI = 1458048 v_BeforeStep24 := v_T430fl - 2.0; t_T430.start(v_T430fl); // WI = 1458048 t_BeforeStep24.start(v_BeforeStep24); t_BeforeStep24.timeout; //@siclog "Step 24" siclog@ //SS stops UL periodic grant allocation f_NR_ULGrantConfiguration_Stop(nr_Cell1); … … }

After Change:

import from NR_RRC_Templates all; import from NR5GC_RRCSteps all; import from NR_CommonFunctions all; import from NR_ASP_TypeDefs all; ….. function f_TC_8_1_5_14_1_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-255092 R5s250232 sic@ … … var DRB_Identity v_DRBIdTemp; var template(omit) NR_RLC_BearerToAddModList_Type v_RLC_ConfigList:= omit; var template (omit) NR_RadioBearer_Type v_SS_Drb_Config:= omit; //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); … … //@siclog "Step 22" siclog@ //SS stops broadcasting SIB19 f_NR_SetSysinfoCombinationToNR_1(nr_Cell1); //@sic R5s250232 sic@ f_NR_ModifySysinfo_SIB19(nr_Cell1,false); // WI = 1458048 //@siclog "Step 22A" siclog@ v_DRBIdTemp := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_Config := f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBIdTemp,-, -, -,-,-,ms1000); v_RLC_ConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBIdTemp, cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC_8_1_5_14_1(f_NR_CellInfo_GetIsFR1(nr_Cell1)),f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_38508_LogicalChannelConfig_DRB(-,-,-,true) ); f_NR_RRCReconfigExistingDRBs(nr_Cell1,omit,v_RLC_ConfigList,-,-,cs_TimingInfo_Now,{v_SS_Drb_Config}); //@siclog "Step 23" siclog@ //@sic R5-255092 R5s250232 sic@ //SS start a timer T1 with value = T430-(((Current SFN- epochsfn)*10)/1000) v_Timing := f_NR_GetCurrentTiming(nr_Cell1); v_T430fl := 20.0 - (int2float(v_Timing.SFN.Number * 10)) / 1000.0; //considering epoch time from SFN=0 and SIB modifiction // WI = 1458048 v_BeforeStep24 := v_T430fl - 2.0; t_T430.start(v_T430fl); // WI = 1458048 t_BeforeStep24.start(v_BeforeStep24); t_BeforeStep24.timeout; //@siclog "Step 24" siclog@ //SS stops UL periodic grant allocation f_NR_ULGrantConfiguration_Stop(nr_Cell1); … … }

  - Addition of f_NR_UL_AM_RLC_8_1_5_14_1

| Template name | f_NR_UL_AM_RLC_8_1_5_14_1 |
| --- | --- |
| Reason for change | Please refer to change 2.1 |
| Summary of change | Please refer to change 2.1 |
| TTCN module | NR_RRC_Templates.ttcn |
| MCCTF160 comment | Accepted conditional to prose CR agreement at RAN5#110 |

Change:

function f_NR_UL_AM_RLC_8_1_5_14_1(boolean p_IsFR1) return template (value) UL_AM_RLC { var T_PollRetransmit v_T_PollRetransmit; if (p_IsFR1) { v_T_PollRetransmit := ms1000; } else { v_T_PollRetransmit := ms30; } return cs_38508_UL_AM_RLC(v_T_PollRetransmit); }