# Positioning enhancements for GERAN - Workplan/Agreements

# Introduction

At RAN#73 a new Work Item called Positioning Enhancements for GERAN was approved, see [1].

This document is intended to capture the work plan for the Work Item, reached agreements and working assumptions as well as conclusions from the initial study phase.  The document contains the status as of closing of RAN6#1.

# Work Plan

| Meeting | Plan | Outcome |
| --- | --- | --- |
| RAN#72 13th -16th June, 2016 Busan, South Korea | - Approval of work item | Work Item approved including an initial study phase with objectives to: study the performance of proposed positioning enhancements, including: - evaluate the accuracy of proposed positioning enhancements such as TA multilateration, with and without tighter assessment of timing by MS, and compare with existing positioning methods with the same hardware support as for the proposed enhancement. - procedures and signalling for the proposed positioning enhancements - evaluate suitability of the proposed positioning enhancements for EC-GSM-IoT, including impacts on MS complexity and power consumption. |
| RAN6#1 22nd – 26th Aug, 2016 Gothenburg, Sweden | - Complete initial study phase including identification of candidate positioning enhancements for further specification work. | At the closing of RAN6#1 meeting the following conclusions with respect to the initial study phase objectives were reached: - To continue with specification work of the Timing Advance (TA) Multilateration method as simulations have shown that the TA Multilateration method provide considerable accuracy gains over the Cell ID+TA method (existing positioning method with the same hardware support), even without tightening the current MS synchronization requirement. A targeted 100 m accuracy was shown to be reached by the Cell ID+TA method in 6% of positioning attempts, while for TA Multilateration between 20-40% (depending on number of cells used for positioning). - Further positioning accuracy improvements (60-95% @ 100m) have been shown by simulations if a tighter assessment of timing by MS, together with similar synchronization accuracy at the base station. - Without RAN signaling optimizations the energy consumption of a positioning procedure has been found acceptable, comparing roughly to a CC1 mobile report of 400 bytes. If also RAN signaling optimizations are considered the energy consumption has been shown to be lowered by 50%. - To consider Hybrid TA/OTD Multilateration for possible specification work as analytical evaluations of the Hybrid TA/OTD Multilateration method has shown that the positioning error will increase with a factor <2, compared to the TA Multilateration method, while at the same time providing energy efficiency gains. - To further investigate MS complexity impacts and tightening of MS synchronization requirement. |
| RAN#73 19th -22nd September, 2016 New Orleans, USA | - Presentation of status report and approval to continue with specification work. Initial study phase completed. | - Status report approved and initial study phase completed. |
| RAN6#2 14nd – 18h Nov, 2016 Reno, USA | - Candidate solution(s) selected for further specification work. - Agreement on stage 2 CRs. |  |
| RAN#74 5th -8thd December, 2016 Vienna, Austria | - Presentation of status report and stage 2 CRs approved |  |
| RAN6#3 13thd – 17th Feb, 2017 Athens, Greece | - Agreement on stage 3 CRs for identified positioning enhancements. |  |
| RAN#75 6th -9thd March, 2017 Dubrovnik, Croatia | - Approval of CR set to introduce the identified positioning enhancements - Completion of work item. |  |

# Agreements and working assumptions

| Area | WA# | Working assumption/agreement | Status | Comment |
| --- | --- | --- | --- | --- |
| - General | 1.1 | Candidate solutions will only be introduced as a general positioning enhancement over the PS domain – CS domain is excluded. | Open |  |
|  | 1.2 | The evaluations for positioning accuracy shall focus on the stationary mobile use case; worse accuracy is acceptable for moving devices. | Agreed |  |
| - Simulations | 2.1 | For link and system level simulations, re-use the assumptions from 3GPP TR 45.820 with the limitation/exceptions that: - For system simulations: - Use building penetration loss model 1 and inter-site correlation coefficient 0.5 - No specific traffic model added in the network, but instead a sufficient number of synchronization attempts are performed. - For link simulations: - Use EPA propagation model | Agreed |  |
|  | 2.2 | The simulations results are presented: - With the used positioning method declared - With a model presented of the synchronization accuracy assumptions for the MS and BTS - With the maximum number of base stations used for positioning declared - With the percent of users out of coverage declared - As percentile of positioning accuracy of 100 m or better | Agreed |  |
| - Energy evaluations | 3.1 | Candidate solutions are compared on a single positioning event basis. | Agreed |  |
|  | 3.2 | Energy evaluations are performed assuming an EC-GSM-IoT device in CC1 except for the TA establishment procedure where one of the neighbor cells shall be assumed to be in CC2. | Agreed |  |
|  | 3.3 | Evaluations should be done assuming an operating voltage of 3.3 V, an operating current of 1.23 A and 0.03 A at transmission and reception, respectively | Agreed |  |
|  | 3.4 | Due to only marginal impact on the total energy contribution the energy consumption in light and deep sleep can be ignored. | Agreed |  |
| - Signaling and procedures | 4.1 | PS LCS CAPABILITY modified to indicate when a UE supports Multilateration procedures. | Agreed |  |
|  | 4.2 | All SMLC to UE communications shall be supported using RRLP messages. | Agreed |  |
|  | 4.3 | Use the existing RRLP TOM protocol discriminator to support the transmission of RRLP messages containing Multilateration information. | Agreed |  |
|  | 4.4 | The selected Multilateration signalling procedure (TBD) shall only be applicable to the PS domain.. | Agreed |  |
|  | 4.5 | Further enhancements to the positioning procedure without impacting the security aspects needs to be considered to improve the energy efficiency of the procedure without impacting the security aspect of the procedure. | Agreed |  |

# Initial study phase conclusions

At the closing of RAN6#1 meeting the following conclusions with respect to the initial study phase objectives were reached:

- To continue with specification work of the Timing Advance (TA) Multilateration method as simulations have shown that the TA Multilateration method provide considerable accuracy gains over the Cell ID+TA method (existing positioning method with the same hardware support), even without tightening the current MS synchronization requirement. A targeted 100 m accuracy was shown to be reached by the Cell ID+TA method in 6% of positioning attempts, while for TA Multilateration between 20-40% (depending on number of cells used for positioning).

- Further positioning accuracy improvements (60-95% @ 100m) have been shown by simulations if a tighter assessment of timing by MS, together with similar synchronization accuracy at the base station.

- Without RAN signaling optimizations the energy consumption of a positioning procedure has been found acceptable, comparing roughly to a CC1 mobile report of 400 bytes. If also RAN signaling optimizations are considered the energy consumption has been shown to be lowered by 50%.

- To consider Hybrid TA/OTD Multilateration for possible specification work as analytical evaluations of the Hybrid TA/OTD Multilateration method has shown that the positioning error will increase with a factor  <2, compared to the TA Multilateration method, while at the same time providing energy efficiency gains.

- To further investigate MS complexity impacts and tightening of MS synchronization requirement.

# Conclusion

The document contains the Work Plan for ePOS_GERAN, working assumptions/agreements as well as Initial study phase agreements as of close of RAN6#1 meeting.

# References

- RP-161260, “New Work Item on Positioning Enhancements for GERAN”, source Ericsson LM, Orange, MediaTek Inc., Sierra Wireless, Nokia. RAN#72.