# Enhanced Positioning – Positioning performance evaluation

# Introduction

At RAN#72 a Work item on “Positioning Enhancements for GERAN” was approved. Part of the WI scope is until RAN#73 to:

- “study the performance of proposed positioning enhancements, including:

  - evaluate the accuracy of proposed positioning enhancements such as TA multilateration, with and without tighter assessment of timing by MS, and compare with existing positioning methods with the same hardware support as for the proposed enhancement.

  - procedures and signalling for the proposed positioning enhancements

  - evaluate suitability of the proposed positioning enhancements for EC-GSM-IoT, including impacts on MS complexity and power consumption”

This document focuses on the first bullet above, by using the mentioned TA multilateration method. The positioning performance is evaluated by system simulations. A SINR-dependent model of the measurement error is introduced, see [6]. Also, the impact of network guidance that allows the MS to avoid using co-sited base stations for positioning is evaluated.

# TA multilateration

## General

Detailed information regarding TA multilateration can be found in [2]. Here a short recap is provided.

The basic idea is based on extending the current Cell ID + timing advance method by performing a multilateration based on timing advance values. For this to work the timing advance value from at least three neighboring base stations are needed.

The base stations to be used for multilateration could be chosen either:

- Based on selection from the network by knowing the position of the base stations and choosing a set of base stations with favorable geometry,

- Based on MS selection, selecting the strongest cells visible to the MS, or,

- Based on MS selection with NW guidance, selecting the strongest cells visible to the MS while avoiding co-sited cells.

In this paper these three methods are referred to as “NW assisted selection”, “MS autonomous selection” and “MS autonomous selection with NW guidance” respectively.

The accuracy of the TA multilateration will depend on the number of base stations used, the placement of the base stations, the accuracy of the TA estimates, and the algorithm used to derive the position. The

Figure 1: Illustration of timing advance trilateration method

## TA accuracy

A model of the MS-BTS distance estimation error has been derived from link simulations in [6]:

- The MS synchronization error eMS [GSM symbol periods] is modelled as the average of N independent Laplace distributed random numbers, where N=5.

- The BTS synchronization error eBTS [GSM symbol periods] is modelled either (in case only access bursts are sent on EC-RACH) as a Laplace distributed random number, or (in case a single block is transmitted on EC-PDTCH in addition to the access bursts on EC-RACH) as the average of two Laplace distributed random numbers.

- From this the MS-BTS distance estimation error is calculated as  ½(eMS+eBTS)CT, where C is the speed of light [m/s] and T is the GSM symbol period [s].

The standard deviation of the Laplace distributions depends on the SINR and the logical channel used. For more details, see [6].

For comparison, the uniform error distributions used in the contribution to RAN6#1 [5] have also been simulated.

## NW assisted selection

The placement of the base stations will be chosen based on the best geometry to the serving cell.

Assume that only three base stations are to be used for multilateration (in this case trilateration), then one of the base stations is assumed to always be the serving cell. The selection of the two remaining cells is based on the ideal angle from the serving cell. Since the network does not know where the MS is located the MS position is assumed to be according to the measured TA value in a direction of the main lobe of the cell.

The selection process can be detailed in the following steps (and is also illustrated in Figure 2):

- Get TA for serving cell [blue square]

- Estimate MS position to be in the direction of the main lobe of the serving cell according to the measured TA [red diamond]

- Out of all cells visible to the MS (within the MCL) [13 green lines; one to each base station] select the one closest to 2/3 [the purple line pointing to the left]

- Out of the remaining visible cells, select the one closest to 4/3 [the purple line pointing downwards]

Hence, no consideration is taken to the SINR experienced by the MS in this selection, but could probably improve the location estimation if done properly (there will be a trade-off between good geometry of the selected cells, and the accuracy provided by the TA estimate). If less than three cells are visible to the MS, the location is based on cell ID and TA see Section 2.6.

Figure 2: NW assisted cell selection

## MS autonomous selection

No consideration is with this method taken to the geometry of the cells, and it is assumed that the MS will select base stations based on descending signal strength. If less than three cells are visible to the MS, the location is based on cell ID and TA see Section 2.6.

## MS autonomous selection with NW guidance

This method is the same as the MS autonomous selection, except that only the strongest of a set of cells with co-located base stations may be selected.

## Reference selection

The reference selection method in the simulations is based on the TA + cell ID method. This means that the TA is estimated in the serving cell and the MS position is determined to be the distance of the estimated TA in the direction of the main lobe in the cell.

## Multilateration method

The method used for the multilateration was Gauss-Newton.

# Simulation assumptions

The simulation assumptions in [4] have been followed.

GSM specific assumptions are shown in Table 1.

Table 1: GSM specific simulation assumptions

| Parameter | Value |
| --- | --- |
| MS synchronization accuracy | - SINR-dependent model (see [6]) based on average of 5 EC-SCH readings - Uniform -1/2 to +1/2 symbol - Uniform -1/4 to +1/4 symbol - Uniform -1/8 to +1/8 symbol |
| BTS synchronization accuracy | - SINR-dependent model (see [6]) based on average of 1 EC-RACH and 1 EC-PDTCH reading - Uniform -1/2 to +1/2 symbol - Uniform -1/4 to +1/4 symbol - Uniform -1/8 to +1/8 symbol |
| MCL | 164 dB (33 dBm EC-GSM-IoT devices) |
| Building penetration loss | On (see [4], Model 1, 0.5 inter-site correlation) |
| Maximum number of base stations used for multilateration | 3, 5 or 10 |

The “maximum number of base stations” in the table above is an upper limit on the number of base stations for which TA estimation is done. The actual number may be lower in case fewer are visible (i.e., above the MCL of 164 dB) to the MS.

# Simulation results

## Comparison of link simulation based model and uniform models

In the contribution to RAN6#1 [5], uniform distributions of the MS-BTS distance estimation error were used. The positioning performance of the new SINR-dependent model is compared to the uniform models in Figure 3.

The MS autonomous method with maximum three base stations has been used.

It can be seen that the SINR-dependent model is slightly better than the uniform model with -1/8 to +1/8 symbol range. This shows that it is important to model the SINR dependency rather than to assume a worst case spread.

Figure 3: CDFs of the positioning error with different models for the MS-BTS distance estimation error. MS autonomous method using 3 base stations.

## Comparison of methods for base station selection

In [5] the “MS autonomous selection” and the “NW assisted selection” were simulated with a SINR-independent uniform measurement error model. The results were in favour of the “NW assisted selection”. However, since the “NW assisted selection” may select a BTS with lower SINR over a BTS with higher SINR in order to improve the geometry, the SINR dependency of the measurement error is important to model.

Figure 4 shows the positioning error CDFs of the “MS autonomous selection” (blue line) and the “NW assisted selection” (red line) with the SINR-dependent measurement error model when the maximum number of base stations used is three. It is now less obvious which of the two is better. The green line shows the performance of the “MS autonomous selection with NW guidance” that seems to outperform the other two.

Figure 4: CDFs of the positioning error with different methods for selecting base stations. Maximum number of base stations used is 3.

In contrast to this, Figure 5 shows the same CDFs but for the case where up to 10 base stations may be used for positioning. In this case, the “MS autonomous selection” is the best. Avoiding co-sited base stations (i.e., the “MS autonomous selection with NW guidance”) degrades performance due to the possibility that base stations with lower SNR could be used instead of using multiple co-sited base stations with higher SNR.

Figure 5: CDFs of the positioning error with different methods for selecting base stations. Maximum number of base stations used is 10.

When only three base stations are used, the situation that some of them are co-sited gives an under-determined problem. Therefore, avoiding co-sited base stations improves performance. On the other hand, when 10 base stations are used, the problem is over-determined even if some of them are co-sited. Replacing these co-sited base stations with other base stations with lower SNR will only degrade performance.

## Impact of number of used base stations

In this section the positioning accuracy impact of number of used base stations is investigated. The best method for selecting base stations is identified for the case of 3, 5 and 10 base stations (i.e., the “MS autonomous selection with NW guidance” is the best method when three base stations are used and the “MS autonomous selection” otherwise).

The positioning performance CDFs are shown in Figure 6. It can be noted that when NW guidance is used in the case of three base stations, the gain of increasing the number of base stations is moderate.

Figure 6: Impact of maximum number of used base stations.

# Conclusion

This paper has investigated the positioning accuracy for three different timing advance Multilateration methods: Network assisted, MS autonomous and MS autonomous with network guidance. It has been shown that the MS autonomous with network guidance method has the best performance when three base stations are used for the positioning procedure and that the gain of increasing the number of base stations is moderate. In addition, the lower battery consumption realized by the MS when using only three base station per positioning event is a significant advantage compared to using five or ten base stations. It is therefore proposed to agree a working assumption to only pursue the MS autonomous with network guidance in further specification work.

Working assumption 1:  Based on the fact the MS autonomous with network guidance has the best performance when three base stations are used in the positioning and that the gains are moderate when increasing the number of base stations, it is proposed to only pursue this method in further specification work.

Another aspect that has been shown in this paper is that it is important to correctly model the TA estimation error and use a SINR-dependent model in order to obtain reliable results. Therefore, is it proposed to agree the following working assumption.

Working assumption 2: When modelling the TA estimation error in system level simulations an SINR dependent model is used.

Another aspect that is important to note is that with the assumptions taken in this paper as well as in the supporting link level simulations paper [6] that a very good positioning accuracy of 50 m at the 67% can be achieved using three base stations.  One key contributing factor to this accuracy is the fact that the MS averages a set of 5 EC-SCH readings to achieve a good synchronization accuracy and but it can also be noted that the synchronization accuracy is SINR dependent.  This means that for the SMLC node to be able to more accurately estimate the overall positioning error it would be advantageous for the SMLC node not only assume the worst case synchronization estimation error but the actual synchronization error as estimated by the MS.  It is therefore also proposed to agree the following working assumption.

Working assumption 3: The MS shall report its estimated synchronization accuracy to the network in the RLC data block part of the timing advance estimation procedure.

# References

RP-161260, “New Work Item on Positioning Enhancements for GERAN”, source Ericsson LM, Orange, MediaTek Inc., Sierra Wireless, Nokia. RAN#72.

RP-161034, “Positioning enhancements for GERAN – introducing Timing Advance trilateration”, source Ericsson LM. RAN#72

3GPP TS 45.010, “Radio subsystem synchronization”

R6-160087, “System level simulations for positioning enhancements - Assumptions”, source Ericsson LM.

R6-160012, “System level simulations for positioning enhancements – Methods and Results”, source Ericsson LM

R6-160157, “Enhanced Positioning – Link level evaluations and performance”, source Ericsson LM