3GPP TSG RAN WG6 #2  	R6-160179

# Reno, Nevada, USA, November 14-18, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.001 | CR | 0092 | rev | - | Current version: | 13.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to EC-GSM-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM-Core |  |  |  |  |  |  | Date: |  | 2016-11-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - The usage of TS3 to TS7 on EC-RACH is incorrect. - Frequency Hopping is not permitted on CCCH channels as specified in TS 45.002 Table 3, where PCH, AGCH, RACH and NCH are mandated to use C0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 5.2a.4 the usage of Training sequences on EC-RACH is corrected to apply to TS3, TS5, TS6 and TS7 only. - In clause 6 the requirement for non-hopping channels is corrected to apply also for CCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in the same specification and to other specifications exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2a.4, 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

### 5.2a.4	EC-GSM-IoT

For EC operation, an EC-CCCH/D block designated to MSs in the lower coverage class (CC1) only shall use a TSC from the TSC Set 1 for GMSK corresponding to the BCC (see 3GPP TS 45.002) and EC-CCCH/D blocks designated to at least one MS in a higher coverage class (CC2 to CC4) shall use the paired TSC with the same TSC number from the TSC Set 2 for GMSK (see 3GPP TS 45.002).

For EC-RACH a set of four synchronization sequences is defined, denoted TS3, TS5, TS6 and TS7. Different synchronization sequences are used to indicate the UL CC selected to assist the BTS in detecting the MS access and separate accesses from different CCs. TS3 is used for CC1, TS5 is used for CC2, TS6 is used for CC3 and TS7 is used for CC4.

2nd modification

# 6	Frequency hopping capability

The frequency hopping capability is optionally used by the network operator on all or part of its network. The main advantage of this feature is to provide diversity on one transmission link (especially to increase the efficiency of coding and interleaving for slowly moving mobile stations) and also to average the quality on all the communications through interferers diversity. It is implemented on all mobile stations.

The principle of slow frequency hopping is that every mobile transmits its time slots according to a sequence of frequencies that it derives from an algorithm. The frequency hopping occurs between time slots and, therefore, a mobile station transmits (or receives) on a fixed frequency during one time slot ( 577 µs) and then must hop before the time slot on the next TDMA frame. Due to the time needed for monitoring other base stations the time allowed for hopping is approximately 1 ms, according to the receiver implementation. The receive and transmit frequencies are always duplex frequencies.

The frequency hopping sequences are orthogonal inside one cell (i.e. no collisions occur between communications of the same cell), and independent from one cell to an homologue cell (i.e. using the same set of RF channels, or cell allocation). The hopping sequence is derived by the mobile from parameters broadcast at the channel assignment, namely, the mobile allocation (set of frequencies on which to hop), the hopping sequence number of the cell (which allows different sequences on homologue cells) and the index offset (to distinguish the different mobiles of the cell using the same mobile allocation). The non-hopping case is included in the algorithm as a special case. The different parameters needed and the algorithm are specified in 3GPP TS 45.002.

In case of multi band operation frequency hopping channels in different bands of operation, e.g. between channels in GSM and DCS, is not supported. Frequency hopping within each of the bands supported shall be implemented in the mobile station.

It must be noted that the basic physical channels supporting any of the BCCH, CCCH, EC-BCCH and EC-CCCH do not hop.

For COMPACT, frequency hopping is not permitted on CPBCCH or CPCCCH for a specific amount of blocks. On other frequency hopping channels, a reduced mobile allocation is used on the corresponding blocks.

In CTS, the frequency hopping capability shall be used. The frequency hopping sequences are independently chosen by each CTS-FP. The hopping sequence is derived by the CTS-MS from parameters transmitted during the attachment procedure. The different parameters needed and the algorithm are specified in 3GPP TS 45.002. It must be noted that the basic physical channels supporting the CTSBCH and some other particular channels do not hop (see 3GPP TS  45.002).

End of modifications