3GPP TSG RAN WG6 Meeting #02		R6-160251

Reno, USA, 14 – 18 November 2016	Agenda Item: 5

3GPP TSG RAN WG2 Meeting #96		R2-169085

Reno, USA, 14 – 18 November 2016

Title:	Response LS on Enhanced Coverage authorization impact on cell and PLMN selection procedures

Response to:	LS R2-167426/S2-166286 & R2-167403/C1-164703 on Enhanced Coverage authorization impact on cell and PLMN selection procedures from SA2 and CT1.

Release:	Release 14

Work Item:	FS_CIoT_ext

Source:	RAN2

To:	SA2, CT1

Cc:	RAN6

Contact person:	Mungal Dhanda

mdhanda (at) qti (dot) qualcomm (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN2 thanks SA2 and CT1 on their LSes on Enhanced Coverage authorization impact on cell and PLMN selection procedures.

RAN2 has discussed Enhanced Coverage authorization for eMTC UEs and LTE UEs that support CE and reached the following agreements.

0:	The authorisation determines whether the UE is allowed to use the enhanced cell selection criteria or not.

1: 	UE not authorized to use coverage enhancements shall consider a cell that can only be camped in normal service in CE mode as not suitable. UE can camp in limited service.

2	The existing cell re-selection procedures and rules are used with authorization of coverage enhancements.

3	RAN2 informs CT1 about the above RAN2 agreements, and that AS needs to know from NAS whether coverage enhancements is authorized. RAN2 have not discussed PLMN selection and leave that to CT1.

4	Authorization of coverage enhancements will not impact specifications for connected mode mobility (i.e. left to network implementation). In case Idle mode procedures apply in connected mode (e.g. RRC re-establishment) the Idle mode procedure handling of authorization of coverage enhancements applies.

RAN2 also discussed Enhanced Coverage authorization for NB-IoT and reached the following agreement.

It is applicable to NB-IoT

RAN2 only modify the suitability threshold that the UE takes into account for the additional coverage enhancement level, where it is assumed that NAS provides an on/off indication to the UE.

Note: For NB-IoT currently there is no coverage enhancement level with respect to suitability thresholds

# 2	Actions

To 3GPP SA2 and CT1

ACTION: 	RAN2 asks SA2 and CT1 to take the above information into account.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #97	13-17 February 2017, Athens, Greece

TSG RAN WG2 Meeting #97bis	03-07 April 2017, TBD, USA