3GPP TSG RAN WG6 #2  	R6-160266

# Reno, Nevada, USA, November 14-18, 2016                                Revision of R6-160246

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.002 | CR | 0203 | rev | 2 | Current version: | 13.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Alternative Mappings for Higher Coverage Classes with 2 PDCHs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Core |  |  |  |  |  |  | Date: |  | 2016-11-04 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of alternative blind physical layer mappings for dedicated channels (EC-PDTCH and EC-PACCH) on DL/UL with 2 PDCH resources has been agreed as part of the Rel-14 work item on Radio Interface Enhancements for EC-GSM-IoT in RP-161806. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclauses 6.3.2.1, 6.3.2.2.1.1 and 6.3.2.3.1 the alternative mappings of logical channels to physical channels for higher coverage classes using 2 PDCH operation for uplink and downlink are added. - In subclause 6.3.2.2.6 changes to Overlaid CDMA applied for EC-PDTCH/EC-PACCH channels with 2 PDCH assigned in EC operation are added. - In subclause 6.4, Note 18, it is specified that the channel configurations for EC-PDTCH are also applicable to the new alternative mappings for higher coverage classes using the EC-PDTCH/TS logical channel type. - In clause 7 mapping table 6a for EC channels is updated to include the alternative mappings for higher coverage classes. Figure 9c is added to reflect the mapping for EC-PDTCH/2TS for CC2, CC3 and CC4 onto the 52-multiframe, with one RLC block in CC4 being mapped onto 2 51-multifarmes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Alternative coverage class mappings with 2 PDCHs will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1, 6.3.2.2.1.1, 6.3.2.2.6, 6.3.2.3.1, 6.4, 7 (table 6a, figure 9b, figure 9c) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064, TS 45.001, TS 45.003, TS 45.005, TS 45.008, TS 44.018, TS 44.060 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010, TS 51.021 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

### 6.3.2	Mapping in time of packet logical channels onto physical channels

#### 6.3.2.1	General

A physical channel allocated to carry packet logical channels is called a packet data channel (PDCH). A PDCH shall carry packet logical channels only.

In RTTI configuration, physical channels are paired, forming PDCH-pairs. The two physical channels shall have the same parameters (see subclause 5.6.3) except for the timeslot number (TN). The two PDCHs constituting a PDCH-pair shall be located on the same carrier. The two PDCHs constituting a PDCH-pair need not be contiguous. In each direction, physical channels shall be assigned so that PDCH-pairs do not partially overlap.

On a given PDCH, PDTCHs in both BTTI configuration and RTTI configuration (assigned to different MSs) may be carried. Alternatively, both PDCHs forming a PDCH-pair may be assigned to only carry PDTCHs in RTTI configuration.

Packet switched logical channels are mapped dynamically onto a 52-multiframe.

-	For a PDCH/F in BTTI configuration the 52-multiframe consists of 12 blocks of 4 consecutive frames, 2 idle frames and 2 frames used for the PTCCH (see 3GPP TS 45.010), as shown in Figure 9. Table 6 in clause 7, indicates the frame numbers for each of the blocks (B0...B11) transmitted in the multiframe. The ordered list of block is defined as B0, B6, B3, B9, B1, B7, B4, B10, B2, B8, B5, B11.

-	For PDCH/H, the 52-multiframe consists of 6 blocks of 4 frames each, and two idle frames.  Table 6 in clause 7 indicates the frame numbers for each of the blocks (B0…B5) transmitted in the multiframe.

-	For a PDCH-pair in RTTI configuration the 52-multiframe consists of 24 RTTI blocks of 2 consecutive frames, plus 2 idle frames and 2 frames used for the PTCCH (see 3GPP TS 45.010) on each PDCH of the PDCH-pair, as shown in Figure 9a. Table 6 in clause 7 indicates the frame numbers for each of the 24 RTTI blocks (B0a, B0b, ...B11a, B11b) transmitted in the 52-multiframe.

-	For a PDCH/F, when in EC operation, the 52-multiframe consists of 12 BTTI blocks of 4 consecutive frames, and 4 idle frames, as shown in Figure 9b. In case blind physical layer transmissions are used, multiple BTTI blocks constitute the block that is mapped onto the physical channel, see Figure 9b. Table 6a in clause 7 indicates the frame numbers of each of the blocks depending on the downlink Coverage Class or uplink Coverage Class respectively.

-	Depending on the intended coverage range, EC-PDTCH/F and EC-PACCH/F are defined for different Coverage Classes (CCs), see 3GPP TS 43.064. Depending on the CC and the number of PDCHs assigned for EC operation (see 3GPP 44.018 and 3GPP 44.060) one EC-PDTCH/F and/or EC-PACCH/F are mapped onto different number of PDCHs according to:

-	CC1: 				1 PDCH

-	CC2, CC3, CC4: 4 PDCHs or 2 PDCHs

The PDCHs that an EC-PDTCH/F and/or EC-PACCH/F are mapped to are contiguous, see Table 6a in clause 7.

A block allocated to a given logical channel comprises one radio block, or in the case of uplink only, 4 random access bursts.  The type of channel may vary on a block-by-block basis. In case of EC operation a block allocated to an EC-channel can consist of blind physical layer transmissions of a radio block. The number of blind physical layer transmissions is dependent on the assigned Coverage Class.

In the downlink direction, the logical channel type shall be indicated by the message type contained in the block header part for GPRS, or indicated by Stealing Flags for EGPRS and EGPRS2, or detected by the MS in case of EC operation.

In the uplink part for channels other than (EC-)PACCH transmitted as access bursts or PRACH, or CPRACH or (EC-)RACH, the logical channel type shall be indicated by the message type contained in the block header part for GPRS, or indicated by Stealing Flags for EGPRS and EGPRS2, or known by the BTS in case of EC operation. For (EC-)PACCH transmitted as access bursts, the logical channel type is indicated by the corresponding polling message on the downlink (see 3GPP TS 44.060). For the PRACH or CPRACH case the logical channel type is indicated by the USF (see 3GPP TS 44.060), set on the downlink on a block by block basis.

For COMPACT, timeslot mapping and rotation of the control channels is used such that control channels belonging to a serving time group are rotated over odd timeslot numbers as follows: 7, 5, 3, 1, 7, 5  . The rotation occurs between frame numbers (FN) mod 52 = 3 and 4. The mapping of the control channels on timeslot numbers is defined by the following formula:

-	for 0  FN mod 52  3, TN = ((6 x ((FN div 52) mod 4)) + 1 + (2 x TG)) mod 8;

-	for 4  FN mod 52  51, TN = ((6 x ((FN div 52) mod 4)) + 7 + (2 x TG)) mod 8.

Packet switched logical channels PDTCH, PACCH, and PTCCH are never rotated.

2nd modification

#### 6.3.2.2	Mapping of the uplink channels

##### 6.3.2.2.1	Mapping of (EC-)PDTCH/U and (EC-)PACCH/U

###### 6.3.2.2.1.1	BTTI configuration

The PDCH's where the MS may expect occurrence of its (EC-)PDTCH/U(s) or (EC-)PACCH/U for a mobile originated transfer is indicated in resource assignment messages (see 3GPP TS 44.060 and 3GPP 44.018). (EC-)PACCH/U shall be allocated respecting the resources assigned to the MS and the MS multislot class. For each PDCH assigned to the MS, an Uplink State Flag (R0... R7) is given to the MS. An exception is when using Fixed Uplink Allocation where the Uplink State Flag is not used to allocate resources, and hence no Uplink State Flag is given to the MS.

In case of USF based allocation, the occurrence of the PDTCH/U and/or the PACCH/U at given block(s) Bx (where Bx = B0...Bn; n=5 for the PDTCH/HU and n=11 for the PDTCH/FU) in the 52-multiframe structure for a given MS on a given PDCH shall be indicated by the value of the Uplink State Flag (USF) contained in the header of the preceding block transmitted in the downlink of the same PDCH (or in the case of shifted USF on the downlink of a PDCH with a relationship to the uplink PDCH as defined in 3GPP TS 44.060), that is to say B(x-1) in the same multiframe if x1 or B(n) in the previous multiframe if x=0. If the USF in block B(x-1) indicates that block B(x) shall be used by an MS for which the USF_GRANULARITY is set to 1 (corresponding to 4 blocks) in the last assignment message, that MS shall also use the three following blocks. The USF corresponding to the last three blocks shall be set to an unused value. The MS may transmit a PDTCH block or a PACCH block on any of the uplink blocks used by the MS. The occurrence of the PACCH/U associated to a PDTCH/D shall be indicated by the network by polling the MS (see 3GPP TS 44.060).

In case of Fixed Uplink Allocation, the occurrence of the EC-PDTCH/U at given block(s) in the 52-multiframe structure for a given MS on given PDCH(s) shall be indicated by an EC Immediate Assignment Type 1, EC-Immediate Assignment Type 2, EC Packet Uplink Ack/Nack or EC Packet Uplink Assignment message. The number of blocks available per 52-multiframe depends on the uplink Coverage Class and the number of PDCHs assigned for EC operation as shown in table 6.3-3. The mapping of each block onto the 52-multiframe is shown in table 6a, and illustrated in figure 9b and figure 9c.

Table 6.3-3: Blocks per 52-multiframe depending on uplink Coverage Class for EC-GSM-IoT

| Uplink Coverage Class | Number of PDCH assigned |  | Number of blocks per 52-multiframe |
| --- | --- | --- | --- |
| 1 | 1 |  | 12 |
| 2 | 4 |  | 12 |
| 3 | 4 |  | 6 |
| 4 | 4 |  | 3 |
| 2 | 2 | 6 |
| 3 | 2 | 3 |
| 4 | 2 | 3/21 |
| Note 1: This corresponds to 3 blocks per 2*52-multiframes. |  |  |

The occurrence of the EC-PACCH/U associated with EC-PDTCH/D shall be indicated by the network by polling the MS (see 3GPP TS 44.060).

NOTE 1:	This subclause specifies how the network shall signal that the MS is allowed to use the uplink. The operation of the MS is specified in 3GPP TS 44.060. In particular cases of extended dynamic allocation or exclusive allocation, the MS may not need to monitor the USF on all the downlink timeslots corresponding to the assigned uplink PDCHs. In case of Fixed Uplink Allocation the MS shall not monitor the USF.

NOTE 2:	The PDCH/HU is only assigned in exclusive allocation (see 3GPP TS 44.060).

NOTE 3: 	A MS using packet uplink traffic channels mapped to the same physical channel than an uplink PCCCH in extended dynamic allocation MAC mode is not required to check if allocated uplink PDTCH/U or PACCH/U blocks also belong to the PRACH.

In a dual carrier or a DLMC configuration, the uplink block(s) shall be allocated on the corresponding physical channel on which the USF or poll (see 3GPP TS 44.060) is received. Uplink blocks shall not be allocated on physical channels having different frequency domain descriptions (see subclause 5.4) simultaneously in the same block period.

For COMPACT, USF_GRANULARITY should be set to 0 (corresponding to 1 block) for dynamic allocation for the following cases:

i)	for odd timeslot numbers (TN) 1, 3, 5, and 7 in nominal and large cells;

ii)	for even timeslot numbers (TN) 0, 2, 4, and 6 in large cells.

3rd modification

##### 6.3.2.2.6	Mapping of Overlaid CDMA sub channels

In case overlaid CDMA is used, see 3GPP TS 43.064 [6], up to four MS can be multiplexed on the same physical resource, simultaneously transmitting, using different orthogonal codes assigned by the network if four PDCHs are assigned for EC operation, see 3GPP 44.018 [10] and 3GPP TS 44.060 [11]. Else if two PDCHs are assigned for EC operation, two MS can be multiplexed on the same physical resource. The codes are applied per burst over the assigned PDCHs for EC-PDTCH and EC-PACCH in each TDMA frame from the lowest to the highest numbered assigned TN, according to Table 6.3-2 , and are only applied in case blind physical layer transmissions are used (i.e. for CC2, CC3 and CC4). The code value is either 0 or 1. In case 0 is used, the burst is transmitted as if no code was applied. In case 1 is used, the whole burst is shifted in phase by  radians, see 3GPP TS 45.004 [14]. Since the code is applied over the blind physical layer transmissions within the TDMA frame, and CC2, CC3, CC4 all map to four or two PDCHs in a TDMA frame, there is no restriction on the pairing of MS on the same physical resources, of different Coverage Classes, as long as blind physical layer transmissions are used (CC2, CC3 and CC4) and the same number of PDCHs (i.e. four or two) are assigned for these MS assigned on those resources. Overlaid CDMA is only applied for transmissions on the uplink, i.e. for EC-PACCH/U and EC-PDTCH/U.

Table 6.3-2. Overlaid CDMA codes

| Overlaid CDMA code1 | Code sequence2 | Code sequence3 |
| --- | --- | --- |
| 0 | 0000 | 00 |
| 1 | 0101 | 01 |
| 2 | 0011 | - |
| 3 | 0110 | - |
| NOTE1: see 3GPP TS 44.060 [11] NOTE2: The first number in the sequence is applied to the lowest numbered assigned PDCH. The second number to the second lowest numbered assigned PDCH etc, in case four PDCHs are assigned for EC operation. NOTE 3: The first number in the sequence is applied to the lower numbered assigned PDCH and the second number to the higher numbered assigned PDCH. This code sequence is used if two PDCHs are assigned for EC operation. |  |  |

4th modification

#### 6.3.2.3	Mapping of the downlink channels

##### 6.3.2.3.1	Mapping of the (EC-)PDTCH/D and (EC-)PACCH/D

The PDCH(s) in BTTI configuration or the PDCH-pair(s) in RTTI configuration where the MS may expect occurrence of its (EC-)PDTCH/D(s) for a mobile terminated transfer or its (EC-)PACCH/D, for both mobile originated and mobile terminated transfer, are indicated in resource assignment messages (see 3GPP TS 44.060). (EC-)PDTCH/D and (EC-)PACCH/D can be mapped dynamically on all blocks except those used for PBCCH (see subclause 6.3.2.3.3). The logical channel type shall be indicated in the block header for GPRS, it shall be indicated by Stealing Flags for EGPRS and EGPRS2, and it shall be detected by the MS in case of EC operation. The mobile owner of the (EC-)PDTCH/D or (EC-)PACCH/D shall be indicated by the TFI (Temporary Flow Identity) (see 3GPP TS 44.060).

For EC-PDTCH/D and EC-PACCH/D the number of blocks available per 52-multiframe depends on the downlink Coverage Class and the number of PDCHs assigned for EC operation, and is the same as for uplink shown in table 6.3-3.

If PDTCH/D is mapped on blocks, which may be used for PCCCH and where paging may appear, the network shall only use coding schemes CS-1 to CS-4.

NOTE: 	This restriction is needed to avoid the expiry of the downlink signalling counter (DSC) for non-EGPRS capable mobile stations in case the network uses MCS-1 to MCS-9. CS-1 should be favoured, as it provides the strongest error protection.

5th modification

## 6.4	Permitted channel combinations

### 6.4.1	Permitted channel combinations onto a basic physical channel

The following are the permitted ways, as defined by 3GPP TS 44.003, in which channels can be combined onto basic physical channels for one or several MSs.

The following definitions are used in the list of combinations below.

| Combination designation | Channel combination |
| --- | --- |
| SUB_TA | TCH/H + FACCH/H + SACCH/TH |
| SUB_T | TCH/H |
| SUB_PA | PDTCH/H + PACCH/H |
| SUB_TE | TCH/H + FACCH/H + SACCH/TPH + EPCCH/H |
| SUB_OTA | O-TCH/H + O-FACCH/H + SACCH/TH |
| SUB_OT | O-TCH/H |
| SUB_OTE | O-TCH/H + O-FACCH/H + SACCH/TPH + EPCCH/H |

Numbers appearing in parenthesis after channel designations indicate sub-channel numbers. (0..n) shall be interpreted as subchannel 0, 1,…, n-1 and n. Channels and sub-channels need not necessarily be assigned.

- TCH/F + FACCH/F + SACCH/TF

- O-TCH/F + O-FACCH/F + SACCH/TF

- FCCH + SCH + BCCH + CCCH

- FCCH + SCH + BCCH + CCCH + SDCCH/4(0..3) + SACCH/C4(0..3)

- BCCH + CCCH

- SDCCH/8(0 .7) + SACCH/C8(0 . 7)

- TCH/F + FACCH/F + SACCH/M

- TCH/F + SACCH/M

- TCH/FD + SACCH/MD

- PBCCH + PCCCH + PDTCH/F + PACCH/F + PTCCH/F

- PCCCH + PDTCH/F + PACCH/F + PTCCH/F

- PDTCH/F + PACCH/F + PTCCH/F

- CTSBCH + CTSPCH + CTSARCH + CTSAGCH

- CTSPCH + CTSARCH + CTSAGCH

- CTSBCH

- CTSBCH + TCH/F + FACCH/F + SACCH/CTS

- E-TCH/F + E-IACCH/F + E-FACCH/F + SACCH/TF

- E-TCH/F + E-IACCH/F + E-FACCH/F + SACCH/M

- E-TCH/F + E-IACCH/F + SACCH/M

- E-TCH/FD + E-IACCH/F + SACCH/MD

- CFCCH + CSCH + CPBCCH + CPCCCH + PDTCH/F + PACCH/F + PTCCH/F

- CPCCCH + PDTCH/F + PACCH/F + PTCCH/F

- O-TCH/F + O-FACCH/F + SACCH/TPF + EPCCH/F

- TCH/F + FACCH/F + SACCH/TPF + EPCCH/F

- TCH/F + FACCH/F + SACCH/MP + EPCCH/M

- TCH/F + SACCH/MP + EPCCH/M

- TCH/FD + SACCH/MPD + EPCCH/MD

xxix)		PDTCH/F + PACCH/F + SACCH/TF

- PDTCH/F + PACCH/F + SACCH/TPF + EPCCH/F

xxxi)		PDTCH/F + PACCH/F + SACCH/M

xxxii)		PDTCH/FD + PACCH/FD + SACCH/MD

xxxiii)		PDTCH + PACCH/F + SACCH/M + EPCCH/M

xxxiv)		PDTCH + PACCH/F + SACCH/M + EPCCH/MD

xxxv)		PRACH + PDTCH/U + PACCH/U + PTCCH/U + MPRACH

xxxvi)		PDTCH/U + PACCH/U + PTCCH/U + MPRACH

xxxvii)	EC-SCH + EC-BCCH + EC-CCCH

xxxviii)	FCCH + SCH + BCCH + CCCH + EC-CCCH/U

xxxix)		EC-CCCH

xl)			BCCH + CCCH + EC-CCCH/U

xli)			EC-PDTCH/F + EC-PACCH/F

xlii)		EC-PDTCH/F + EC-PACCH/F + PDTCH/F + PACCH/F + PTCCH/F

The following combinations of half rate channels are allowed on a basic physical channel for a single mobile, where the second half rate channel need not be assigned:

a1)	SUB_TA + SUB_T	(Lm + Lm configuration)

a2)	SUB_TA + SUB_OT	(Lm + Lm configuration)

a3)	SUB_TA + SUB_PA	(DTM single slot)

a4)	SUB_TE + SUB_T	(Lm + Lm configuration)

a5)	SUB_TE + SUB_OT	(Lm + Lm configuration)

a6)	SUB_TE + SUB_PA	(DTM single slot)

a7)	SUB_OTA + SUB_OT	(Lm + Lm configuration)

a8)	SUB_OTA + SUB_T	(Lm + Lm configuration)

a9)	SUB_OTA + SUB_PA	(DTM single slot)

a10)	SUB_OTE + SUB_OT	(Lm + Lm configuration)

a11)	SUB_OTE + SUB_T	(Lm + Lm configuration)

a12)	SUB_OTE + SUB_PA	(DTM single slot)

The following combinations of half rate channels are allowed on a basic physical channel for two mobiles:

b1)	SUB_TA + SUB_TA

b2)	SUB_TA + SUB_TE

b3)	SUB_TA + SUB_OTA

b4)	SUB_TA + SUB_OTE

b5)	SUB_TE + SUB_TE

b6)	SUB_TE + SUB_OTA

b7)	SUB_TE + SUB_OTE

b8)	SUB_OTA + SUB_OTA

b9)	SUB_OTA + SUB_OTE

b10)	SUB_OTE + SUB_OTE

The following combinations of full rate channels are allowed on a basic physical channel capable of VAMOS for two mobiles in VAMOS mode (where the 2 full rate channels constitute a VAMOS pair):

c11)	i) + i)

c12)	i) + xxv)

c13)	xxv) + xxv)

The following combinations of full rate and half rate channels are allowed on a basic physical channel capable of VAMOS for three mobiles in VAMOS mode (where the half rate channel on sub-channel number 0 and the full rate channel constitute one VAMOS pair whilst the half rate channel on sub-channel number 1 and the full rate channel constitute a different VAMOS pair on the same basic physical channel capable of VAMOS)

d1)	SUB_TA + SUB_TA + i)

d2)	SUB_TA + SUB_TA + xxv)

d3)	SUB_TA + SUB_TE + i)

d4)	SUB_TA + SUB_TE + xxv)

d5)	SUB_TE + SUB_TE + i)

d6)	SUB_TE + SUB_TE + xxv)

The following combinations of half rate channels are allowed on a basic physical channel capable of VAMOS for 3 mobiles of which 2 are in VAMOS mode (where the pair of half rate channels sharing the same sub-channel number constitutes the VAMOS pair whilst the other half rate channel is not in VAMOS mode)

e1)	SUB_TA + SUB_TA + SUB_TA

e2)	SUB_TA + SUB_TA + SUB_TE

e3)	SUB_TE + SUB_TE + SUB_TA

e4)	SUB_TE + SUB_TE + SUB_TE

The following combinations of half rate channels are allowed on a basic physical channel capable of VAMOS for four mobiles in VAMOS mode (where the pair of half rate channels on the sub-channel number 0 constitutes one VAMOS pair and the other pair of half rate channels on the sub-channel number 1 constitutes the other VAMOS pair):

f1)	SUB_TA + SUB_TA + SUB_TA + SUB_TA

f2)	SUB_TA + SUB_TA + SUB_TA + SUB_TE

f3)	SUB_TA + SUB_TA + SUB_TE + SUB_TE

f4)	SUB_TA + SUB_TE + SUB_TE + SUB_TE

f5)	SUB_TE + SUB_TE + SUB_TE + SUB_TE

NOTE 0:	CCCH = PCH+ RACH + AGCH + NCH.

EC-CCCH = EC-PCH+EC-RACH+EC-AGCH.

PCCCH = PPCH+PRACH+PAGCH

CPCCCH = CPPCH + CPRACH + CPAGCH

EC-CCCH/U = EC-RACH

NOTE 1:	Where the SMSCB is supported, the CBCH replaces SDCCH number 2 in cases v) and vii) above.

NOTE 2:	A combined CCCH/SDCCH allocation (case v) above) may only be used when no other CCCH channel is allocated.

NOTE 3:	Combinations viii), ix), x), xix), xx), xxi), xxix), xxx), xxxi) and xxxii) are used without EPC in multislot configurations as defined in subclause 6.4.2.

NOTE 4:	Combinations xiv), xv), xvi) and xvii) shall be used in CTS; combinations xiv), xvi) and xvii) shall be mutually exclusive; combinations xiv) and xv) shall also be mutually exclusive.

NOTE 5:	Combinations xxii) and xxiii) shall be used for COMPACT on serving time groups.

NOTE 6:	Combinations i), ii), xiii), xxv), xxiv) or any of a1) to a12) shall be used for single timeslot operation in DTM.

NOTE 7:	A unidirectional TCH combination i), viii), ix) or x) may be combined with the corresponding E-TCH combination xviii), xix), xx) or xxi) respectively in the other direction.

NOTE 8:	Combinations xxvi), xxvii), xxviii) , xxxii), xxxiii) and xxxiv) are used with EPC in multislot configurations as defined in subclause 6.4.2.

NOTE 9:	The basic physical channel onto which channels can be combined according to combinations i), ii), viii), ix), x), xviii), xix), xx), xxi), xxiv), xxv), xxvi), xxvii), xxviii), xxix), xxx), xxxi), xxxii), xxxiii) and xxxiv) is referred to as dedicated basic physical subchannel full rate (DBPSCH/F) in Iu mode.

NOTE 10:	The basic physical channel onto which channels can be combined according to combinations xi), xii) and xiii) is referred to as shared basic physical subchannel full rate (SBPSCH/F) in Iu mode.

NOTE 11:	The part of the basic physical channel onto which channels can be combined according to combinations SUB_TA, SUB_T, SUB_PA, SUB_TE, SUB_OTA, SUB_OT, SUB_OTE is referred to as dedicated basic physical subchannel half rate (DBPSCH/H) in Iu mode.

NOTE 12:	The part of the basic physical channel onto which channels can be combined according to combination SUB_PA is referred to as shared basic physical subchannel half rate (SBPSCH/H) in Iu mode.

NOTE 13:	The parts of the basic physical channel onto which SDCCH and SACCH can be combined according to combination v) are referred to as dedicated basic physical subchannels for SDCCH/4 and SACCH/C4 (DBPSCH/S4) in Iu mode.

NOTE 14:	The parts of the basic physical channel onto which SDCCH and SACCH can be combined according to combination vii) are referred to as dedicated basic physical subchannels for SDCCH/8 and SACCH/C8 (DBPSCH/S8) in Iu mode.

NOTE 15:	Combinations xxxv) and xxxvi) are used only for MBMS.

NOTE 16:	In RTTI configuration only combination xiii) shall be used.

NOTE 17: 	Combinations xxxviii) and xl) are only applicable if 2 TS EC-RACH mapping is used, and in that case to the lower numbered timeslot.

NOTE 18: Combinations xli) and xlii) with EC-PDTCH/F apply also to EC-PDTCH/2TS/F using 2 PDCHs for blind physical layer transmissions.

6th modification

Table 6a : Mapping of Extended Coverage logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

| Channel designation | Dir. | Allowable timeslot assignment | Allowable RF channel assignment | Burst type | Repeat length TDMA frames | Coverage class (CC) | Interleaved block TDMA frame mapping |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EC-SCH | D | 1 | C0 | SB | 204 | - | B([0,1,2,3,4,5,6] + 51 N), N=0,1,2,34 |
| EC-BCCH | D | 1 | C0 | NB | 408 | - | B([7,8,9,10,11,12,13,14]+51N), N=0,…,75 |
| EC-PCH | D | 1,3,5,7 | C0 | NB | 51 | 1 | B0(19,20),B1(21,22),…,B15(49,50) |
|  |  |  |  |  | 102 | 2 | B0([19,…,26] + 51N),B1([27,…,34] + 51N),…,B3([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,34 |
| EC-AGCH | D | 1 | C0 | NB | 51 | 1 | B0(15,16),B1(17,18),…,B17(49,50) |
|  |  |  |  |  | 102 | 2 | B0([19,…,26] + 51N),B1([27,…,34] + 51N),…,B3([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,34 |
|  |  | 3,5,7 |  |  | 51 | 1 | B0(1,2),B1(3,4),…,B24(49,50) |
|  |  |  |  |  | 102 | 2 | B0([3,…,10] + 51N),B1([11,…,18] + 51N),…,B5([43,…,50] + 51N) N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,1,2,34 |
| EC-RACH, 1 TS | U | 1,3,5,7 | C0 | AB | 51 | 1 | B0(0),B1(1),..B50(50) |
|  |  |  |  |  |  | 2 | B0(1,…,4), B1(5,…,8),...B11(45,…,48) |
|  |  |  |  |  |  | 3 | B0(2,…, 17),B1(18,…,33), B2(34,…,49) |
|  |  |  |  |  | 102 | 4 | B0([0,…,23] + 51N),B1([24,…,47] + 51N), N=0,13 |
| EC-RACH, 2 TS | U | 0,1, or, 2,3, or, 4,5, or, 6,7 | C0 | AB | 51 | 2 | B0(1,2,1’,2’), B1(3,4,3’,4’),...,B24(49,50,49’,50’) |
|  |  |  |  |  |  | 3 | B0(2,…,9,2’,…,9’), B1(10,…,17,10’,…,17’),...,B5(42...49,42’...49’) |
|  |  |  |  |  | 102 | 4 | B0([0,…,11,0’,…,11’] + 51N),…,B3([36,…,47,36’,…,47’] + 51N), N=0,13 |
| EC-PACCH, EC-PDTCH | D&U | 0…7 | C0…Cn | NB2 | 52 | 1 | B0(0...3), B1(4...7), B2(8...11), B3(13...16), B4(17...20), B5(21…24), B6(26...29), B7(30...33), B8(34...37), B9(39...42), B10(43...46), B11(47...50) |
|  |  |  |  |  |  | 2 | B0(0...3,0'...3',0''...3'',0'''...3'''),B1(4...7,4'...7',4''...7'',4'''...7'''), B2(8...11, 8'...11',8''...11'', 8'''...11'''), B3(13...16,13'...16',13''...16'',13'''...16'''), B4(17...20,17'...20',17''...20'',17'''...20'''), B5(21...24,21'...24',21''...24'',21'''...24'''), B6(26...29,26'...29',26''...29'',26'''...29'''), B7(30...33,30'...33',30''...33'',30'''...33'''), B8(34...37,34'...37',34''...37'',34'''...37'''), B9(39...42,39'...42',39''...42'',39'''...42'''), B10(43...46,43'...46',43''...46'',43'''...46'''), B11(47...50,47'...50',47''...50'',47'''...50''')1 |
|  |  |  |  |  |  | 3 | B0(0...7,0'...7',0''...7'',0'''...7'''), B1(8...11,13…16,8'...11',13'…16',8''...11'',13''…16'',8'''...11''',13''',…,16'''), B2(17...24, 17'...24',17''...24'', 17'''...24'''),B3(26...33,26'...33',26''...33'',26'''...33'''), B4(34...37,39…42,34'...37',39'…42',34''...37'',39''…42'',34'''...37''',39'''…42'''), B5(43...50,43'...50',43''...50'',43'''...50''')1 |
|  |  |  |  |  |  | 4 | B0(0...11,13…16,0'...11',13'…16',0''...11'',13''…16'',0'''...11''',13'''…16'''), B1(17...24,26…33,17'...24',26'…33',17''...24'',26''…33'',17'''...24''',26'''…33'''), B2(34...37,39...50,34'...37',39'...50',34’’...37’’,39'’...50'’,34'’'...37'’',39'''...50''')1 |
|  |  |  |  |  | 52 | 26 | B0(0..7,0'..7'), B1(8..11,13..16,8'..11',13'..16'), B2(17..24,17'..24'), B3(26..33,26'..33'), B4(34..37,39..42,34'..37',39'..42'), B5(43..50,43'..50') |
|  |  |  |  |  | 52 | 36 | B0(0..11,13..16,0’..11,13..16), B1(17..24,26..33, 17'..24',26'..33'), B2(34..37,39..50,34'..37',39'..50') |
|  |  |  |  |  | 104, | 46 | B0(0..11,13..24,26..33,0’..11’,13’..24’,26’..33’), B1(34..37,39..50,52..63,34’..37’,39’..50’,52’..63’), B2(65..76,78..90,92..103,65’..76’,78’..90’,92’..103’) |
| NOTE 1: EC-PDTCH and EC-PACCH are mapped onto four PDCHs. Number n indicates mapping on the PDCH with the lowest timeslot number in TDMA frame n, whereas number n', n'', and n''' indicates mapping on the PDCH with the second lowest, second highest, and highest timeslot number in TDMA frame n respectively. The four timeslots shall be consecutive. NOTE 2: Access Bursts (AB) instead of Normal Bursts (NB) may be used on the uplink as polling response on EC-PACCH/U. The same resources are used by the Access Burst. NOTE 3 N = (FN div 51) mod 2 NOTE 4 N = (FN div 51) mod 4 NOTE 5 N = (FN div 51) mod 8 NOTE 6: EC-PDTCH/2TS and EC-PACCH/2TS are mapped onto two PDCHs in case of blind physical layer transmissions. |  |  |  |  |  |  |  |

Figure 9b: 52-multiframe for EC-GSM-IoT for different Coverage Classes (CC) using 4 PDCHs for CC2 to CC4 in EC operation.

Figure 9c: 52-multiframe for EC-GSM-IoT for different Coverage Classes (CC) using 2 PDCHs for CC2 to CC4 in EC operation.