3GPP TSG RAN WG6 #3 	R6-170060

# Athens, Greece, February 13-17, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.005 | CR | 0598 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Extended Access Burst for connectionless Multilateration Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePOS_GERAN-Perf |  |  |  |  |  |  | Date: |  | 2017-02-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The Extended Access Burst is used for Multilateration Timing Advance (MTA) for accessing neighbour cells that are not in extended coverage if indicated by the network in the Multilateration TA Request message and if the cell indicates support for MTA as depicted in R6-170047. The usage of Extended Access Burst within the Connectionless MTA procedure provides energy saving benefits compared to the Connection-oriented MTA procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In Table 1a the reference input level performance is specified for the new RACH access message carried in the Extended Access burst. - In Table 1z the reference input level performance is specified for the new EC-RACH access message for EC operation in CC1. - In Table 2a the co-channel interference performance is specified for the new RACH access message carried in the Extended Access burst. - In Table 2ah the co-channel interference performance is specified for the new EC-RACH access message for EC operation in CC1. - In Table 2al the adjacent channel interference performance is specified for the new EC-RACH access message for EC operation in CC1. - In Annex B, figure B.1, the abbreviation ‘dB’ is corrected to apply for the Dummy Burst ‘DB’ as specified in 45.002. The Extended Access Burst reuses the power versus time mask for normal duration burst at GMSK modulation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No radio performance requirements are defined for the Extended Access Burst, hence missed energy saving for MS requested to perform MTA. Inconsistency to stage 2 content. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 1a, Table 1z, Table 2a, Table 2ah, Table 2al, Annex B (figure B.1) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.059 CR 0083 TS 45.001 CR 0096 TS 45.002 CR 0205 TS 45.003 CR 0144 TS 44.018 CR 1060 TS 44.031 CR 0229 TS 48.008 CR 0411 TS 49.031 CR 0068 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010 CR xxxx TS 51.021 CR xxxx |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modification

Table 1a: Input signal level (for normal BTS) at reference performance for GMSK modulated signals

| GSM 900, ER-GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |  |
| channel |  |  | static |  | TU50 (no FH) |  | TU50 (ideal FH) |  | RA250 (no FH) |  | HT100 (no FH) |  |  |
| PDTCH/CS-1 |  | dBm | -104(x) |  | -104 |  | -104(x) |  | -104(x) |  | -103 |  |  |
| PDTCH/CS-2 |  | dBm | -104(x) |  | -100 |  | -101 |  | -101 |  | -99 |  |  |
| PDTCH/CS-3 |  | dBm | -104(x) |  | -98 |  | -99 |  | -98 |  | -96 |  |  |
| PDTCH/CS-4 |  | dBm | -101 |  | -90 |  | -90 |  | * |  | * |  |  |
| USF/CS-1 |  | dBm | -104(x) |  | -101 |  | -103 |  | -103 |  | -101 |  |  |
| USF/CS-2 to 4 |  | dBm | -104(x) |  | -103 |  | -104(x) |  | -104(x) |  | -104 |  |  |
| PRACH/11 bits1) |  | dBm | -104(x) |  | -104 |  | -104 |  | -103 |  | -103 |  |  |
| PRACH/8 bits1) |  | dBm | -104(x) |  | -104 |  | -104 |  | -103 |  | -103 |  |  |
| GSM 900, ER-GSM 900, GSM 850 and MXM 850 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |  |
| Channel |  |  | static |  | TU50 (no FH) |  | TU50 (ideal FH) |  | RA250 (no FH) |  | HT100 (no FH) |  |  |
| PDTCH/MCS-06) | dBm |  | -104 |  | -103 |  | -103,5 |  | -103 |  | -102,5 |  |  |
| PDTCH/MCS-1 | dBm |  | -104(x) |  | -102.5 |  | -103 |  | -103 |  | -102 |  |  |
| PDTCH/MCS-2 | dBm |  | -104(x) |  | -100,5 |  | -101 |  | -100,5 |  | -100 |  |  |
| PDTCH/MCS-3 | dBm |  | -104(x) |  | -96,5 |  | -96,5 |  | -92,5 |  | -95,5 |  |  |
| PDTCH/MCS-4 | dBm |  | -101,5 |  | -91 |  | -91 |  | * |  | * |  |  |
| USF/MCS-1 to 4 | dBm |  | -104(x) |  | -102,5 |  | -104 |  | -104(x) |  | -102,5 |  |  |
| PRACH/11 bits2), 3) |  | dBm | -104(x) |  | -104 |  | -104 |  | -103 |  | -103 |  |  |
| PRACH/30 bits2), 3), 8) |  | dBm | [-104(x)] |  | [-104] |  | [-104] |  | [-103] |  | [-103] |  |  |
| PRACH/8 bits 1) |  | dBm | -104(x) |  | -104 |  | -104 |  | -103 |  | -103 |  |  |
| DCS 1 800 & PCS 1 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |
| channel |  |  | static | TU50 (no FH) |  | TU50 (ideal FH) |  | RA130 (no FH) |  | HT100 (no FH) |  |  |
| PDTCH/CS-1 |  | dBm | -104(x) | -104 |  | -104 |  | -104(x) |  | -103 |  |  |
| PDTCH/CS-2 |  | dBm | -104(x) | -100 |  | -100 |  | -101 |  | -99 |  |  |
| PDTCH/CS-3 |  | dBm | -104(x) | -98 |  | -98 |  | -98 |  | -94 |  |  |
| PDTCH/CS-4 |  | dBm | -101 | -88 |  | -88 |  | * |  | * |  |  |
| USF/CS-1 |  | dBm | -104(x) | -103 |  | -103 |  | -103 |  | -101 |  |  |
| USF/CS-2 to 4 |  | dBm | -104(x) | -104(x) |  | -104(x) |  | -104(x) |  | -103 |  |  |
| PRACH/11 bits1) |  | dBm | -104(x) | -104 |  | -104 |  | -103 |  | -103 |  |  |
| PRACH/8 bits1) |  | dBm | -104(x) | -104 |  | -104 |  | -103 |  | -103 |  |  |
| DCS 1800, PCS 1900 and MXM 1900 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |
| channel |  |  | static | TU50 (no FH) |  | TU50 (ideal FH) |  | RA130 (no FH) |  | HT100 (no FH) |  |  |
| PDTCH/MCS-06) |  | dBm | -104 | -103 |  | -103,5 |  | -103 |  | -102,5 |  |
| PDTCH/MCS-1 |  | dBm | -104(x) | -102,5 |  | -103 |  | -103 |  | -101,5 |  |
| PDTCH/MCS-2 |  | dBm | -104(x) | -100,5 |  | -101 |  | -100,5 |  | -99,5 |  |
| PDTCH/MCS-3 |  | dBm | -104(x) | -96,5 |  | -96,5 |  | -92,5 |  | -94,5 |  |
| PDTCH/MCS-4 |  | dBm | -101,5 | -90,5 |  | -90,5 |  | * |  | * |  |
| USF/MCS-1 to 4 |  | dBm | -104(x) | -104 |  | -104 |  | -104(x) |  | -102,5 |  |
| PRACH/11 bits2), 3) |  | dBm | -104(x) | -104 |  | -104 |  | -103 |  | -103 |  |  |
| PRACH/30 bits2), 3), 8) |  | dBm | [-104(x)] | [-104] |  | [-104] |  | [-103] |  | [-103] |  |  |
| PRACH/8 bits1) |  | dBm | -104(x) | -104 |  | -104 |  | -103 |  | -103 |  |  |
| NOTE 1: The specification for PDTCH/CS-1 applies also for PACCH, PBCCH, PAGCH, PPCH, PTCCH/D. NOTE 2: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU50 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. NOTE 3: PDTCH/CS-4 and PDTCH/MCS-x can not meet the reference performance for some propagation conditions (*). NOTE 4 : The complete conformance should not be restricted to the logical channels and channel models identified with (x) NOTE 5: Identification of the correct Training sequence is required. Cases identified by 1) include one training sequence and cases identified by 2) include 3 training sequences according to 3GPP TS 45.002. The specification marked by 3) also applies to CPRACH. NOTE 6: The specification of MCS-0 only applies in DL RTTI configuration. NOTE 7: The specification for PDTCH/CS-3 applies also when used for PACCH in DLMC configuration (see 3GPP TS 44.060). NOTE 8: The performance requirement applies for a random access channel carrying 30 information bits, see 3GPP TS 45.003. |  |  |  |  |  |  |  |  |  |  |  |  |  |

2nd Modification

# Table 1z: Input signal level (for normal BTS) at reference performance for GMSK modulated signals for different Coverage Classes (CC)

| E-GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dBm | -120,5 | -115,5 | -117,5 | -115,5 |
| EC-PACCH/U/4 | CC2 | dBm | -126,5 | -122,0 | -123,5 | -122,0 |
| EC-PACCH/U/8 | CC3 | dBm | -129,5 | -124,5 | -126,0 | -125,0 |
| EC-PACCH/U/16 | CC4 | dBm | -132,0 | -128,0 | -128,0 | -127,5 |
| EC-PDTCH/MCS-14) | CC1 | dBm | -117,0 | -115,5 | -115,5 | -115,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | -123,0 | -121,5 | -121,5 | -121,5 |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | -126,0 | -124,5 | -124,0 | -123,5 |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | -128,5 | -128,0 | -126,5 | -126,0 |
| 1 TS EC-RACH2) 5) | CC1 | dBm | -116,0 | -113,5 | - | -113,0 |
| 1 TS EC-RACH/4 | CC2 | dBm | -122,0 | -119,0 | - | -118,5 |
| 1 TS EC-RACH/16 | CC3 | dBm | -126,0 | -123,0 | - | -123,0 |
| 1 TS EC-RACH/48 | CC4 | dBm | -128,0 | -126,0 | - | -125,0 |
| 2 TS EC-RACH/4 | CC2 | dBm | -122,5 | -119,5 | - | -119,0 |
| 2 TS EC-RACH/16 | CC3 | dBm | -127,0 | -124,5 | - | -124,0 |
| 2 TS EC-RACH/48 | CC4 | dBm | -129,5 | -128,0 | - | -127,0 |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dBm | -120,5 | -115,5 | -117,5 | -115,0 |
| EC-PACCH/U/4 | CC2 | dBm | -126,5 | -122,0 | -123,5 | -122,0 |
| EC-PACCH/U/8 | CC3 | dBm | -129,5 | -124,5 | -126,0 | -124,5 |
| EC-PACCH/U/16 | CC4 | dBm | -132,0 | -128,0 | -128,0 | -127,5 |
| EC-PDTCH/MCS-14) | CC1 | dBm | -117,0 | -115,5 | -115,5 | -115,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | -123,0 | -121,5 | -121,5 | -121,0 |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | -126,0 | -124,5 | -124,0 | -123,5 |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | -129,0 | -128,0 | -126,5 | -125,5 |
| 1 TS EC-RACH2) 5) | CC1 | dBm | -116,5 | -113,5 | - | -113,5 |
| 1 TS EC-RACH/4 | CC2 | dBm | -122,0 | -119,0 | - | -118,5 |
| 1 TS EC-RACH/16 | CC3 | dBm | -126,0 | -123,0 | - | -122,5 |
| 1 TS EC-RACH/48 | CC4 | dBm | -128,0 | -126,5 | - | -125,0 |
| 2 TS EC-RACH/4 | CC2 | dBm | -122,5 | -119,5 | - | -119,0 |
| 2 TS EC-RACH/16 | CC3 | dBm | -127,5 | -124,5 | - | -124,0 |
| 2 TS EC-RACH/48 | CC4 | dBm | -129,5 | -128,0 | - | -126,5 |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 1a apply for Static and TU50 (no FH) and TU50 (ideal FH) propagation conditions NOTE 5: The performance requirement also applies for EC-RACH carrying 30 information bits, see 3GPP 45.003. |  |  |  |  |  |  |

3rd Modification

Table 2a: Interference ratio at reference performance for GMSK modulated signals

| GSM 900, ER-GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU3 (no FH) | TU3 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA250 (no FH) |
| PDTCH/CS-1 | dB | 13 | 9 | 10 | 9 | 9 |
| PDTCH/CS-2 | dB | 15 | 13 | 14 | 13 | 13 |
| PDTCH/CS-3 | dB | 16 | 15 | 16 | 15 | 16 |
| PDTCH/CS-4 | dB | 21 | 23 | 24 | 24 | * |
| USF/CS-1 | dB | 19 | 10 | 12 | 10 | 10 |
| USF/CS-2 to 4 | dB | 18 | 9 | 10 | 9 | 8 |
| PRACH/11 bits1) | dB | 8 | 8 | 8 | 8 | 10 |
| PRACH/8 bits1) | dB | 8 | 8 | 8 | 8 | 9 |
| GSM 850, MXM 850, ER-GSM 900 and GSM 900 |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU3 (no FH) | TU3 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA250 (no FH) |
| PDTCH/MCS-05) | dB | 10 | 8.5 | 9 | 8.5 | 9 |
| PDTCH/MCS-1 | dB | 13 | 9.5 | 10.5 | 9.5 | 10 |
| PDTCH/MCS-2 | dB | 15 | 12 | 12.5 | 12 | 12 |
| PDTCH/MCS-3 | dB | 16.5 | 16.5 | 17 | 17 | 19 |
| PDTCH/MCS-4 | dB | 19 | 21.5 | 22 | 22 | * |
| USF/MCS-1 to 4 | dB | 18 | 10 | 11 | 9.5 | 9.5 |
| PRACH/11 bits2), 3) | dB | 8 | 8 | 8 | 8 | 10 |
| PRACH/30 bits2), 3), 7) | dB | [8] | [8] | [8] | [8] | [10] |
| PRACH/8 bits1) | dB | 8 | 8 | 8 | 8 | 9 |
| DCS 1 800 & PCS 1 900 |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU1,5 (no FH) | TU1,5 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA130 (no FH) |
| PDTCH/CS-1 | dB | 13 | 9 | 9 | 9 | 9 |
| PDTCH/CS-2 | dB | 15 | 13 | 13 | 13 | 13 |
| PDTCH/CS-3 | dB | 16 | 15 | 16 | 16 | 16 |
| PDTCH/CS-4 | dB | 21 | 23 | 27 | 27 | * |
| USF/CS-1 | dB | 19 | 10 | 10 | 10 | 10 |
| USF/CS-2 to 4 | dB | 18 | 9 | 9 | 9 | 7 |
| PRACH/11 bits1) | dB | 9 | 9 | 9 | 9 | 10 |
| PRACH/8 bits1) | dB | 8 | 8 | 8 | 8 | 9 |
| DCS 1800, PCS 1 900 and MXM 1900 |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU1,5 (no FH) | TU1,5 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA130 (no FH) |
| PDTCH/MCS-05) | dB | 10 | 8.5 | 9 | 8.5 | 9 |
| PDTCH/MCS-1 | dB | 13 | 9.5 | 10 | 9.5 | 10 |
| PDTCH/MCS-2 | dB | 15 | 12 | 12 | 12 | 12 |
| PDTCH/MCS-3 | dB | 16.5 | 16.5 | 17 | 18 | 19 |
| PDTCH/MCS-4 | dB | 19 | 21.5 | 23 | 23 | * |
| USF/MCS-1 to 4 | dB | 18 | 10 | 9.5 | 9.5 | 9.5 |
| PRACH/11 bits2), 3) | dB | 9 | 9 | 9 | 9 | 10 |
| PRACH/30 bits2), 3), 7) | dB | [9] | [9] | [9] | [9] | [10] |
| PRACH/8 bits1) | dB | 8 | 8 | 8 | 8 | 9 |
| Performance is specified at 30% BLER for those cases identified with mark **. NOTE 1: The specification for PDTCH/CS-1 applies also for PACCH, PBCCH, PAGCH, PPCH, PTCCH/D. NOTE 2: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU50 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. The TU3 (ideal FH) and TU1.5 (ideal FH), sufficient decorrelation cannot easily be achieved. These performance requirements are given for information purposes and need not be tested. NOTE 3: PDTCH/CS-4 and PDTCH/MCS-x cannot meet the reference performance for some propagation conditions (*). NOTE 4: Identification of the correct Training sequence is required. Cases identified by 1) include one training sequence and cases identified by 2) include 3 training sequences according to 3GPP TS 45.002. The specification identified by 3) also applies to CPRACH. NOTE 5: The specification of MCS-0 only applies in DL RTTI configuration. NOTE 6: The specification for PDTCH/CS-3 applies also when used for PACCH in DLMC configuration (see 3GPP TS 44.060). NOTE 7: The performance requirement applies for a random access channel carrying 30 information bits, see 3GPP TS 45.003. |  |  |  |  |  |  |

4th Modification

Table 2ah: Cochannel interference ratio at reference performance (for normal BTS) for GMSK modulated signals

| E-GSM 900 and GSM 850 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | -1,0 | -1,0 | 0,0 |
| EC-PACCH/U/4 | CC2 | dB | -8,0 | -9,0 | -8,0 |
| EC-PACCH/U/8 | CC3 | dB | -11,0 | -12,0 | -11,5 |
| EC-PACCH/U/16 | CC4 | dB | -13,5 | -14,5 | -14,5 |
| EC-PDTCH/MCS-14) | CC1 | dB | 0,0 | -0,5 | -0,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -7,5 | -7,5 | -7,5 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -11,0 | -10,5 | -10,5 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -14,0 | -13,0 | -13,0 |
| 1 TS EC-RACH2) 5) | CC1 | dB | 2,0 | - | 2,0 |
| 1 TS EC-RACH/4 | CC2 | dB | -5,5 | - | -4,5 |
| 1 TS EC-RACH/16 | CC3 | dB | -10,0 | - | -10,0 |
| 1 TS EC-RACH/48 | CC4 | dB | -12,5 | - | -12,5 |
| 2 TS EC-RACH/4 | CC2 | dB | -5,0 | - | -4,5 |
| 2 TS EC-RACH/16 | CC3 | dB | -10,5 | - | -10,5 |
| 2 TS EC-RACH/48 | CC4 | dB | -13,5 | - | -13,5 |
| DCS 1800 and PCS 1900 |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | -0,5 | -1,0 | 1,0 |
| EC-PACCH/U/4 | CC2 | dB | -7,5 | -9,0 | -7,5 |
| EC-PACCH/U/8 | CC3 | dB | -10,5 | -11,5 | -11,0 |
| EC-PACCH/U/16 | CC4 | dB | -13,5 | -14,0 | -14,0 |
| EC-PDTCH/MCS-14) | CC1 | dB | -0,5 | -0,5 | -0,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -7,0 | -7,5 | -7,0 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -10,5 | -10,0 | -10,0 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -13,5 | -12,5 | -13,0 |
| 1 TS EC-RACH2) 5) | CC1 | dB | 2,0 | - | 2,0 |
| 1 TS EC-RACH/4 | CC2 | dB | -5,5 | - | -4,0 |
| 1 TS EC-RACH/16 | CC3 | dB | -9,5 | - | -10,0 |
| 1 TS EC-RACH/48 | CC4 | dB | -12,5 | - | -13,0 |
| 2 TS EC-RACH/4 | CC2 | dB | -5,0 | - | -4,5 |
| 2 TS EC-RACH/16 | CC3 | dB | -10,5 | - | -10,0 |
| 2 TS EC-RACH/48 | CC4 | dB | -13,5 | - | -13,5 |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 2a apply for TU3 (low band) and TU1,5 (high band) and TU50 (no FH) propagation conditions together with the conditions for EGPRS in subclause 6.3.4. NOTE 5: The performance requirement also applies for EC-RACH carrying 30 information bits, see 3GPP 45.003. |  |  |  |  |  |

5th Modification

Table 2al: Adjacent channel interference ratio at reference performance (for normal BTS) for GMSK modulated signals

| E-GSM 900 and GSM 850 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | -19,5 | -21,0 | -19,0 |
| EC-PACCH/U/4 | CC2 | dB | -27,0 | -29,5 | -27,0 |
| EC-PACCH/U/8 | CC3 | dB | -28,0 | -32,5 | -29,0 |
| EC-PACCH/U/16 | CC4 | dB | -29,0 | -35,0 | -30,5 |
| EC-PDTCH/MCS-14) | CC1 | dB | -21,0 | -20,5 | -20,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -30,0 | -29,5 | -29,5 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -32,0 | -32,5 | -32,0 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -33,0 | -35,0 | -33,0 |
| 1 TS EC-RACH2) 5) | CC1 | dB | -14,5 | - | -14,5 |
| 1 TS EC-RACH/4 | CC2 | dB | -23,0 | - | -23,0 |
| 1 TS EC-RACH/16 | CC3 | dB | -26,0 | - | -26,0 |
| 1 TS EC-RACH/48 | CC4 | dB | -27,5 | - | -27,5 |
| 2 TS EC-RACH/4 | CC2 | dB | -24,5 | - | -24,5 |
| 2 TS EC-RACH/16 | CC3 | dB | -27,0 | - | -27,5 |
| 2 TS EC-RACH/48 | CC4 | dB | -29,5 | - | -29,5 |
| DCS 1800 and PCS 1900 |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | -19,5 | -21,0 | -19,5 |
| EC-PACCH/U/4 | CC2 | dB | -25,0 | -30,0 | -27,0 |
| EC-PACCH/U/8 | CC3 | dB | -26,5 | -32,5 | -28,5 |
| EC-PACCH/U/16 | CC4 | dB | -28,0 | -35,0 | -30,0 |
| EC-PDTCH/MCS-14) | CC1 | dB | -21,0 | -20,5 | -20,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -30,0 | -29,5 | -29,5 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -31,5 | -31,5 | -32,0 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -32,5 | -35,0 | -32,5 |
| 1 TS EC-RACH2) 5) | CC1 | dB | -14,5 | - | -14,5 |
| 1 TS EC-RACH/4 | CC2 | dB | -23,0 | - | -23,0 |
| 1 TS EC-RACH/16 | CC3 | dB | -26,0 | - | -26,5 |
| 1 TS EC-RACH/48 | CC4 | dB | -27,5 | - | -27,5 |
| 2 TS EC-RACH/4 | CC2 | dB | -24,5 | - | -24,5 |
| 2 TS EC-RACH/16 | CC3 | dB | -27,5 | - | -28,5 |
| 2 TS EC-RACH/48 | CC4 | dB | -30,0 | - | -30,5 |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 2a apply for TU3 (low band) and TU1,5 (high band) and TU50 (no FH) propagation conditions together with the conditions for EGPRS in table 6.3-1b and subclause 6.3.4. NOTE 5: The performance requirement also applies for EC-RACH carrying 30 information bits, see 3GPP 45.003. |  |  |  |  |  |

6th Modification

Annex B (normative):
Transmitted power level versus time

Figure B.1: Time mask for normal duration bursts (NB, FB, DB, SB and EAB) at GMSK modulation

End of Modifications