3GPP TSG RAN WG6 #4 	R6-170160

# Hangzhou, P.R. China, May 15-19, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 44.018 | CR | 1062 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of ESAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Core |  |  |  |  |  |  | Date: |  | 2017-05-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of ESAB for EC-RACH CC5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | EC-RACH CC5 is considered in affected clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ESAB for EC-RACH CC5 will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.5.2.1.2a, 3.10.5, 9.1.43q, 9.1.65 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064 CR XXXX TS 45.001 CR XXXX TS 45.002 CR XXXX TS 45.003 CR XXXX TS 45.004 CR XXXX TS 45.005 CR XXXX TS 45.008 CR XXXX |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.021 CR XXXX |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modification

##### 3.5.2.1.2a	EC Packet access procedure (EC-CCCH)

The RR entity of a mobile station initiates the packet access procedure by scheduling the sending of EC PACKET CHANNEL REQUEST messages (see Table 3.5.2.1.2a.1 and sub-clause 9.1.65) as follows:

-	Using the 1TS EC-RACH Mapping method if CC1 has been selected on the uplink or if Access_Timeslots=0, sent in EC SYSTEM INFORMATION TYPE 2.

-	Using the 2TS EC-RACH Mapping method if CC2, CC3, CC4 or CC5 has been selected on the uplink and Access_Timeslots=1, sent in EC SYSTEM INFORMATION TYPE 2.

-	When the 2TS EC-RACH Mapping method is used for CC2, CC3 and CC4 the EC-RACH is mapped over two consecutive timeslots, and the EC PACKET CHANNEL REQUEST messages sent across the 2 timeslots shall be identical and shall use the same training sequence code (i.e. one of TS5, TS6 or TS7 is used according to the uplink CC).

-	When the 2TS EC-RACH Mapping method is used for CC5 the EC-RACH is mapped over two consecutive timeslots and the EC PACKET CHANNEL REQUEST message sent across the 2 timeslots using ESAB and TS8.

See 3GPP TS 45.002 for details regarding coverage class specific rules for multiplexing blind physical layer transmissions using the 2 TS EC-RACH.

Prior to sending an EC PACKET CHANNEL REQUEST message on the EC-RACH a MS shall determine the applicable EC_CCCH_GROUP as per idle mode for EC-GSM-IoT (see 3GPP TS 45.002 [32]).

After scheduling the transmission of EC PACKET CHANNEL REQUEST messages it leaves idle mode (in particular, the mobile station shall ignore EC PAGING REQUEST messages).

Table 3.5.2.1.2a.1: EC Packet Access Procedure

| Purpose of the packet access procedure | EC-GSM-IoT Capable Cell |
| --- | --- |
| User data transfer (requested RLC mode = acknowledged) or Upper layer signalling transfer (e.g. page response, cell update, MM signalling, etc) | EC PACKET CHANNEL REQUEST sent using either TS3, TS5, TS6, TS7 or TS8 - see sub-clause 9.1.65 (NOTE 1) |
| NOTE 1: An EC capable MS in a cell that does not support EC-GSM-IoT shall operate according to Table 3.5.2.1.2.1 if it supports GPRS TBFs or EGPRS TBFs. |  |

The mobile station then sends maximally M + 1 EC PACKET CHANNEL REQUEST messages where each message is repeated according to the number of blind physical layer transmissions corresponding to its uplink CC.  The first EC PACKET CHANNEL REQUEST message is sent using a transmission opportunity corresponding to its uplink coverage class randomly selected (with uniform probability distribution) from a set of Tcc 51-multiframes on the EC-RACH/RACH as shown in Table 3.5.2.1.2a.2.  When sending a page response the set of Tcc 51-multiframes starts with the first 51-multiframe immediately following the last 51-multiframe used for receiving the corresponding paging request. For a mobile originated access attempt the set of Tcc 51-multiframes starts with the first 51-multiframe (following the 51-multiframe during which upper layers requested the transfer of a LLC PDU) that contains a transmission opportunity corresponding to its uplink coverage class.

Table 3.5.2.1.2a.2: Values of parameters Scc and Tcc

|  | Scc | Tcc |
| --- | --- | --- |
| CC1 | Sm | Tm |
| CC2 | Sm | Tm |
| CC3 | 2*Sm | 2*Tm |
| CC4 | 4*Sm | 2*(Tm+1) |
| CC5 | 4*Sm | 3*(Tm+2) |

The value for Sm is sent in the EC SYSTEM INFORMATION TYPE 2 message and is used for determining the Scc value applicable for monitoring the EC-AGCH according to the selected downlink coverage class of the mobile station.

The value for Tm is sent in the EC SYSTEM INFORMATION TYPE 2 message and is used for determining the Tcc value applicable for randomly selecting an EC-RACH (and possibly RACH) transmission opportunity corresponding to the selected uplink coverage class of the mobile station.

M is the value of the parameter "EC_Max_Retrans" sent in the EC SYSTEM INFORMATION TYPE 2 message.

The EC PACKET CHANNEL REQUEST message (see sub-clause 9.1.65) contains the following parameters:

-	a random reference field which is drawn randomly from a uniform probability distribution for every new transmission;

-	a field indicating the downlink coverage class selected by the mobile station (only included when the message is sent on EC-RACH);

- 	a field indicating the signal strength measured by the MS (only included when the message is sent on RACH);

-	a field indicating the purpose of the data transfer ("page response", "cell update", "RLC/MAC control message" or "uplink data transfer") and the number of uplink data blocks it has to send (assuming MCS-1 channel coding is used);

-	a field indicating the priority of the packet access request.

After sending the first EC PACKET CHANNEL REQUEST message or a subsequent retransmission, the mobile station shall start reading the EC-AGCH (according to the downlink coverage class indicated within the corresponding EC PACKET CHANNEL REQUEST message) in an attempt to find a response matching its last transmission.

- A MS that has selected downlink CC1 shall begin looking for a matching response starting within downlink 51-multiframe N (i.e. N = TDMA FN div 51) if it used uplink 51-multiframe N to send the last blind physical layer transmission of the EC PACKET CHANNEL REQUEST message and there is at least one remaining valid CC1 reception opportunity in downlink 51-multiframe N. If a matching response is not found or there are no remaining valid CC1 reception opportunities within downlink 51-multiframe N it shall start reading 51-multiframe N+1in an attempt to find a matching response. The total number of downlink 51-multiframes it reads (excluding downlink 51-multiframe N) in an attempt to find a matching response is determined by Scc (see Table 3.5.2.1.2a.2).

- A MS that has selected downlink CC2 shall begin looking for a matching response starting within downlink 51-multiframe N if it used uplink 51-multiframe N to send the last blind physical layer transmission of the EC PACKET CHANNEL REQUEST message, N mod 2 = 0 and there is at least one remaining valid CC2 reception opportunity that starts in downlink 51-multiframe N. If a matching response is not found using downlink 51-multiframes N and N+1 or there are no remaining valid CC2 reception opportunities that start within downlink 51-multiframe N it shall start reading downlink 51-multiframe N+1 (respectively N+2) if N mod 2 = 1 (respectively N mod 2 = 0). The total number of downlink 51-multiframes it reads (excluding downlink 51-multiframe N) in an attempt to find a matching response is determined by Scc (see Table 3.5.2.1.2a.2).

- A MS that has selected downlink CC3 shall begin looking for a matching response starting with downlink 51-multiframe N+1 (respectively N+2) when it used uplink 51-multiframe N to send the last blind physical layer transmission of the EC PACKET CHANNEL REQUEST message where N mod 2 = 1 (respectively N mod 2 = 0). The total number of downlink 51-multiframes it reads in an attempt to find a matching response is determined by Scc (see Table 3.5.2.1.2a.2).

- A MS that has selected downlink CC4 shall begin looking for a matching response starting with downlink 51-multiframe N+1 (respectively N+2, N+3, N+4) when it used uplink 51-multiframe N to send the last blind physical layer transmission of the EC PACKET CHANNEL REQUEST message where N mod 4 = 3 (respectively N mod 4 = 2, 1, 0). The total number of downlink 51-multiframes it reads in an attempt to find a matching response is determined by Scc (see Table 3.5.2.1.2a.2).

A mobile station accessing the network when the low priority indicator is set to “MS is configured to NAS signalling low priority” (see 3GPP TS 24.008) that has sent one or more EC PACKET CHANNEL REQUEST messages shall proceed as follows:

-	If, while reading the total number of downlink 51-multiframes determined by Scc, the mobile station receives an EC IMMEDIATE ASSIGNMENT TYPE 2 or an EC IMMEDIATE ASSIGNMENT REJECT message corresponding to its last transmitted EC PACKET CHANNEL REQUEST message it shall act on that message as described in sub-clause 3.5.2.1.3a.

-	If a matching response is not found and the mobile station is not sending an exception report (see sub-clause 9.1.65) it shall examine the Implicit Reject Status (IRS) field sent as part of the EC-SCH INFORMATION message (see sub-clause 9.1.30c).  If the IRS field indicates the access attempt is rejected the mobile station shall abort the packet access procedure, start timer T3146 (if not already running) and initiate the EC Implicit Reject procedure (see sub-clause 3.5.2a.2).

-	Otherwise, the MS schedules the sending of another EC PACKET CHANNEL REQUEST message (if allowed according to "EC_Max_Retrans") using a transmission opportunity corresponding to its uplink coverage class randomly selected (with uniform probability distribution) from a set of Tcc 51-multiframes on the EC-RACH (determined using Tm as shown in Table 3.5.2.1.2a.2).

-	The set of Tcc 51-multiframes starts with the first 51-multiframe (following the set of Scc downlink 51-multiframe read in attempt to find a matching response or following the last downlink 51-multiframe read to acquire the IRS field) that contains a transmission opportunity corresponding to its uplink coverage class.

-	When scheduling another EC PACKET CHANNEL REQUEST message the MS uses the “CC_Access_Adaptation” parameter (broadcast in EC SYSTEM INFORMATION TYPE 2 message) to determine if it is allowed to increment both its uplink CC and downlink CC to the next CC supported by the network (unless it is already using CC4).

-	Upon incrementing its uplink/downlink CC the number of subsequent EC PACKET CHANNEL REQUEST message transmission attempts that may occur before it is allowed to increment its uplink/downlink CC once again is the same as when sending the first EC PACKET CHANNEL REQUEST message (i.e. determined by the “CC_Access_Adaptation” parameter).

- When sending up to M + 1 EC PACKET CHANNEL REQUEST messages the MS may increment its uplink CC and downlink CC a maximum of 2 times if CC adaptations are allowed according to the “CC_Access_Adaptation” parameter.

Having sent M + 1 EC PACKET CHANNEL REQUEST messages, the RR entity of the mobile station starts timer T3146 at the start of the first of the set of downlink 51-multiframes (excluding downlink 51-multiframe N) it reads in an attempt to find a matching response as determined by Scc (see Table 3.5.2.1.2a.2). At expiry of timer T3146, the packet access procedure is aborted, a random access failure is indicated to upper layers and autonomous cell re-selection is performed according to 3GPP TS 43.022.

2nd Modification

### 3.10.5	Extended Coverage

An EC-GSM-IoT capable mobile station supports extended coverage whereby blind physical layer transmissions are used on both the uplink and downlink according to the current coverage conditions and the logical channel used (see 3GPP TS 45.002). Four different coverage classes are defined for downlink and five different coverage classes are defined for uplink, each corresponding to a fixed number of blind physical layer transmissions for each logical channel, where the fixed number of blind physical layer transmissions need not be the same for different logical channels belonging to the same coverage class. A mobile station that has enabled EC operation selects its uplink and downlink coverage class based on thresholds broadcast by the network. The mobile station uses its selected uplink coverage class to determine the number of blind physical layer transmissions to use when sending an EC PACKET CHANNEL REQUEST message (see sub-clause 3.5.2a) and includes the selected downlink coverage class in the message (see sub-clause 9.1.65).

Upon determining that its currently selected downlink coverage class has changed, a mobile station that has enabled EC operation shall trigger the transmission of a cell update under certain conditions (see 3GPP TS 45.008).

Blind physical layer transmissions in a cell on EC-PDTCH and EC-PACCH channels in uplink and downlink are mapped on to either 4 consecutive PDCH resources or 2 consecutive PDCH resources. The number of consecutive PDCH resources used for EC TBF operation in an EC-GSM-IoT capable cell is controlled via the EC System information type 2 message. The mapping of EC channels onto physical channels shall be as specified in 3GPP TS 45.002.

3rd Modification

9.1.43q	EC System information type 2

Table 9.1.43q.1: EC SYSTEM INFORMATION TYPE 2 information element details

| Message Type (3 bits) The Message Type field is encoded according to Table 10.4.4. |  |
| --- | --- |
| EC SI 2_INDEX (2 bits) and EC SI 2_COUNT (2 bits) The purpose of these fields is to indicate the number of individual message instances within the sequence of EC SI 2 messages and to assign an index to identify each instance. The EC SI 2_INDEX field is binary coded, range 0 to 3, and provides an index to identify an individual EC SI 2 message instance. The EC SI 2_COUNT field is binary coded, range 0 to 3, and provides the value of the highest indexed message instance in the sequence of EC SI 2 messages. |  |
| EC SI_CHANGE_MARK (5 bits) This field is defined in Table 9.1.43p.1. |  |
| EC Cell Selection Parameters struct This structure provides the cell selection parameters for the serving cell. Location Area Identification (40 bits) The Location Area Identification field is coded as the value part of the Location Area Identification IE specified in sub-clause 10.5.1.3. |  |
| Routing Area Code (8 bits) This field is the binary representation of the Routing Area Code, see 3GPP TS 23.003. |  |
| Cell Identity (16 bits) The Cell Identity field is coded as the value part of the Cell Identity IE specified in sub-clause 10.5.1.1. |  |
| EC_BS_CC_CHANS (2 bits) This field indicates the number of extended coverage common control channels (EC-CCCHs) supported in the cell. Value 00 1 EC-CCCH supported 01 2 EC-CCCHs supported 10 3 EC-CCCHs supported 11 4 EC-CCCHs supported If the Access_Timeslots field provided in this message indicates that 2 TS EC-RACH mapping shall be applied in the cell, then the number of EC-CCCHs supported in the cell shall be equal to or lower than the number of common control channels (CCCHs) configured in the same cell. |  |
| EC_RXLEV_ACCESS_MIN (6 bits) The EC_RXLEV_ACCESS_MIN field is coded as the binary representation of the minimum received signal level at the MS for which it is permitted to access the system, see 3GPP TS 45.008. |  |
| MS_TXPWR_MAX_CCH (5 bits) The MS_TXPWR_MAX_CCH field is coded as the binary representation of the "power control level" in 3GPP TS 45.005 corresponding to the maximum TX power level an MS may use when accessing the system on the (EC-)CCCH. This value shall be used by the Mobile Station according to 3GPP TS 45.008. |  |
| LB_MS_TXPWR_MAX_CCH (5 bits) The LB_MS_TXPWR_MAX_CCH field is coded as the binary representation of the "power control level" in 3GPP TS 45.005 corresponding to the maximum TX power level an MS may use on all other than DCS 1800 and PCS 1900 frequency bands when accessing the system on the (EC-)CCCH. This value shall be used by the Mobile Station according to 3GPP TS 45.008. |  |
| CELL_SELECTION_RLA_MARGIN (3 bits) The CELL_SELECTION_RLA_MARGIN field provides the MS with information whether RLA_EC and RLA_GC based measurements may be omitted or not. The use of this field is defined in 3GPP TS 45.008. |  |
| Coverage Class Selection Parameters struct This structure provides information for the MS to estimate its uplink and downlink coverage class. |  |
| DL_CC_Selection (1 bit) This field indicates the method for selecting the downlink coverage class to be used by the MS. Bit 0 RLA_EC based coverage class selection 1 SLA based coverage class selection The use of this field is defined in 3GPP TS 45.008. |  |
| BT_Threshold_DL (5 bits) This field indicates the threshold below which blind physical layer transmissions are used on EC-CCCH. The use of this field is defined in 3GPP TS 45.008. |  |
| CC2_Range_DL (5 bits) CC3_Range_DL (5 bits) These fields are optionally sent by the network to indicate the signal level range of the indicated downlink coverage classes. The presence of one or both of the above fields indicates network support of the associated downlink coverage class. The use of these fields is defined in 3GPP TS 45.008. |  |
| BT_Threshold_UL (5 bits) This field indicates the signal level threshold below which blind physical layer transmissions are used on EC-RACH. The use of this field is defined in 3GPP TS 45.008. |  |
| CC2_Range_UL (5 bits) CC3_Range_UL (5 bits) These fields are optionally sent by the network to indicate the signal level range of the indicated uplink coverage classes. The presence of one or both of the above fields indicates network support of the associated uplink coverage class. The use of these fields is defined in 3GPP TS 45.008. |  |
| BSPWR (6 bits) This field indicates the BTS output power transmitted on FCCH and EC-SCH. The use of this field is defined in 3GPP TS 45.008. |  |
| DL_Signal_Strength_Step_Size (2 bits) This field indicates the step-size in signal level above BT_Threshold_DL possible to report by the MS in the EC Packet Channel Request message (see sub-clause 9.1.65). It is encoded as follows: Bits 2 1 0 0 X = 2 0 1 X = 4 1 0 X = 6 1 1 X = 8 |  |
| EC_Reduced_PDCH_Allocation (1 bit) This field indicates that the number of consecutive PDCHs the network allocates when assigning an EC TBF to a MS indicating Coverage Class CC2, CC3 or CC4 on the uplink or downlink during packet access. Bit 0 Four consecutive PDCHs are always allocated to MS for an EC TBF. 1 Two consecutive PDCHs are always allocated to MS for an EC TBF. |  |
| EC-RACH Control Parameters struct The EC-RACH Control Parameters contains the access control parameters needed when accessing the network on the EC-RACH. EC_Max_Retrans (2 bits) This field indicates the maximum number of retransmissions on EC-RACH (see sub-clause 3.5.2.1.2a). It is encoded as the Max retrans field in the RACH Control Parameters IE defined in sub-clause 10.5.2.29. |  |
| Sm (2 bits) This field is used by a MS to determine the number of multiframes it needs to read on the EC-AGCH in an attempt to find a response matching its last EC-RACH transmission (see sub-clause 3.5.2.1.2a). Bits 2 1 0 0 1 or more multiframes 0 1 2 or more multiframes 1 0 3 or more multiframes 1 1 4 or more multiframes |  |
| Tm (2 bits) This field is used by a MS to determine the number of multiframes on the EC-RACH from which it randomly selects a transmission/retransmission opportunity (see sub-clause 3.5.2.1.2a). Bits 2 1 0 0 1 or more multiframes 0 1 2 or more multiframes 1 0 3 or more multiframes 1 1 4 or more multiframes |  |
| Access_Timeslots (1 bit) This field indicates whether random access mapped over two timeslots (e.g. TS 0 and TS 1, or, TS 2 and TS 3) shall be applied or not (see sub-clause 3.5.2.1.2a). Bit 0 1 TS EC-RACH mapping shall be applied. 1 2 TS EC-RACH mapping shall be applied if uplink Coverage Class 2, Coverage Class 3 or Coverage Class 4 has been selected. The number of EC-RACH supported in the cell is indicated by the EC_BS_CC_CHANS field provided in this message. If uplink Coverage Class 5 has been selected, this bit will be ignored. |  |
| CC_Access_Adaptation (2 bits) This field indicates whether the mobile station is allowed to increment its Coverage Class after one or more failed access attempts on the EC-RACH (see sub-clause 3.5.2.1.2a). Bits 2 1 0 0 Coverage Class adaptation not allowed 0 1 Coverage Class adaptation after 1 failed EC-RACH transmission attempt 1 0 Coverage Class adaptation after 2 failed EC-RACH transmission attempts 1 1 Coverage Class adaptation after 3 failed EC-RACH transmission attempts |  |
| Cell_Bar_Access (1 bit) This field indicates whether the cell is bared for access. It is encoded as the CELL_BAR_ACCESS field in the RACH Control Parameters IE defined in sub-clause 10.5.2.29. |  |
| EC_Access_Control_Class (7 bits) This field contains a bitmap indicating the barring status for Access Control Classes 0 to 15 (AC C0 to AC C15) applicable for the Common PLMN provided in this message. The EC_Access_Control_Class field is encoded as follows. Bits 7 6 5 4 3 2 1 x x x x x x x AC C15 AC C14 AC C13 AC C12 AC C11 AC C10 AC C0 to AC C9 For a mobile station with AC C = N access is not barred if the AC CN bit is coded with a "0"; N = 0, 1, .. 9, 11, .., 15. Access control for mobile stations belonging to one of the access control classes 0 to 9 (AC C0 to AC C9) is indicated by the first bit in the bitmap (bit 1). If this field is not present and the MS has selected the common PLMN it shall assume its access class is not barred. For an MS in EC operation the AC C10 field is not used in this version of the specification. |  |
| Exception_Report_Status (1 bit) This field indicates whether sending of exception reports are allowed or not. It is encoded as follows: Bit 0 Sending of exception reports allowed in the cell for all MSs 1 Sending of exception reports not allowed in the cell except for the MSs that belong to one of the authorized Access Control Classes 11 to 15. If this field is not present and the MS has selected the common PLMN it shall assume exception reports are not barred. |  |
| BT_Threshold_UL_Margin (3 bits) This optional field indicates the power margin above BT_Threshold_UL used for (EC-)RACH open-loop power control. The use of this field is defined in 3GPP TS 45.008. |  |
| Short RACH Control Parameters struct The Short RACH Control Parameters contains the access control parameters needed when accessing the network on the RACH on the common control channel (CCCH) on TS 0. Max_Retrans (2 bits) This field indicates the maximum number of retransmissions on RACH. It is encoded as the Max retrans field in the RACH Control Parameters IE defined in sub-clause 10.5.2.29. |  |
| Tx-integer (4 bits) This field indicates the number of slots to spread the transmission on RACH. It is encoded as the Tx-integer field in the RACH Control Parameters IE defined in sub-clause 10.5.2.29. |  |
| Access_Control_Class (16 bits) This field contains a bitmap indicating the barring status for Access Control Classes 0 to 15 (AC C0 to AC C15) applicable for the Common PLMN provided in this message. In the absence of this field, the Access Control Class information provided in the EC-RACH Control Parameters struct shall be used instead. The Access_Control_Class field is encoded as follows. Bits 16 15 … 1 x x … x AC C15 AC C14 … AC C0 For an MS in EC operation the AC C10 field is not used in this version of the specification. |  |
| EC Cell Options struct ALPHA (4 bits) This field is the binary representation of the parameter for MS output power control in units of 0.1, see 3GPP TS 45.008. For encoding and description of the ALPHA field see the Global Power Control Parameters IE in 3GPP TS 44.060. T3168 (3 bits) T3192 (3 bits) These fields are defined in 3GPP TS 44.060. T3226 (3 bits) This field is the binary representation of the timeout value of timer T3226. Range: 0 to 7. The timeout values are given in the following table. Bits 3 2 1 0 0 0 20 ms 0 0 1 60 ms 0 1 0 100 ms 0 1 1 200 ms 1 0 0 500 ms 1 0 1 700 ms 1 1 0 1000 ms 1 1 1 1200 ms T3248 (2 bits) This field is the binary representation of the timeout value of timer T3248. Range: 0 to 3. The timeout values are given in the following table. Bits 2 1 0 0 not used 0 1 1 second 1 0 2 seconds 1 1 3 seconds |  |

4th Modification

### 9.1.65	EC Packet Channel Request

This message may be sent by an EC capable mobile station attempting system access using the EC-RACH (see sub-clause 3.5.2.1.2a) in which the message format is as shown in Tables 9.1.65.1 and 9.1.65.2. This message can also be sent using the RACH (see sub-clause 3.5.2.1.2) in which the message format is as shown in Tables 9.1.65.3 and 9.1.65.4.

The uplink access burst block format is defined in 3GPP TS 44.004. The order of bit transmission is defined in 3GPP TS 44.004. The message is coded in 11-bit format.

The EC capability is implied when a MS attempts system access using the EC-RACH or by the use of TS3 when a MS attempts system access using the RACH.

Table 9.1.65.1: EC PACKET CHANNEL REQUEST messages (EC-RACH)

| Training sequence (see 3GPP TS 45.002) | bits 11…...1 | Packet Channel Access |
| --- | --- | --- |
| TS3 | < EC PACKET CHANNEL REQUEST message content > | Uplink CC1 MS |
| TS5 | < EC PACKET CHANNEL REQUEST message content > | Uplink CC2 MS |
| TS6 | < EC PACKET CHANNEL REQUEST message content > | Uplink CC3 MS |
| TS7 | < EC PACKET CHANNEL REQUEST message content > | Uplink CC4 MS |
| TS8 | < EC PACKET CHANNEL REQUEST message content > (Note 1) | Uplink CC5 MS |
| Note 1: Only sent on EC-RACH when EC SYSTEM INFORMATION TYPE 2 message indicates CC5 access is supported (i.e. CC4_Range_UL parameter is included) and is sent using ESAB (see 3GPP TS 45.002) |  |  |

Table 9.1.65.2: EC PACKET CHANNEL REQUEST message content (EC-RACH)

| TS | Bit 11 | Bit 10 | Bit 9 | Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 (CC1) | EGPRS Capability | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Selected DL Coverage Class : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 5 (CC2) | 0 | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Selected DL Coverage Class : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 5 (CC2) | 1 | Spare (10) |  |  |  |  |  |  |  |  |  |
| 6 (CC3) | 0 | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Selected DL Coverage Class : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 6 (CC3) | 1 | Spare (10) |  |  |  |  |  |  |  |  |  |
| 7 (CC4) | 0 | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Selected DL Coverage Class : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 7 (CC4) |  | 1 |  | Spare (10) |  |  |  |  |  |  |  |  |  |
| 8 (CC5) |  | 0 |  | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Selected DL Coverage Class : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 8 (CC5) | 1 | Spare (10) |  |  |  |  |  |  |  |  |  |

Table 9.1.65.3: EC PACKET CHANNEL REQUEST messages (RACH)

| Training sequence (see 3GPP TS 45.002) | bits 11…...1 | Packet Channel Access |
| --- | --- | --- |
| TS3 | < EC PACKET CHANNEL REQUEST message content > (Note 1) | Uplink CC1 MS |
| TS5 | < EC PACKET CHANNEL REQUEST message content > (Note 2) | Uplink CC2 MS |
| TS6 | < EC PACKET CHANNEL REQUEST message content > (Note 2) | Uplink CC3 MS |
| TS7 | < EC PACKET CHANNEL REQUEST message content > (Note 2) | Uplink CC4 MS |
| TS8 | < EC PACKET CHANNEL REQUEST message content > (Note 3) | Uplink CC5 MS |
| Note 1: Only sent on RACH when RACH Access Control is enabled (see sub-clause 9.1.30c) Note 2: Only sent on RACH when 2TS EC-RACH is enabled (see sub-clause 9.1.43q) Note 3: Only sent on RACH when EC SYSTEM INFORMATION TYPE 2 message indicates CC5 access is supported (i.e. CC4_Range_UL parameter is included) and is sent using ESAB (see 3GPP TS 45.002) |  |  |

Table 9.1.65.4: EC PACKET CHANNEL REQUEST message content (RACH)

| TS | Bit 11 | Bit 10 | Bit 9 | Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 (CC1) | EGPRS Capability | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Signal Strength : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 5 (CC2) | 0 | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Selected DL Coverage Class : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 5 (CC2) | 1 | Spare (10) |  |  |  |  |  |  |  |  |  |
| 6 (CC3) | 0 | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Selected DL Coverage Class : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 6 (CC3) | 1 | Spare (10) |  |  |  |  |  |  |  |  |  |
| 7 (CC4) | 0 | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Selected DL Coverage Class : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 7 (CC4) |  | 1 |  | Spare (10) |  |  |  |  |  |  |  |  |  |
| 8 (CC5) |  | 0 |  | < EC PACKET CHANNEL REQUEST message content > :: = < EC-NumberOfBlocks : bit (3) > < EC Priority : bit (1) > < RandomBits : bit (3) > < Selected DL Coverage Class : bit (3) > ; |  |  |  |  |  |  |  |  |  |
| 8 (CC5) |  | 1 |  | Spare (10) |  |  |  |  |  |  |  |  |  |

Table 9.1.65.5: EC PACKET CHANNEL REQUEST details

EC-NumberOfBlocks (3 bit field) This field indicates the type of payload a mobile station has to send as follows: bit 3 2 1 0 0 0 page response 0 0 1 RLC/MAC control message 0 1 0 cell update or uplink data transfer requiring 1 MCS-1 coded RLC data block 0 1 1 uplink data transfer – 2 MCS-1 coded RLC data blocks required 1 0 0 uplink data transfer – 3 MCS-1 coded RLC data blocks required . . . 1 1 1 uplink data transfer – 6or more MCS-1 coded RLC data blocks required

EC Priority (1 bit field) This field indicates the priority of the uplink data transfer being requested by the mobile station as follows: 0 normal priority (normal report) 1 high priority (exception report)

Selected DL Coverage Class (3 bit field) This field indicates the Downlink Coverage Class that the mobile station has determined to be applicable at the point of sending an EC PACKET CHANNEL REQUEST and, when CC1 applies to the downlink, the extent to which the C-value exceeds the value indicated by the BT_Threshold_DL parameter sent in EC SI2 (see sub-clause 9.1.43q): Case 1: EC SYSTEM INFORMATION indicates 4 of 4 possible coverage classes are supported: bit 3 2 1 0 0 0 DL CC4 0 0 1 DL CC3 0 1 0 DL CC2 0 1 1 DL CC1; BT_Threshold_DL ≤ C_VALUE < BT_Threshold_DL + X dB 1 0 0 DL CC1; BT_Threshold_DL + X ≤ C_VALUE < BT_Threshold_DL + 2X dB 1 0 1 DL CC1; BT_Threshold_DL + 2X ≤ C_VALUE < BT_Threshold_DL + 3X dB 1 1 0 DL CC1; BT_Threshold_DL + 3X ≤ C_VALUE < BT_Threshold_DL + 4X dB 1 1 1 DL CC1; C_VALUE ≥ BT_Threshold_DL + 4X dB Case 2: EC SYSTEM INFORMATION indicates 3 of 4 possible coverage classes are supported: bit 3 2 1 0 0 0 DL CC4 0 0 1 DL CC2 or CC3 0 1 0 DL CC1; BT_Threshold_DL ≤ C_VALUE < BT_Threshold_DL + X dB 0 1 1 DL CC1; BT_Threshold_DL + X ≤ C_VALUE < BT_Threshold_DL + 2X dB 1 0 0 DL CC1; BT_Threshold_DL + 2X ≤ C_VALUE < BT_Threshold_DL + 3X dB 1 0 1 DL CC1; BT_Threshold_DL + 3X ≤ C_VALUE < BT_Threshold_DL + 4X dB 1 1 0 DL CC1; BT_Threshold_DL + 4X ≤ C_VALUE < BT_Threshold_DL + 5X dB 1 1 1 DL CC1; C_VALUE ≥ BT_Threshold_DL + 5X dB

Case 3: EC SYSTEM INFORMATION indicates coverage classes 1 and 4 are supported: bit 3 2 1 0 0 0 DL CC4 0 0 1 DL CC1; BT_Threshold_DL ≤ C_VALUE < BT_Threshold_DL + X dB 0 1 0 DL CC1; BT_Threshold_DL + X ≤ C_VALUE < BT_Threshold_DL + 2X dB 0 1 1 DL CC1; BT_Threshold_DL + 2X ≤ C_VALUE < BT_Threshold_DL + 3X dB 1 0 0 DL CC1; BT_Threshold_DL + 3X ≤ C_VALUE < BT_Threshold_DL + 4X dB 1 0 1 DL CC1; BT_Threshold_DL + 4X ≤ C_VALUE < BT_Threshold_DL + 5X dB 1 1 0 DL CC1; BT_Threshold_DL + 5X ≤ C_VALUE < BT_Threshold_DL + 6X dB 1 1 1 DL CC1; C_VALUE ≥ BT_Threshold_DL + 6X dB X is defined by the DL_Signal_Strength_Step_Size field (see sub-clause 9.1.43q). When determining the value of this field the received signal level based measurements (RLA_EC) or SINR based measurements (SLA) are used to determine the C-value, see 3GPP TS 45.008 [34].

Signal Strength (3 bit field) This field indicates the extent to which the received power exceeds the CC1 threshold: bit 3 2 1 0 0 0 DL CC1; BT_Threshold_DL ≤ C_VALUE < BT_Threshold_DL + X dB 0 0 1 DL CC1; BT_Threshold_DL + X ≤ C_VALUE < BT_Threshold_DL + 2X dB 0 1 0 DL CC1; BT_Threshold_DL + 2X ≤ C_VALUE < BT_Threshold_DL + 3X dB 0 1 1 DL CC1; BT_Threshold_DL + 3X ≤ C_VALUE < BT_Threshold_DL + 4X dB 1 0 0 DL CC1; BT_Threshold_DL + 4X ≤ C_VALUE < BT_Threshold_DL + 5X dB 1 0 1 DL CC1; BT_Threshold_DL + 5X ≤ C_VALUE < BT_Threshold_DL + 6X dB 1 1 0 DL CC1; BT_Threshold_DL + 6X ≤ C_VALUE < BT_Threshold_DL + 7X dB 1 1 1 DL CC1; C_VALUE ≥ BT_Threshold_DL + 7X dB X is defined by the DL_Signal_Strength_Step_Size field (see sub-clause 9.1.43q). When determining the value of this field the received signal level based measurements (RLA_EC) or SINR based measurements (SLA) are used to determine the C-value, see 3GPP TS 45.008 [34].

EGPRS Capability (1 bit field) This field indicates whether the MS supports GMSK only (EGPRS without 8PSK) or GMSK and 8PSK (EGPRS with 8PSK on uplink and downlink) 0 EGPRS without 8PSK 1 EGPRS with 8PSK (uplink and downlink)

RandomBits (3 bit field) For the definition of these three last information fields see 3GPP TS 44.060.

End of Modifications