3GPP TSG RAN WG6 #4 	R6-170167

# Hangzhou, P.R. China, May 15-19, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 51.021 | CR | 0279 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of ESAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Perf |  |  |  |  |  |  | Date: |  | 2017-05-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of ESAB for EC-RACH CC5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some updates for the descriptions related to the coverage classes of EC-RACH are introduced; Test requirement for EC-RACH CC5 using ESAB is added in corresponding tables |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EC-RACH CC5 will not be described |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.11, 7.3.2, 7.3.4, 7.4.2, 7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064 CR XXXX TS 45.001 CR XXXX TS 45.002 CR XXXX TS 45.003 CR XXXX TS 45.004 CR XXXX TS 45.005 CR XXXX TS 45.008 CR XXXX TS 44.018 CR XXXX |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modification

### 4.10.11	Support of EC-GSM-IoT

A BTS supporting EC-GSM-IoT shall, in addition to fulfilling EGPRS requirements, unless otherwise stated, fulfil the requirements for EC-channels in at least coverage classes CC1 and CC4 (no Overlaid CDMA applied). The BTS shall support 1 TS EC-RACH in CC1 and at least one of 1 TS EC-RACH and 2 TS EC-RACH in Coverage Classes higher than CC1. If the BTS supports CC5 then it shall support 2 TS EC-RACH using ESAB (see 3GPP TS 45.002). Unless otherwise stated, the BTS requirements for EC-GSM-IoT are defined with RX diversity with two antennas with uncorrelated fast fading on fading channels and with equal gain between the two receive branches in case of sensitivity performance and interference level performance. For the performance with Overlaid CDMA the requirements are defined for single RX antenna configuration. The other receiver performance requirements for EC-GSM-IoT are defined for single RX antenna configuration.

The support of EC operation and the set of coverage classes supported, whether the BTS supports Overlaid CDMA, and in this case the number of subchannels supported, shall be included in the manufacturer’s declaration. The BTS shall be equipped with RX diversity and shall support at least MCS-1 and MCS-1/16 for EC-PDTCH.

If supported, EC-PDTCH channels in CC1 using MCS-2 to MCS-4 and EC-PDTCH channels in CC1 using MCS-6 to MCS-9 need not be tested if corresponding channels for EGPRS with the same MCSs have been tested. Otherwise, for these EC-channels the same antenna configuration applies as for EGPRS, i.e. single RX antenna connector and no RX diversity.

In case an EC-channel in higher Coverage Class than 1 is used (i.e. CC2, CC3 or CC4), the BTS shall be able to distinguish between signals from up to 4 different assigned MSs on the same timeslot in case Overlaid CDMA (see 3GPP TS 45.002 and 3GPP TS 45.004) is supported.

Performance is defined for two SCPIR_UL configurations:

-	0 dB, i.e. all Overlaid CDMA subchannels are received with the same power.

-	9 dB in case of two users per timeslot, or [3]/ [6]/ [9] dB in case of four users per timeslot.

To verify performance, tests are performed for both SCPIR_UL configurations for the supported number of subchannels. In case the manufacturer declares support for multiplexing of three users per timeslot the test for two users per timeslot applies. The requirements for the receiver performance for packet data and control channels in table 7.4-2b apply to Overlaid CDMA subchannel 1 when in specified and supported coverage class and the other subchannel(s) always is/are in CC2.

2nd Modification

### 7.3.2	Test Case

Table 7.3-2: Test Signal input level for Static reference sensitivity measurement for packet switched channels without PAN, ECSD and AMR-WB channels, and 8-PSK modulated AMR and AMR-WB channels

| Normal BTS (GSM400, GSM850, GSM 900, ER-GSM 900, DCS 1800, PCS1900, GSM 700, MXM850* , MXM1900*) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of Channel |  |  | Static | TTI (2) | Comment |
| PDTCH/CS-1 to 3 |  | dBm | -104 | BTTI |  |
| PDTCH/CS-4 |  | dBm | -101 | BTTI |  |
| PRACH/11 bits |  | dBm | -104 | - |  |
| PRACH/8 bits |  | dBm | -104 | - |  |
| PACCH |  | dBm | -104 | - |  |
| PDTCH/MCS-1 |  | dBm | -104 | BTTI & RTTI |  |
| PDTCH/MCS-2 |  | dBm | -104 | BTTI & RTTI |  |
| PDTCH/MCS-3 |  | dBm | -104 | BTTI & RTTI |  |
| PDTCH/MCS-4 |  | dBm | -101.5 | BTTI & RTTI |  |
| PDTCH/MCS-5 |  | dBm | -101 | BTTI & RTTI |  |
| PDTCH/MCS-6 |  | dBm | -99.5 | BTTI & RTTI |  |
| PDTCH/MCS-7 |  | dBm | -96 | BTTI & RTTI |  |
| PDTCH/MCS-8 |  | dBm | -93 | BTTI & RTTI |  |
| PDTCH/MCS-9 |  | dBm | -91.5 | BTTI & RTTI |  |
| PDTCH/UAS-7 |  | dBm | -97.5 | BTTI & RTTI |  |
| PDTCH/UAS-8 |  | dBm | -96.5 | BTTI & RTTI |  |
| PDTCH/UAS-9 |  | dBm | -96 | BTTI & RTTI |  |
| PDTCH/UAS-10 |  | dBm | -95 | BTTI & RTTI |  |
| PDTCH/UAS-11 |  | dBm | -93 | BTTI & RTTI |  |
| PDTCH/UBS-5 |  | dBm | -103.5 | BTTI & RTTI | Input signal generated with narrow pulse-shaping filter |
| PDTCH/UBS-6 |  | dBm | -101.5 | BTTI & RTTI |  |
| PDTCH/UBS-7 |  | dBm | -95 | BTTI & RTTI |  |
| PDTCH/UBS-8 |  | dBm | -93 | BTTI & RTTI |  |
| PDTCH/UBS-9 |  | dBm | -91.5 | BTTI & RTTI |  |
| PDTCH/UBS-10 |  | dBm | -88 | BTTI & RTTI |  |
| PDTCH/UBS-11 |  | dBm | -85.5 | BTTI & RTTI |  |
| PDTCH/UBS-12 |  | dBm | -84.5 | BTTI & RTTI |  |
| PDTCH/UBS-5 |  | dBm | -104 | BTTI & RTTI | Input signal generated with wide pulse-shaping filter |
| PDTCH/UBS-6 |  | dBm | -104 | BTTI & RTTI |  |
| PDTCH/UBS-7 |  | dBm | -101 | BTTI & RTTI |  |
| PDTCH/UBS-8 |  | dBm | -99.5 | BTTI & RTTI |  |
| PDTCH/UBS-9 |  | dBm | -98.5 | BTTI & RTTI |  |
| PDTCH/UBS-10 |  | dBm | -95 | BTTI & RTTI |  |
| PDTCH/UBS-11 |  | dBm | -93 | BTTI & RTTI |  |
| PDTCH/UBS-12 |  | dBm | -91.5 | BTTI & RTTI |  |
| E-FACCH/F |  | dBm | -104 |  |  |
| E-TCH/F43.2 (NT) |  | dBm | -97 |  |  |
| E-TCH/F32 (T) |  | dBm | -104 |  |  |
| E-TCH/F28.8 (T) |  | dBm | -99.5 |  |  |
| E-TCH/F28.8 (NT) |  | dBm | -100 |  |  |
| TCH/WFS12.65 |  | dBm | -104 |  |  |
| TCH/ WFS8.85 |  | dBm | -104 |  |  |
| TCH/WFS6.60 |  | dBm | -104 |  |  |
| O-FACCH/F |  | dBm | -104 |  |  |
| O-FACCH/H |  | dBm | -104 |  |  |
| O-TCH/AHS12.2 |  | dBm | -100,5 |  |  |
| O-TCH/AHS10.2 |  | dBm | -101 |  |  |
| O-TCH/AHS7.95 |  | dBm | -102,5 |  |  |
| O-TCH/AHS7.4 |  | dBm | -102,5 |  |  |
| O-TCH/AHS6.7 |  | dBm | -103 |  |  |
| O-TCH/AHS5.9 |  | dBm | -103,5 |  |  |
| O-TCH/AHS5.15 |  | dBm | -104 |  |  |
| O-TCH/AHS4.75 |  | dBm | -104 |  |  |
| O-TCH/AHS-INB (FER) |  | dBm | -103,5 |  |  |
| O-TCH/AHS (EVSIDUR) |  | dBm | -104 |  |  |
| O-TCH/AHS (EVRFR) |  | dBm | -104 |  |  |
| O-TCH/WFS23.85 |  | dBm | -100,5 |  |  |
| O-TCH/WFS15.85 |  | dBm | -102,5 |  |  |
| O-TCH/WFS12.65 |  | dBm | -104 |  |  |
| O-TCH/WFS8.85 |  | dBm | -104 |  |  |
| O-TCH/WFS6.60 |  | dBm | -104 |  |  |
| O-TCH/WFS-INB (FER) |  | dBm | -104 |  |  |
| O-TCH/WFS (EVSIDUR) |  | dBm | -104 |  |  |
| O-TCH/WFS (EVRFR) |  | dBm | -104 |  |  |
| O-TCH/WHS12.65 |  | dBm | -100,5 |  |  |
| O-TCH/WHS8.85 |  | dBm | -102,5 |  |  |
| O-TCH/WHS6.60 |  | dBm | -103 |  |  |
| O-TCH/WHS-INB (FER) |  | dBm | -103,5 |  |  |
| O-TCH/WHS (EVSIDUR) |  | dBm | -104 |  |  |
| O-TCH/WHS (EVRFR) |  | dBm | -104 |  |  |
| Normal BTS GSM 850 & E-GSM 900 |  |  |  |  |  |
| Type of Channel |  |  | Static | TTI (2) | Comment |
| EC-PACCH/U | CC1 | dBm | [tbd] | - | Two antenna configuration applies (RX diversity) see Note 5 |
| EC-PACCH/U/4 | CC2 | dBm | [tbd] | - |  |
| EC-PACCH/U/8 | CC3 | dBm | [tbd] | - |  |
| EC-PACCH/U/16 | CC4 | dBm | [tbd] | - |  |
| EC-PDTCH/MCS-16) | CC1 | dBm | [tbd] | BTTI |  |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | [tbd] | BTTI |  |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | [tbd] | BTTI |  |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | [tbd] | BTTI |  |
| EC-PDTCH/MCS-57) | CC1 | dBm | [tbd] | BTTI |  |
| 1 TS EC-RACH4) | CC1 | dBm | [tbd] | - |  |
| 1 TS EC-RACH/4 | CC2 | dBm | [tbd] | - |  |
| 1 TS EC-RACH/16 | CC3 | dBm | [tbd] | - |  |
| 1 TS EC-RACH/48 | CC4 | dBm | [tbd] | - |  |
| 2 TS EC-RACH/4 | CC2 | dBm | [tbd] | - |  |
| 2 TS EC-RACH/16 | CC3 | dBm | [tbd] | - |  |
| 2 TS EC-RACH/48 | CC4 | dBm | [tbd] | - |  |
| 2 TS EC-RACH/66 | CC5 | dBm | [tbd] | - |  |
| Normal BTS DCS 1800 & PCS 1900 |  |  |  |  |  |
| Type of Channel |  |  | Static | TTI (2) | Comment |
| EC-PACCH/U | CC1 | dBm | [tbd] | - | Two antenna configuration applies (RX diversity) see Note 5 |
| EC-PACCH/U/4 | CC2 | dBm | [tbd] | - |  |
| EC-PACCH/U/8 | CC3 | dBm | [tbd] | - |  |
| EC-PACCH/U/16 | CC4 | dBm | [tbd] | - |  |
| EC-PDTCH/MCS-16) | CC1 | dBm | [tbd] | BTTI |  |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | [tbd] | BTTI |  |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | [tbd] | BTTI |  |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | [tbd] | BTTI |  |
| EC-PDTCH/MCS-57) | CC1 | dBm | [tbd] | BTTI |  |
| 1 TS EC-RACH4) | CC1 | dBm | [tbd] | - |  |
| 1 TS EC-RACH/4 | CC2 | dBm | [tbd] | - |  |
| 1 TS EC-RACH/16 | CC3 | dBm | [tbd] | - |  |
| 1 TS EC-RACH/48 | CC4 | dBm | [tbd] | - |  |
| 2 TS EC-RACH/4 | CC2 | dBm | [tbd] | - |  |
| 2 TS EC-RACH/16 | CC3 | dBm | [tbd] | - |  |
| 2 TS EC-RACH/48 | CC4 | dBm | [tbd] | - |  |
| 2 TS EC-RACH/66 | CC5 | dBm | [tbd] | - |  |
| *NOTE 1: PDTCH/CS-1 to 4 and ECSD are not applicable for MXM850 and MXM1900 NOTE 2: Test Signal input levels for Static reference sensitivity apply to BTTI without PAN as well as RTTI without PAN for PDTCH indicated in the table. NOTE 3: The requirements for all channel types assume single antenna configuration unless otherwise stated. NOTE 4: Identification of the correct Training sequence is required, see 3GPP TS 45.002 NOTE 5: For the notation of EC-channels, see 3GPP TS 45.003 NOTE 6: For EC-PDTCH/MCS-2 to MCS-4 the input levels and conditions for PDTCH/MCS-2 to MCS-4 apply, assuming BTTI and no receive diversity. NOTE 7: For EC-PDTCH/MCS-6 to MCS-9 the input levels and conditions for PDTCH/MCS-6 to MCS-9 apply, assuming BTTI and no receive diversity. |  |  |  |  |  |

3rd Modification

### 7.3.4	Conformance requirement

Table 7.3-6: Static error performance limits at RX sensitivity level

| Channel type: |  | Error Parameter |  | Limit Value |  |
| --- | --- | --- | --- | --- | --- |
| FACCH/H |  | FER |  | 0,10 % |  |
| FACCH/F |  | FER |  | 0,10 % |  |
| E-FACCH/F |  | FER |  | 5 % |  |
| SDCCH and SACCH |  | FER |  | 0,10 % |  |
| RACH |  | FER |  | 0,50 % |  |
| E-TCH/F43,2 NT |  | BLER |  | 10 % |  |
| E-TCH/F32,0 T |  | BER |  | 0,1 % |  |
| E-TCH/F28,8 T |  | BER |  | 0,1 % |  |
| E-TCH/F28,8 NT |  | BLER |  | 10 % |  |
| TCH/F 14.4 |  | BER |  | 10-5 |  |
| TCH/F9.6 |  | BER |  | 10-5 |  |
| TCH/H4.8 |  | BER |  | 10-5 |  |
| TCH/FS |  | FER |  | 0,10 a % |  |
| - class Ib |  | RBER |  | 0,40/a % |  |
| - class II |  | RBER |  | 2,0 % |  |
| TCH/HS |  | FER |  | 0,025 % |  |
| - class Ib, BFI=0 |  | RBER |  | 0,001 % |  |
| - class II, BFI=0 |  | RBER |  | 0,72 % |  |
| - |  | UFR |  | 0,048 % |  |
| - class Ib, UFI=0 |  | RBER |  | 0,001 % |  |
| - |  | EVSIDR |  | 0,06 % |  |
| SID=0 and BFI=0 |  | RBER |  | 0,001 % |  |
| - |  | ESIDR |  | 0,01 % |  |
| SID=1 or 2 |  | RBER |  | 0,003 % |  |
| TCH/EFS |  | FER |  | 0,1 % |  |
| - class Ib |  | RBER |  | 0,1 % |  |
| - class II |  | RBER |  | 2,0 % |  |
| TCH/AFS12.2, class Ib |  | RBER |  | 0,001 % |  |
| TCH/AFS10.2, class Ib |  | RBER |  | 0,001 % |  |
| TCH/AHS7.95 |  | FER |  | 0,01 % |  |
| - class Ib |  | RBER |  | 0,004 % |  |
| - class II |  | RBER |  | 0,66 % |  |
| TCH/AHS7.4 |  | FER |  | 0,01 % |  |
| - class Ib |  | RBER |  | 0,001 % |  |
| - class II |  | RBER |  | 0,66 % |  |
| TCH/AHS6.7 |  | FER |  | 0,01 % |  |
| - class Ib |  | RBER |  | 0,001 % |  |
| - class II |  | RBER |  | 0,66 % |  |
| TCH/AHS5.9, class II |  | RBER |  | 0,66 % |  |
| TCH/AHS5.15, class II |  | RBER |  | 0,66 % |  |
| TCH/AHS4.75, class II |  | RBER |  | 0,66 % |  |
| TCH/AHS-INB |  | FER |  | 0,013 % |  |
| PDTCH/CS-1 to 4 |  | BLER |  | 10 % |  |
| PDTCH/MCS-1 to 9 |  | BLER |  | 10 % (Note 2) |  |
| PDTCH/UAS-7 to 11 |  | BLER |  | 10 % (Note 3) |  |
| PDTCH/UBS-5 to 12 |  | BLER |  | 10 % (Note 3) |  |
| PAN |  | BLER |  | 5 % |  |
| PRACH/11 bits |  | BLER |  | 15 % |  |
| PRACH/8 bits |  | BLER |  | 15 % |  |
| PACCH |  | BLER |  | 10 % |  |
| EC-PDTCH/U MCS-14) |  | BLER |  | 50% |  |
| EC-PDTCH/U MCS-54) |  | BLER |  | 50% |  |
| EC-RACH5) |  | BLER |  | 20% |  |
| EC-PACCH/U5) |  | BLER |  | 10% |  |
| TCH/WFS12.65 |  | FER |  | 1 % |  |
|  |  | RBER |  | 0,50 % |  |
| TCH/ WFS8.85 |  | FER |  | 1 % |  |
|  |  | RBER |  | 0,50 % |  |
| TCH/WFS6.60 |  | FER |  | 1 % |  |
|  |  | RBER |  | 0,24 % |  |

| Channel type: | Error Parameter | Limit Value |
| --- | --- | --- |
| O-FACCH/F | FER | 5 % |
| O-FACCH/H | FER | 5 % |
| O-TCH/AHS12.2 | FER | 1 % |
| - Class Ib | RBER | 0,71 % |
| O-TCH/AHS10.2 | FER | 1 % |
| - Class Ib | RBER | 0,35 % |
| O-TCH/AHS7.95 | FER | 1 % |
| - Class Ib | RBER | 0,10 % |
| O-TCH/AHS7.4 | FER | 1 % |
| - Class Ib | RBER | 0,16 % |
| O-TCH/AHS6.7 | FER | 1 % |
| - Class Ib | RBER | 0,22 % |
| O-TCH/AHS5.9 | FER | 1 % |
| - Class Ib | RBER | 0,57 % |
| O-TCH/AHS5.15 | FER | 1 % |
| - Class Ib | RBER | 0,15 % |
| O-TCH/AHS4.75 | FER | 1 % |
| - Class Ib | RBER | 0,18 % |
| O-TCH/AHS-INB | FER | 0.5 % |
| O-TCH/AHS (EVSIDUR) | FER | 1 % |
| O-TCH/AHS (EVRFR) | FER | 1 % |
| O-TCH/WFS23.85 | FER | 1 % |
| - Class Ib | RBER | 1,00 % |
| O-TCH/WFS15.85 | FER | 1 % |
| - Class Ib | RBER | 0,50 % |
| O-TCH/WFS12.65 | FER | 1 % |
| - Class Ib | RBER | 0,89 % |
| O-TCH/WFS8.86 | FER | 1 % |
| - Class Ib | RBER | 0,77 % |
| O-TCH/WFS6.6 | FER | 1 % |
| - Class Ib | RBER | 0,05 % |
| O-TCH/WFS-INB | FER | 0.5 % |
| O-TCH/WFS (EVSIDUR) | FER | 1 % |
| O-TCH/WFS (EVRFR) | FER | 1 % |
| O-TCH/WHS12.65 | FER | 1 % |
| - Class Ib | RBER | 0,57 % |
| O-TCH/WHS8.86 | FER | 1 % |
| - Class Ib | RBER | 0,19 % |
| O-TCH/WHS6.6 | FER | 1 % |
| - Class Ib | RBER | 0,23 % |
| O-TCH/WHS-INB | FER | 0.5 % |
| O-TCH/WHS (EVSIDUR) | FER | 1 % |
| O-TCH/WHS (EVRFR) | FER | 1 % |
| NOTE 1: The performance requirements for TCH/WFS inband signalling, SID_UPDATE and RATSCCH are the same as those given for TCH/AFS. It is sufficient to test inband signalling, SID_UPDATE and RATSCCH requirements for only one of the channel types TCH/AFS and TCH/WFS. NOTE 2: The performance requirements for MCS-1 to 4 and MCS-5 to 9 apply to BTTI & RTTI without PAN included. For MCS-1 to 3 and MCS-5 to 8 the requirements also apply for BTTI&RTTI with PAN. NOTE 3: The performance requirements for all respective PDTCH type apply to BTTI & RTTI with or without PAN included. NOTE 4: Applies for all supported Coverage Classes (CC1 to CC4) using MCS-1. For EC-PDTCH/MCS-2 to MCS-4 the requirements for PDTCH/MCS-2 to MCS-4 and similarly for EC-PDTCH/MCS-6 to MCS-9 the requirements for PDTCH/MCS-6 to MCS-9 apply assuming BTTI, no PAN and no receive diversity. NOTE 5: Applies for all supported Coverage Classes (CC1 to CC5). |  |  |

4th Modification

### 7.4.2	Test Case

Table 7.4-2a: Test signal input level for Multipath Reference Sensitivity measurements for EC-channels in different Coverage Classes (CC)

| GSM 900 and GSM 850 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | [TU1.2 (ideal FH)] | TU50 (no FH) |
| EC-PACCH/U | CC1 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-11) | CC1 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-52) | CC1 | dBm | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH3) | CC1 | dBm | [tbd] | - | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dBm | [tbd] | - | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dBm | [tbd] | - | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dBm | [tbd] | - | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dBm | [tbd] | - | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dBm | [tbd] | - | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dBm | [tbd] | - | [tbd] |
| 2 TS EC-RACH/66 | CC5 | dBm | [tbd] | - | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |
| channel |  |  | TU1.2 (no FH) | [TU1.2 (ideal FH)] | TU50 (no FH) |
| EC-PACCH/U | CC1 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-11) | CC1 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-52) | CC1 | dBm | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH3) | CC1 | dBm | [tbd] | - | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dBm | [tbd] | - | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dBm | [tbd] | - | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dBm | [tbd] | - | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dBm | [tbd] | - | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dBm | [tbd] | - | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dBm | [tbd] | - | [tbd] |
| 2 TS EC-RACH/66 | CC5 | dBm | [tbd] | - | [tbd] |
| NOTE 1: For MCS-2, MCS-3 and MCS-4 in CC1, see table 7.4-2. NOTE 2: For MCS-6, MCS-7, MCS-8 and MCS-9 in CC1, see table 7.4-2. NOTE 3: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 4: For the notation of EC-channels, see 3GPP TS 45.003. |  |  |  |  |  |

5th Modification

### 7.5.2	Test Case

Table 7.5-4a: Co-channel interference ratios (C/Ic) for EC-channels

| GSM 900 and GSM 850 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | [TU1.2 (ideal FH)] | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-11) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-52) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH3) | CC1 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/66 | CC5 | dB | [tbd] | - | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | [TU1.2 (Ideal FH)] | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-11) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-52) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH3) | CC1 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/66 | CC5 | dB | [tbd] | - | [tbd] |
| NOTE 1: For MCS-2, MCS-3 and MCS-4 in CC1, see table 7.5-4. NOTE 2: For MCS-6, MCS-7, MCS-8 and MCS-9 in CC1, see table 7.5-4. NOTE 3: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 4: For the notation of EC-channels, see 3GPP TS 45.003. |  |  |  |  |  |

Table 7.5-5c: Adjacent channel interference ratios (C/Ia) for EC-channels

| GSM 900 and GSM 850 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no SFH) | [TU1.2 (ideal SFH)] | TU50 (no SFH) |
| EC-PACCH/U | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-11) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-52) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH3) | CC1 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/66 | CC5 | dB | [tbd] | - | [tbd] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no SFH) | [TU1.2 (Ideal SFH)] | TU50 (no SFH) |
| EC-PACCH/U | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PACCH/U/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-11) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/4 | CC2 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/8 | CC3 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-1/16 | CC4 | dB | [tbd] | [tbd] | [tbd] |
| EC-PDTCH/MCS-52) | CC1 | dB | [tbd] | [tbd] | [tbd] |
| 1 TS EC-RACH3) | CC1 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/4 | CC2 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/16 | CC3 | dB | [tbd] | - | [tbd] |
| 1 TS EC-RACH/48 | CC4 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/4 | CC2 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/16 | CC3 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/48 | CC4 | dB | [tbd] | - | [tbd] |
| 2 TS EC-RACH/66 | CC5 | dB | [tbd] | - | [tbd] |
| NOTE 1: For MCS-2, MCS-3 and MCS-4 in CC1, see table 7.5-3. NOTE 2: For MCS-6, MCS-7, MCS-8 and MCS-9 in CC1, see table 7.5-5. NOTE 3: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 4: For the notation of EC-channels, see 3GPP TS 45.003. |  |  |  |  |  |

End of Modifications