3GPP TSG RAN WG6 #4 	R6-170168

# Hangzhou, P.R. China, May 15-19, 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.005 | CR | 0602 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCL Performance of CC1 to CC4 with 2 PDCHs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Perf |  |  |  |  |  |  | Date: |  | 2017-05-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Compact burst mapping is agreed to be used in uplink for EC-PDTCH/EC-PACCH with 2PDCH allocation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Performance requirement is added in affected tables for EC-PDTCH/EC-PACCH with 2PDCH allocation for CC2 to CC4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Alternative coverage class mappings with 2 PDCHs will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 1z, Table 1aa, Table 1ad, Table 2ah, Table 2ai, Table 2al, Table 2am, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Only performance requirement for CC2 to CC4 with 2PDCHs is considered in affected tables. |  |  |  |  |  |  |  |  |

1st Modification

# Table 1z: Input signal level (for normal BTS) at reference performance for GMSK modulated signals for different Coverage Classes (CC)

| E-GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |
| Channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dBm | -120,5 | -115,5 | -117,5 | -115,5 |
| EC-PACCH/U/4 | CC2 | dBm | -126,5 | -122,0 | -123,5 | -122,0 |
| EC-PACCH/U/8 | CC3 | dBm | -129,5 | -124,5 | -126,0 | -125,0 |
| EC-PACCH/U/16 | CC4 | dBm | -132,0 | -128,0 | -128,0 | -127,5 |
| EC-PACCH/U/2TS/4 | CC2 | dBm | [-126,5] | [-121,9] | [-123,2] | [-122,1] |
| EC-PACCH/U/2TS/8 | CC3 | dBm | [-129,2] | [-124,2] | [-125,2] | [-125,0] |
| EC-PACCH/U/2TS/16 | CC4 | dBm | [-131,3] | [-127,4] | [-126,9] | [-127,2] |
| EC-PACCH/U/2TS/48 | CC5 | dBm | [TBD] | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/MCS-14) | CC1 | dBm | -117,0 | -115,5 | -115,5 | -115,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | -123,0 | -121,5 | -121,5 | -121,5 |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | -126,0 | -124,5 | -124,0 | -123,5 |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | -128,5 | -128,0 | -126,5 | -126,0 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dBm | [-123,1] | [-121,6] | [-121,3] | [-121,3] |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dBm | [-126,1] | [-124,6] | [-123,3] | [-123,4] |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dBm | [-128,3] | [-127,8] | [-125,5] | [-125,9] |
| EC-PDTCH/2TS/MCS-1'/48 | CC5 | dBm | [TBD] | [TBD] | [TBD] | [TBD] |
| 1 TS EC-RACH2) | CC1 | dBm | -116,0 | -113,5 | - | -113,0 |
| 1 TS EC-RACH/4 | CC2 | dBm | -122,0 | -119,0 | - | -118,5 |
| 1 TS EC-RACH/16 | CC3 | dBm | -126,0 | -123,0 | - | -123,0 |
| 1 TS EC-RACH/48 | CC4 | dBm | -128,0 | -126,0 | - | -125,0 |
| 2 TS EC-RACH/4 | CC2 | dBm | -122,5 | -119,5 | - | -119,0 |
| 2 TS EC-RACH/16 | CC3 | dBm | -127,0 | -124,5 | - | -124,0 |
| 2 TS EC-RACH/48 | CC4 | dBm | -129,5 | -128,0 | - | -127,0 |
| 1 TS EC-RACH/30 bits5) | CC1 | dBm | [TBD] | [TBD] | - | [TBD] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |
| channel |  |  | Static | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dBm | -120,5 | -115,5 | -117,5 | -115,0 |
| EC-PACCH/U/4 | CC2 | dBm | -126,5 | -122,0 | -123,5 | -122,0 |
| EC-PACCH/U/8 | CC3 | dBm | -129,5 | -124,5 | -126,0 | -124,5 |
| EC-PACCH/U/16 | CC4 | dBm | -132,0 | -128,0 | -128,0 | -127,5 |
| EC-PACCH/U/2TS/4 | CC2 | dBm | [-126,5] | [-121,9] | [-123,2] | [-122,0] |
| EC-PACCH/U/2TS/8 | CC3 | dBm | [-129,2] | [-124,2] | [-125,2] | [-124,8] |
| EC-PACCH/U/2TS/16 | CC4 | dBm | [-131,3] | [-127,5] | [-126,9] | [-127,2] |
| EC-PACCH/U/2TS/48 | CC5 | dBm | [TBD] | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/MCS-14) | CC1 | dBm | -117,0 | -115,5 | -115,5 | -115,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dBm | -123,0 | -121,5 | -121,5 | -121,0 |
| EC-PDTCH/MCS-1/8 | CC3 | dBm | -126,0 | -124,5 | -124,0 | -123,5 |
| EC-PDTCH/MCS-1/16 | CC4 | dBm | -129,0 | -128,0 | -126,5 | -125,5 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dBm | [-123,1] | [-121,7] | [-121,3] | [-120,8] |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dBm | [-126,0] | [-124,5] | [-123,3] | [-122,9] |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dBm | [-128,3] | [-127,7] | [-125,6] | [-125,4] |
| EC-PDTCH/2TS/MCS-1'/48 | CC5 | dBm | [TBD] | [TBD] | [TBD] | [TBD] |
| 1 TS EC-RACH2) | CC1 | dBm | -116,5 | -113,5 | - | -113,5 |
| 1 TS EC-RACH/4 | CC2 | dBm | -122,0 | -119,0 | - | -118,5 |
| 1 TS EC-RACH/16 | CC3 | dBm | -126,0 | -123,0 | - | -122,5 |
| 1 TS EC-RACH/48 | CC4 | dBm | -128,0 | -126,5 | - | -125,0 |
| 2 TS EC-RACH/4 | CC2 | dBm | -122,5 | -119,5 | - | -119,0 |
| 2 TS EC-RACH/16 | CC3 | dBm | -127,5 | -124,5 | - | -124,0 |
| 2 TS EC-RACH/48 | CC4 | dBm | -129,5 | -128,0 | - | -126,5 |
| 1 TS EC-RACH/30 bits5) | CC1 | dBm | [TBD] | [TBD] | - | [TBD] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 1a apply for Static and TU50 (no FH) and TU50 (ideal FH) propagation conditions. NOTE 5: The performance requirement applies for the random access message carrying 30 information bits (see 3GPP TS 45.003) sent using the Extended AB burst format (see 3GPP TS 45.002) as part of the Multilateration Timing Advance procedure using the Extended Access Burst method in EC operation. |  |  |  |  |  |  |

2nd Modification

# Table 1aa: Input signal level (for EC-GSM-IoT MS) at reference performance for GMSK modulated signals for different Coverage Classes (CC)

| E-GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |
| Channel |  |  | Static |  | TU1.2 (no FH) |  | TU1.21) (ideal FH) |  | TU50 (no FH) |  |
| EC-SCH | - | dBm |  | -124.5 |  | -121.5 |  | - |  | -123.0 |  |
| EC-BCCH | - | dBm |  | -123.5 |  | -119.5 |  | - |  | -121.5 |  |
| EC-PACCH/D | CC1 | dBm |  | -114.5 |  | -106.5 |  | -110.0 |  | -109.0 |  |
| EC-PACCH/D/4 | CC2 | dBm |  | -120.5 |  | -112.5 |  | -116.0 |  | -115.0 |  |
| EC-PACCH/D/8 | CC3 | dBm |  | -123.5 |  | -115.0 |  | -120.5 |  | -119.5 |  |
| EC-PACCH/D/16 | CC4 | dBm |  | -126.0 |  | -118.0 |  | -124.0 |  | -123.0 |  |
| EC-PACCH/D/2TS/4 | CC2 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-PACCH/D/2TS/8 | CC3 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-PACCH/D/2TS/16 | CC4 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-CCCH/D2) | CC1 | dBm |  | -111.0 |  | -103.0 |  | - |  | -103.5 |  |
| EC-CCCH/D/8 | CC2 | dBm |  | -119.5 |  | -114.5 |  | - |  | -116.5 |  |
| EC-CCCH/D/16 | CC3 | dBm |  | -122.0 |  | -117.0 |  | - |  | -120.0 |  |
| EC-CCCH/D/32 | CC4 | dBm |  | -124.0 |  | -120.5 |  | - |  | -122.5 |  |
| EC-PDTCH/MCS-14) | CC1 | dBm |  | -112.0 |  | -106.5 |  | -108.5 |  | -108.0 |  |
| EC-PDTCH/MCS-1/4 | CC2 | dBm |  | -118.0 |  | -112.5 |  | -114.5 |  | -114.0 |  |
| EC-PDTCH/MCS-1/8 | CC3 | dBm |  | -121.0 |  | -115.5 |  | -118.0 |  | -117.5 |  |
| EC-PDTCH/MCS-1/16 | CC4 | dBm |  | -124.0 |  | -118.5 |  | -122.0 |  | -121.5 |  |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |
| channel |  |  | Static |  | TU1.2 (no FH) |  | TU1.21) (ideal FH) |  | TU50 (no FH) |  |
| EC-SCH | - | dBm |  | -124.5 |  | -122.0 |  | - |  | -123.0 |  |
| EC-BCCH | - | dBm |  | -123.5 |  | -120.0 |  | - |  | -121.5 |  |
| EC-PACCH/D | CC1 | dBm |  | -114.5 |  | -106.5 |  | -109.5 |  | -109.5 |  |
| EC-PACCH/D/4 | CC2 | dBm |  | -120.5 |  | -112.0 |  | -116.0 |  | -115.5 |  |
| EC-PACCH/D/8 | CC3 | dBm |  | -123.5 |  | -115.5 |  | -120.5 |  | -120.0 |  |
| EC-PACCH/D/16 | CC4 | dBm |  | -126.0 |  | -118.5 |  | -124.0 |  | -123.5 |  |
| EC-PACCH/D/2TS/4 | CC2 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-PACCH/D/2TS/8 | CC3 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-PACCH/D/2TS/16 | CC4 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-CCCH/D2) | CC1 | dBm |  | -111.0 |  | -103.5 |  | - |  | -104.0 |  |
| EC-CCCH/D/8 | CC2 | dBm |  | -119.5 |  | -114.5 |  | - |  | -117.0 |  |
| EC-CCCH/D/16 | CC3 | dBm |  | -122.0 |  | -117.5 |  | - |  | -120.0 |  |
| EC-CCCH/D/32 | CC4 | dBm |  | -124.0 |  | -121.0 |  | - |  | -122.5 |  |
| EC-PDTCH/MCS-14) | CC1 | dBm |  | -112.0 |  | -106.5 |  | -108.5 |  | -108.5 |  |
| EC-PDTCH/MCS-1/4 | CC2 | dBm |  | -118.0 |  | -112.5 |  | -114.5 |  | -114.0 |  |
| EC-PDTCH/MCS-1/8 | CC3 | dBm |  | -121.0 |  | -115.5 |  | -118.0 |  | -118.0 |  |
| EC-PDTCH/MCS-1/16 | CC4 | dBm |  | -124.0 |  | -118.5 |  | -122.0 |  | -121.5 |  |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dBm |  | [TBD] |  | [TBD] |  | [TBD] |  | [TBD] |  |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test NOTE 2: The performance requirements for EC-CCCH apply for EC-PCH and EC-AGCH. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 1a apply for Static and TU50 (no FH) and TU50 (ideal FH) propagation conditions |  |  |  |  |  |  |  |  |  |  |  |  |

3rd Modification

# Table 1ad: Input signal level (for normal BTS) at reference performance for GMSK modulated signals in Coverage Classes 2, 3 and 4 (CC2-CC4) using Overlaid CDMA

| GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of channel |  |  |  |  | Propagation conditions: TU1.2 no FH |  |  |  |  |  |  |
|  |  |  |  |  | 2 users per time slot |  |  |  | 4 users per time slot |  |  |
| SCPIR_UL |  |  |  |  | 0 dB |  | 9 dB |  | 0 dB1) |  | 3, 6, 9 dB |
| EC-PACCH/U/4 | CC2 |  | dBm | -116,0 |  | -115,5 |  | -115,5 |  | -114,5 |  |
| EC-PACCH/U/8 | CC3 |  | dBm | -119,0 |  | -118,5 |  | -118,5 |  | -118,0 |  |
| EC-PACCH/U/16 | CC4 |  | dBm | -121,5 |  | -121,5 |  | -121,5 |  | -121,5 |  |
| EC-PACCH/U/2TS/4 | CC2 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PACCH/U/2TS/8 | CC3 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PACCH/U/2TS/16 | CC4 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PACCH/U/2TS/48 | CC5 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PDTCH/MCS-1/4 | CC2 |  | dBm | -118,5 |  | -118,0 |  | -118,0 |  | -117,5 |  |
| EC-PDTCH/MCS-1/8 | CC3 |  | dBm | -121,5 |  | -121,0 |  | -121,5 |  | -121,0 |  |
| EC-PDTCH/MCS-1/16 | CC4 |  | dBm | -124,5 |  | -124,0 |  | -124,5 |  | -124,0 |  |
| EC-PDTCH/2TS/MCS-1/4 | CC2 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PDTCH/2TS/MCS-1/8 | CC3 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PDTCH/2TS/MCS-1/16 | CC4 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PDTCH/2TS/MCS-1'/48 | CC5 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| DCS 1800 & PCS 1900 |  |  |  |  |  |  |  |  |  |  |  |
| Type of channel |  |  |  |  | Propagation conditions: TU1.2 no FH |  |  |  |  |  |  |
|  |  |  |  |  | 2 users per time slot |  |  |  | 4 users per time slot |  |  |
| SCPIR_UL |  |  |  |  | 0 dB |  | 9 dB |  | 0 dB1) |  | 3, 6, 9 dB |
| EC-PACCH/U/4 | CC2 | dBm |  |  | -116,0 |  | -115,5 |  | -115,5 |  | -114,5 |
| EC-PACCH/U/8 | CC3 | dBm |  |  | -119,0 |  | -118,5 |  | -118,5 |  | -118,0 |
| EC-PACCH/U/16 | CC4 | dBm |  |  | -122,0 |  | -121,5 |  | -122,0 |  | -121,5 |
| EC-PACCH/U/2TS/4 | CC2 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PACCH/U/2TS/8 | CC3 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PACCH/U/2TS/16 | CC4 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PACCH/U/2TS/48 | CC5 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PDTCH/MCS-1/4 | CC2 | dBm |  |  | -118,5 |  | -118,0 |  | -118,0 |  | -117,5 |
| EC-PDTCH/MCS-1/8 | CC3 | dBm |  |  | -121,5 |  | -121,0 |  | -121,5 |  | -121,0 |
| EC-PDTCH/MCS-1/16 | CC4 | dBm |  |  | -124,5 |  | -124,0 |  | -124,0 |  | -124,0 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PDTCH/2TS/MCS-1/8 | CC3 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PDTCH/2TS/MCS-1/16 | CC4 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| EC-PDTCH/2TS/MCS-1'/48 | CC5 |  | dBm | [TBD] |  | [TBD] |  | - |  | - |  |
| NOTE 1: All subchannels signals have the same signal level, i.e. all SCPIR are 0 dB. |  |  |  |  |  |  |  |  |  |  |  |

4th Modification

# Table 2ah: Cochannel interference ratio at reference performance (for normal BTS) for GMSK modulated signals

| E-GSM 900 and GSM 850 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | -1,0 | -1,0 | 0,0 |
| EC-PACCH/U/4 | CC2 | dB | -8,0 | -9,0 | -8,0 |
| EC-PACCH/U/8 | CC3 | dB | -11,0 | -12,0 | -11,5 |
| EC-PACCH/U/16 | CC4 | dB | -13,5 | -14,5 | -14,5 |
| EC-PACCH/U/2TS/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/48 | CC5 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/MCS-14) | CC1 | dB | 0,0 | -0,5 | -0,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -7,5 | -7,5 | -7,5 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -11,0 | -10,5 | -10,5 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -14,0 | -13,0 | -13,0 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1'/48 | CC5 | dB | [TBD] | [TBD] | [TBD] |
| 1 TS EC-RACH2) | CC1 | dB | 2,0 | - | 2,0 |
| 1 TS EC-RACH/4 | CC2 | dB | -5,5 | - | -4,5 |
| 1 TS EC-RACH/16 | CC3 | dB | -10,0 | - | -10,0 |
| 1 TS EC-RACH/48 | CC4 | dB | -12,5 | - | -12,5 |
| 2 TS EC-RACH/4 | CC2 | dB | -5,0 | - | -4,5 |
| 2 TS EC-RACH/16 | CC3 | dB | -10,5 | - | -10,5 |
| 2 TS EC-RACH/48 | CC4 | dB | -13,5 | - | -13,5 |
| 1 TS EC-RACH/30bits5) | CC1 | dBm | [TBD] | - | [TBD] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | -0,5 | -1,0 | 1,0 |
| EC-PACCH/U/4 | CC2 | dB | -7,5 | -9,0 | -7,5 |
| EC-PACCH/U/8 | CC3 | dB | -10,5 | -11,5 | -11,0 |
| EC-PACCH/U/16 | CC4 | dB | -13,5 | -14,0 | -14,0 |
| EC-PACCH/U/2TS/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/48 | CC5 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/MCS-14) | CC1 | dB | -0,5 | -0,5 | -0,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -7,0 | -7,5 | -7,0 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -10,5 | -10,0 | -10,0 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -13,5 | -12,5 | -13,0 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1'/48 | CC5 | dB | [TBD] | [TBD] | [TBD] |
| 1 TS EC-RACH2) 5) | CC1 | dB | 2,0 | - | 2,0 |
| 1 TS EC-RACH/4 | CC2 | dB | -5,5 | - | -4,0 |
| 1 TS EC-RACH/16 | CC3 | dB | -9,5 | - | -10,0 |
| 1 TS EC-RACH/48 | CC4 | dB | -12,5 | - | -13,0 |
| 2 TS EC-RACH/4 | CC2 | dB | -5,0 | - | -4,5 |
| 2 TS EC-RACH/16 | CC3 | dB | -10,5 | - | -10,0 |
| 2 TS EC-RACH/48 | CC4 | dB | -13,5 | - | -13,5 |
| 1 TS EC-RACH/30bits5) | CC1 | dBm | [TBD] | - | [TBD] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 2a apply for TU3 (low band) and TU1,5 (high band) and TU50 (no FH) propagation conditions together with the conditions for EGPRS in subclause 6.3.4. NOTE 5: The performance requirement applies for the random access message carrying 30 information bits (see 3GPP TS 45.003) sent using the Extended AB burst format (see 3GPP TS 45.002) as part of the Multilateration Timing Advance procedure using the Extended Access Burst method in EC operation. |  |  |  |  |  |

5th Modification

# Table 2ai: Cochannel interference ratio at reference performance (for EC-GSM-IoT MS) for GMSK modulated signals

| E-GSM 900 and GSM 850 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  |  |  |  |  | Propagation conditions |  |  |
| Channel |  |  |  |  |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-SCH | - |  |  | dB |  |  | -12.0 | - | -13.5 |
| EC-BCCH | - |  |  | dB |  |  | -13.5 | - | -15.0 |
| EC-PACCH/D | CC1 |  |  | dB |  |  | -1.5 | -5.0 | -4.0 |
| EC-PACCH/D/4 | CC2 |  |  | dB |  |  | -5.5 | -10.0 | -9.0 |
| EC-PACCH/D/8 | CC3 |  |  | dB |  |  | -6.5 | -13.0 | -12.0 |
| EC-PACCH/D/16 | CC4 |  |  | dB |  |  | -8.0 | -15.5 | -15.0 |
| EC-PACCH/D/2TS/4 | CC2 |  |  | dB |  |  | [TBD] | [TBD] | [TBD] |
| EC-PACCH/D/2TS/8 | CC3 |  |  | dB |  |  | [TBD] | [TBD] | [TBD] |
| EC-PACCH/D/2TS/16 | CC4 |  |  | dB |  |  | [TBD] | [TBD] | [TBD] |
| EC-CCCH/D2) | CC1 |  |  | dB |  |  | 2.5 | - | 0.5 |
| EC-CCCH/D/8 | CC2 |  |  | dB |  |  | -7.5 | - | -10.5 |
| EC-CCCH/D/16 | CC3 |  |  | dB |  |  | -9.0 | - | -13.0 |
| EC-CCCH/D/32 | CC4 |  |  | dB |  |  | -12.0 | - | -15.0 |
| EC-PDTCH/MCS-14) | CC1 |  |  | dB |  |  | -1.5 | -3.0 | -2.5 |
| EC-PDTCH/MCS-1/4 | CC2 |  |  | dB |  |  | -6.5 | -8.0 | -7.5 |
| EC-PDTCH/MCS-1/8 | CC3 |  |  | dB |  |  | -8.5 | -12.0 | -11.5 |
| EC-PDTCH/MCS-1/16 | CC4 |  |  | dB |  |  | -10.5 | -14.5 | -14.5 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 |  |  | dB |  |  | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/8 | CC3 |  |  | dB |  |  | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/16 | CC4 |  |  | dB |  |  | [TBD] | [TBD] | [TBD] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |  |  |  |  |
| Type of |  |  |  |  |  |  | Propagation conditions |  |  |
| Channel |  |  |  |  |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU50 (no FH) |
| EC-SCH |  | - |  |  | dB |  | -12.5 | - | -13.5 |
| EC-BCCH |  | - |  |  | dB |  | -14.0 | - | -15.0 |
| EC-PACCH/D |  | CC1 |  |  | dB |  | -1.0 | -5.0 | -5.0 |
| EC-PACCH/D/4 |  | CC2 |  |  | dB |  | -5.0 | -10.0 | -10.0 |
| EC-PACCH/D/8 |  | CC3 |  |  | dB |  | -7.0 | -13.0 | -12.5 |
| EC-PACCH/D/16 |  | CC4 |  |  | dB |  | -9.0 | -15.5 | -15.0 |
| EC-PACCH/D/2TS/4 |  | CC2 |  |  | dB |  | [TBD] | [TBD] | [TBD] |
| EC-PACCH/D/2TS/8 |  | CC3 |  |  | dB |  | [TBD] | [TBD] | [TBD] |
| EC-PACCH/D/2TS/16 |  | CC4 |  |  | dB |  | [TBD] | [TBD] | [TBD] |
| EC-CCCH/D2) |  |  | CC1 |  |  | dB | 2.0 | - | 0.5 |
| EC-CCCH/D/8 |  |  | CC2 |  |  | dB | -7.5 | - | -11.0 |
| EC-CCCH/D/16 |  |  | CC3 |  |  | dB | -9.5 | - | -13.5 |
| EC-CCCH/D/32 |  |  | CC4 |  |  | dB | -13.0 | - | -15.0 |
| EC-PDTCH/MCS-14) |  | CC1 |  |  | dB |  | -1.0 | -2.5 | -2.5 |
| EC-PDTCH/MCS-1/4 |  | CC2 |  |  | dB |  | -6.5 | -8.0 | -8.0 |
| EC-PDTCH/MCS-1/8 |  | CC3 |  |  | dB |  | -8.5 | -11.5 | -11.5 |
| EC-PDTCH/MCS-1/16 |  | CC4 |  |  | dB |  | -10.5 | -14.5 | -14.5 |
| EC-PDTCH/2TS/MCS-1/4 |  | CC2 |  |  | dB |  | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/8 |  | CC3 |  |  | dB |  | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/16 |  | CC4 |  |  | dB |  | [TBD] | [TBD] | [TBD] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test NOTE 2: The performance requirements for EC-CCCH apply for EC-PCH and EC-AGCH. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 2a apply for TU3 (low band) and TU1,5 (high band) and TU50 (no FH) propagation conditions together with the conditions for EGPRS in subclause 6.3.4. |  |  |  |  |  |  |  |  |  |

6th Modification

# Table 2al: Adjacent channel interference ratio at reference performance (for normal BTS) for GMSK modulated signals

| E-GSM 900 and GSM 850 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | -19,5 | -21,0 | -19,0 |
| EC-PACCH/U/4 | CC2 | dB | -27,0 | -29,5 | -27,0 |
| EC-PACCH/U/8 | CC3 | dB | -28,0 | -32,5 | -29,0 |
| EC-PACCH/U/16 | CC4 | dB | -29,0 | -35,0 | -30,5 |
| EC-PACCH/U/2TS/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/48 | CC5 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/MCS-14) | CC1 | dB | -21,0 | -20,5 | -20,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -30,0 | -29,5 | -29,5 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -32,0 | -32,5 | -32,0 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -33,0 | -35,0 | -33,0 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1'/48 | CC5 | dB | [TBD] | [TBD] | [TBD] |
| 1 TS EC-RACH2) | CC1 | dB | -14,5 | - | -14,5 |
| 1 TS EC-RACH/4 | CC2 | dB | -23,0 | - | -23,0 |
| 1 TS EC-RACH/16 | CC3 | dB | -26,0 | - | -26,0 |
| 1 TS EC-RACH/48 | CC4 | dB | -27,5 | - | -27,5 |
| 2 TS EC-RACH/4 | CC2 | dB | -24,5 | - | -24,5 |
| 2 TS EC-RACH/16 | CC3 | dB | -27,0 | - | -27,5 |
| 2 TS EC-RACH/48 | CC4 | dB | -29,5 | - | -29,5 |
| 1 TS EC-RACH/30bits5) | CC1 | dBm | [TBD] | - | [TBD] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU50 (no FH) |
| EC-PACCH/U | CC1 | dB | -19,5 | -21,0 | -19,5 |
| EC-PACCH/U/4 | CC2 | dB | -25,0 | -30,0 | -27,0 |
| EC-PACCH/U/8 | CC3 | dB | -26,5 | -32,5 | -28,5 |
| EC-PACCH/U/16 | CC4 | dB | -28,0 | -35,0 | -30,0 |
| EC-PACCH/U/2TS/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/U/2TS/48 | CC5 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/MCS-14) | CC1 | dB | -21,0 | -20,5 | -20,5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -30,0 | -29,5 | -29,5 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -31,5 | -31,5 | -32,0 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -32,5 | -35,0 | -32,5 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1'/48 | CC5 | dB | [TBD] | [TBD] | [TBD] |
| 1 TS EC-RACH2) | CC1 | dB | -14,5 | - | -14,5 |
| 1 TS EC-RACH/4 | CC2 | dB | -23,0 | - | -23,0 |
| 1 TS EC-RACH/16 | CC3 | dB | -26,0 | - | -26,5 |
| 1 TS EC-RACH/48 | CC4 | dB | -27,5 | - | -27,5 |
| 2 TS EC-RACH/4 | CC2 | dB | -24,5 | - | -24,5 |
| 2 TS EC-RACH/16 | CC3 | dB | -27,5 | - | -28,5 |
| 2 TS EC-RACH/48 | CC4 | dB | -30,0 | - | -30,5 |
| 1 TS EC-RACH/30bits5) | CC1 | dBm | [TBD] | - | [TBD] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: Identification of the correct Training sequence is required, see 3GPP TS 45.002. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 2a apply for TU3 (low band) and TU1,5 (high band) and TU50 (no FH) propagation conditions together with the conditions for EGPRS in table 6.3-1b and subclause 6.3.4. NOTE 5: The performance requirement applies for the random access message carrying 30 information bits (see 3GPP TS 45.003) send using the Extended AB burst format (see 3GPP TS 45.002) as part of the Multilateration Timing Advance procedure using the Extended Access Burst method in EC operation. |  |  |  |  |  |

7th Modification

# Table 2am: Adjacent channel interference ratio at reference performance (for EC-GSM-IoT MS) for GMSK modulated signals

| E-GSM 900 and GSM 850 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (ideal FH) | TU50 (no FH) |
| EC-SCH | - | dB | -28.5 | - | -30.0 |
| EC-BCCH | - | dB | -31.0 | - | -32.5 |
| EC-PACCH/D | CC1 | dB | -18.5 | -23.0 | -22.0 |
| EC-PACCH/D/4 | CC2 | dB | -21.0 | -26.5 | -25.5 |
| EC-PACCH/D/8 | CC3 | dB | -22.0 | -29.0 | -28.0 |
| EC-PACCH/D/16 | CC4 | dB | -23.5 | -30.5 | -30.0 |
| EC-PACCH/D/2TS/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/D/2TS/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/D/2TS/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-CCCH/D2) | CC1 | dB | -15.0 | - | -17.5 |
| EC-CCCH/D/8 | CC2 | dB | -24.0 | - | -27.5 |
| EC-CCCH/D/16 | CC3 | dB | -25.0 | - | -29.0 |
| EC-CCCH/D/32 | CC4 | dB | -27.5 | - | -30.5 |
| EC-PDTCH/MCS-14) | CC1 | dB | -20.0 | -21.5 | -21.5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -24.0 | -26.0 | -25.5 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -26.0 | -29.5 | -29.0 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -27.5 | -32.0 | -32.0 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| DCS 1800 and PCS 1900 |  |  |  |  |  |
| Type of |  |  | Propagation conditions |  |  |
| Channel |  |  | TU1.2 (no FH) | TU1.21) (Ideal FH) | TU50 (no FH) |
| EC-SCH | - | dB | -29.0 | - | -30.5 |
| EC-BCCH | - | dB | -32.0 | - | -33.0 |
| EC-PACCH/D | CC1 | dB | -18.0 | -23.0 | -23.0 |
| EC-PACCH/D/4 | CC2 | dB | -21.0 | -26.5 | -26.5 |
| EC-PACCH/D/8 | CC3 | dB | -22.5 | -29.0 | -28.5 |
| EC-PACCH/D/16 | CC4 | dB | -24.0 | -30.5 | -30.5 |
| EC-PACCH/D/2TS/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/D/2TS/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PACCH/D/2TS/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| EC-CCCH/D2) | CC1 | dB | -15.5 | - | -18.5 |
| EC-CCCH/D/8 | CC2 | dB | -24.5 | - | -27.5 |
| EC-CCCH/D/16 | CC3 | dB | -25.5 | - | -29.5 |
| EC-CCCH/D/32 | CC4 | dB | -28.5 | - | -31.0 |
| EC-PDTCH/MCS-14) | CC1 | dB | -20.0 | -21.5 | -21.5 |
| EC-PDTCH/MCS-1/4 | CC2 | dB | -24.0 | -26.0 | -26.0 |
| EC-PDTCH/MCS-1/8 | CC3 | dB | -26.0 | -29.5 | -29.5 |
| EC-PDTCH/MCS-1/16 | CC4 | dB | -28.0 | -32.0 | -32.0 |
| EC-PDTCH/2TS/MCS-1/4 | CC2 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/8 | CC3 | dB | [TBD] | [TBD] | [TBD] |
| EC-PDTCH/2TS/MCS-1/16 | CC4 | dB | [TBD] | [TBD] | [TBD] |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. NOTE 2: The performance requirements for EC-CCCH apply for EC-PCH and EC-AGCH. NOTE 3: For the notation of EC-channels, see 3GPP TS 45.003. NOTE 4: For MCS-2, MCS-3 and MCS-4 in CC1 the requirements in table 2a apply for TU3 (low band) and TU1,5 (high band) and TU50 (no FH) propagation conditions together with the conditions for EGPRS in table 6.3-1a and subclause 6.3.4. |  |  |  |  |  |

End of Modifications