3GPP TSG RAN WG6 #4 	R6-170226

# Hangzhou, P.R. China, May 15-19, 2017	    	(Revision of R6-170162)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.002 | CR | 0209 | rev | 1 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of ESAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Core |  |  |  |  |  |  | Date: |  | 2017-05-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of ESAB for EC-RACH CC5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | New mapping method and new synchronization sequence of ESAB are considered in the affected clauses for EC-RACH CC5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ESAB for EC-RACH CC5 will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.2, 5.2.7, 5.2.10, 6.3.4.3, 6.5.1, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064 CR XXXX TS 45.001 CR XXXX TS 45.003 CR XXXX TS 45.004 CR XXXX TS 45.005 CR XXXX TS 45.008 CR XXXX TS 44.018 CR XXXX |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.021 CR XXXX |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modification

### 5.2.1	General

A burst is a period of RF carrier which is modulated by a data stream. A burst therefore represents the physical content of a timeslot except for the case of ESAB (see sub-clause 5.2.10) which represents the physical content of 2 consecutive timeslots.

### 5.2.2	Types of burst and burst timing

A timeslot is divided into 156,25 normal symbol periods or 187,5 reduced symbol periods (see 3GPP TS 45.010). For normal symbol period bursts, an integral symbol period implementation option with a timeslot length of 157 normal symbol periods on timeslots with TN = 0 and 4, and 156 normal symbol periods on timeslots with TN = 1, 2, 3, 5, 6 and 7 can be used. In this case, the same timeslot length is also used for reduced symbol period bursts, resulting in a timeslot length of 188.4 reduced symbol periods on timeslots with TN = 0 and 4, and 187.2 reduced symbol periods on timeslots with TN = 1, 2, 3, 5, 6 and 7. In case of EC-GSM-IoT, only normal symbol period bursts with the integral symbol period implementation option shall be used.

The modulating rate is assumed to be the normal symbol rate (see 3GPP TS 45.004) unless otherwise stated.

For GMSK modulation (see 3GPP TS 45.004) a symbol is equivalent to a bit. A particular bit period within a timeslot is referenced by a bit number (BN), with the first bit period being numbered 0, and the last (1/4) bit period being numbered 156, in case of non-integer symbol timeslot.

For AQPSK modulation (see 3GPP TS 45.004) one symbol corresponds to a pair of bits mapped as described in 3GPP TS 45.004. Each bit in the bit pair belongs to a different VAMOS subchannel in the VAMOS pair. A particular bit period within a timeslot is referenced by a bit number (BN), with the first bit being numbered 0, and the last (1/2) bit being numbered 312, in case of non-integer symbol timeslot. The bit pairs are mapped to the symbols as described in 3GPP TS 45.004. The AQPSK modulation is used only at the normal symbol rate and only in the downlink.

For 8PSK modulation (see 3GPP TS 45.004) one symbol corresponds to three bits. A particular bit period within a timeslot is referenced by a bit number (BN), with the first bit being numbered 0, and the last (3/4) bit being numbered 468, in case of non-integer symbol timeslot. The bits are mapped to symbols in ascending order according to 3GPP TS 45.004.

For 16QAM modulation (see 3GPP TS 45.004) one symbol corresponds to four bits. A particular bit period within a timeslot is referenced by a bit number (BN), with the first bit being numbered 0, and the last bit being numbered 624 for normal symbol rate bursts. When the modulating rate is the higher symbol rate, the last bit is numbered 749. The bits are mapped to symbols in ascending order according to 3GPP TS 45.004.

For 32QAM modulation (see 3GPP TS 45.004) one symbol corresponds to five bits. A particular bit period within a timeslot is referenced by a bit number (BN), with the first bit being numbered 0, and the last (1/4) bit being numbered 781, in case of non-integer symbol timeslot for normal symbol rate bursts. When the modulating rate is the higher symbol rate, the last (1/2, in case of uniform timeslot length) bit is numbered 937. The bits are mapped to symbols in ascending order according to 3GPP TS 45.004.

For QPSK modulation (see 3GPP TS 45.004) one symbol corresponds to two bits. A particular bit period within a timeslot is referenced by a bit number (BN), with the first bit being numbered 0, and the last bit being numbered 374. The bits are mapped to symbols in ascending order according to 3GPPTS45.004. QPSK modulation is used only at the higher symbol rate.

In the subclauses following, the transmission timing of a burst within a timeslot is defined in terms of bit number. The bit with the lowest bit number is transmitted first.

Different types of burst exist in the system. One characteristic of a burst is its useful duration. This document, in the subclauses following, defines full bursts of 147 normal symbol periods useful duration, full bursts of 176 reduced symbol periods useful duration (see 3GPP TS 45.010), a short burst of 87 normal symbol periods useful duration (see subclause 5.2.7) and a long burst of 244 normal symbol periods useful duration (see subclause 5.2.10). The useful part of a burst is defined as beginning from half way through symbol number 0. The definition of the useful part of a burst needs to be considered in conjunction with the requirements placed on the phase and amplitude characteristics of a burst as specified in 3GPP TS 45.004 and 45.005.

The period between bursts appearing in successive timeslots is termed the guard period. Subclause 5.2.8 details constraints which relate to the guard period.

*** For Information ***

### 5.2.8	Guard period

The guard period is provided because it is required for the MSs that transmission be attenuated for the period between bursts with the necessary ramp up and down occurring during the guard periods as defined in 3GPP TS 45.005. A base transceiver station is not required to have a capability to ramp down and up between adjacent bursts, but is required to have a capability to ramp down and up for non-used time-slots, as defined in 3GPP TS 45.005. In any case where the amplitude of transmission is ramped up and down, then by applying an appropriate modulation bit stream interference to other RF channels can be minimized.

In some cases, there is more than one option for the duration of the guard period.  The actual duration of the guard period between bursts with different symbol rates is defined in 3GPP TS 45.010.

2nd Modification

### 5.2.7	Access burst (AB)

Table 5.2.7-1: Access Burst

| Bit Number (BN) | Length of field | Content of field | Definition |
| --- | --- | --- | --- |
| 0-7 | 8 | Extended tail bits | see table 5.2.7-2 |
| 8-48 | 41 | Synch. sequence bits | see table 5.2.7-3 and table 5.2.7-4 |
| 49-84 | 36 | Encrypted bits (e0..e35) | see 3GPP TS 45.003 |
| 85-87 | 3 | Tail bits | see table 5.2.7-2 |
| 88-155 88-156 | 68, 68.25, or 691 | Extended guard period (bits) | see subclause 5.2.8 |
| NOTE1: 68 or 69 always applies in case of an EC-GSM-IoT access burst sent using CC1 to CC4, see 3GPP TS 45.010 |  |  |  |

The “extended tail bits” and the “tail bits” are defined as modulating bits with states according to table 5.2.7-2.

Table 5.2.7-2: Tail bits and extended tail bits

| Tail bit type | Bit number (BN) | Modulating bits |
| --- | --- | --- |
| Extended tail bits | (BN0, BN1, BN2 .. BN7) | (0, 0, 1, 1, 1, 0, 1, 0) |
| Tail bits | (BN85, BN86, BN87) | (0, 0, 0) |

The “Synch. sequence bits” are defined as modulating bits with states according to table 5.2.7-3.

Table 5.2.7-3: Synch. Sequence bits

| Bit number (BN) | Modulating bits | Applicability |
| --- | --- | --- |
| (BN8, BN9 .. BN48) | (0, 1, 0, 0, 1, 0, 1, 1, 0, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 1, 1, 0, 0, 1, 1, 0, 1, 0, 1, 0, 1, 0, 0, 0, 1, 1, 1, 1, 0, 0, 0) | unless explicitly stated otherwise this training (synchronization) sequence "TS0" shall be used (includes PEO and EC operation), see 3GPP TS 44.060 |
|  | (0, 1, 0, 1, 0, 1, 0, 0, 1, 1, 1, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0, 1, 1, 0, 0, 0, 1, 0, 1, 1, 1, 1, 0, 0, 1, 0, 0, 1, 1, 0, 1) | in case alternative training (synchronization) sequence "TS1" is used except in EC operation. |
|  | (1, 1, 1, 0, 1, 1, 1, 1, 0, 0, 1, 0, 0, 1, 1, 1, 0, 1, 0, 1, 0, 1, 1, 0, 0, 0, 0, 0, 1, 1, 0, 1, 1, 0, 1, 1, 1, 0, 1, 1, 1) | in case alternative training (synchronization) sequence "TS2" is used except in EC operation. |
|  | (1, 1, 0, 0, 1, 0, 0, 1, 1, 1, 0, 0, 0, 1, 0, 0, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 1, 1, 0, 1, 0, 1, 0, 1, 1, 0, 0, 1, 0) | in case alternative training (synchronization) sequence "TS4" is used for PEO operation. |

Table 5.2.7-4: Synch. Sequence bits, in EC operation

| Bit number (BN) | Modulating bits | Uplink CC | Applicability |
| --- | --- | --- | --- |
| (BN8, BN9 .. BN48) | (1, 0, 0, 0, 1, 0, 0, 0, 1, 1, 1, 0, 1, 0, 1, 1, 1, 0, 1, 1, 0, 1, 0, 0, 0, 0, 0, 1, 0, 0, 0, 0, 1, 0, 1, 1, 0, 0, 0, 1, 0) | 1 | For operation on RACH or EC-RACH in case "TS3" is used1 |
|  | (0, 1, 0, 1, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 0, 1, 0, 1, 1, 1, 0, 1, 0, 1, 1, 0, 1, 1, 0, 0, 1, 1, 0, 0, 1, 0, 1, 0, 0) | 2 | For operation on EC-RACH in case "TS5" is used1 |
|  | (0, 1, 0, 1, 1, 1, 1, 0, 0, 1, 1, 1, 0, 1, 0, 1, 1, 1, 1, 0, 1, 1, 0, 1, 0, 0, 0, 1, 0, 0, 1, 1, 0, 0, 0, 0, 1, 0, 1, 1, 1) | 3 | For operation on EC-RACH in case "TS6" is used1 |
|  | (0,1, 0, 0, 0, 0, 1, 0, 1, 1, 0, 0, 0, 0, 0, 1, 1, 1, 0, 1, 0, 0, 1, 0, 1, 0, 1, 1, 1, 0, 1, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0) | 4 | For operation on EC-RACH in case "TS7" is used1 |
| NOTE1: see 3GPP TS 44.018 |  |  |  |

3rd Modification

### 5.2.10	Extended Synchronization Access burst (ESAB)

Table 5.2.10-1: Extended Synchronization Access Burst

| Bit Number (BN) | Length of field | Content of field | Definition |
| --- | --- | --- | --- |
| 0-139 | 140 | Synch. sequence bits | see table 5.2.10-3 |
| 140-241 | 102 | Encrypted bits (e0..e101) | see 3GPP TS 45.003 |
| 242-244 | 3 | Tail bits | see table 5.2.10-2 |
| 245-311, 245-312 | 67 or 681 | Extended guard period (bits) | see subclause 5.2.8 |
| NOTE1: 67 or 68 always applies in case of an EC-GSM-IoT access burst sent using CC5, see 3GPP TS 45.010 |  |  |  |

The “tail bits” are defined as modulating bits with states according to table 5.2.10-2.

Table 5.2.10-2: Tail bits

| Tail bit type | Bit number (BN) | Modulating bits |
| --- | --- | --- |
| Tail bits | (BN242, BN243, BN244) | (0, 0, 0) |

The “Synch. sequence bits” are defined as modulating bits with states according to table 5.2.10-3.

Table 5.2.10-3: Synch. Sequence bits

| Bit number (BN) | Modulating bits | Uplink CC | Applicability |
| --- | --- | --- | --- |
| (BN0, BN1 .. BN139) | ([1, 1, 1, 0, 1, 1, 1, 0, 1, 1, 0, 0, 0, 1, 1, 1, 0, 0, 0, 0, 0, 1, 0, 0, 1, 1, 1, 1, 0, 1, 0, 0, 1, 0, 0, 1, 1, 1, 0, 0, 1, 0, 0, 0, 1, 1, 0, 1, 1, 0, 1, 0, 0, 1, 1, 1, 0, 0, 0, 1, 1, 1, 1, 0, 1, 0, 1, 0, 1, 0, 1, 1, 0, 1, 1, 0, 0, 1, 0, 1, 1, 1, 0, 0, 0, 1, 1, 1, 0, 1, 1, 1, 1, 1, 1, 1, 1, 0, 1, 0, 1, 1, 0, 1, 0, 1, 1, 1, 0, 1, 0, 0, 0, 0, 0, 1, 1, 0, 1, 1, 0, 1, 1, 1, 1, 1, 0, 1, 1, 1, 1, 0, 0, 1, 1, 1, 0, 0, 1, 0]) | 5 | For operation on EC-RACH in case "TS8" is used1 |
| NOTE1: see 3GPP TS 44.018 |  |  |  |  |

4th Modification

#### 6.3.4.3	Mapping of the uplink Extended Coverage CCCH (EC-CCCH/U)

The mapping of the EC-RACH (EC-CCCH/U) is defined in table 6a of clause 7 and illustrated in figure 16 and figure 17, where the possible blocks are indicated for each uplink Coverage Class. Furthermore, two different EC-RACH mappings exist. The mapping to be used is signalled on cell level in EC SI, see 3GPP TS 44.018. The EC-RACH is either mapped onto a single TS, or over 2 consecutive TS for CC2, CC3, CC4 and CC5. CC1 EC-RACH is always mapped onto 1 TS.

The EC-RACH is fixed allocated in the full 51-multiframe. The EC-RACH blocks constitute different number of EC-RACH bursts depending on the uplink Coverage Class used, see table 6a and illustration in figure 16 and figure 17. One, four, sixteen, forty-eight, one-hundred-and-thirty-two bursts are used for CC1, CC2, CC3, CC4 and CC5 EC-RACH blocks respectively. In case of CC4 and CC5, the bursts are mapped over two and three 51-multiframes respectively, and for other Coverage Classes each block is contained in a single 51-multiframe.

5th Modification

### 6.5.1	General

i)	A base transceiver station must transmit a burst in every timeslot of every TDMA frame in the downlink of radio frequency channel C0 of the cell allocation (to allow mobiles to make power measurements of the radio frequency channels supporting the BCCH, see 3GPP TS 45.008). In order to achieve this requirement a dummy burst is defined in subclause 5.2.6 which shall be transmitted by the base transceiver station on all timeslots of all TDMA frames of radio frequency channel C0 for which no other channel requires a burst to be transmitted.

ii)	Timeslot number 0 of radio frequency channel C0 of the cell allocation must support either channel combinations iv), v) or xxxviii) in subclause 6.4.1. No other timeslot or allocated channel from the cell allocation is allowed to support channel combinations iv), v) and xxxviii) in subclause 6.4.1.

iii)	The parameter BS_CC_CHANS in the BCCH defines the number of basic physical channels supporting common control channels (CCCHs). All shall use timeslots on radio frequency channel C0 of the cell allocation. The first CCCH shall use timeslot number 0, the second timeslot number 2, the third timeslot number 4 and the fourth timeslot number 6. Each CCCH carries its own CCCH_GROUP of mobiles in idle mode. Mobiles in a specific CCCH_GROUP will listen for paging messages and make random accesses only on the specific CCCH to which the CCCH_GROUP belongs. The method by which a mobile determines the CCCH_GROUP to which it belongs is defined in subclause 6.5.2.

iv)	The parameter BS_CCCH_SDCCH_COMB in the BCCH (see subclause 3.3.2) defines whether the common control channels defined are combined with SDCCH/4(0.3) + SACCH/C4(0.3) onto the same basic physical channel. If they are combined then the number of available random access channel blocks (access grant channel blocks and paging channel blocks; see following), are reduced as defined in table 5 of clause 7.

v)	The PCH, AGCH, NCH and BCCH Ext may share the same TDMA frame mapping (considered modulo 51) when combined onto a basic physical channel. The channels are shared on a block by block basis, and information within each block, when de-interleaved and decoded allows a mobile to determine whether the block contains paging messages, notification message, system information messages or access grants. However, to ensure a mobile satisfactory access to the system a variable number of the available blocks in each 51-multiframe can be reserved for access grants and system information messages, only. The number of blocks not used for paging (BS_AG_BLKS_RES) starting from, and including block number 0 is broadcast in the BCCH (see subclause 3.3.2). As above the number of paging blocks per 51-multiframe considered to be "available" shall be reduced by the number of blocks reserved for access grant messages.

The number of paging blocks per 51-multiframe shall be the same for all cells in a routing area where eDRX is supported to ensure the nominal paging group of a MS that uses eDRX (see 3GPP TS 44.018 [10]) occurs at the same location (i.e. in the same paging block) within the set of 51-multiframes comprising its eDRX cycle in all cells in that routing area.

If system information messages are sent on BCCH Ext, BS_AG_BLKS_RES shall be set to a value greater than zero.

Table 5 of clause 7 defines the access grant blocks and paging blocks available per 51-multiframe.

vi)	Another parameter in the BCCH, BS_PA_MFRMS indicates the number of 51-multiframes between transmissions of paging messages to mobiles in idle mode of the same paging group. The "available" paging blocks per CCCH are then those "available" per 51-multiframe on that CCCH (determined by the two above parameters) multiplied by BS_PA_MFRMS. An exception case is where eDRX is supported in a given routing area in which case all cells therein shall have the same number of paging blocks per 51-multiframe (see 3GPP TS 44.018 [10]). In this exception case the "available" paging blocks per eDRX cycle on a CCCH is determined by multiplying the number of paging blocks per 51-multiframe by the number of 51-multiframes per eDRX cycle (BS_ePA_MFRMS) (see Table 6.5.6a-1). Mobiles are normally only required to monitor every Nth block of their paging channel, where N equals the number of "available" blocks in total (determined by the above BCCH parameters) on the paging channel of the specific CCCH which their CCCH_GROUP is required to monitor. Note that when eDRX is used then N is determined using BS_ePA_MFRMS which is not a parameter sent on the BCCH but is derived directly from the eDRX cycle length negotiated by the mobile station with the network (see sub-clause 6.5.2a and Table 6.5.6a-1). Other paging modes (e.g. page reorganize or paging overload conditions described in 3GPP TS 44.018) may require the mobile to monitor paging blocks more frequently than this. All the mobiles listening to a particular paging block are defined as being in the same PAGING_GROUP. The method by which a particular mobile determines to which particular PAGING_GROUP it belongs and hence which particular block of the available blocks on the paging channel is to be monitored is defined in subclause 6.5.2 and 6.5.2a.

vii)	An MS which has its membership of at least one voice group or voice broadcast call group set to the active state shall, in addition to monitoring the paging blocks as described above, monitor the notification channel, NCH. This logical channel is always mapped onto contiguous blocks reserved for access grants, in a position and number as given by the parameter NCP, defined in 3GPP TS 44.018, broadcast on the BCCH. The channel may be present when a cell supports voice group or voice broadcast calls. The coding of the various structural parameters described above in this subclause is not changed. Information within a block, when deinterleaved and decoded, allows the MS to determine whether the block contains access grant messages or notification messages.

viii)	In presence of PCCCH, the parameter BS_PCC_CHANS in the PBCCH defines the number of physical channels for packet data (PDCH) carrying PCCCH. The (P)BCCH shall in addition indicate the physical description of those channels. Each PCCCH carries its own PCCCH_GROUP of MSs in GPRS attached mode. MS in a specific PCCCH_GROUP will listen for paging messages and make random accesses only on the specific PCCCH to which the PCCCH_GROUP belongs. The method by which an MS determines the PCCCH_GROUP to which it belongs is defined in subclause 6.5.6.

ix)	In CTS, the CTSBCH (CTSBCH-SB and CTSBCH-FB) shall always be transmitted by the CTS-FP according to the rules defined in Clause 6 and table 8 of clause 7.

In CTS idle mode, a CTS-MS shall be assigned a CTS_PAGING_GROUP, as specified in subclause 6.5.7. Several CTS-MS can be assigned the same CTS_PAGING_GROUP. The CTS-MS shall determine the specific 52-multiframe where a paging block may be sent to it according to the rule defined in subclause 6.5.7, and shall listen to the CTSBCH of the previous 52-multiframe. In this 52-multiframe, the CTS-MS shall decode the CTSBCH-SB information bits : if the flag indicating the presence of a CTSPCH in the next 52-multiframe is properly set (see 3GPP TS 44.056), the CTS-MS shall listen to the next CTSPCH and read the paging block. With this method, it is not necessary to maintain on the physical channel the CTSPCH : the CTSPCH shall only be transmitted when a paging message shall be addressed to one or several CTS-MS in a CTS paging group.

When using the CTSARCH, the CTS-MS shall send two bursts on the CTSARCH: these two bursts shall be sent on two successive frames and shall fulfil the mapping defined in table 8 of clause 7, with the requirement of the first burst being sent in a TDMA frame with even FN. They shall contain the same access request message, which is specified in 3GPP TS 44.056. The first sent burst can be used by the CTS-FP to assess the path loss between the CTS-MS and itself, in order to effectively decode the second burst.

x)	For COMPACT, the base transceiver station shall transmit a burst in a PDCH allocated to carry CPBCCH, in all TDMA Frames where CPBCCH, CFCCH, CSCH is allocated or where CPPCH can appear. In TDMA Frames where CPPCH can appear on the physical channel where CPBCCH is allocated, the base transceiver station shall transmit a dummy block in case no block is required to be transmitted.

xi)	For COMPACT, a base station does not transmit a burst in every timeslot of every TDMA frame in the downlink of the COMPACT control carrier (i.e., discontinuous transmission is used).

xii)	For COMPACT, inter base station time synchronization is required. Timeslot number (TN) = i (i = 0 to 7) and frame number (FN) with FN mod 208 =0  shall occur at the same time in all cells.

xiii)	For the primary COMPACT carrier, timeslot numbers (TN) 1, 3, 5, and 7 shall support channel combination xxii) in subclause 6.4.1. TNs 0, 2, 4, and 6 shall support channel combination xiii).

xiv)	For the secondary COMPACT carrier(s) carrying CPCCCH, timeslot numbers (TN) 1, 3, 5, and 7 shall support channel combination xxiii) in subclause 6.4.1. TNs 0, 2, 4, and 6 shall support channel combination xiii). CPCCCHs on secondary COMPACT carrier(s) shall be allocated on same time group as for primary COMPACT carrier.

xv)	For the secondary COMPACT carrier(s) not carrying CPCCCH, timeslot numbers (TN) 0 through 7 shall support channel combination xiii) in subclause 6.4.1.

xvi)	For COMPACT, BS_PAG_BLKS_RES shall be less than or equal to 8 and less than or equal to 10-BS_PBCCH_BLKS.

xvii)	For COMPACT, CFCCH, CSCH, CPBCCH, and CPCCCH are rotated as described in subclause 6.3.2.1. PDTCH, PACCH, and PTCCH do not rotate.

xviii)	For COMPACT, the parameters NIB_CCCH_0, NIB_CCCH_1, NIB_CCCH_2, and NIB_CCCH_3 shall not be broadcast for a serving time group.

xix)	For the COMPACT, NIB_CCCH_0, NIB_CCCH_1, NIB_CCCH_2, and NIB_CCCH_3 blocks shall be idle for non-serving time groups and rotate in accordance with the non-serving time groups.

The downlink position of the NIB_CCCH idle blocks is based on the ordered list as defined in subclause 6.3.2.1. The MS shall ignore these downlink idle blocks and shall interpret this action as not having detected an assigned USF value on an assigned PDCH.

xx)	For COMPACT large cells, NIB_CCCH_0, NIB_CCCH_1, NIB_CCCH_2, and NIB_CCCH_3 blocks shall be idle on timeslots immediately preceding and succeeding non-serving time groups and rotate in accordance with the non-serving time groups. The MS shall ignore these downlink idle blocks and shall interpret this action as not having detected an assigned USF value on an assigned PDCH.

The downlink position of the NIB_CCCH idle blocks is based on the ordered list as defined in subclause 6.3.2.1.

xxi)	For COMPACT, the MS attempts uplink random access on its designated serving time group (TG) by monitoring for USF=FREE in every downlink block.

For dynamic allocation, while in the uplink transfer state, the MS monitors all of the downlink non-idle blocks of its assigned PDCH for uplink assignments. The MS shall ignore downlink idle blocks and shall interpret this action as not having detected an assigned USF value on an assigned PDCH.

USF should be set equal to FREE for downlink non-idle blocks B0 on timeslot numbers (TN) 1, 3, 5, and 7.

xxii)	While in broadcast/multicast receive mode (see 3GPP TS 45.008), the MS shall continue to monitor system information either on the BCCH or, if present, on the PBCCH unless the network has indicated that system information and paging messages are sent on the PACCH for the MBMS radio bearer the MS listens to. If the network has not indicated that system information and paging messages are sent on the PACCH, or if the MS does not have an MS_ID, the MS shall additionally read paging messages either from the CCCH or, if the PBCCH is present, from the PCCCH. The MS shall then monitor the same paging group as in packet idle mode, i.e. shall determine the paging blocks to monitor using the methods described in subclause 6.5.2 or subclause 6.5.6. If the location of the MBMS radio bearer with respect to the control channels does not allow the mobile station to satisfy this requirement, the mobile station shall not read those radio blocks of the MBMS radio bearer that would prevent the monitoring of its paging group on the paging channel(s).

xxiii)	For a mobile in DLMC configuration, fallback to reception of a single carrier, irrespective of the number of assigned carriers,  is performed with regular periodicity. The periodicity is based on BS_PA_MFRMS, which indicates the number of 52-multiframes between two fallback periods. A mobile belongs to a particular single carrier fallback group which identifies a specific basic radio block period, or in case of RTTI mode two consecutive reduced radio block periods, within the set of “available” single carrier fallback blocks, during which it performs single carrier fallback. The single carrier fallback applies to all assigned PDCHs (or PDCH pairs, in case of RTTI mode) on the carrier where single carrier fallback is performed.

The method by which a particular mobile determines to which single carrier fallback group it belongs is defined in subclause 6.5.8. Single carrier fallback is performed on the carrier where the PTCCH is assigned. In case no PTCCH is assigned the single carrier fallback is performed on the carrier with the lowest number (see 3GPP TS 44.060 for numbering of carriers). Irrespective of which carrier is used, the single carrier fallback applies to all assigned PDCHs (in BTTI configuration)/PDCH-pairs (in RTTI configuration) on that carrier. In case of inter-band reception, the single carrier fallback only applies in one frequency band.

xxiv)	For EC-GSM-IoT, timeslot number 1 of radio frequency channel C0 of the cell allocation must support channel combination xxxvii) in subclause 6.4.1. No other timeslot or allocated channel from the cell allocation is allowed to support channel combination xxxvii).

xxv)	For EC-GSM-IoT, the parameter EC_BS_CC_CHANS in the EC SI on EC-BCCH defines the number of basic physical channels supporting extended coverage common control channels (EC-CCCHs). All shall use timeslots on radio frequency channel C0 of the cell allocation and shall support channel combination xxxvii) (timeslot number 1) or xxxix) (timeslot number 3,5, or 7) in subclause 6.4.1. The first EC-CCCH shall use timeslot number 1, the second timeslot number 3, the third timeslot number 5 and the fourth timeslot number 7. Each EC-CCCH carries its own EC_CCCH_GROUP of mobiles in idle mode. Mobiles in a specific EC_CCCH_GROUP will listen for paging messages and make random accesses only on the specific EC-CCCH to which the EC_CCCH_GROUP belongs. In case 2 TS EC-RACH mapping is configured by the network in EC SI (see 3GPP TS 44.018), the physical resources where the EC-RACH is mapped for CC2, CC3, CC4 and CC5 shall be the physical channel used by EC_CCCH_GROUP and one timeslot number lower, see table 6a. In this case the network shall ensure that channel combination xxxviii) is used on the lower timeslot number. The method by which a mobile determines the EC_CCCH_GROUP to which it belongs is defined in subclause 6.5.2b. In case the RACH is used (RACH Access Control in 3GPP TS 44.018) by a MS in EC operation, then channel combination shall be used xxxviii), and timeslot number 0 shall be used.

xxvi)	For EC-GSM-IoT, the EC-AGCH and EC-PCH may share the same TDMA frame mapping (considered modulo 51) when combined onto a basic physical channel. The channels are shared on a block by block basis, and information within each block, when de-interleaved and decoded allows a mobile to determine whether the block contains paging messages or access grants. Table 6a of clause 7 defines the access grant blocks and paging blocks available per 51-multiframe for each Coverage Class respectively. For a given Coverage Class, the same number of paging blocks are always available per 51-multiframe. The number of access grant blocks can differ depending on if the basic physical channel for EC-CCCH is timeslot number 1 or not.

xxvii)	For EC-GSM-IoT, the "available" paging blocks per eDRX cycle on an EC-CCCH is Coverage Class specific. It is determined by BS_ePA_MFRMS, see Table 6.5.6a-1, and the downlink Coverage Class of the MS, see subclause 6.5.2b. A mobile station selects one of the "available" paging blocks per eDRX cycle as its PAGING_GROUP based on IMSI. All MSs belonging to the same Coverage Class, using the same eDRX cycle and that has selected the same paging block are defined as being in the same PAGING_GROUP. The method by which a particular mobile determines to which particular PAGING_GROUP it belongs and hence which particular block of the “available” paging blocks on the paging channel is to be monitored is defined in subclause 6.5.2b.

6th Modification

# 7	Mapping tables

Table 6a : Mapping of Extended Coverage logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

| Channel designation | Dir. | Allowable timeslot assignment | Allowable RF channel assignment | Burst type | Repeat length TDMA frames | Coverage class (CC) | Interleaved block TDMA frame mapping |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EC-SCH | D | 1 | C0 | SB | 204 | - | B([0,1,2,3,4,5,6] + 51 N), N=0,1,2,34 |
| EC-BCCH | D | 1 | C0 | NB | 408 | - | B([7,8,9,10,11,12,13,14]+51N), N=0,…,75 |
| EC-PCH | D | 1,3,5,7 | C0 | NB | 51 | 1 | B0(19,20),B1(21,22),…,B15(49,50) |
|  |  |  |  |  | 102 | 2 | B0([19,…,26] + 51N),B1([27,…,34] + 51N),…,B3([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,34 |
| EC-AGCH | D | 1 | C0 | NB | 51 | 1 | B0(15,16),B1(17,18),…,B17(49,50) |
|  |  |  |  |  | 102 | 2 | B0([19,…,26] + 51N),B1([27,…,34] + 51N),…,B3([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,34 |
|  |  | 3,5,7 |  |  | 51 | 1 | B0(1,2),B1(3,4),…,B24(49,50) |
|  |  |  |  |  | 102 | 2 | B0([3,…,10] + 51N),B1([11,…,18] + 51N),…,B5([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,1,2,34 |
| EC-RACH, 1 TS | U | 1,3,5,7 | C0 | AB, Exten-ded AB7 | 51 | 1 | B0(0),B1(1),..B50(50) |
|  |  |  |  |  |  | 2 | B0(1,…,4), B1(5,…,8),...B11(45,…,48) |
|  |  |  |  |  |  | 3 | B0(2,…, 17),B1(18,…,33), B2(34,…,49) |
|  |  |  |  |  | 102 | 4 | B0([0,…,23] + 51N),B1([24,…,47] + 51N), N=0,13 |
| EC-RACH, 2 TS | U | 0,1, or, 2,3, or, 4,5, or, 6,7 | C0 | AB | 51 | 2 | B0(1,2,1’,2’), B1(3,4,3’,4’),...,B24(49,50,49’,50’) |
|  |  |  |  |  |  | 3 | B0(2,…,9,2’,…,9’), B1(10,…,17,10’,…,17’),...,B5(42,...,49,42’,...,49’) |
|  |  |  |  |  | 102 | 4 | B0([0,…,11,0’,…,11’] + 51N),…,B3([36,…,47,36’,…,47’] + 51N), N=0,13 |
|  |  |  |  | ESAB | 153 | 5 | B0([0,…,21,0’,…,21’] + 51N), B1([22,…,43,22’,…,43’] + 51N), N=0,1,28 |
| EC-PACCH, EC-PDTCH | D&U | 0…7 | C0…Cn | NB2 | 52 | 1 | B0(0...3), B1(4...7), B2(8...11), B3(13...16), B4(17...20), B5(21…24), B6(26...29), B7(30...33), B8(34...37), B9(39...42), B10(43...46), B11(47...50) |
|  |  |  |  |  |  | 2 | B0(0...3,0'...3',0''...3'',0'''...3'''),B1(4...7,4'...7',4''...7'',4'''...7'''), B2(8...11, 8'...11',8''...11'', 8'''...11'''), B3(13...16,13'...16',13''...16'',13'''...16'''), B4(17...20,17'...20',17''...20'',17'''...20'''), B5(21...24,21'...24',21''...24'',21'''...24'''), B6(26...29,26'...29',26''...29'',26'''...29'''), B7(30...33,30'...33',30''...33'',30'''...33'''), B8(34...37,34'...37',34''...37'',34'''...37'''), B9(39...42,39'...42',39''...42'',39'''...42'''), B10(43...46,43'...46',43''...46'',43'''...46'''), B11(47...50,47'...50',47''...50'',47'''...50''')1 |
|  |  |  |  |  |  | 3 | B0(0...7,0'...7',0''...7'',0'''...7'''), B1(8...11,13…16,8'...11',13'…16',8''...11'',13''…16'',8'''...11''',13''',…,16'''), B2(17...24, 17'...24',17''...24'', 17'''...24'''),B3(26...33,26'...33',26''...33'',26'''...33'''), B4(34...37,39…42,34'...37',39'…42',34''...37'',39''…42'',34'''...37''',39'''…42'''), B5(43...50,43'...50',43''...50'',43'''...50''')1 |
|  |  |  |  |  |  | 4 | B0(0...11,13…16,0'...11',13'…16',0''...11'',13''…16'',0'''...11''',13'''…16'''), B1(17...24,26…33,17'...24',26'…33',17''...24'',26''…33'',17'''...24''',26'''…33'''), B2(34...37,39...50,34'...37',39'...50',34’’...37’’,39'’...50'’,34'’'...37'’',39'''...50''')1 |
|  |  |  |  |  | 52 | 26 | B0(0..7,0'..7'), B1(8..11,13..16,8'..11',13'..16'), B2(17..24,17'..24'), B3(26..33,26'..33'), B4(34..37,39..42,34'..37',39'..42'), B5(43..50,43'..50') |
|  |  |  |  |  | 52 | 36 | B0(0..11,13..16,0’..11,13..16), B1(17..24,26..33, 17'..24',26'..33'), B2(34..37,39..50,34'..37',39'..50') |
|  |  |  |  |  | 104 | 46 | B0(0..11,13..24,26..33,0’..11’,13’..24’,26’..33’), B1(34..37,39..50,52..63,34’..37’,39’..50’,52’..63’), B2(65..76,78..90,92..103,65’..76’,78’..90’,92’..103’) |
| NOTE 1: EC-PDTCH and EC-PACCH are mapped onto four PDCHs. Number n indicates mapping on the PDCH with the lowest timeslot number in TDMA frame n, whereas number n', n'', and n''' indicates mapping on the PDCH with the second lowest, second highest, and highest timeslot number in TDMA frame n respectively. The four timeslots shall be consecutive. NOTE 2: Access Bursts (AB) instead of Normal Bursts (NB) may be used on the uplink as polling response on EC-PACCH/U. The same resources are used by the Access Burst. NOTE 3 N = (FN div 51) mod 2 NOTE 4 N = (FN div 51) mod 4 NOTE 5 N = (FN div 51) mod 8 NOTE 6: EC-PDTCH/2TS and EC-PACCH/2TS are mapped onto two PDCHs in case of blind physical layer transmissions. NOTE 7: For Multilateration Timing Advance using the Extended Access Burst method in EC operation, the Extended AB format is used in case of CC1 selected in downlink and uplink. NOTE 8: N = (FN div 51) mod 3 |  |  |  |  |  |  |  |

Figure 17: TDMA frame mapping for 2TS EC-RACH

End of Modifications