# 3GPP RAN WG6 Meeting #4				R6-170268

May 15 – 19, 2017, Hangzhou, China 		(Revision of R6-170175)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.010 | CR | 0071 | rev | 5 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Multilateration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePOS_GERAN-Perf |  |  |  |  |  |  | Date: |  | 2017-05-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The accuracy of Multilateration using Multilateration Timing Advance (see 3GPP TS 43.059) relies on accuracy of TA estimation by the BTS as well as on accuracy of the estimation of the BTS timing by the MS. In addition, the accuracy of Multilateration using Multilateration Observed Time Difference, see 3GPP TS 43.059, also relies on the accuracy of the determination of the observed time differences between serving cell transmission and neighbour cell transmission The corresponding requirements need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A requirement on - The MS assessment of BTS timing, at and above the reference sensitivity level of the SCH or, in the case of EC operation, at and above the input level of EC-SCH, in response to a Multilateration Request of better than ± ¼ normal symbol period is introduced. - The MS timing of its transmissions to the BTS according to signals received from the BTS, at and above the reference sensitivity level or, in the case of EC operation, at and above the input level of EC-SCH reference performance, in response to a Multilateration Request of better than ± 1/8 normal symbol period is introduced. - The absolute value of an OTD report's error of at most ¼ normal symbol period, at and above the reference sensitivity level or, in the case of EC operation, at and above the input level of EC-SCH reference performance, is introduced. - MS estimation of the MS Transmission Offset is introduced - MS estimation of the error in the estimation of the BTS timing is introduced - BSC estimation of the BTS Reception Acccuracy Level is introduced - BSC reporting of the BTS Reception Accuracy Capability is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multilateration using TA and OTD will be inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1 (new), 5.4.2 (new), 6.3.1 (new), 6.3.2 (new) 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 24.008 TS 43.059 CR 0081 TS 44.018 CR 1047 TS 44.031 CR 0227 TS 44.060 CR 1631 TS 45.008 CR 0648 TS 48.008 CR 0410 TS 48.018 CR 0430 TS 48.018 CR 0435 TS 49.031 CR 0066 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.021 CR 0282 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

## 1.1	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 25.123: “Requirements for support of radio resource management  (TDD)”.

[3]	3GPP TS 25.133: “Requirements for support of radio resource management  (FDD)”.

[4]	3GPP TR 43.030: “Radio network planning aspects”.

[5]	3GPP TS 43.052: “Lower layers of the Cordless Telephony System (CTS) Radio Interface; Stage 2”.

[6]	3GPP TS 43.059: “Functional stage 2 description of Location Services (LCS) in GERAN”.

[7]	3GPP TS 43.064: “Overall description of the GPRS radio interface; Stage 2”.

[8]	3GPP TS 44.018: “Mobile radio interface layer 3 specification, Radio Resource Control Protocol”.

[9]	3GPP TS 44.060: “General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/ Medium Access Control (RLC/MAC) protocol”.

[10]	3GPP TS 45.002: “Multiplexing and multiple access on the radio path”.

[11]	3GPP TS 45.005: “Radio transmission and reception”.

[12]	3GPP TS 45.008: “Radio subsystem link control”.

[13]	3GPP TS 45.050: “Background for RF Requirements”.

[14]	3GPP TS 45.056: “CTS-FP Radio Sub-system”.

[15]	3GPP TS 45.004: “Modulation”.

[16]	3GPP TS 36.133: “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”.

[17]	3GPP TS 36.211: “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation”.

[18]	3GPP TS 49.031: “Location Services (LCS); Base Station System Application Part, LCS Extension (BSSAP-LE)”.

*** Next change ***

## 5.4	Timing advance estimation

### 5.4.1	Initial timing advance estimation

When the BTS detects an access burst transmission on RACH, PRACH, or one or a sequence of access burst(s) on EC-RACH, it shall measure the delay of this signal relative to the expected signal from an MS at zero distance under static channel conditions. This delay, called the timing advance, shall be rounded to the nearest normal symbol period and included in a response from the BTS when applicable.

For the pico-BTS and Local Area multicarrier BTS, there is no requirement to measure this timing advance. However, either this measured value or a programmable value of timing advance shall be included in the response from the BTS when a timing advance value needs to be sent.

### 5.4.2	BTS Timing Advance Estimation for Positioning

A higher level of accuracy of the timing advance estimation by the BTS (reported to the SMLC, see 3GPP TS 43.059 [6]) is desired when the following positioning procedures are used:

- Multilateration Timing Advance procedure for assessing the timing advance in the serving cell and in non-serving cells, and

- Multilateration Observed Time Difference procedure for assessing the timing advance in the serving cell,

The actual algorithms and methods for estimation of the timing advance value are implementation dependent.

Moreover, a BSC that reports a timing advance value based on transmissions from an MS (e.g. the EC Multilateration Request message or EGPRS Multilateration Request message, see 3GPP TS 44.018 [8] and 3GPP TS 44.060 [9]) using the RLC Data Block or the Extended Access Burst method, shall establish a reported timing advance value = tAReported . It does so by first using the BTS estimated TA value = tAEstimated (based on the AB received on the RACH or on the EC-RACH (including blind physical layer transmissions for CC2, CC3 and CC4) identified when the MS starts the positioning procedure and using it as the assigned TA value = tAAssigned (i.e. the TA value sent to the MS using an assignment message on the AGCH/EC-AGCH is the value of the tAEstimated  rounded off to the nearest symbol). It then adjusts the most recent estimated TA value as it receives subsequent updated timing advance estimation values from the BTS (i.e. the BTS provides an updated timing advance estimation for each subsequent burst it receives from the MS for the remainder of the positioning procedure wherein each burst sent by the MS uses the assigned TA).The final reported TA value (tAReported) is calculated by the BSC after it has received the last estimated TA value from the BTS based on the last transmission from the MS for the current positioning procedure and takes into account the MS Transmission Offset sent by the MS during the procedure (see 3GPP TS 44.018 [8] and 3GPP TS 44.060 [9] and 3GPP TS 43.059 [6]). This is shown in the formulas below.

tAReported=1Ni=1NtAi 		(1)

where

tA1= tAEstimated1+tMS Transmisson Offset		(2)

tAi=2…N= tAEstimatedi+tAAssigned+tMS Transmisson Offset	(3)

where tMS Transmisson Offset corresponds to the time as reported by the MS in the “MS Transmission Offset IE” (see 3GPP TS 49.031 [18]).

For the Extended Access burst method the number of timing advance estimations is N= 2 wherein both the RACH burst and the Extended Access burst are used for timing advance estimation. Similarly, for the RLC Data Block method the number of timing advance estimations correspond to N=5 (for the case of no HARQ retransmissions needed on the (EC-)PDTCH) wherein the AB received on RACH or EC-RACH  and the 4 normal bursts used to receive the RLC data block are used for timing advance estimation.

Moreover, a BTS estimating the timing advance value for an MS using the RLC Data Block or the Extended Access Burst methods will be subject to an accuracy limitation inherent to its implementation and the radio conditions applicable when receiving transmissions from the MS performing the Multilateration Timing Advance procedure. This accuracy limitation is expressed as an assessment error of the reported TA value tAReported and shall be reported (to the SMLC via the BSS) as the BTS Reception Accuracy Level (see 3GPP TS 43.059 [6] and TS 3GPP 49.031 [18]) wherein the assessment error corresponds to the variance of the reported timing advance value of multiple timing advance estimations according to the equations below where N denotes the number of timing advance estimations and tAi, i=1...N denotes the estimated timing advance in estimation 'i'.

The variance vartAReportedof the estimated timing advance value shall be evaluated using the formula:

var(tA_Reported)=s2N		(4)

s2 is the unbiased sample variance

s2=1N-1i=1NtAi-tAReported2		(5)

where tAReportedand  tAi are calculated per the equations (1), (2) and (3) above.

For the Access Burst method the reported timing advance value corresponds to the estimated timing advance value derived from receiving the AB containing the EC Multilateration Request message or EGPRS Multilateration Request message.

When reporting the timing advance value to the SMLC the tAReported is rounded off to the nearest 1/64 of a normal symbol period (see 3GPP TS 49.031 [18].

Similarly, for the SMLC to be able to accurately estimate the number of required BTSs to be used during the Multilateration Timing Advance procedure (in order to meet a targeted positioning accuracy) each individual BTS shall, provide the BSC with a guaranteed timing advance assessment error it shall always be capable of supporting regardless of radio conditions, see 3GPP TS 49.031 [18]. The BSC, in turn, reports this guaranteed timing advance assessment error value as the applicable BTS Reception Accuracy Capability in the Assistance Information Response message sent to the SMLC in response to an Assistance Information Request message (see 3GPP TS 43.059 [6]).

The BTS Reception Accuracy Capability shall be evaluated at the reference sensitivity level of the RACH or, in case of EC operation (see 3GPP TS 44.018 [36]), at the input level of the EC-RACH (CC1) reference performance (see 3GPP TS 45.005 [11]). The BTS shall comply with the indicated BTS reception Accuracy Capability in [90] % of the timing advance estimations.

*** For information only ***

## 6.2	Internal timebase

The MS shall keep its internal timebase in line with that of signals received from the BTS. If the MS determines that the timing difference exceeds 2 µ seconds, it shall adjust its timebase in steps of ¼ normal symbol period. This adjustment shall be performed at intervals of not less than 1 second and not greater than 2 seconds until the timing difference is less than ½ normal symbol periods.

*** Second change ***

## 6.3	Assessment of BTS timing

### 6.3.1 	General

In determining the timing of signals from the BTS, the timings shall be assessed in such a way that the timing assessment error is less than ½ normal symbol periods. The assessment algorithm must be such that the requirements of 6.2 can be met.

### 6.3.2	MS Assessment of BTS timing for Positioning

A higher level of accuracy of the assessment of the BTS timing by the MS (reported to the SMLC, see 3GPP TS 43.059 [6]) is desired when the following positioning procedures are used:

- Multilateration Timing Advance procedure for assessing the TA in the serving cell and in non-serving cells, and

- Multilateration Observed Time Difference procedure for assessing the TA in the serving cell,The Initial LLC-PDU received from the MS;

During the positioning procedure the minimum requirement on the error assessment of the BTS timing by the MS at and above the reference sensitivity level of the SCH or, in the case of EC operation, at and above the input level of the EC-SCH reference performance, shall be [± ¼] normal symbol period unless the MS Sync Accuracy the MS can realize when performing the positioning procedure in a given cell meets a tighter requirement for the assessment of the BTS timing than the default value (indicated in the MSRAC IE - see 3GPP TS 24.008). In this case the value of the MS Sync Accuracy sent by the MS indicates the tighter requirement. Furthermore, to meet the targeted positioning accuracy for a targeted number of BTSs the SMLC determines a target assessment of BTS timing required of the MS and includes this as the Requested MS Sync Accuracy in the triggering RRLP message when indicating to the MS to use the RLC Data block or Extended Access burst method (see 3GPP TS 43.059 [6] and 3GPP TS 49.031[18]).

The MS shall attempt to achieve the Requested MS Sync Accuracy using a minimum of 2 or a maximum of [10] repeated synchronizations to the same BTS and report the achieved error in the assessment of the sync accuracy (i.e. assessment of BTS timing on the downlink) in the uplink RLC data block or the Extended Access Burst as the MS Sync Accuracy. The error of the sync accuracy corresponds to the variance of the estimated timing of multiple repeated synchronizations according to the equations below where N denotes the number of independent synchronization attempts and ti, i=1...N denotes the estimated timing in synchronization 'i'.

The variance var(t) of the estimated timing shall be evaluated using the formula:

var(t)=s2N

s2 is the unbiased sample variance

s2=1N-1i=1N(ti-t)2

and t is the mean of ti, i.e.

t=1Ni=1Nti

Similarly, to support accurate positioning using the Multilateration Observed Time Difference (OTD) procedure, a higher level of accuracy of the assessment of BTS timing is required in order to determine the difference in arrival time between (EC-)SCH synchronization sequences of two base station transmissions which is reported to the network in the RRLP Multilateration OTD Response message. At and above the reference senstivity level of the SCH or, in the case of EC operation, at and above the input level of the EC-SCH reference performance (see 3GPP TS 45.005 [11]), the absolute value of a Multilateration OTD report's error shall be ≤ ¼ normal symbol period. An MS applying the Multilateration OTD procedure shall report in the OTD Measurement Information IE the achieved assessment error of the BTS timing, according to the formulas above in this clause, as the MS Sync Accuracy for the respective neighbour cell.

*** Third change ***

## 6.4	Timing of transmission

The MS shall time its transmissions to the BTS according to signals received from the BTS. The MS transmissions to the BTS, measured at the MS antenna, shall be 468,75-TA normal symbol periods (i.e. 3 timeslots-TA) behind the transmissions received from the BTS, where TA is the last timing advance received from the current serving BTS. The tolerance on these timings shall be ± 1 normal symbol period. For CTS, the tolerance on these timings shall be ± ½ normal symbol period.

For an MS that transmits to a BTS in response to a RRLP Multilateration Timing Advance Request or RRLP Multilateration Observed Time Difference message (see 3GPP TS 43.059 [6]) the requirement on tolerance on the timings of its transmissions, when operating at and above the reference sensitivity level of the SCH or, in the case of EC operation, at and above the input level of the EC-SCH reference performance (see 3GPP TS 45.005 [11]), shall be [ ±1/8] normal symbol period or better. In addition, immediately prior to transmitting to a BTS while performing Multilateration Timing Advance procedure using the RLC Data Block method or the Extended Access Burst Method (see 3GPP TS 44.018 [8] and 3GPP TS 43.059 [6]), the MS shall also calculate the offset between the nominal transmission opportunity, as determined in relation to the estimated timing of the BTS, and the selected transmission opportunity, as dictated by the internal timebase. The offset shall be reported as the MS Transmission Offset in the uplink RLC data block or in the Extended Access Burst [6]. If the selected uplink transmission opportunity occurs earlier than the nominal transmission opportunity, the MS transmission Offset shall be reported as a positive value. Similarly, if the selected uplink transmission opportunity occurs later than the nominal transmission opportunity, the MS transmission Offset shall be reported as a negative value. Note that upon receiving an assigned TA value the resulting new transmission opportunity (selected according to the nearest available timebase specific transmission opportunity identified after applying the indicated TA value) needs to be in the same direction as before, i.e., if before receiving an assigned TA value the selected uplink transmission opportunity (selected using the nearest available timebase specific transmission opportunity) occurred earlier than the nominal transmission opportunity then the MS shall establish a new nominal transmission opportunity using the indicated TA value and then select an uplink transmission opportunity (selected using the nearest available timebase specific transmission opportunity) that occurs earlier than the new nominal transmission opportunity (and similarly for the case where the selected uplink transmission opportunity occurred later than the nominal transmission opportunity).

In case of a multislot configuration, the MS shall use a common timebase for transmission of all channels. In this case, if the MS, not in EC operation, does not support transmission of reduced symbol period bursts, it may optionally use a timeslot length of 157 normal symbol periods on timeslots TN = 0 and 4, and 156 normal symbol periods on timeslots with TN = 1, 2, 3, 5, 6 and 7, rather than 156,25 normal symbol periods on all timeslots. An EC-GSM-IoT MS, when in EC operation, shall use the timeslot length with integer symbol periods, see subclause 5.7.2.

If the MS supports reduced symbol period transmissions, it shall use a timeslot length of 187.5 reduced symbol periods or a timeslot of length 156.25 normal symbol periods. When there is a pair of different symbol period bursts on adjacent timeslots, then the guard period between the two bursts shall be 8.5 normal symbol periods which equals 10.2 reduced symbol periods. The active part of a reduced symbol period burst shall start a quarter of a normal symbol period later compared to a normal symbol period burst as shown in Figure 5.7.3.

In case of a circuit switched multislot configuration, the common timebase shall be derived from the main channel and the TA values received on other channels shall be neglected. In case of a packet switched multislot configuration the common timebase shall be derived from all timeslots monitored by the MS. In this case, the MS may assume that the BTS uses a timeslot length of 156,25 normal symbol periods on all timeslots using normal symbol period and a timeslot length of 187,5 reduced symbol periods on all timeslots using reduced symbol period. In the case of a combination of circuit and packet switched channel configuration the MS may derive the common timebase from the circuit switched channel only.

*** End of Changes ***