Solving the Security Threat for MTA Positioning Methods

# introduction

As described in [1], one drawback of the MTA procedure, when performed using the RLC Data Block method is that a bandit MS can monitor MTA procedure related transmissions made by a valid MS in a given cell and mimic these transmissions in a neighbor cell. The information provided by the bandit MS in the mimicked transmissions can be selected with the express purpose of misleading a BSS/BTS receiving those transmissions and thereby cause it to estimate a substantially inaccurate timing advance value for that MS. The BSS then forwards the inaccurate timing advance information to a SMLC for further processing. When the SMLC processes the full set of timing advance values it receives for the MS it will therefore have a substantially increased probability of estimating a position for the corresponding MS with degraded accuracy. This same security threat applies when the MTA procedure is performed using the Extended Access Burst method since the same MTA Reference ID is used in all cells used for the MTA procedure. When the Access Burst method is used this security threat does not exist since this method already involves the SMLC generating cell specific Short ID values and including them as part of the assistance information sent encrypted within the triggering RRLP message.

# Solution for RLC data block method

The key objective is to introduce a means for reducing the possibility of a bandit MS triggering a BSS/BTS to generate false timing advance information and reporting it to the SMLC.  This can be achieved through the introduction of a cell specific identifier, securely delivered to the MS in the triggering RRLP message, that can be sent by the MS when performing the MTA procedure in a given cell. When a BTS/BSS estimates a timing advance value applicable for a MS in that cell it relays the timing advance value, the identity of the cell and the cell specific identifier to a SMLC in the context of a MTA report. The SMLC then associates the received MTA report to a MS and determines if the cell specific identifier received in the MTA report matches the cell specific identifier it provided for that cell in the assistance information included in the associated instance of the triggering RRLP message. The SMLC discards any MTA report that does not include a matching cell specific identifier. The signaling associated with this solution are as follows:

- The SMLC includes a cell specific “Random ID” parameter (e.g. 8 bits long) having a value randomly generated by the SMLC for each cell for which it provides assistance information within the RRLP Multilateration Timing Advance Request message.

- When using a cell identified by assistance information to perform MTA using the RLC Data Block method the MS includes the corresponding “Random ID” value as a parameter within the transmitted RLC Data Block.

- The SMLC may also include a “Spare Random ID” parameter that does not correspond to a cell for which it provides cell specific assistance information. This allows the MS to select a cell (for which cell specific assistance information is not provided) and perform the MTA procedure therein and still include the “Spare Random ID” parameter as the “Random ID” parameter in the RLC Data Block.

- Upon receiving the RLC Data Block the BSS forwards the “Random ID” parameter (along with timing advance and cell ID information) to the SMLC, thereby allowing the SMLC to confirm the validity of the reported timing advance information for any given cell before using it to estimate the position of the corresponding MS.

# Solution for the extended access burst method

The key objective is the same as described for the RLC Data Block method and the solution can be realized using the following signaling:

- The SMLC can include a cell specific “Mini Random ID” parameter (e.g. 3 bits long) having a value randomly generated by the SMLC for each cell for which it provides assistance information within the RRLP Multilateration Timing Advance Request message. The number of bits used for this parameter will be determined by the available payload space within the first and second multilateration access request messages sent by a MS using the Extended Access Burst method.

- When using a cell identified by assistance information to perform MTA using the Extended Access Burst method the MS shall include the corresponding “Mini Random ID” value as a parameter within the second multilateration access request message (also referred to as the Extended Access Burst).

- The SMLC may also include a “Mini Spare Random ID” parameter that does not correspond to a cell for which it provides cell specific assistance information. This allows the MS to select a cell (for which cell specific assistance information is not provided) and perform the MTA procedure therein and still include a “Mini Spare Random ID” parameter as the “Mini Random ID” parameter in the second multilateration access request message.

- Upon receiving the first and second multilateration access request messages the BSS forwards the “Mini Random ID” parameter (along with the timing advance and cell ID information) to the SMLC, thereby allowing the SMLC to confirm the validity of the reported timing advance information for any given cell before using it to estimate the position of the corresponding MS.

Observation 1: Solutions to the MTA reporting security issue identified in [1] can be realized using simple enhancements to the RLC Data Block and Extended Access Burst methods for the MTA procedure whereby assistance information sent in the RRLP message can include “Random ID” information that a MS can relay to the BSS when performing the radio access part of the MTA procedure.

Observation 2: To keep the solution as generic as possible for both the RLC Data Block and Extended Access Burst methods the “Random ID” value sent by a MS performing the radio access part of the MTA procedure can be 16 bits long.

Observation 3: After reception of a “Random ID” value by the BSS it is sent to the SMLC along with corresponding MTA timing advance information using a SCCP connection that the Random ID is logically associated with (i.e. when the SMLC first allocates a set of Random ID values for inclusion in a RRLP message they will always be associated with a SCCP connection used by the SMLC for a specific MTA procedure). The reception of any one of the allocated Random ID values along with corresponding MTA timing advance information therefore allows the SMLC to validate the timing advance information (i.e. the MTA timing advance information is accepted only if the corresponding “Random ID” is received for the first time during the corresponding MTA procedure).

Working Assumption 1: The RLC Data Block and Extended Access Burst methods used for MTA will always make use of a 16 bit “Random ID” value that the MS selects from a set of up to “Random ID” values included as part of the assistance information sent in the RRLP MTA Request message.

Working Assumption 2: When a MS performs the radio access part of the MTA procedure using the RLC Data Block and Extended Access Burst methods it will always convey the “Random ID” value it has selected to the BSS over the radio interface.

Working Assumption 3: Each “Random ID” value provided by the RRLP MTA Request message can only be used in only 1 cell selected by the MS for performing the radio access part of the MTA procedure.

# summary

The sourcing company proposes that the solutions described in sections 2 and 3 and captured in the proposed working assumptions below should be included within the scope of the ongoing efforts towards completing the Release 14 “ePOS_GERAN“ work item for GERAN.

Working Assumption 1: The RLC Data Block and Extended Access Burst methods used for MTA will always make use of a 16 bit “Random ID” value that the MS selects from a set of up to “Random ID” values included as part of the assistance information sent in the RRLP MTA Request message.

Working Assumption 2: When a MS performs the radio access part of the MTA procedure using the RLC Data Block and Extended Access Burst methods it will always convey the “Random ID” value it has selected to the BSS over the radio interface.

Working Assumption 3: Each “Random ID” value provided by the RRLP MTA Request message can only be used in only 1 cell selected by the MS for performing the radio access part of the MTA procedure.

# references

[1]	R6-170200 “On Security of the MTA Method for Position Estimation”, source Nokia, RAN6#4