3GPP TSG-RAN6 Meeting #4	R6-170224

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 44.018 | CR | 1065 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of enhanced DECOR for GERAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eDecor |  |  |  |  |  | Date: |  |  | 2017-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enhanced DECOR has been finalized in SA2#116 and for UTRAN and LTE in RAN#75. It needs to be introduced in GERAN specifications to support the functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A bit is introduced in SI13 to indicate if a cell supports enhanced DECOR or not. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhanced DECOR is not supported in GERAN system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.1.43q, 10.5.2.37b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 44.060 CR 1641 TS 48.018 CR 0438 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	(void)

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	(void)

[4]	(void)

[5]	(void)

[6]	3GPP TS 22.011: "Technical Specification Group Services and System Aspects; Service accessibility".

[7]	(void)

[8]	(void)

[9]	(void)

[10]	3GPP TS 23.003: "Numbering, addressing and identification".

[11]	3GPP TS 43.013: "Discontinuous Reception (DRX) in the GSM system".

[12]	(void)

[12a]	3GPP TS 43.059: "Functional Stage 2 Description of Location Services in GERAN".

[13]	(void)

[14]	3GPP TS 23.022: "Functions related to Mobile Station (MS) in idle mode and group receive mode".

[15]	3GPP TS 24.002: "GSM - UMTS Public Land Mobile Network (PLMN) access reference configuration".

[16]	3GPP TS 44.003: "Mobile Station - Base Station System (MS - BSS) interface; Channel structures and access capabilities".

[17]	3GPP TS 44.004: "Layer 1; General requirements".

[18]	3GPP TS 44.005: "Data Link (DL) layer; General aspects".

[19]	3GPP TS 44.006: "Mobile Station - Base Station System (MS - BSS) interface; Data Link (DL) layer specification".

[20]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[21]	3GPP TS 24.010: "Mobile radio interface layer 3 Supplementary services specification; General aspects".

[22]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[23]	3GPP TS 44.012: "Short Message Service Cell Broadcast (SMSCB) support on the mobile radio interface".

[23a]	3GPP TS 44.071: "Location Services (LCS); Mobile radio interface layer 3 Location Services (LCS) specification".

[23b]	3GPP TS 44.031 "Location Services; Mobile Station (MS) - Serving Mobile Location Centre (SMLC); Radio Resource LCS Protocol (RRLP)".

[24]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification - Formats and coding".

[25]	(void)

[26]	(void)

[27]	(void)

[28]	(void)

[29]	(void)

[30]	(void)

[31]	(void)

[32]	3GPP TS 45.002: "Multiplexing and multiple access on the radio path".

[33]	3GPP TS 45.005: "Radio transmission and reception".

[34]	3GPP TS 45.008: "Radio subsystem link control".

[35]	3GPP TS 45.010: "Radio subsystem synchronization".

[36]	(void)

[37]	(void)

[38]	(void)

[39]	3GPP TS 51.010: "Mobile Station (MS) conformance specification".

[40]	(void)

[41]	(void)

[42]	(void)

[43]	(void)

[44]	(void)

[45]	(void)

[46]	(void)

[47]	(void)

[48]	(void)

[49]	(void)

[50]	(void)

[51]	(void)

[52]	(void)

[53]	ITU-T Recommendation Q.931: "ISDN user-network interface layer 3 specification for basic control".

[54]	(void)

[55]	(void)

[56]	(void)

[57]	(void)

[58]	(void)

[59]	(void)

[60]	(void)

[61]	(void)

[62]	(void)

[63]	(void)

[64]	(void)

[65]	(void)

[66]	(void)

[67]	(void)

[68]	(void)

[69]	(void)

[70]	(void)

[71]	(void)

[72]	(void)

[73]	(void)

[74]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service Description; Stage 2".

[75]	(void)

[76]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control / Medium Access Control (RLC/MAC) protocol".

[77]	(void)

[78]	(void)

[79]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols - Stage 3".

[80]	TIA/EIA/IS-2000-5-A: "Upper Layer (Layer 3) Signaling Standard for cdma2000 Spread Spectrum Systems".

[81]	TIA/EIA/IS-833: "G3G CDMA-MC to GSM-MAP".

[82]	TIA/EIA/IS-2000-4-A: "Link Access Control (LAC) Standard for cdma2000 Spread Spectrum Systems".

[83]	3GPP TS 45.009: "Link Adaptation".

[84]	3GPP TS 26.103: "Speech Codec List for GSM and UMTS".

[85]	Michel MOULY, "CSN.1 Specification, Version 2.0", Cell & Sys, ISBN: 2-9510062-0-9. 
Web: http://perso.wanadoo.fr/cell.sys/.

[86]	3GPP TS 45.003: "Channel Coding".

[87]	3GPP TS 31.102: "Characteristics of the USIM application".

[88]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[89]	3GPP TS 25.101: "User Equipment (UE) radio transmission and reception (FDD)".

[90]	3GPP TS 25.102: "User Equipment (UE) radio transmission and reception (TDD)".

[91]	3GPP TS 25.224: "Physical Layer Procedures (TDD)".

[92]	3GPP TS 28.062: "Inband Tandem Free Operation (TFO) of speech codecs; stage 3".

[93]	3GPP TS 43.068: "Voice Group Call Service (VGCS); Stage 2".

[94]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[95]	3GPP TS 29.280: "3GPP EPS Sv interface (MME to MSC) for SRVCC".

[96]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[97]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[98]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[99]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[100]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[101]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[102]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[103]	3GPP TS 23.251: "Network Sharing; Architecture and functional description".

[104]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[105]	3GPP TS 25.123: "Requirements for support of radio resource management (TDD)".

[106]	3GPP TS 25.133: "Requirements for support of radio resource management (FDD)".

[107]	3GPP TS 48.018: "BSS GPRS Protocol (BSSGP)".

[108]	3GPP TS 23.682: “Architecture enhancements to facilitate communications with packet data networks and applications”

[109]	3GPP TS 43.064: “Overall description of the GPRS Radio Interface; Stage 2”.

[110]	3GPP TS 24.301: “General Packet Radio Service (GPRS) enhancements for  Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

Second Change

9.1.43q	EC System information type 2

This message is sent on the EC-BCCH if EC-GSM-IoT is supported in the cell. It is sent by the network providing EC-RACH/RACH control information and cell selection information to mobile stations that have enabled EC operation. Multiple instances of this message may be sent by the network. The requirements for transmission of this message are defined in 3GPP TS 45.002.

Message type:	EC SYSTEM INFORMATION TYPE 2

Significance:	dual

Direction: network to mobile station< EC System Information Type 2 > ::= < Message Type : bit (3) > < EC SI 2_INDEX : bit (2) > < EC SI 2_COUNT : bit (2) > < EC SI_CHANGE_MARK : bit (5) > { 0 ¦ 1 < EC Cell Selection Parameters : < EC Cell Selection Parameters struct >> } { 0 ¦ 1 < Coverage Class Selection Parameters : < Coverage Class Selection Parameters struct >> } { 0 ¦ 1 < EC-RACH Control Parameters : < EC-RACH Control Parameters struct >> } { 0 ¦ 1 < Short RACH Control Parameters : < Short RACH Control Parameters struct >> } { 0 ¦ 1 < EC Cell Options: < EC Cell Options struct >> } -- Additions in release Rel-14: < MS_ASSISTED_DCN : bit > < spare padding > ; < EC Cell Selection Parameters struct > ::= < Location Area Identification : bit (40) > < Routing Area Code : bit (8) > < Cell Identity : bit (16) > < EC_BS_CC_CHANS : bit (2) > < EC_RXLEV_ACCESS_MIN : bit (6) > -- used in the path loss criterion C1 < MS_TXPWR_MAX_CCH : bit (5) > -- used in the path loss criterion C1 { 0 ¦ 1 < LB_MS_TXPWR_MAX_CCH : bit (5) > } { 0 ¦ 1 < CELL_SELECTION_RLA_MARGIN : bit (3) > }; < Coverage Class Selection Parameters struct > ::= < DL_CC_Selection: bit (1) > < BT_Threshold_DL: bit (5) > { 0 ¦ 1 < CC2_Range_DL : bit (5) > } { 0 ¦ 1 < CC3_Range_DL : bit (5) > } < BT_Threshold_UL: bit (5) > { 0 ¦ 1 < CC2_Range_UL : bit (5) > } { 0 ¦ 1 < CC3_Range_UL : bit (5) > } < BSPWR : bit (6) > { 0 ¦ 1 < DL_Signal_Strength_Step_Size : bit (2) > } < EC_Reduced_PDCH_Allocation : bit(1) >; < EC-RACH Control Parameters struct > ::= < EC_Max_Retrans : bit (2) > < Sm : bit (2) > < Tm : bit (2) > < Access_Timeslots : bit (1) > < CC_Access_Adaptation: bit (2) > < Cell_Bar_Access : bit (1) > < EC_Access_Control_Class : bit (7) > < Exception_Report_Status : bit (1) > { 0 ¦ 1 < BT_Threshold_UL_Margin : bit (3) > } ; < Short RACH Control Parameters struct > ::= < Max_Retrans : bit (2) > < Tx-integer : bit (4) > < Cell_Bar_Access : bit (1) > { 0 -- Access Control Class information and Exception Report Status in EC-RACH Control Parameters applies ¦ 1 -- The indicated Access Control Class bitmap and Exception Report Status field applies < Access_Control_Class : bit (16) > < Exception_Report_Status : bit (1) > } ; < EC Cell Options struct > ::= { 0 ¦ 1 < ALPHA : bit (4) > } { 0 ¦ 1 < T3168 : bit (3) > } { 0 ¦ 1 < T3192 : bit (3) > } { 0 ¦ 1 < T3226 : bit (3) > } < T3248 : bit (2) > ;

Figure 9.1.43q.1: EC SYSTEM INFORMATION TYPE 2 message content

Table 9.1.43q.1: EC SYSTEM INFORMATION TYPE 2 information element details

| Message Type (3 bits) The Message Type field is encoded according to Table 10.4.4. |  |
| --- | --- |
| EC SI 2_INDEX (2 bits) and EC SI 2_COUNT (2 bits) The purpose of these fields is to indicate the number of individual message instances within the sequence of EC SI 2 messages and to assign an index to identify each instance. The EC SI 2_INDEX field is binary coded, range 0 to 3, and provides an index to identify an individual EC SI 2 message instance. The EC SI 2_COUNT field is binary coded, range 0 to 3, and provides the value of the highest indexed message instance in the sequence of EC SI 2 messages. |  |
| EC SI_CHANGE_MARK (5 bits) This field is defined in Table 9.1.43p.1. |  |
| EC Cell Selection Parameters struct This structure provides the cell selection parameters for the serving cell. Location Area Identification (40 bits) The Location Area Identification field is coded as the value part of the Location Area Identification IE specified in sub-clause 10.5.1.3. |  |
| Routing Area Code (8 bits) This field is the binary representation of the Routing Area Code, see 3GPP TS 23.003. |  |
| Cell Identity (16 bits) The Cell Identity field is coded as the value part of the Cell Identity IE specified in sub-clause 10.5.1.1. |  |
| EC_BS_CC_CHANS (2 bits) This field indicates the number of extended coverage common control channels (EC-CCCHs) supported in the cell. Value 00 1 EC-CCCH supported 01 2 EC-CCCHs supported 10 3 EC-CCCHs supported 11 4 EC-CCCHs supported If the Access_Timeslots field provided in this message indicates that 2 TS EC-RACH mapping shall be applied in the cell, then the number of EC-CCCHs supported in the cell shall be equal to or lower than the number of common control channels (CCCHs) configured in the same cell. |  |
| EC_RXLEV_ACCESS_MIN (6 bits) The EC_RXLEV_ACCESS_MIN field is coded as the binary representation of the minimum received signal level at the MS for which it is permitted to access the system, see 3GPP TS 45.008. |  |
| MS_TXPWR_MAX_CCH (5 bits) The MS_TXPWR_MAX_CCH field is coded as the binary representation of the "power control level" in 3GPP TS 45.005 corresponding to the maximum TX power level an MS may use when accessing the system on the (EC-)CCCH. This value shall be used by the Mobile Station according to 3GPP TS 45.008. |  |
| LB_MS_TXPWR_MAX_CCH (5 bits) The LB_MS_TXPWR_MAX_CCH field is coded as the binary representation of the "power control level" in 3GPP TS 45.005 corresponding to the maximum TX power level an MS may use on all other than DCS 1800 and PCS 1900 frequency bands when accessing the system on the (EC-)CCCH. This value shall be used by the Mobile Station according to 3GPP TS 45.008. |  |
| CELL_SELECTION_RLA_MARGIN (3 bits) The CELL_SELECTION_RLA_MARGIN field provides the MS with information whether RLA_EC and RLA_GC based measurements may be omitted or not. The use of this field is defined in 3GPP TS 45.008. |  |
| Coverage Class Selection Parameters struct This structure provides information for the MS to estimate its uplink and downlink coverage class. |  |
| DL_CC_Selection (1 bit) This field indicates the method for selecting the downlink coverage class to be used by the MS. Bit 0 RLA_EC based coverage class selection 1 SLA based coverage class selection The use of this field is defined in 3GPP TS 45.008. |  |
| BT_Threshold_DL (5 bits) This field indicates the threshold below which blind physical layer transmissions are used on EC-CCCH. The use of this field is defined in 3GPP TS 45.008. |  |
| CC2_Range_DL (5 bits) CC3_Range_DL (5 bits) These fields are optionally sent by the network to indicate the signal level range of the indicated downlink coverage classes. The presence of one or both of the above fields indicates network support of the associated downlink coverage class. The use of these fields is defined in 3GPP TS 45.008. |  |
| BT_Threshold_UL (5 bits) This field indicates the signal level threshold below which blind physical layer transmissions are used on EC-RACH. The use of this field is defined in 3GPP TS 45.008. |  |
| CC2_Range_UL (5 bits) CC3_Range_UL (5 bits) These fields are optionally sent by the network to indicate the signal level range of the indicated uplink coverage classes. The presence of one or both of the above fields indicates network support of the associated uplink coverage class. The use of these fields is defined in 3GPP TS 45.008. |  |
| BSPWR (6 bits) This field indicates the BTS output power transmitted on FCCH and EC-SCH. The use of this field is defined in 3GPP TS 45.008. |  |
| DL_Signal_Strength_Step_Size (2 bits) This field indicates the step-size in signal level above BT_Threshold_DL possible to report by the MS in the EC Packet Channel Request message (see sub-clause 9.1.65). It is encoded as follows: Bits 2 1 0 0 X = 2 0 1 X = 4 1 0 X = 6 1 1 X = 8 |  |
| EC_Reduced_PDCH_Allocation (1 bit) This field indicates that the number of consecutive PDCHs the network allocates when assigning an EC TBF to a MS indicating Coverage Class CC2, CC3 or CC4 on the uplink or downlink during packet access. Bit 0 Four consecutive PDCHs are always allocated to MS for an EC TBF. 1 Two consecutive PDCHs are always allocated to MS for an EC TBF. |  |
| EC-RACH Control Parameters struct The EC-RACH Control Parameters contains the access control parameters needed when accessing the network on the EC-RACH. EC_Max_Retrans (2 bits) This field indicates the maximum number of retransmissions on EC-RACH (see sub-clause 3.5.2.1.2a). It is encoded as the Max retrans field in the RACH Control Parameters IE defined in sub-clause 10.5.2.29. |  |
| Sm (2 bits) This field is used by a MS to determine the number of multiframes it needs to read on the EC-AGCH in an attempt to find a response matching its last EC-RACH transmission (see sub-clause 3.5.2.1.2a). Bits 2 1 0 0 1 or more multiframes 0 1 2 or more multiframes 1 0 3 or more multiframes 1 1 4 or more multiframes |  |
| Tm (2 bits) This field is used by a MS to determine the number of multiframes on the EC-RACH from which it randomly selects a transmission/retransmission opportunity (see sub-clause 3.5.2.1.2a). Bits 2 1 0 0 1 or more multiframes 0 1 2 or more multiframes 1 0 3 or more multiframes 1 1 4 or more multiframes |  |
| Access_Timeslots (1 bit) This field indicates whether random access mapped over two timeslots (e.g. TS 0 and TS 1, or, TS 2 and TS 3) shall be applied or not (see sub-clause 3.5.2.1.2a). Bit 0 1 TS EC-RACH mapping shall be applied. 1 2 TS EC-RACH mapping shall be applied if uplink Coverage Class 2, Coverage Class 3 or Coverage Class 4 has been selected. The number of EC-RACH supported in the cell is indicated by the EC_BS_CC_CHANS field provided in this message. |  |
| CC_Access_Adaptation (2 bits) This field indicates whether the mobile station is allowed to increment its Coverage Class after one or more failed access attempts on the EC-RACH (see sub-clause 3.5.2.1.2a). Bits 2 1 0 0 Coverage Class adaptation not allowed 0 1 Coverage Class adaptation after 1 failed EC-RACH transmission attempt 1 0 Coverage Class adaptation after 2 failed EC-RACH transmission attempts 1 1 Coverage Class adaptation after 3 failed EC-RACH transmission attempts |  |
| Cell_Bar_Access (1 bit) This field indicates whether the cell is bared for access. It is encoded as the CELL_BAR_ACCESS field in the RACH Control Parameters IE defined in sub-clause 10.5.2.29. |  |
| EC_Access_Control_Class (7 bits) This field contains a bitmap indicating the barring status for Access Control Classes 0 to 15 (AC C0 to AC C15) applicable for the Common PLMN provided in this message. The EC_Access_Control_Class field is encoded as follows. Bits 7 6 5 4 3 2 1 x x x x x x x AC C15 AC C14 AC C13 AC C12 AC C11 AC C10 AC C0 to AC C9 For a mobile station with AC C = N access is not barred if the AC CN bit is coded with a "0"; N = 0, 1, .. 9, 11, .., 15. Access control for mobile stations belonging to one of the access control classes 0 to 9 (AC C0 to AC C9) is indicated by the first bit in the bitmap (bit 1). For an MS in EC operation the AC C10 field is not used in this version of the specification. |  |
| Exception_Report_Status (1 bit) This field indicates whether sending of exception reports are allowed or not. It is encoded as follows: Bit 0 Sending of exception reports allowed in the cell for all MSs 1 Sending of exception reports not allowed in the cell except for the MSs that belong to one of the authorized Access Control Classes 11 to 15. |  |
| BT_Threshold_UL_Margin (3 bits) This optional field indicates the power margin above BT_Threshold_UL used for (EC-)RACH open-loop power control. The use of this field is defined in 3GPP TS 45.008. |  |
| Short RACH Control Parameters struct The Short RACH Control Parameters contains the access control parameters needed when accessing the network on the RACH on the common control channel (CCCH) on TS 0. Max_Retrans (2 bits) This field indicates the maximum number of retransmissions on RACH. It is encoded as the Max retrans field in the RACH Control Parameters IE defined in sub-clause 10.5.2.29. |  |
| Tx-integer (4 bits) This field indicates the number of slots to spread the transmission on RACH. It is encoded as the Tx-integer field in the RACH Control Parameters IE defined in sub-clause 10.5.2.29. |  |
| Access_Control_Class (16 bits) This field contains a bitmap indicating the barring status for Access Control Classes 0 to 15 (AC C0 to AC C15) applicable for the Common PLMN provided in this message. In the absence of this field, the Access Control Class information provided in the EC-RACH Control Parameters struct shall be used instead. The Access_Control_Class field is encoded as follows. Bits 16 15 … 1 x x … x AC C15 AC C14 … AC C0 For an MS in EC operation the AC C10 field is not used in this version of the specification. |  |
| EC Cell Options struct ALPHA (4 bits) This field is the binary representation of the parameter for MS output power control in units of 0.1, see 3GPP TS 45.008. For encoding and description of the ALPHA field see the Global Power Control Parameters IE in 3GPP TS 44.060. T3168 (3 bits) T3192 (3 bits) These fields are defined in 3GPP TS 44.060. T3226 (3 bits) This field is the binary representation of the timeout value of timer T3226. Range: 0 to 7. The timeout values are given in the following table. Bits 3 2 1 0 0 0 20 ms 0 0 1 60 ms 0 1 0 100 ms 0 1 1 200 ms 1 0 0 500 ms 1 0 1 700 ms 1 1 0 1000 ms 1 1 1 1200 ms T3248 (2 bits) This field is the binary representation of the timeout value of timer T3248. Range: 0 to 3. The timeout values are given in the following table. Bits 2 1 0 0 not used 0 1 1 second 1 0 2 seconds 1 1 3 seconds MS_ASSISTED_DCN See sub-clause 10.5.2.37b. |  |

Third Change

#### 10.5.2.37b	SI 13 Rest Octets

The SI 13 Rest Octets information element is coded according to the syntax specified below and described in tables 10.5.2.37b.1 and 10.5.2.37b.2.

The SI 13 Rest Octets information element is a type 5 information element with 20 octets length.

Table 10.5.2.37b.1: SI 13 Rest Octets information element content

< SI 13 Rest Octets > ::= { L ¦ H < BCCH_CHANGE_MARK : bit (3) > < SI_CHANGE_FIELD : bit (4) > { 0 ¦ 1 < SI13_CHANGE_MARK : bit (2) > < GPRS Mobile Allocation : GPRS Mobile Allocation IE > } -- Defined in 3GPP TS 44.060 { 0 -- PBCCH not present in cell (This shall always be indicated. See sub-clause 1.8) : < RAC : bit (8) > < SPGC_CCCH_SUP : bit > < PRIORITY_ACCESS_THR : bit (3) > < NETWORK_CONTROL_ORDER : bit (2) > < GPRS Cell Options : GPRS Cell Options IE > -- Defined in 3GPP TS 44.060 < GPRS Power Control Parameters : GPRS Power Control Parameters IE > ¦ 1 -- PBCCH present in cell (This shall never be indicated. See sub-clause 1.8) : < PSI1_REPEAT_PERIOD : bit (4) > < PBCCH Description : PBCCH Description struct > } { null ¦ L -- Receiver compatible with earlier release ¦ H -- Additions in release 99 : < SGSNR : bit > { null ¦ L -- Receiver compatible with earlier release ¦ H -- Additions in release Rel-4: < SI_STATUS_IND : bit > { null ¦ L -- Receiver compatible with earlier release ¦ H -- Additions in release Rel-6: { 0 ¦ 1 < LB_MS_TXPWR_MAX_CCH : bit (5) > } < SI2n_SUPPORT : bit(2) > { -- Additions in release Rel-11: < SI_CHANGE_ALT > -- Additions in release Rel-13: { 0 ¦ 1 < PEO_DSC : bit (2) > < RCC : bit (3) > } { 0 ¦ 1 < C1_DELTA_MIN : bit (2) > < C1_DELTA_MAX : bit (3) > } -- Additions in release Rel-14: < MS_ASSISTED_DCN : bit > -- Additions in future releases shall be indicated above this line } // -- truncation according to sub-clause 8.9 is allowed, bits "L" assumed } } } } < spare padding > ;

< PBCCH Description struct > ::= < Pb : bit (4) > < TSC : bit (3) > < TN : bit (3) > { 00 -- BCCH carrier ¦ 01 < ARFCN : bit (10) > ¦ 1 < MAIO : bit (6) > } ; < SI_CHANGE_ALT > ::= L ¦ H ;

Table 10.5.2.37b.2: SI 13 Rest Octets information element

BCCH_CHANGE_MARK (3 bit field) This field indicates the status of the information on BCCH. The value of this field may be changed when information on BCCH is changed, see 3GPP TS 44.060.

SI_CHANGE_FIELD (4 bit field) This field is the binary representation of which information was changed at the last indication in BCCH_CHANGE_MARK, see 3GPP TS 44.060. Range 0 to 15: 0 Update of unspecified SI message or SI messages; 1 Update of SI1 message; 2 Update of SI2, SI2 bis or SI2 ter message or any instance of SI2quater messages; 3 Update of SI3, SI4, SI7, SI8, SI16 or SI17 message; 4 Update of SI9 message; 5 Update of SI18 or SI20 message; 6 Update of SI19 message; 7 Update of SI15 message; 8 Update of SI2n message; 9 Update of SI22 or SI23 message; All other values shall be interpreted as 'update of unknown SI message type'.

SI13_CHANGE_MARK (2 bit field) This field is the binary representation of the SI change mark identifying the GPRS Mobile Allocation provided in SI13 and PSI13 messages. Range: 0 to 3.

GPRS Mobile Allocation (information element) This information element is the representation of the GPRS mobile allocation provided in SI13 and PSI13 messages. It is identified by MA_NUMBER = 14 when referenced from a packet assignment message. The GPRS Mobile Allocation information element is defined in 3GPP TS 44.060. When used in SI13 or PSI13 message, this information element shall refer to the cell allocation defined for the cell in SI1 or PSI2.

RAC (8 bit field) This field is the binary representation of the Routing Area Code, see 3GPP TS 23.003.

SPGC_CCCH_SUP (bit field) This field indicates the support of the parameter SPLIT_PG_CYCLE on CCCH from the network side: 0 SPLIT_PG_CYCLE is not supported on CCCH in this cell; 1 SPLIT_PG_CYCLE is supported on CCCH in this cell.

The PRIORITY_ACCESS_THR field (3 bit) is the binary representation of the parameter PRIORITY_ACCESS_THR: 0 0 0 packet access is not allowed in the cell; 0 0 1 spare, shall be interpreted as '000' (packet access not allowed); 0 1 0 spare, shall be interpreted as '000' (packet access not allowed); 0 1 1 packet access is allowed for priority level 1; 1 0 0 packet access is allowed for priority level 1 to 2; 1 0 1 packet access is allowed for priority level 1 to 3; 1 1 0 packet access is allowed for priority level 1 to 4; 1 1 1 spare, shall be interpreted as '110' (packet access allowed).

The NETWORK_CONTROL_ORDER field (2 bit) is the binary representation of the parameter NETWORK_CONTROL_ORDER, see 3GPP TS 44.060: 0 0 NC0: MS controlled cell re-selection, no measurement reporting. 0 1 NC1: MS controlled cell re-selection, MS sends measurement reports. 1 0 NC2: Network controlled cell re-selection, MS sends measurement reports. 1 1 Reserved for future use, interpreted as NC0 by mobile station.

GPRS Cell Options (information element) The GPRS Cell Option information element is defined in 3GPP TS 44.060.

PSI1_REPEAT_PERIOD (4 bit field) This field is the representation of the PSI1 repeat period. The field is coded according to the following table: 0000 PSI1 repeat period = 1 multiframe 0001 PSI1 repeat period = 2 multiframes : 1111 PSI1 repeat period = 16 multiframes

GPRS Power Control Parameters The GPRS Power Control Parameters information element is defined in 3GPP TS 44.060.

PBCCH Description struct The PBCCH description struct provides the channel description for the PBCCH. The frequency description for the PBCCH may be specified by an ARFCN (non-hopping radio frequency channel) or a MAIO (hopping radio frequency channel) field. In case of a hopping radio frequency channel, the PBCCH shall use the GPRS mobile allocation specified in this message. If none of the ARFCN or MAIO fields are present, the PBCCH shall use the BCCH carrier. Pb (4bit) (for encoding and description see the Global Power Control Parameters IE) The TSC field (3 bit) is the binary representation of the training sequence code used for PBCCH. Range: 0 to 7. The TN field (3 bit) is the binary representation of the timeslot number for the PBCCH. Range: 0 to 7. The ARFCN field (10 bit) is the binary representation of the absolute RF channel number. Range: 0 to 1023. The MAIO field (6 bit) is the binary representation of the mobile allocation index offset. Range: 0 to 63.

SGSNR, SGSN Release (1 bit field) 0 SGSN is Release '98 or older 1 SGSN is Release '99 onwards

SI_STATUS_IND (1 bit field) 0 The network does not support the PACKET SI STATUS message; 1 The network supports the PACKET SI STATUS message.

LB_MS_TXPWR_MAX_CCH (5 bit field) The LB_MS_TXPWR_MAX_CCH field is coded as the binary representation of the 'power control level' in 3GPP TS 45.005 corresponding to the maximum TX power level a mobile station may use when accessing on a packet control channel. This value shall be used by the mobile station according to 3GPP TS 45.008.

SI2n_SUPPORT (2 bit field) The SI2n_SUPPORT field indicates whether the network supports the SI2n message provision. 00 SI2n is not supported 01 SI2n is supported on PACCH 10 SI2n is supported on PACCH and broadcast on BCCH 11 SI2n is supported on PACCH and broadcast on BCCH Ext.

SI_CHANGE_ALT This field indicates how a mobile station supporting network sharing shall take into account a change of system information signalled by a SI_CHANGE_FIELD value '2': L A mobile station supporting network sharing should not attempt to re-read the SI2quater message; H A mobile station supporting network sharing shall fully take into account a change of system information signalled by the SI_CHANGE_FIELD value '2', SI2quater included. PEO_DSC (2 bit field) This field provides an intial value for the downlink signalling counter (DSC) used for verifying the ongoing ability of a MS that has enabled eDRX to receive paging messages on the PCH (3GPP TS 45.008). The presence of the PEO_DSC field indicates the serving cell supports PEO. The PEO_DSC field is coded according to the following table: 00 DSC = 4 01 DSC = 6 10 DSC = 8 11 DSC = 10 RCC (3 bit) See sub-clause 10.5.2.16. C1_DELTA_MIN (2 bit field) See sub-clause 9.1.43r. C1_DELTA_MAX (3 bit field) See sub-clause 9.1.43r. MS_ASSISTED_DCN This field indicates if the cell supports MS assisted Dedicated Core Network selection, see 3GPP TS 23.401 [110]. 0 The network does not support MS assisted Dedicated Core Network selection; 1 The network supports MS assisted Dedicated Core Network selection. All remaining bit positions are reserved.

NOTE:	The SGSN Release bit indicates the version of the SGSN specific protocols and is not applicable to access stratum protocols.