3GPP TSG RAN WG6 #7 	R6-180019

# Athens, Greece, February 26- March 2, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 48.018 | CR | 0443 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Restricted Use of Enhanced Coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  |  | Date: |  | 2018-02-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Restricted use of enhanced coverage was recently introduced in SA2 specification TS 23.060 by S2-166937 and in CT1 specification TS 24.008 by C1-170529 and C1-172085 as part of the CIoT_Ext work item in SA. However, for EC-GSM-IoT the impacts to existing radio interface procedures, such as cell reselection, DL and UL CC selection in connected mode and the use of the paging procedure for granting again unrestricted use of enhanced coverage, from introduction of this feature have not been considered so far. These aspects require changes to GERAN specifications as outlined in R6-180015, which also relate to the present specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In sub-clause 6.1 SGSN behaviour to include the status of enhanced coverage restriction as part of DL UNITDATA message. - In sub-clause 7.1 SGSN behaviour is specified regarding the removal of the enhanced coverage restriction status, sent in the PAGING PS message to the BSS, if the MS camps on a non-EC-GSM-IoT supporting cell. - In sub-clause 7.1a SGSN behaviour is specified regarding the removal of the enhanced coverage restriction status, sent in the PAGING PS message to the BSS, if the MS camps on an EC-GSM-IoT supporting cell. - In sub-clauses 10.2.1, 10.3.1 and 11.3.141 the Enhanced Coverage Additional Information IE (optional) of the DL UNITDATA and PAGING PS PDU carrying the status for removal of the enhanced coverage restriction is specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The restricted use of enhanced coverage is not correctly implemented on radio layer, hence it cannot be used in a Rel-14 network supporting EC-GSM-IoT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 7.1, 7.1a, 10.2.1,10.3.1, 11.3.141 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064 CR 0116 TS 44.018 CR 1081 TS 45.008 CR 0655 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010 CR XXXX |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |

1st Modification

# 6	User data and signalling procedures between RL and BSSGP SAPs

## 6.1	Downlink UNITDATA procedure

On the downlink, a DL-UNITDATA PDU shall contain information elements to be used by the RLC/MAC function and an LLC-PDU. There shall be only one LLC-PDU per DL-UNITDATA PDU. The LLC-PDU shall always be the last information element in the DL-UNITDATA PDU, and shall be aligned on a 32 bit boundary for efficient processing.

An SGSN provides the BSSGP with a current TLLI, identifying the MS. If an SGSN provides a second TLLI, indicating that an MS has recently changed its TLLI, this shall be considered as the "old" TLLI. A BSS uses the "old" TLLI to locate an MS's existing context. Subsequent uplink data transfers for this MS shall reference the current TLLI, and not the old TLLI.

The SGSN shall include the IMSI in the PDU. As an exception, the SGSN may omit the IMSI in the PDU if the mobile station identified by the TLLI is in MM non-DRX mode period (i.e. during a GMM procedure for GPRS attach or routing area updating defined in 3GPP TS 24.008) and the SGSN does not have a valid IMSI.

The SGSN may include the Service UTRAN CCO (Cell Change Order) information element in the PDU (relevant if the network initiated cell change order to UTRAN, network initiated cell change order to E-UTRAN, PS handover to UTRAN or PS handover to E-UTRAN procedures are used). If this information element is received in multiple PDUs (either DL-UNITDATA PDU(s), CREATE-BSS-PFC PDU(s) or PS-HANDOVER-REQUEST PDU(s)), the information element contained in the last received PDU shall take precedence.

If the SGSN has valid DRX Parameters for a TLLI, then the SGSN shall include them in the PDU. Nevertheless, the SGSN can omit the DRX Parameters if the MS identified with the TLLI is in MM non-DRX mode period to speed up the transmission of the LLC-PDU on the radio interface. The SGSN shall not send a DL-UNITDATA PDU without the DRX Parameters IE if the MS identified with the TLLI is not in MM non-DRX mode period.

An exception case is when the SGSN has uplink and downlink Coverage Class information (previously received from the BSS in the UL-UNITDATA PDU) or eDRX information (negotiated during NAS signalling) or both available for a given MS in which case it shall proceed as follows when sending a DL-UNITDATA PDU for that MS:

-	If uplink and downlink Coverage Class information is available for the MS then the SGSN shall include the Coverage Class information in the DL-UNITDATA. If a negotiated eDRX value is available then it shall also be included. The SGSN considers the eDRX to be negotiated upon accepting the eDRX cycle value requested by a MS during NAS signalling. If a negotiated eDRX value is not available but DRX information is available then it shall be included. If the target cell supports EC-GSM-IoT the corresponding LLC-PDU is delivered as follows:

-	If the MS is in idle mode a downlink packet resource assignment is sent on the EC-AGCH of its EC_CCCH_GROUP (see 3GPP TS 45.002 [32]) using the indicated downlink Coverage Class information and the lowest eDRX cycle and the LLC PDU is sent thereon.

-	If the MS is in packet transfer mode the LLC PDU is delivered using either an existing downlink EC TBF or using a new downlink EC TBF established by sending a downlink packet resource assignment on the EC-PACCH (see 3GPP TS 44.060 [22]) using the downlink Coverage Class information the BSS is currently using for that MS.

-	If Coverage Class information is not available but a negotiated eDRX value is available for the MS then the SGSN shall include the negotiated eDRX value in the DL-UNITDATA PDU. If the target cell supports eDRX the corresponding LLC-PDU is delivered as follows:

-	If the MS is in idle mode a downlink packet resource assignment is sent on the AGCH of its CCCH_GROUP (see 3GPP TS 45.002 [32]) using the lowest eDRX cycle and the LLC PDU is sent thereon.

-	If the MS is in packet transfer mode the LLC PDU is delivered using either an existing downlink TBF or using a new downlink TBF established by sending a downlink packet resource assignment on the PACCH (see 3GPP TS 44.060 [22]).

-	The SGSN shall apply these rules also when sending a DL-UNITDATA PDU to a MS for which the GMM procedure for GPRS attach or routing area updating (defined in 3GPP TS 24.008) has not yet completed.

An SGSN provides the BSSGP with MS specific information, enabling the RLC/MAC entity in a BSS to transmit an LLC-PDU to the MS in a user specific manner. The information made available to the radio interface includes:

-	MS Radio Access Capability. This defines the radio capabilities of the ME. If there is valid MS Radio Access Capability information known by the SGSN for the associated MS, the SGSN shall include it in the DL-UNITDATA PDU. Otherwise, MS Radio Access Capability shall not be present;

-	Packet Flow Identifier. This identifies the packet flow context associated with the LLC PDU and is included by the SGSN if the packet flow context feature is negotiated. If the mobile station does not support the PFC feature or if the PFI is not known (e.g. the new SGSN did not get the PFI from the old SGSN during a RAU) then the SGSN shall use the pre-defined PFI to indicate best-effort QoS;

-	QoS Profile. This defines the (peak) bit rate, the type of BSSGP's SDU (signalling or data), the type of LLC frame (ACK, SACK, or not), the precedence class, and the transmission mode to be used when transmitting the LLC-PDU across the radio interface;

-	PDU Lifetime. This defines the remaining time period that the PDU is considered as valid within the BSS. If the PDU is held for a period exceeding the "PDU Lifetime" time period, the PDU shall be locally discarded. The PDU Lifetime is set within the SGSN by the upper layers.

An SGSN provides the BSSGP with Enhanced Coverage Additional Information which indicates to the BSS whether the use of enhanced coverage is restricted or not for a given MS, thus enabling the BSS to take this status into account for deciding the appropriate coverage class (uplink and/or downlink) in packet transfer mode.

A BSS may incorporate the PDU Lifetime, the Precedence and the (peak) bit rate into its radio resource scheduler. If the PFI is present then the BSS may incorporate the information from the associated ABQP into its radio resource scheduler. The algorithm to do this is out of scope of the present document.

If the PFI is known in the BSS and does not correspond to a predefined PFI, then:

-	the (peak) bit rate and the precedence class fields present in the QoS Profile IE shall be ignored by the BSS;

-	if the Allocation/Retention Priority was provided at the time the corresponding PFC was created or last modified, then the ‘Priority’ IE, if present in the downlink UNITDATA PDU, is discarded.

Two types of BSSGP SDU are distinguished within the QoS Profile: layer 3 signalling and data. Layer 3 signalling may be transmitted over the Um interface with higher protection. If the MS has an RR connection to the network (see 3GPP TS 44.018), Layer 3 signalling may be transmitted over the Um interface on the main signalling link of the RR connection, provided that the LLC PDU meets length restrictions imposed by the BSS. In this case, the BSS shall include the LLC PDU contained in the BSSGP PDU in the correspondent Layer 3 Um interface message (see 3GPP TS 44.018).

The type of LLC frame indicates if the LLC frame type is an ACK or SACK command/response, or not (see 3GPP TS 44.064). An ACK or SACK command/response frame type may be transmitted over the Um interface with higher protection.

Two transmission modes across the radio interface are possible: acknowledged (using RLC/MAC ARQ functionality) and unacknowledged (using RLC/MAC unitdata functionality). These transmission modes do not apply when the MS has an RR connection to the network and BSS uses the main signalling link of the RR connection, in which case the acknowledged transmission mode is used.

If Priority is present, assuming it shall not be discarded according to the rule above, only the priority-level field shall be regarded. The management of priority levels is implementation dependent and under operator control. The preemption capability indicator, the queuing allowed indicator and preemption vulnerability indicator shall be ignored in this case.

In addition to constructing the DL-UNITDATA, the SGSN supplies the LSP, the BVCI, the NSEI, and for an IP sub-network the NS Change IP endpoint, associated with the MS to the lower layer network service, enabling network service routeing to the peer entity. These parameters are not transmitted as part of the BSSGP across the Gb-interface for the purpose of identifying the receiving endpoint (they are sent in the BSSGP Perform-Location-Request PDU to identify the serving cell of the target MS).

If the Gb-interface is supported using an IP sub-network, then the Resource Distribution function at the SGSN may transmit a BSSGP DL-UNITDATA PDU with an LLC-PDU Length Indicator set to 0. The BSS uses this DL-UNITDATA to change the IP endpoint at the SGSN to which any future UL-UNITDATA for the TLLI (indicated in the DL-UNITDATA) is sent. The LLC-PDU with a Length Indicator set to 0 is not sent across the radio interface.

In the case where localised service area is supported the SGSN may inform the BSS as to which LSA identities that the mobile has preferences by sending the LSA INFORMATION element. The BSS stores this information and uses it e.g. for network controlled cell re-selection when determining specific cell selection parameters for the mobile. The algorithm for determining specific cell selection parameters for the mobile is not defined further in the present document. The SGSN may inform the BSS about the contents of SPID in the DL-UNITDATA PDU. In this case the SPID is stored in the BSS.

When the “Service Identification for improved Radio Utilization for GERAN” (SIRUG) feature (see clause 5.3.5.3 of TS 23.060 [7]) is supported, and the SGSN receives SCI information in the GTP-U header, the SGSN inserts the SCI IE and the GGSN/P-GW location IE in the associated DL-UNITDATA PDU(s). The BSS uses the Home PLMN ID contained within the IMSI and the GGSN/P-GW location information to determine whether or not it can handle the SCI information for that GGSN/P-GW. If it cannot, the BSS discards the SCI information and shall treat the rest of the DL-UNITDATA normally.

In abnormal cases, if the SCI IE is received but the GGSN/P-GW location IE is missing, then the BSS shall assume that the GGSN/P-GW is located in the VPLMN; and, if the GGSN/P-GW location IE is received but the SCI IE is missing, then the BSS shall ignore the GGSN/P-GW location IE.

Specific handling related to MOCN and GWCN configurations of network sharing, is described in sub-clauses 6.6 and 6.7.

When the SGSN is using a GWCN configuration, and the BSSGP DL-UNITDATA PDU contains a local TLLI, then the BSC shall use the registered Operator specific NRI value to identify the corresponding serving operator.

Specific handling releted to Dedicated Core Networks as well as MS assisted Dedicated Core Network selection is described in sub-clause 6.6.

An exception case is when the SGSN determines that there is data to be delivered to a MS for which the MPM Timer is running (see sub-clause 8b.2.1). In this case it shall not attempt delivery of that data until the MPM Timer expires or is stopped.

### 6.1.1	Abnormal conditions

The following actions are defined in periods of congestion.

To satisfy the maximum number of service requests, the BSS may redistribute MSs among cells (i.e. network controlled cell reselection is initiated). If this occurs, the BSS may inform the SGSN through the RADIO-STATUS PDU (Radio Cause value: cell reselection ordered). The BSS shall update any internal references that indicate the location of the MS. The BSS may attempt to internally re-route queued LLC frames to an MS that has been moved to a new cell. If this functionality is not supported, or if it is not possible to internally re-route LLC frames, the LLC frame shall be discarded.

It is the responsibility of the higher layer protocols in the SGSN to cope with discarded LLC frames.

2nd Modification

## 7.1	Paging procedure

When an SGSN initiates the paging procedure for GPRS services as defined in 3GPP TS 24.008, it shall send one or more PAGING-PS PDUs to the BSS.

When instructed by an MSC/VLR to initiate a paging procedure for non-GPRS services as defined in 3GPP TS 24.008, an SGSN shall send one or more PAGING-CS PDUs to the BSS.

These paging PDUs shall contain the information elements necessary for the BSS to initiate paging for an MS within a group of cells.

The SGSN provides an indication of the cells within which the BSS shall page the MS. The levels of resolution within one BSS are: all cells within the BSS, all cells on the BSS within one location area, all cells on the BSS within one routing area, and one BVCI (i.e. cell). A routing area, a location area, or a BSS area is associated with one or more NSEIs. If the cells in which to page the MS are served by several NSEIs then one paging PDU must be sent to each of these NSEIs.

A paging PDU shall be used to generate the corresponding radio interface paging request message(s) to be transmitted at the appropriate time.

It should be noted that each paging PDU relates to only one MS and therefore a BSS may pack pages for different MSs into the relevant 3GPP TS 24.008 or 3GPP TS 44.060 radio interface paging request messages.

In the case of paging for non-GPRS services, the SGSN shall provide the MS's IMSI and DRX Parameters. The SGSN shall also include the Global CN-Id information element in the paging PDU when this information element is received from the MSC/VLR. The Global CN-Id information element is received from the MSC/VLR if paging using only the IMSI parameter as identifier of the MS is performed via the SGSN when the MSC/VLR applies intra domain connection of RAN nodes to multiple CN nodes as described in 3GPP TS 23.236. The BSS shall then buffer this information element until receiving the paging response from the MS in order to route the paging response to the correct MSC/VLR.

In the case of paging for GPRS services, the SGSN shall provide the MS's IMSI. If DRX Parameters are available, the SGSN shall also provide the DRX Parameters. If eDRX Parameters are available, the SGSN shall also provide the eDRX Parameters (see sub-clause 7.1a).

NOTE:	The IMSI and the DRX Parameters enable the BSS to derive the paging population number. Paging without DRX parameters may require a considerable extension of the paging duration.

An SGSN may provide the BSSGP with MS specific information, enabling a BSS to execute the paging procedure in an MS specific manner. This includes:

-	QoS Profile. The Precedence parameter is set by the upper layers (in the SGSN). The SGSN shall set the bit rate parameter to "best effort". The SGSN shall set the transmission mode to unacknowledged. The BSS shall ignore the received bit rate, the BSSGP SDU type, LLC type, and transmission mode parameters;

-	PFI or an aggregate BSS QoS profile information which indicates if the page is for signalling, for SMS, for TOM8, for best-effort, or for a specific packet flow. The aggregate BSS QoS profile in this case is used for paging only and is not stored by the BSS. If both of the optional PFI and ABQP IEs are present, the ABQP takes precedence.

-	the most recently received downlink Coverage Class and the cell identity for the cell where the Coverage Class was reported by the MS, if previously received from the BSS in the UL-UNITDATA PDU.

-	MS Radio Access Capability defining the radio capabilities of the MS. If there is valid MS Radio Access Capability information known by the SGSN for the associated MS, the SGSN shall include it in the PAGING-PS PDU. Otherwise, the MS Radio Access Capability shall not be present.

-	Paging Attempt Information consisting of Paging Attempt Count and Intended Number of Paging Attempts information. If Paging Attempt Information is provided then the Paging Attempt Count field shall be increased by one at each new paging attempt performed for delivering a packet data payload available for delivery. If the SGSN cannot determine the number of intended paging attempts the SGSN shall use the code point 0000 corresponding to ´Information not available´.

If an SGSN provides a P-TMSI in a PAGING-PS PDU, then the BSS shall use the P-TMSI to address the MS. If the SGSN does not provide the P-TMSI in the PAGING-PS PDU, then the BSS shall use the IMSI to address the MS.

If the BSS determines that the nominal paging group of the MS occurs too far into the future (e.g. the BSS is unable to buffer the paging request until the next occurrence of the nominal paging group for the indicated MS) it responds to the PAGING-PS PDU by sending a PAGING-PS-REJECT PDU to the SGSN and includes information indicating the time until the next paging occasion therein.

If an SGSN provides a TLLI in a PAGING-CS PDU and a radio context identified by the TLLI exists within the BSS, then the paging request message shall be directly sent to the MS. If the SGSN does not provide the TLLI in the PAGING-CS PDU or if no radio context identified by the TLLI exists within the BSS, then the BSS shall use the TMSI, if provided in the PAGING-CS PDU, else the IMSI, to address the MS.

The PAGING-CS PDU consists of the parameters described above for a PAGING-PS PDU (except the P-TMSI, PFI, ABQP and QoS profile parameters) and, optionally, some or all of the following parameters; TMSI, TLLI, Global CN-Id, Channel Needed and eMLPP-Priority. The Channel Needed and eMLPP-Priority information shall be handled transparently by the BSS.

The SGSN may send the BSS a DUMMY-PAGING-PS PDU at any time to determine the time until the next paging occasion for the MS indicated therein. The BSS uses the information provided within the DUMMY-PAGING-PS PDU to calculate the time until the next paging occasion for the indicated MS and includes it within a DUMMY-PAGING-PS-RESPONSE PDU it sends back to the SGSN.

A SGSN that supports Multilateration Timing Advance (MTA) procedure (see 3GPP TS 44.031 and 3GPP TS 43.059 [23]) shall not send a PAGING-PS PDU to a BSS if the MPM timer is running for the corresponding MS. Otherwise, if neither the MPM timer nor Ready timer is running it may send a PAGING-PS PDU indicating ‘positioning event triggered’ in response to receiving a location request (see sub-clause 8b.1). If the SGSN receives a corresponding page response it sends the BSS managing the serving cell a PERFORM-LOCATION-REQUEST PDU and starts the Ready timer. If the UL-UNITDATA PDU containing the page response includes the “MultilaterationTiming Advance”, “MS Sync Accuracy” and “BTS Reception Accuracy Level” information elements the SGSN includes them in the PERFORM-LOCATION-REQUEST PDU it sends to the serving BSS.

In case the network supports restricted use of enhanced coverage (see 3GPP TS 24.008 [46]), and the MS camps on a cell not supporting EC-GSM-IoT, the SGSN checks the status of Enhanced Coverage Restriction (RestrictEC indication in 3GPP TS 24.008 [46]) obtained from HSS/HLR or retrieves the status from the stored MS context and includes it in the PAGING-PS PDU (see sub-clauses 10.3.1 and 11.3.141) in order to inform the BSS whether to sustain or remove the restriction on use of enhanced coverage.

## 7.1a	Paging procedure for Extended Coverage and eDRX

An SGSN may have any combination of uplink and downlink Coverage Class information (previously received from the BSS in an UL-UNITDATA PDU) and eDRX information (negotiated during NAS signalling) available for a given MS when it determines that paging is necessary for that MS. When sending a PAGING-PS PDU to a BSS to trigger paging for a given MS the SGSN shall include the most recently received Coverage Class information and the most recent eDRX value negotiated during NAS signalling for that MS. If the BVC-RESET procedure (see sub-clauses 8.4 and 11.3.84) is performed and indicates a change to the eDRX capability for a BSS, it shall not impact the most recent eDRX value negotiated during NAS signalling for that MS.

In case the network supports restricted use of enhanced coverage (see 3GPP TS 24.008 [46]), and the MS camps on a cell supporting EC-GSM-IoT, the SGSN checks the status of Enhanced Coverage Restriction (RestrictEC indication in 3GPP TS 24.008 [46]) obtained from HSS/HLR or retrieves the status from the stored MS context and includes it in the PAGING-PS PDU (see sub-clauses 10.3.1 and 11.3.141) in order to inform the BSS whether to sustain or remove the restriction on use of enhanced coverage.

3rd Modification

### 10.2.1	DL-UNITDATA

This PDU is sent to the BSS to transfer an LLC-PDU across the radio interface to an MS.

PDU type:	DL-UNITDATA

Direction:	SGSN to BSS

Table 10.2.1: DL-UNITDATA PDU contents

| Information element | Type / Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| PDU type | PDU type/11.3.26 | M | V | 1 |
| TLLI (current) | TLLI/11.3.35 | M | V | 4 |
| QoS Profile (note 1) | QoS Profile/11.3.28 | M | V | 3 |
| PDU Lifetime | PDU Lifetime/11.3.25 | M | TLV | 4 |
| MS Radio Access Capability (note 2) | MS Radio Access Capability/11.3.22 | O | TLV | 7-? |
| Priority (note 3) | Priority/11.3.27 | O | TLV | 3 |
| DRX Parameters (note 11) | DRX Parameters/11.3.11 | O | TLV | 4 |
| IMSI (note 14) | IMSI/11.3.14 | O | TLV | 5-10 |
| TLLI (old) | TLLI/11.3.35 | O | TLV | 6 |
| PFI | PFI/11.3.42 | O | TLV | 3 |
| LSA Information | LSA Information/11.3.19 | O | TLV | 7-? |
| Service UTRAN CCO | Service UTRAN CCO/11.3.47 | O | TLV | 3 |
| Subscriber Profile ID for RAT/Frequency priority (note 5) | Subscriber Profile ID for RAT/Frequency priority/11.3.105 | O | TLV | 3 |
| Redirection Indication (note 6) | Redirection Indication/11.3.112 | O | TLV | 3 |
| Redirection Completed (note 7) | Redirection Completed/11.3.113 | O | TLV | 3 |
| Unconfirmed send state variable (note 9) | Unconfirmed send state variable/11.3.114 | C | TLV | 4 |
| SCI (note 10) | SCI/ 11.3.116 | O | TLV | 3 |
| GGSN/P-GW location (note 10) | GGSN/P-GW location/11.3.117 | O | TLV | 3 |
| eDRX Parameters (note 11) | eDRX Parameters/11.3.122 | O | TLV | 3 |
| Coverage Class | Coverage Class/11.3.124 | O | TLV | 3 |
| Old Routing Area Identification (note 12) | Old Routing Area Identification/11.3.127 | O | TLV | 8 |
| Attach Indicator (note 13) | Attach Indicator/11.3.128 | O | TLV | 3 |
| SGSN Group Identity (note 15) | SGSN Group Identity/11.3.131 | C | TLV | 5 |
| Additional P-TMSI (note 15) | Additional P-TMSI/11.3.132 | C | TLV | 6 |
| UE Usage Type (note 15) | UE Usage Type/11.3.133 | C | TLV | 3 |
| Alignment octets | Alignment octets/11.3.1 | O | TLV | 2-5 |
| LLC-PDU (note 4) | LLC-PDU/11.3.15 | M | TLV | 2-? |
| Initial LLC-PDU (note 8) | LLC-PDU/11.3.15 | O | TLV | 2-? |
| Timing Advance Request | Timing Advance Request/11.3.140 | O | TLV | 3 |
| Enhanced Coverage Additional Information | Enhanced Coverage Additional Information/11.3.141 | O | TLV | 3 |
| NOTE 1: Some attributes of the QoS Profile shall be discarded if the PFI field is present and corresponds to a known PFC in the BSS. NOTE 2: The field shall be present if there is valid MS Radio Access Capability information known by the SGSN; the field shall not be present otherwise. NOTE 3: The priority field shall be discarded if the PFI field is present and corresponds to a known PFC in the BSS for which the ARP field was received. NOTE 4: The LLC-PDU Length Indicator may be zero. NOTE 5: This IE may be included if available in the SGSN. If the Service UTRAN CCO IE is present with the value of "shall not" the Service UTRAN CCO IE takes precedence over this IE. NOTE 6: This IE shall be included if Redirect Attempt flag was present in UL-UNITDATA and the CN requests rerouting by the BSC to another CN operator. NOTE 7: This IE shall be included if Redirect Attempt flag was present in UL-UNITDATA and the redirection is completed. NOTE 8: The initial Layer 3 Information received from MS. Only present when Redirection Indication is present. NOTE 9: Contains the value of the V(U) as defined in 3GPP TS 44.064 [12] if Redirection Indication IE is present. NOTE 10: These IEs are included when the SGSN supports the SIRUG feature and the received GTP-U packet contained the SCI IE. NOTE 11: If the SGSN has valid eDRX Parameters for a TLLI it shall include the eDRX Parameters IE (see sub-clause 6.1) in which case the DRX Parameters IE shall not be included. NOTE 12: This IE is only included when Redirection Indication is present and the SGSN supports CS/PS coordination enhancements. NOTE 13: This IE indicates a GPRS attach request from the MS. It may only be included if Redirection Indication is present and the SGSN supports CS/PS coordination enhancements. NOTE 14: This IE is included if the IMSI was retrieved unencrypted from the MS and the SGSN supports Dedicated Core Networks. NOTE 15: This IE is included if the SGSN supports Dedicated Core Networks and is included when the “Redirection Indication” IE is present. |  |  |  |  |

4th Modification

### 10.3.1	PAGING PS

This PDU indicates that a BSS shall initiate the packet paging procedure for an MS within a group of cells.

PDU type:	PAGING-PS

Direction:	SGSN to BSS

Table 10.3.1: PAGING PS PDU content

| Information elements | Type / Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| PDU type | PDU type/11.3.26 | M | V | 1 |
| IMSI | IMSI/11.3.14 | M | TLV | 5 -10 |
| DRX Parameters (note 3) | DRX Parameters/11.3.11 | O | TLV | 4 |
| BVCI (note 1) | BVCI/11.3.6 | C | TLV | 4 |
| Location Area (note 1) | Location Area/11.3.17 | C (note 2) | TLV | 7 |
| Routeing Area (note 1) | Routeing Area/11.3.31 | C (note 2) | TLV | 8 |
| BSS Area Indication (note 1) | BSS Area Indication/11.3.3 | C | TLV | 3 |
| PFI | PFI/11.3.42 | O | TLV | 3 |
| ABQP | ABQP/11.3.43 | O | TLV | 13-? |
| QoS Profile | QoS Profile/11.3.28 | M | TLV | 5 |
| P-TMSI | TMSI/11.3.36 | O | TLV | 6 |
| eDRX Parameters (note 3) | eDRX Parameters/11.3.122 | O | TLV | 3 |
| Coverage Class | Coverage Class/11.3.124 | O | TLV | 3 |
| Enhanced Coverage Additional Information | Enhanced Coverage Additional Information /11.3.141 | O | TLV | 3 |
| Cell Identifier (note 4) | Cell Identifier/11.3.9 | O | TLV | 10 |
| MS Radio Access Capability (note 5) | MS Radio Access Capability/11.3.22 | O | TLV | 7-? |
| Paging Attempt Information (note 6) | Paging Attempt Information/11.3.125 | O | TLV | 3 |
| NOTE 1: One and only one of the conditional IEs shall be present. No repeated instances of the conditional IEs are permissible (e.g. one and only one Location Area shall be present). NOTE 2: When network sharing is supported, the PLMN included in the Location Area/ Routeing Area elements can be either the Common PLMN or an Additional PLMN (see 3GPP TS 44.018 [25]). NOTE 3: If the SGSN has valid eDRX Parameters for a TLLI it shall include the eDRX Parameters IE in which case the DRX Parameters IE shall not be included. For the case where PSM is enabled with eDRX and the Active timer is running the SGSN shall always include the negotiated eDRX value in the eDRX Parameters IE. NOTE 4: The cell identity for the cell where the Coverage Class was reported by the MS shall be included if available at the SGSN. NOTE 5: The field shall be present if there is valid MS Radio Access Capability information for the MS known by the SGSN; the field shall not be present otherwise. NOTE 6: The field shall be present if the SGSN is paging a MS due to a positioning event. |  |  |  |  |

5th Modification

### 11.3.141	Enhanced Coverage Additional Information

This information element provides the BSS with the Enhanced Coverage Additional Information.

The element coding is:

Table 11.3.141.a: Enhanced Coverage Additional Information

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet 1 | IEI |  |  |  |  |  |  |  |
| Octet 2 | Length Indicator |  |  |  |  |  |  |  |
| Octet 3 | Spare |  |  |  |  |  |  | ECRR |

The Enhanced Coverage Restriction Removed (ECRR) field is coded as shown below:

Table 11.3.141.b: ECRR field

| Coding | Semantic |
| --- | --- |
| 0 | Restriction on use of enhanced coverage is not removed |
| 1 | No restriction on use of enhanced coverage or restriction on use of enhanced coverage is removed |

End of Modifications