Performance Evaluation for EC-PICH Channel

# introduction

At RAN#79 the WID in [1] was approved which specifies the introduction of a paging indication channel to reduce energy consumption of the EC-GSM-IoT device in idle mode. This document describes the link level performance of EC-PICH channel as described in the companion paper [2]. The changes with respect to earlier version of the document are highlighted in yellow.

# Design of WAKE-UP SIGNAL (WUS) / GO TO SLEEP (GTS) Signals

In the following, the burst format used for EC-PICH is shown.

Wake-up signal (WUS):

Indicates the presence of paging in a set of EC-PCH blocks. Table 1 depicts the burst format.

| Tail Bits (3) | As in existing Normal Burst |
| --- | --- |
| WUS Sequence (142)* | [0,1,0,0,0,1,0,0,0,1,1,1,0,1,0,1,1,1,0,1,1,0,1,0,0,0,0,0,1,0,0,0,0,1,0,1,1, 0,0,0,1,0,0,0,1,1,1,0,1,0,1,1,1,0,1,1,0,1,1,1,1,0,1,1,1,1,0,0,0,1,0,0,1,0,1 ,1,1,0,1,1,1,1,0,0,0,1,1,1,0,1,1,0,1,0,0,0,0,0,1,0,0,0,0,1,0,1,1,0,0,0,1,0, 0,0,1,1,1,0,1,0,1,1,1,0,1,1,0,1,0,0,0,0,0,1,0,0,0,0,1,0,1,0] |
| Tail Bits (3) | As in existing Normal Burst |
| Guard Bits (8) | As in existing Normal Burst |

Table 1: Burst format design for WUS.

Go-to-sleep signal (GTS):

Indicates that no page is present in a set of EC-PCH blocks. Table 2 depicts the burst format.

| Tail Bits (3) | As in existing Normal Burst |
| --- | --- |
| GTS Sequence (142)* | [0,0,1,0,0,0,0,1,0,1,1,0,0,0,0,0,1,1,1,0,1,0,0,1,0,1,0,1,1,1,0,1,1,1,0,0 ,0,1,0,0,0,0,1,0,1,1,0,0,0,0,0,1,1,1,0,1,0,0,0,1,0,0,0,1,0,1,1,1,0,0,1,1, 1,1,1,1,0,0,1,0,1,0,0,1,0,0,1,1,1,0,1,0,0,1,0,1,0,1,1,1,0,1,1,1,0,0,0,1, 0,0,0,0,1,0,1,1,0,0,0,0,0,1,1,1,0,1,0,0,1,0,1,0,1,1,1,0,1,1,1,0,0,0] |
| Tail Bits (3) | As in existing Normal Burst |
| Guard Bits (8) | As in existing Normal Burst |

Table 2: Burst format design for GTS.

*middle 26 bits in both WUS and GTS Sequence are maintained to be equal to Normal Burst TSC.

TS 7 in Table 5.2.3a of TS 45.002 is used for WUS.

TS 7 in Table 5.2.3b of TS 45.002 is used for GTS.

Note: The sequences will be further investigated for performance optimization until RAN6#9 in terms of improved cross correlation properties.

# SIMULATION ASSUMPTIONS

Table 3 depicts the simulation environment.

| Environment | TU 1.2 nFH (900 MHz) |
| --- | --- |
| Downlink higher coverage classes | CC4, CC3 |
| RxLev (dBm) | -120.5 (for CC4) / -117 (for CC3) |
| Number of blocks | 10000 |

Table 3: Simulation environment.

Note: Simulation assumptions remain the same as was in TR 45.820 [3].

Note: SNR is chosen as per Table 1aa in 45.005 (for EC-CCCH).

For CC4, two bursts in two TDMA frames (as mentioned in the companion paper [2]) per 4*51 MF are transmitted with either WUS/GTS. This is considered as one block. This means, if the paging message for CC4 is intended, WUS is transmitted in these two TDMA frames on TN1.

For CC3, each burst of single EC-PICH block carry single bit information. Thus the EC-PICH block carries two 2 bits of information with each bit mapping to one of the CC3 paging block as mentioned in companion paper [2].

For the simulation, at the receiver, the correlation is performed against WUS/GTS sequence and accordingly decision is taken if it is WUS / GTS.

Since, missed detection of WUS should be avoided because missed detection of WUS will lead to missing the paging message, more weightage is given for WUS (to limit the missed detection here to 1%).

Note: This is one method and there could be other methods to detect WUS / GTS.

Thus, the following performance requirements for WUS / GTS reception are proposed in Table 4.

| Tx Signal | Proposed Requirement for False Detection | Remark |
| --- | --- | --- |
| WUS | 1% | acceptable missed paging detection rate |
| GTS | 10% | acceptable missed go-to-sleep detection rate (i.e. page reception without matching page |

Table 4: Proposed Performance Requirements for WUS and GTS.

This yields following working assumption:

WA1: The acceptable missed paging detection rate is 1% and the acceptable rate for page reception without matching page is 10%.

# PERFORMANCE RESULTS

The initial performance results are shown in Tables 5 and 6 as follows.

For CC4:

| Tx Signal | Proposed Requirement for False Detection | Simulation Result for False Detection |
| --- | --- | --- |
| WUS | 1% | 1% |
| GTS | 10% | 8% |

Table 5: Simulated performance for CC4.

For CC3:

| Tx Signal | Proposed Requirement for False Detection | Simulation Result for False Detection |
| --- | --- | --- |
| WUS | 1% | 1% |
| GTS | 10% | 6% |

Table 6: Simulated performance for CC3.

Two observations are made:

Observation 1: 2 bursts are sufficient in 4*51 MF for detecting WUS / GTS intended for CC4 users.

Observation 2: 1 burst is sufficient for detecting WUS / GTS intended for CC3 users. Single EC-PICH block containing 2 bursts can be used to indicate paging for two CC3 paging blocks.

# CONCLUSION

In this paper, initial performance results of EC-PICH are given. Initial results suggest that the performance of the investigated sequences is good which can help in better battery savings for the EC-GSM IoT devices. Further performance optimization of the sequences will be investigated.

The sourcing companies propose to agree working assumption WA1.

# References

[1]	RP-180541, “New WID on Further Enhanced EC-GSM-IoT”, Nokia, Nokia Shanghai Bell, Sierra Wireless S.A., Prima Telecom Testing, RAN#79

[2]	R6-180067, “Paging Indication Channel, Concept Design”, Nokia, Nokia Shanghai Bell, RAN6#8

[3]	3GPP TR 45.820 v13.1.0, “Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)”