| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.002 | CR | 0214 | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Energy efficiency enhancements for EC-GSM-IoT MS in idle mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_fenh-Core |  |  |  |  |  |  | Date: |  | 2018-05-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of new logical channel EC-PICH for energy efficient paging reception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New sub-clause 6.5.8 is added for determining the mapping of the EC-PICH block onto the control multifarme which needs to be monitored by MS using CC3 or CC4 prior to receiving the corresponding paging block within paging multiframes defined for each of these two coverage classes. Table 6a in clause 7 is updated to include EC-PICH block locations. The TDMA frame mapping for EC-PICH is included in clause 7 in Figure 18 for CC3 and in Figure 19 for CC4. Figures 14 and 15 are clarified to be valid without EC-PICH usage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Energy efficient paging reception using EC-PICH will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.8 (new), 7: Table 6a, Figure 14, Figure 15, Figure 18 (new), Figure 19 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | 3GPP TS 43.064 CR 0117 3GPP TS 44.018 CR 1082 3GPP TS 45.005 CR XXXX 3GPP TS 45.008 CR 0660 3GPP TS 48.018 CR 0445 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | 3GPP TS 51.021 CR XXX |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |

For Information only

### 6.5.3a	Determination of specific paging multiframe and paging block index for EC-GSM-IoT

#### 6.5.3a.1	CC1

The 51-multiframe where the paging block is mapped for CC1 occurs when:

PAGING_GROUP div 16 = (RFNQH div 51) mod (BS_ePA_MFRMS)

where

RFNQH is the TDMA frame number known with an accuracy of a quarter hyperframe (see sub-clause 3.3.2.2.3)

The index to the required paging block of the "available" blocks in the 51-multiframe:

Paging block index = PAGING_GROUP mod 16

where the index is then used with the look-up table 6a of clause 7 (also illustrated in figure 13) to determine the actual paging channel interleaved block to be monitored.

#### 6.5.3a.2	CC2

The two 51-multiframes where the paging block is mapped for CC2 occurs when:

PAGING_GROUP div 4 = (RFNQH div 102) mod (BS_ePA_MFRMS div 2)

The index to the required paging block of the "available" blocks in the 51-multiframe:

Paging block index = PAGING_GROUP mod 4

where the index is then used with the look-up table 6a of clause 7 (also illustrated in figure 13) to determine the actual paging channel interleaved block to be monitored.

#### 6.5.3a.3	CC3

The two 51-multiframes where the paging block is mapped for CC3 occurs when:

PAGING_GROUP div 2 = (RFNQH div 102) mod (BS_ePA_MFRMS div 2)

The index to the required paging block of the "available" blocks in the 51-multiframe:

Paging block index = PAGING_GROUP mod 2

where the index is then used with the look-up table 6a of clause 7 (also illustrated in figure 13) to determine the actual paging channel interleaved block to be monitored.

#### 6.5.3a.4	CC4

The four 51-multiframes where the paging block is mapped for CC4 occurs when:

PAGING_GROUP div 2 = (RFNQH div 204) mod (BS_ePA_MFRMS div 4)

The index to the required paging block of the "available" blocks in the 51-multiframe:

Paging block index = PAGING_GROUP mod 2

where the index is then used with the look-up table 6a of clause 7 (also illustrated in figure 13) to determine the actual paging channel interleaved block to be monitored.

1st Modification

### 6.5.8	Determination of EC-PICH block for EC-GSM-IoT

For CC4, two EC-PICH blocks occur in every four 51-multiframes and for CC3 one EC-PICH block occurs in every two 51-multiframes. The location of the EC-PICH blocks is specified in Table 6a.

The MS in CC4 coverage condition, after determining the four 51-multiframes where the paging block occurs and the paging block index within four 51-multiframes, as specified in subclause 6.5.3a4, monitors the EC-PICH block for a paging indication corresponding to its paging block, with the EC-PICH block occurring before the actual paging block. The paging indication for paging block index 0 occurs in the second EC-PICH block sent in the previous four 51-multiframes. The paging indication for paging block index 1 occurs in the first EC PICH in the same four 51-multiframes.

The MS in CC3 coverage condition, after determining the two 51-multiframes where the paging block occurs and the paging block index within two 51-multiframes, as specified in subclause 6.5.3a3, monitors the EC-PICH block for a paging indication corresponding to its paging block, with the EC-PICH block occurring before the actual paging block. The EC-PICH block occurs in previous two 51-multiframes indicating the paging for both paging blocks.

2nd Modification

Table 6a : Mapping of Extended Coverage logical channels onto physical channels (see subclauses 6.3, 6.4, 6.5)

| Channel designation | Dir. | Allowable timeslot assignment | Allowable RF channel assignment | Burst type | Repeat length TDMA frames | Coverage class (CC) | Interleaved block TDMA frame mapping |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EC-SCH | D | 1 | C0 | SB | 204 | - | B([0,1,2,3,4,5,6] + 51 N), N=0,1,2,34 |
| EC-BCCH | D | 1 | C0 | NB | 408 | - | B([7,8,9,10,11,12,13,14]+51N), N=0,…,75 |
| EC-PCH | D | 1,3,5,7 | C0 | NB | 51 | 1 | B0(19,20),B1(21,22),…,B15(49,50) |
|  |  |  |  |  | 102 | 2 | B0([19,…,26] + 51N),B1([27,…,34] + 51N),…,B3([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,34 |
| EC-AGCH | D | 1 | C0 | NB | 51 | 1 | B0(15,16),B1(17,18),…,B17(49,50) |
|  |  |  |  |  | 102 | 2 | B0([19,…,26] + 51N),B1([27,…,34] + 51N),…,B3([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([19,…,34] + 51N), B1([35,…,50] + 51N), N=0,1,2,34 |
|  |  | 3,5,7 |  |  | 51 | 1 | B0(1,2),B1(3,4),…,B24(49,50) |
|  |  |  |  |  | 102 | 2 | B0([3,…,10] + 51N),B1([11,…,18] + 51N),…,B5([43,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 102 | 3 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,13 |
|  |  |  |  |  | 204 | 4 | B0([3,…,18] + 51N), B1([19,…,34] + 51N), B2([35,…,50] + 51N), N=0,1,2,34 |
| EC-RACH, 1 TS | U | 1,3,5,7 | C0 | AB, Exten-ded AB7 | 51 | 1 | B0(0),B1(1),..B50(50) |
|  |  |  |  |  |  | 2 | B0(1,…,4), B1(5,…,8),...B11(45,…,48) |
|  |  |  |  |  |  | 3 | B0(2,…, 17),B1(18,…,33), B2(34,…,49) |
|  |  |  |  |  | 102 | 4 | B0([0,…,23] + 51N),B1([24,…,47] + 51N), N=0,13 |
| EC-RACH, 2 TS | U | 0,1, or, 2,3, or, 4,5, or, 6,7 | C0 | AB | 51 | 2 | B0(1,2,1’,2’), B1(3,4,3’,4’),...,B24(49,50,49’,50’) |
|  |  |  |  |  |  | 3 | B0(2,…,9,2’,…,9’), B1(10,…,17,10’,…,17’),...,B5(42,...,49,42’,...,49’) |
|  |  |  |  |  | 102 | 4 | B0([0,…,11,0’,…,11’] + 51N),…,B3([36,…,47,36’,…,47’] + 51N), N=0,13 |
|  |  |  |  | EDAB, ESAB | 153 | 5 | B0([0,…,21,0’,…,21’] + 51N), B1([22,…,43,22’,…,43’] + 51N), N=0,1,28 |
| EC-PACCH, EC-PDTCH | D&U | 0…7 | C0…Cn | NB2 | 52 | 1 | B0(0...3), B1(4...7), B2(8...11), B3(13...16), B4(17...20), B5(21…24), B6(26...29), B7(30...33), B8(34...37), B9(39...42), B10(43...46), B11(47...50) |
|  |  |  |  |  |  | 2 | B0(0...3,0'...3',0''...3'',0'''...3'''),B1(4...7,4'...7',4''...7'',4'''...7'''), B2(8...11, 8'...11',8''...11'', 8'''...11'''), B3(13...16,13'...16',13''...16'',13'''...16'''), B4(17...20,17'...20',17''...20'',17'''...20'''), B5(21...24,21'...24',21''...24'',21'''...24'''), B6(26...29,26'...29',26''...29'',26'''...29'''), B7(30...33,30'...33',30''...33'',30'''...33'''), B8(34...37,34'...37',34''...37'',34'''...37'''), B9(39...42,39'...42',39''...42'',39'''...42'''), B10(43...46,43'...46',43''...46'',43'''...46'''), B11(47...50,47'...50',47''...50'',47'''...50''')1 |
|  |  |  |  |  |  | 3 | B0(0...7,0'...7',0''...7'',0'''...7'''), B1(8...11,13…16,8'...11',13'…16',8''...11'',13''…16'',8'''...11''',13''',…,16'''), B2(17...24, 17'...24',17''...24'', 17'''...24'''),B3(26...33,26'...33',26''...33'',26'''...33'''), B4(34...37,39…42,34'...37',39'…42',34''...37'',39''…42'',34'''...37''',39'''…42'''), B5(43...50,43'...50',43''...50'',43'''...50''')1 |
|  |  |  |  |  |  | 4 | B0(0...11,13…16,0'...11',13'…16',0''...11'',13''…16'',0'''...11''',13'''…16'''), B1(17...24,26…33,17'...24',26'…33',17''...24'',26''…33'',17'''...24''',26'''…33'''), B2(34...37,39...50,34'...37',39'...50',34’’...37’’,39'’...50'’,34'’'...37'’',39'''...50''')1 |
|  | U |  |  |  | 52 | 5 | B0(0..11,13..24,26..37,39..50,0’..11’,13’..24’,26’..37’,39’..50’, 0’’..11’’,13’’..24’’,26’’..37’’,39’’..50’’,0’’’..11’’’,13’’’..24’’’,26’’’..37’’’,39’’’..50’’’) |
|  | D&U |  |  |  | 52 | 26 | B0(0..7,0'..7'), B1(8..11,13..16,8'..11',13'..16'), B2(17..24,17'..24'), B3(26..33,26'..33'), B4(34..37,39..42,34'..37',39'..42'), B5(43..50,43'..50') |
|  | D&U |  |  |  | 52 | 36 | B0(0..11,13..16,0’..11,13..16), B1(17..24,26..33, 17'..24',26'..33'), B2(34..37,39..50,34'..37',39'..50') |
|  | D&U |  |  |  | 104 | 46 | B0(0..11,13..24,26..33,0’..11’,13’..24’,26’..33’), B1(34..37,39..50,52..63,34’..37’,39’..50’,52’..63’), B2(65..76,78..90,92..103,65’..76’,78’..90’,92’..103’) |
|  | U |  |  |  | 104 | 56 | B0([0..11,13..24,26..37,39..50,0’..11’,13’..24’,26’..37’,39’..50’] + 52N), N=0,19 |
| EC-PICH | D | 1,3,5,7 | C0 | NB | 102 | 3 | B0([17,18]+51N) , N=110 |
|  | D | 1,3,5,7 | C0 | NB | 204 | 4 | B0(15,16), B1([15,16]+51N), N=311 |
| NOTE 1: EC-PDTCH and EC-PACCH are mapped onto four PDCHs. Number n indicates mapping on the PDCH with the lowest timeslot number in TDMA frame n, whereas number n', n'', and n''' indicates mapping on the PDCH with the second lowest, second highest, and highest timeslot number in TDMA frame n respectively. The four timeslots shall be consecutive. NOTE 2: Access Bursts (AB) instead of Normal Bursts (NB) may be used on the uplink as polling response on EC-PACCH/U. The same resources are used by the Access Burst. NOTE 3 N = (FN div 51) mod 2 NOTE 4 N = (FN div 51) mod 4 NOTE 5 N = (FN div 51) mod 8 NOTE 6: EC-PDTCH/2TS and EC-PACCH/2TS are mapped onto two PDCHs in case of blind physical layer transmissions. NOTE 7: For Multilateration Timing Advance using the Extended Access Burst method in EC operation, the Extended AB format is used in case of CC1 selected in downlink and uplink. NOTE 8: N = (FN div 51) mod 3 NOTE 9: Transmission for EC-PDTCH and EC-PACCH using 2 PDCHs is distributed over 2 * 52-multiframes. NOTE 10 : One EC-PICH block occurs in every 2*51 multiframes indicating the paging for both paging blocks in next 2*51 multiframes. NOTE 11 : Two EC-PICH blocks occur in every 4*51 multiframes, with first EC-PICH block B0 indicating paging for paging block index 1 in the same 4*51 multiframes and the second EC-PICH block B1 indicating paging for paging block index 0 in the next 4*51 multiframes. |  |  |  |  |  |  |  |

For Information only

Figure 11: TDMA frame mapping for EC-SCH

Figure 12: TDMA frame mapping for EC-BCCH

Figure 13: TDMA frame mapping for EC-PCH

3rd Modification

Figure 14: TDMA frame mapping for EC-AGCH on timeslot number 1 without EC-PICH

Figure 15: TDMA frame mapping for EC-AGCH on timeslot number 3,5 and 7 without EC-PICH

4th Modification

Figure 18: TDMA frame mapping for EC-PICH CC3 in TN1

Figure 19: TDMA frame mapping for EC-PICH CC4 in TN1

End of Modifications