3GPP TSG-RAN WG6 Meeting #9	R6-180144

Gothenburg, Sweden, 20th – 24th Aug 2018

Agenda Item:	6.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Options for NR cell reselection from UTRAN Idle mode

Document for:	Discussion and Agreement

# 1	Background

In [1], SA2 asks RAN2 and RAN6 to provide the list of possible options for Idle mode UE in a UTRAN cell to return to NR.

This paper captures a list of possible options for the UE to reselect NR cell from UTRAN Idle mode after a voice call.

# 2	Discussion

The following options could be considered for NR cell reselection for an Idle mode UE in UTRAN cell after voice call release:

- Network to broadcast the NR frequency list and priority information to be used by the UE in the UTRAN cell for NR cell re-selection in idle mode.

- Provide NR frequency list in the RRC connection release message with no UE measurements from NR cell.

- Provide NR frequency list in the RRC connection release message based on UE measurements from neighbouring NR cells.

- Leave it for UE implementation with no network involvement.

Observation 1: With option 1, operator could have a provision to set cell-based camping priority for NR capable UEs.

Observation 2: For all the above options, the impact is likely limited to UTRAN specs.

Observation 3: Option 3, is an example of measurement-based NR cell reselection.

Proposal: NR cell reselection from UTRAN Idle mode shall be supported with Network involvement.

# 3	Proposals

Proposal: NR cell reselection from UTRAN Idle mode shall be supported with Network involvement.

# 4	Reference

- [1] R6-180060 – SA2 LS in - Return from UTRAN to NG-RAN for SRVCC from 5GS to UTRAN