3GPP TSG RAN WG6 meeting #11 							                                     R6-190002

# Electronic Agreement Meeting (7 January – 4 February, 2019)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.003 | CR | 0150 | rev | - | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EC-SCH cyclic shift operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM-Core |  |  |  |  |  | Date: |  |  | 21/01/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The cyclic shift operation for repeated EC-SCH bursts uses T2'' as input parameter. This parameter is specified in TS 45.003 to be related to the TDMA frame number, in which the burst is sent. However, this description is not consistent with TS 45.002, where T2'' equals the reduced 51-multiframe number (FN div 51) mod 4, in which the repeated EC-SCH burst is sent rather than being related to the TDMA frame number. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 4.7a.4 the interpretation of T2'' as input parameter for the cyclic shift operation for repeated EC-SCH bursts is corrected to apply to the specific 51-multiframe number in the set of 4 contiguous 51-multiframes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains inconsistent with TS 45.002 and TS 45.010. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7a.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |

For information only

## 4.7a	Extended Coverage Synchronization channel (EC-SCH)

### 4.7a.1	Block constitution

The burst carrying the synchronization information on the downlink EC-SCH contains 30 information bits {d(0),d(1),..., d(29)}.

### 4.7a.2	Coding

The 30 information bits {d(0),d(1),..., d(29)} are encoded as follows:

a)	Parity bits:

Ten parity bits {p(0),p(1),,...,p(9)} are defined in such a way that in GF(2) the binary polynomial:

d(0)D39 +...+ d(29)D10 + p(0)D9 +...+ p(9), when divided by:

D10 + D8 + D6 + D5 + D4 + D2 + 1, yields a remainder equal to:

D9 + D8 + D7 + D6 +  D5 +  D4 + D3 + D2 + D+ 1.

b)	Tail bits:

Four tail bits equal to 0 are added to the information and parity bits, the results being a block of 44 bits {u(0),u(1),...,u(43)}:

u(k)	= d(k)			for k = 0,1,...,29

u(k)	= p(k-30)		for k = 30,31,...,39

u(k)	= 0				for k = 40,41,42,43 (tail bits)

c)	Convolutional encoder:

This block of 44 bits {u(0),u(1),...,u(43)} is encoded with the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

This results in a block of 88 coded bits {C(0),C(1),...,C(87)} defined by:

C(2k)		= u(k) + u(k-3) + u(k-4)

C(2k+1)	= u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,....,43 ; u(k) = 0 for k < 0

The block is punctured such that the bits C(k), k=[0,10,19,29,39,48,58,68,77,87] are not transmitted, resulting in a block of 78 encoded bits {e(0),e(1),..., e(77)}.

### 4.7a.3	Blind physical layer transmission

The encoded bits e(j), j=0,…,77 are transmitted 28 times, generating the repeated bursts R(m,B,j)=e(j), m=0,…,27, B=0, j=0,…,77.

1st Modification

### 4.7a.4	Cyclic shift

The repeated bursts are cyclically shifted as:

R’(m,B,j) = R(m,B,(j + T2'') mod 78)

where T2'' = (FN div 51) mod 4, i.e. it refers to the specific 51-multiframe FN in which the burst will be sent in the set of four contiguous 51-multiframes as defined in 3GPP TS 45.002 [8].

### 4.7a.5	Mapping onto a physical channel

Burst R’(m,B,j), is mapped onto burst B’ of the timeslot carrying the EC-SCH, where

B’ = m

NOTE:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

End of Modifications