3GPP TSG RAN WG6 meeting #15 							                                      R6-200016

# Electronic Agreement Meeting (13 January – 3 February, 2020)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.002 | CR | 0222 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of brackets for “tail bits 4” of EDAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Core |  |  |  |  |  | Date: |  |  | 22/01/2020 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Brackets for “tail bits 4” specified for EDAB need to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 5.2.11, in Table 5.2.11-2, brackets for “tail bits 4” used for the second part of the EDAB transmission are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of the EDAB format in this specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modification

### 5.2.11	Extended Dual Slot Access burst (EDAB)

The extended dual slot access burst is specified for uplink coverage class CC5 in EC operation for the 2 TS EC-RACH format. It consists of two elementary bursts, the first part being transmitted in a burst with equal size to the active part of the normal burst, with a timeslot length of 157 normal symbol periods on timeslots with TN = 0 and 4, and 156 normal symbol periods on timeslots with TN = 2 and 6, and the second and last part being transmitted in the consecutive TN in the same TDMA frame in a burst with equal size to the active part of the access burst with a timeslot length of 156 normal symbol periods. Both parts are specified in tables 5.2.11-1a and 5.2.11-1b.

If the Access_Timeslot parameter in EC SI 2 set to ‘1’ (indicating 2 TS EC-RACH mapping), an EC capable mobile in coverage class CC5 using EC-RACH format as EDAB (refer 3GPP TS 44.018 EC SI 2 parameter CC5_EC-RACH_FORMAT_IND), shall access the network using the training sequence TS7.

If the serving cell supports all coverage classes (i.e CC1 to CC5) and if the Access_Timeslot parameter in EC SI 2 set to ‘0’ (indicating 1 TS EC-RACH mapping), an EC capable mobile in coverage class CC5 using EC-RACH format as EDAB (refer 3GPP TS 44.018 EC SI 2 parameter CC5_EC-RACH_FORMAT_IND), shall access the network using the training sequence TS7.

If the serving cell supports four or less coverage classes (i.e. CC2 or CC3 or both may not be supported) and the Access_Timeslot parameter in EC SI 2 set to ‘0’ (indicating 1 TS EC-RACH mapping), an EC capable mobile with coverage class CC5 using EC-RACH format as EDAB (refer 3GPP TS 44.018 EC SI 2 parameter CC5_EC-RACH_FORMAT_IND) shall access the network using a training sequence selected as follows:

-	if only CC3_Range_UL parameter is not present (indicating the absence of CC3 coverage class), TS 6 is selected.

-	if only CC2_Range_UL parameter is not present (indicating the absence of CC2 coverage class), TS 5 is selected.

-	if parameters CC2_Range_UL, CC3_Range_UL are not present (indicating the absence of CC2 and CC3 coverage classes), TS 6 is selected.

Table 5.2.11-1a: Extended Dual slot Access Burst (first part)

| Bit Number (BN) | Length of field | Content of field | Definition |
| --- | --- | --- | --- |
| 0-7 | 8 | Extended tail bits | see table 5.2.11-2 |
| 8-48 | 41 | Synch. sequence bits | see table 5.2.7-4 |
| 49-78 | 30 | Encrypted bits (e0..e29) | see 3GPP TS 45.003 |
| 79-81 | 3 | Tail bits 1 | see table 5.2.11-2 |
| 82-111 | 30 | Encrypted bits (e0..e29) | see 3GPP TS 45.003 |
| 112-114 | 3 | Tail bits 2 | see table 5.2.11-2 |
| 115-144 | 30 | Encrypted bits (e0..e29) | see 3GPP TS 45.003 |
| 145-147 | 3 | Tail bits 3 | see table 5.2.11-2 |
| 148-155 148-156 | 8, or, 91 | Guard period (bits) | see subclause 5.2.8 |
| NOTE1: 8 or 9 always applies in case of EC-GSM-IoT (9 for TN0 and 4; 8 for TN2 and 6), see 3GPP TS 45.010 |  |  |  |

The “extended tail bits”, “tail bits 1”, “tail bits 2” and “tail bits 3” are defined as modulating bits with states according to table 5.2.11-2.

Table 5.2.11-1b: Extended Dual slot Access Burst (second and last part)

| Bit Number (BN) | Length of field | Content of field | Definition |
| --- | --- | --- | --- |
| 0-7 | 8 | Extended tail bits | see table 5.2.11-2 |
| 8-48 | 41 | Synch. sequence bits | see table 5.2.7-4 |
| 49-78 | 30 | Encrypted bits (e0..e29) | see 3GPP TS 45.003 |
| 79-87 | 9 | Tail bits 4 | see table 5.2.11-2 |
| 88-155 | 681 | Guard period (bits) | see subclause 5.2.8 |
| NOTE1: 68 always applies in case of EC-GSM-IoT for TN 1, 3, 5 and 7, see 3GPP TS 45.010 |  |  |  |

The “extended tail bits” and “tail bits 4” are defined as modulating bits with states according to table 5.2.11-2.

Table 5.2.11-2: Tail bits and extended tail bits

| Tail bit type | Bit number (BN) | Modulating bits |
| --- | --- | --- |
| Extended tail bits | (BN0, BN1, BN2, …, BN7) | (0, 0, 1, 1, 1, 0, 1, 0) |
| Tail bits 1 | (BN79, BN80, BN81) | (0, 0, 0)1/(1,1,1)2 /(1,0,0)3 |
| Tail bits 2 | (BN112, BN113, BN114) | (0, 0, 0)1/(1,1,1)2 /(1,0,0)3 |
| Tail bits 3 | (BN145, BN146, BN147) | (0, 0, 0)1/(1,1,1)2 /(1,0,0)3 |
| Tail bits 4 | (BN79, BN80, …, BN87) | (0, 0, 0, 0, 0, 0, 0, 0, 0)1/ (1,1,1,1,1,1,1,1,1)2/ (1,0,0,1,0,0,1,0,0) 3 |
| NOTE 1: These bits are used if TS7 is applied for CC5 EC-RACH operation using EDAB. NOTE 2: These bits are used if TS6 is applied for CC5 EC-RACH operation using EDAB. NOTE 3: These bits are used if TS5 is applied for CC5 EC-RACH operation using EDAB. |  |  |

The “Synch. sequence bits” are defined as modulating bits with states according to table 5.2.7-4 for uplink CC5.

End of Modifications