3GPP TSG RAN WG6 meeting #15 							                                      R6-200030

# Electronic Agreement Meeting (13 January – 3 February, 2020)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.010 | CR | 0080 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of brackets for Multilateration Timing Advance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePOS_GERAN-Perf |  |  |  |  |  | Date: |  |  | 22/01/2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are still square brackets left for MTA in this specification which need to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 5.4.2, brackets are removed for timing advance estimation performance requirement according to the indicated BTS reception Accuracy Capability. In subclause 6.3.2, brackets are removed for the minimum requirement on the error assessment of the BTS timing by the MS and for the maximum number of repeated synchronizations to the same BTS to achieve the Requested MS Sync Accuracy. In subclause 6.4, brackets are removed for the requirement on tolerance on the timings of MS transmissions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation of this feature in this specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2, 6.3.2, 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modification

### 5.4.2	BTS Timing Advance Estimation for Positioning

A higher level of accuracy of the timing advance estimation by the BTS (reported to the SMLC, see 3GPP TS 43.059 [6]) is desired when the following positioning procedures are used:

-	Multilateration Timing Advance procedure for assessing the timing advance in the serving cell and in non-serving cells, and

-	Multilateration Observed Time Difference procedure for assessing the timing advance in the serving cell,

The actual algorithms and methods for estimation of the timing advance value are implementation dependent.

Moreover, a BSC that reports a timing advance value based on transmissions from an MS (e.g. the EC Multilateration Request message or EGPRS Multilateration Request message, see 3GPP TS 44.018 [8] and 3GPP TS 44.060 [9]) using the RLC Data Block or the Extended Access Burst method, shall establish a reported timing advance value = tAReported . It does so by first using the BTS estimated TA value = tAEstimated (based on the AB received on the RACH or on the EC-RACH (including blind physical layer transmissions for CC2, CC3 and CC4) identified when the MS starts the positioning procedure and using it as the assigned TA value = tAAssigned (i.e. the TA value sent to the MS using an assignment message on the AGCH/EC-AGCH is the value of the tAEstimated rounded off to the nearest symbol). It then adjusts the most recent estimated TA value as it receives subsequent updated timing advance estimation values from the BTS (i.e. the BTS provides an updated timing advance estimation for each subsequent burst it receives from the MS for the remainder of the positioning procedure wherein each burst sent by the MS uses the assigned TA).The final reported TA value (tAReported) is calculated by the BSC after it has received the last estimated TA value from the BTS based on the last transmission from the MS for the current positioning procedure and takes into account the MS Transmission Offset sent by the MS during the procedure (see 3GPP TS 44.018 [8] and 3GPP TS 44.060 [9] and 3GPP TS 43.059 [6]). This is shown in the formulas below.

tAReported=1Ni=1NtAi 		(1)

where

tA1= tAEstimated1+tMS Transmisson Offset		(2)

tAi=2…N= tAEstimatedi+tAAssigned+tMS Transmisson Offset	(3)

where tMS Transmisson Offset corresponds to the time as reported by the MS in the “MS Transmission Offset IE” (see 3GPP TS 49.031 [18]).

For the Extended Access burst method the number of timing advance estimations is N= 2 wherein both the RACH burst and the Extended Access burst are used for timing advance estimation. Similarly, for the RLC Data Block method the number of timing advance estimations correspond to N=5 (for the case of no HARQ retransmissions needed on the (EC-)PDTCH) wherein the AB received on RACH or EC-RACH and the 4 normal bursts used to receive the RLC data block are used for timing advance estimation.

Moreover, a BTS estimating the timing advance value for an MS using the RLC Data Block or the Extended Access Burst methods will be subject to an accuracy limitation inherent to its implementation and the radio conditions applicable when receiving transmissions from the MS performing the Multilateration Timing Advance procedure. This accuracy limitation is expressed as an assessment error of the reported TA value tAReported and shall be reported (to the SMLC via the BSS) as the BTS Reception Accuracy Level (see 3GPP TS 43.059 [6] and 3GPP TS  49.031 [18]) wherein the assessment error corresponds to the variance of the reported timing advance value of multiple timing advance estimations according to the equations below where N denotes the number of timing advance estimations and tAi, i=1...N denotes the estimated timing advance in estimation 'i'.

The variance vartAReportedof the estimated timing advance value shall be evaluated using the formula:

var(tA_Reported)=s2N		(4)

s2 is the unbiased sample variance

s2=1N-1i=1NtAi-tAReported2		(5)

where tAReportedand tAi are calculated per the equations (1), (2) and (3) above.

For the Access Burst method the reported timing advance value corresponds to the estimated timing advance value derived from receiving the AB containing the EC Multilateration Request message or EGPRS Multilateration Request message.

When reporting the timing advance value to the SMLC the tAReported is rounded off to the nearest 1/64 of a normal symbol period (see 3GPP TS 49.031 [18]).

Similarly, for the SMLC to be able to accurately estimate the number of required BTSs to be used during the Multilateration Timing Advance procedure (in order to meet a targeted positioning accuracy) each individual BTS shall provide the BSC with BTS Reception Accuracy Capability information (see 3GPP TS 49.031 [18]) as follows:

-	with a guaranteed timing advance assessment error, it shall always be capable of supporting at radio conditions down to the reference sensitivity level for RACH, if the BTS is capable of PEO operation, for all MTA radio access methods supported by the BTS. The BTS Reception Accuracy Capability shall be evaluated at the reference sensitivity level for PRACH/11 bits as specified in TS 45.005 [11].

-	with a guaranteed timing advance assessment error, it shall always be capable of supporting at radio conditions down to the input signal level for reference performance for EC-RACH (CC1), if the BTS is capable of EC operation, for all MTA radio access methods supported by the BTS. The BTS Reception Accuracy Capability shall be evaluated at the input signal level for reference performance for EC-RACH (CC1) as specified in TS 45.005 [11].

The BSC, in turn, reports this guaranteed timing advance assessment error value as the applicable BTS Reception Accuracy Capability in the Assistance Information Response message sent to the SMLC in response to an Assistance Information Request message (see 3GPP TS 43.059 [6]).

The BTS shall comply with the indicated BTS reception Accuracy Capability in 90 % of the timing advance estimations.

2nd Modification

### 6.3.2	MS Assessment of BTS timing for Positioning

A higher level of accuracy of the assessment of the BTS timing by the MS (reported to the SMLC, see 3GPP TS 43.059 [6]) is desired when the following positioning procedures are used:

-	Multilateration Timing Advance procedure for assessing the TA in the serving cell and in non-serving cells, and

-	Multilateration Observed Time Difference procedure for assessing the TA in the serving cell,The Initial LLC-PDU received from the MS;

During the positioning procedure the minimum requirement on the error assessment of the BTS timing by the MS at and above the reference sensitivity level of the SCH or, in the case of EC operation, at and above the input signal level of the EC-SCH at reference performance, shall be ± ¼ normal symbol period unless the MS Sync Accuracy the MS can realize when performing the positioning procedure in a given cell meets a tighter requirement for the assessment of the BTS timing than the default value (indicated in the MSRAC IE - see 3GPP TS 24.008). In this case the value of the MS Sync Accuracy sent by the MS indicates the tighter requirement. Furthermore, to meet the targeted positioning accuracy for a targeted number of BTSs the SMLC determines a target assessment of BTS timing required of the MS and includes this as the Requested MS Sync Accuracy in the triggering RRLP message when indicating to the MS to use the RLC Data block or Extended Access burst method (see 3GPP TS 43.059 [6] and 3GPP TS 49.031[18]).

The MS shall attempt to achieve the Requested MS Sync Accuracy using a minimum of 2 or a maximum of 10 repeated synchronizations to the same BTS and report the achieved error in the assessment of the sync accuracy (i.e. assessment of BTS timing on the downlink) in the uplink RLC data block or the Extended Access Burst as the MS Sync Accuracy. The error of the sync accuracy corresponds to the variance of the estimated timing of multiple repeated synchronizations according to the equations below where N denotes the number of independent synchronization attempts and ti, i=1...N denotes the estimated timing in synchronization 'i'.

The variance var(t) of the estimated timing shall be evaluated using the formula:

var(t)=s2N

s2 is the unbiased sample variance

s2=1N-1i=1N(ti-t)2

and t is the mean of ti, i.e.

t=1Ni=1Nti

Similarly, to support accurate positioning using the Multilateration Observed Time Difference (OTD) procedure, a higher level of accuracy of the assessment of BTS timing is required in order to determine the difference in arrival time between (EC-)SCH synchronization sequences of two base station transmissions which is reported to the network in the RRLP Multilateration OTD Response message. At and above the reference senstivity level of the SCH or, in the case of EC operation, at and above the input signal level of the EC-SCH at reference performance (see 3GPP TS 45.005 [11]), the absolute value of a Multilateration OTD report's error shall be ≤ ¼ normal symbol period. An MS applying the Multilateration OTD procedure shall report in the OTD Measurement Information IE the achieved assessment error of the BTS timing, according to the formulas above in this clause, as the MS Sync Accuracy for the respective neighbour cell.

## 6.4	Timing of transmission

The MS shall time its transmissions to the BTS according to signals received from the BTS. The MS transmissions to the BTS, measured at the MS antenna, shall be 468,75-TA normal symbol periods (i.e. 3 timeslots-TA) behind the transmissions received from the BTS, where TA is the last timing advance received from the current serving BTS. The tolerance on these timings shall be ± 1 normal symbol period. For CTS, the tolerance on these timings shall be ± ½ normal symbol period.

For an MS that transmits to a BTS in response to a RRLP Multilateration Timing Advance Request or RRLP Multilateration Observed Time Difference message (see 3GPP TS 43.059 [6]) the requirement on tolerance on the timings of its transmissions, when operating at and above the reference sensitivity level of the SCH or, in the case of EC operation, at and above the input signal level of the EC-SCH at reference performance (see 3GPP TS 45.005 [11]), shall be ±1/8 normal symbol period or better. In addition, immediately prior to transmitting to a BTS while performing Multilateration Timing Advance procedure using the RLC Data Block method or the Extended Access Burst Method (see 3GPP TS 44.018 [8] and 3GPP TS 43.059 [6]), the MS shall also calculate the offset between the nominal transmission opportunity, as determined in relation to the estimated timing of the BTS, and the selected transmission opportunity, as dictated by the internal timebase. The offset shall be reported as the MS Transmission Offset in the uplink RLC data block or in the Extended Access Burst [6]. If the selected uplink transmission opportunity occurs earlier than the nominal transmission opportunity, the MS transmission Offset shall be reported as a positive value. Similarly, if the selected uplink transmission opportunity occurs later than the nominal transmission opportunity, the MS transmission Offset shall be reported as a negative value. Note that upon receiving an assigned TA value the resulting new transmission opportunity (selected according to the nearest available timebase specific transmission opportunity identified after applying the indicated TA value) needs to be in the same direction as before, i.e., if before receiving an assigned TA value the selected uplink transmission opportunity (selected using the nearest available timebase specific transmission opportunity) occurred earlier than the nominal transmission opportunity then the MS shall establish a new nominal transmission opportunity using the indicated TA value and then select an uplink transmission opportunity (selected using the nearest available timebase specific transmission opportunity) that occurs earlier than the new nominal transmission opportunity (and similarly for the case where the selected uplink transmission opportunity occurred later than the nominal transmission opportunity).

In case of a multislot configuration, the MS shall use a common timebase for transmission of all channels. In this case, if the MS, not in EC operation, does not support transmission of reduced symbol period bursts, it may optionally use a timeslot length of 157 normal symbol periods on timeslots TN = 0 and 4, and 156 normal symbol periods on timeslots with TN = 1, 2, 3, 5, 6 and 7, rather than 156,25 normal symbol periods on all timeslots. An EC-GSM-IoT MS, when in EC operation, shall use the timeslot length with integer symbol periods, see subclause 5.7.2.

If the MS supports reduced symbol period transmissions, it shall use a timeslot length of 187.5 reduced symbol periods or a timeslot of length 156.25 normal symbol periods. When there is a pair of different symbol period bursts on adjacent timeslots, then the guard period between the two bursts shall be 8.5 normal symbol periods which equals 10.2 reduced symbol periods. The active part of a reduced symbol period burst shall start a quarter of a normal symbol period later compared to a normal symbol period burst as shown in Figure 5.7.3.

In case of a circuit switched multislot configuration, the common timebase shall be derived from the main channel and the TA values received on other channels shall be neglected. In case of a packet switched multislot configuration the common timebase shall be derived from all timeslots monitored by the MS. In this case, the MS may assume that the BTS uses a timeslot length of 156,25 normal symbol periods on all timeslots using normal symbol period and a timeslot length of 187,5 reduced symbol periods on all timeslots using reduced symbol period. In the case of a combination of circuit and packet switched channel configuration the MS may derive the common timebase from the circuit switched channel only.

End of Modifications