3GPP TSG RAN conference call on IMT-2030 TPRs

April 29, 2025

Agenda item:	16

Source:	Huawei, HiSilicon

Title:	Proposal on IMT-2030 TPRs

Document for:	Discussion

# Background

According to the endorsed 3GPP 6G timeline [1], RAN#106 approved the new Study Item of “Study on 6G scenarios/use cases and requirements” (ITU focused part) [2]. A reply LS was sent to ITU-R WP5D including initial considerations of possible minimum Technical Performance Requirements (TPRs), evaluation configurations of carrier frequency and bandwidth, as well as the workplan [4]. According to the workplan [5], RAN#107 made progress on the candidate set of TPRs and initially discussed the associated values [9]. Furthermore, RAN#107 decided to organize two conference calls to progress the work between RAN#107 and RAN#108 [10]. To facilitate the meeting discussions, this document provides our suggestions on the planned topics of the conference call.

# Proposal on candidate set of TPRs and definition

As shown in below table 1, there are 23 candidate items under discussion in WP5D. RAN#106 meeting agreed to consider the first 14 items (item #1-#14) as IMT-2030 TPR. A LS was sent to WP5D accordingly [4]. RAN#107 continued the discussion and made further progress on item #15-#18 as follows [9].

Proposal 1: Define positioning accuracy as a quantitative TPR

Proposal 2: Define Sensing-related capabilities as a TPR.

Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection.

Proposal 4: Define energy efficiency as a TPR.

Proposal 5: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL

Proposal 6: For bandwidth TPR, it is proposed to set a target value of 400MHz (same definition as IMT-2020)

-	TBD on other TPR values

Proposal 7: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR.

-	Applicable usage scenario: immersive communication

Table 1: Proposal on candidate set of TPRs for IMT-2030

|  | Candidate IMT-2030 TPRs under discussion in WP5D [6] |  | TPRs in ITU IMT-2020 [7] | Proposed to be defined as ITU IMT-2030 TPR | Progress in 3GPP RAN |
| --- | --- | --- | --- | --- | --- |
| 1 | Peak data rate |  | Legacy TPRs of IMT-2020 |  | Agreed in RAN#106 [4] |
| 2 | Peak spectral efficiency |  |  |  |  |
| 3 | User experienced data rate |  |  |  |  |
| 4 | 5th percentile user spectral efficiency |  |  |  |  |
| 5 | Average spectral efficiency |  |  |  |  |
| 6 | Area traffic capacity |  |  |  |  |
| 7 | User plane latency |  |  |  |  |
| 8 | Control plane latency |  |  |  |  |
| 9 | Connection density |  |  |  |  |
| 10 | Energy efficiency |  |  |  |  |
| 11 | Reliability |  |  |  |  |
| 12 | Mobility |  |  |  |  |
| 13 | Mobility interruption time |  |  |  |  |
| 14 | Bandwidth |  |  |  |  |
| 15 | Positioning |  | New candidate TPRs | (1) | Agreed in RAN#107 [9] |
| 16 | Sensing-related capabilities | [Detection/False alarm probability] (2) |  |  |  |
|  |  | [Localization accuracy] |  |  |  |
|  |  | [Velocity accuracy] |  |  |  |
|  |  | [Sensing resolution] |  |  |  |
| 17 | AI-related capabilities |  |  |  |  |
| 18 | Composite requirement |  |  |  |  |
| 19 | [Coverage] |  |  |  |  |
| 20 | [Security] (3) |  |  |  |  |
| 21 | [Resilience] |  |  |  |  |
| 22 | [Interoperability] (3) |  |  |  |  |
| 23 | [Sustainability] |  |  |  |  |

(1) Note: It is not intended to restrict the implementation details of IMT-2030 technology on how to meet these requirements. For example, setting “positioning accuracy” as performance requirements for IMT-2030 radio interface technology doesn’t imply that LMF (Location management function) is required in IMT-2030 RAN. LMF is located in core network of NR technology. This is up to implementation details of IMT-2030 technology.

(2) Note: Items in square brackets haven’t reached consensus.

(3) Note: Security and Interoperability are important design principle for communication system. However, they are not appropriate to be the technical performance requirement of radio interface technology.

Proposal 1: To adopt the candidate items in above table 1 for IMT-2030 TPRs.

In addition, definition of some TPRs needs further discussion. For example, energy efficiency is expected for a more comprehensive evaluation than IMT-2020, including the possibility of quantitative requirement for both unloaded and loaded scenarios. Sensing, AI and composite requirement are new concepts for IMT-2030, the definition for these items needs discussion. In Annex 1, we propose the definition texts for these items base on the latest progress in WP5D [6].

Proposal 2: To adopt the definitions in Annex 1 for TPR item Energy efficiency, Sensing, AI and Composite requirement.

# Proposal on target values

In previous IMT-2020 discussion the values adopted are several times higher than IMT-Advanced, e.g. ITU IMT-2020 requirements on cell average/cell edge spectral efficiency are 3 x IMT-A, which is to show the advanced capabilities for a new generation compared with the old ones.

Observation 1: ITU-R requirements of IMT-2020 are several times higher than IMT-Advanced, e.g. 3 x IMT-A in case of cell average/cell edge spectral efficiency.

In 2019, 3GPP submitted IMT-2020 self-evaluation report to ITU-R [8]. The evaluation results of NR Release 16 significantly exceeded ITU-R requirements, for example 2~3 times of ITU IMT-2020 requirements in case of cell average and cell edge spectrum efficiency in Dense Urban environment as shown in table below from 3GPP TR 37.910.

Table 5.4.1.2.1-1 from 3GPP TR 37.910, DL spectral efficiency for NR in Dense Urban – eMBB 
(Evaluation configuration A, CF=4 GHz)

(b) NR TDD

| Scheme and antenna configuration | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | Average [bit/s/Hz/TRxP] | 7.8 | 4 | 13.03 | 15.26 | 16.77 | 4 | 13.07 | 15.20 | 16.60 |
|  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.47 | 0.55 | 0.61 |  | 0.45 | 0.53 | 0.57 |
| … |  |  |  |  |  |  |  |  |  |  |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 16.73 | 20.08 | 22.33 | 1 | 16.59 | 19.91 | 22.14 |
|  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.61 | 0.73 | 0.81 |  | 0.57 | 0.68 | 0.76 |

Observation 2: Evaluation result of 3GPP NR R16 in 2019 significantly exceeded ITU-R IMT-2020 requirements, e.g. 2~3 times of ITU IMT-2020 requirements in case of cell average/cell edge spectrum efficiency in Dense Urban environment.

The relation between these target values and the real network performance was mentioned in previous discussion, e.g. due to the limitation of operating conditions (availability of cell sites, spectrum etc.), it may be difficult for real network to achieve downlink user experience data rate > 100Mbps as required by ITU IMT-2020 TPRs. This is true and has been clarified in ITU document with following text in section 4 of IMT-2020 TPR [7].

“These requirements are not intended to restrict the full range of capabilities or performance that candidate RITs/SRITs for IMT-2020 might achieve, nor are they intended to describe how the RITs/SRITs might perform in actual deployments under operating conditions that could be different from those presented in other ITU-R Recommendations and Reports on IMT-2020.”

According to the working method in ITU-R, TPRs are intended for the evaluation of candidate technology proposals (RITs/SRITs: Radio interface technologies/Set of radio interface technologies) on the fulfilment of ITU IMT-2030 requirements. It will be conducted with the associated evaluation methodology (simulation, analysis or inspection), which may not be reflecting the real network conditions.

Observation 3: TPRs are intended for the evaluation of candidate technology proposals on the fulfilment of ITU requirements. They are not intended to describe how the RITs/SRITs might perform in actual deployments.

Furthermore, in RAN#107 it was pointed out that, for the TPR item “user experienced data rate”, the evaluation method may need revision. It was captured as below [9],

“Proposal 5: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL”

The problem is so called “Bandwidth scaling”. In IMT-2020, User experienced data rate is derived analytically by multiplying the 5th percentile user spectral efficiency with bandwidth. The former (5% SE) is the simulation result with configured bandwidth of 20 MHz for TDD or 10 MHz+10 MHz for FDD. The latter (bandwidth used in data rate calculation) could be different from the configured bandwidth used in spectrum efficiency simulation, e.g. 100MHz vs 20MHz, so called “bandwidth scaling”. This may be reasonable for downlink, because base station Tx power may scale linearly as bandwidth increases. But it is not the case for uplink as UE has limited total power irrespective of the bandwidth. “Bandwidth scaling” is not appropriate for the evaluation of uplink data rate.

Therefore, it is proposed to use system level simulation with actual bandwidth for the evaluation of “user experienced data rate”, i.e. no bandwidth scaling. Because of the revised evaluation method, it is proposed to use a new title for IMT-2030 e.g. “cell edge user data rate” to avoid the confusion with previous IMT-2020 values.

Observation 4: “Bandwidth scaling” is not appropriate for the evaluation of uplink data rate

Proposal 3: Use system level simulation with actual bandwidth for the evaluation of “user experienced data rate”, i.e. no bandwidth scaling. Consider to use a new title e.g. “cell edge user data rate” to avoid the confusion with previous IMT-2020 values.

Base on these observations and preliminary technical analysis (see simulations in annex), we propose target values as below table 2.

Table 2: proposal on target value for IMT-2030 TPRs

| ITU IMT-2030 TPR |  |  | Proposal on target value | Comments |
| --- | --- | --- | --- | --- |
| Peak data rate |  |  | DL: 100~200, UL: 50~100Gbps | 5~10 x IMT-2020 |
| Peak spectral efficiency |  |  | DL: 60~90, UL: 30~45bps/Hz | 2~3 x IMT-2020 |
| User experienced data rate/cell edge user data rate |  |  | DL: 500~1000, UL: 30 Mbps | DL: 5~10 x IMT-2020 UL: simulation with actual bandwidth, i.e. no “bandwidth scaling” |
| 5th percentile user spectral efficiency |  |  | Indoor Hotspot 0.9-1.5 (DL), 0.63-1.05 (UL) Dense Urban 0.675-1.125, 0.45-0.75 Rural 0.36-0.6, 0.135-0.225 | 3~5 x IMT-2020, see annex for simulations |
| Average spectral efficiency |  |  | Indoor Hotspot 27-45 (DL), 20.25-33.75 (UL) Dense Urban 23.4-39, 16.2-27 Rural 9.9-16.5, 4.8-8 bps/Hz |  |
| Area traffic capacity |  |  | 50~100 Mbit/s/m2 @ Indoor Hotspot | 5~10 x IMT-2020 |
| latency |  | User plane latency | 0.5-1ms for HRLLC 1ms for Immersive communication | 1/2-1 x IMT-2020 |
|  |  | Control plane latency | 10ms | 1/2 x IMT-2020 |
| Connection density |  |  | 107 - 108 devices/km2 for IoT traffic | 10-100 x IMT-2020 |
| Energy efficiency |  |  | Target value TBD |  |
| Reliability |  |  | 1-10-6 - 1-10-7 for HRLLC | 10-100 x IMT-2020 |
| Mobility |  |  | Target value TBD |  |
| Mobility interruption time |  |  | 0ms |  |
| Bandwidth |  |  | At least 400MHz | 4 x IMT-2020 |
| Positioning |  |  | Horizontal accuracy: [20-50cm] @90% for indoor, [3-5m] @90% for outdoor [Vertical accuracy]: [1-3m] @90% for indoor | Assume 200MHz bandwidth, the necessity of vertical accuracy is FFS |
| Sensing-related capabilities | Detection/False alarm probability |  | Target value TBD | Further updates are expected as the channel model study progresses |
|  | Localization accuracy |  | [meter level] |  |
|  | Velocity accuracy |  | [m/s level] |  |
|  | Sensing resolution |  | Target value TBD |  |
| AI-related capabilities |  |  | N. A | Evaluated by inspection |
| Composite requirement |  |  | 500-1000 users/km2 | Corresponding to 6-12 users per cell of ISD=200m, see annex for simulations |

Proposal 4: To adopt the target values in above table 2 for IMT-2030.

# Conclusion

Proposal 1: To adopt the candidate items in above table 1 for IMT-2030 TPRs.

Proposal 2: To adopt the definitions in Annex 1 for TPR item Energy efficiency, Sensing, AI and Composite requirement.

Proposal 3: Use system level simulation with actual bandwidth for the evaluation of “user experienced data rate”, i.e. no bandwidth scaling. Consider to use a new title e.g. “cell edge user data rate” to avoid the confusion with previous IMT-2020 values.

Proposal 4: To adopt the target values in above table 2 for IMT-2030.

- Observation 1: ITU-R requirements of IMT-2020 are several times higher than IMT-Advanced, e.g. 3 x IMT-A in case of cell average/cell edge spectral efficiency.

- Observation 2: Evaluation result of 3GPP NR R16 in 2019 significantly exceeded ITU-R IMT-2020 requirements, e.g. 2~3 times of ITU IMT-2020 requirements in case of cell average/cell edge spectrum efficiency in Dense Urban environment.

- Observation 3: TPRs are intended for the evaluation of candidate technology proposals on the fulfilment of ITU requirements. They are not intended to describe how the RITs/SRITs might perform in actual deployments.

- Observation 4: “Bandwidth scaling” is not appropriate for the evaluation of uplink data rate

# References

- RP-240823, Additional Considerations for 6G Timeline, TSG Chairs, 3GPP TSG#103 Joint Session, March 2024

- RP-243327 New SID: Study on 6G scenarios/use cases and requirements, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, 3GPP RAN#106, December 2024

- RP-243202 Liaison Statement to External Organizations Minimum requirements related to technical performance for IMT-2030 radio interface(s), source ITU-R WP5D, 3GPP RAN#106, December 2024

- RP-243325	Draft Reply to ITUR_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s) (to: SA; cc: -; contact: CMCC), 3GPP RAN#106, December 2024

- RP-243276	RAN Chair’s Guidance for 6G Study: ITU Focused, 3GPP RAN#106, December 2024

- “Working document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.7 to Working Party 5D Chairman’s Report.

- Report ITU-R M.2410-0 (11/2017), “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”.

- 3GPP TR 37.910 Study on self evaluation towards IMT-2020 submission (Release 16)

- RP-250819, Moderator's summary for IMT-2030 TPRs, Moderator (CMCC), 3GPP RAN#107, March 2025

- RP-250820, Direct Submission to ITU-R from TSG RAN#108 on IMT-2030 TPR values, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, Novamint, 3GPP RAN#107, March 2025

# Annex 1: proposed new TPR definitions for IMT-2030

Note: Following texts are cited from latest TPR working document of ITU-R WP5D. Tracked changes are applied in case of proposing modifications.

## Energy efficiency

Energy efficiency is an important metric of sustainability. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics.

This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario.

The requirement is the relative energy savings between the selected load case(s) and a reference case. Proponents should report the power model used for the evaluation, as well as impacts on communication performance (e.g. QoS, user perceived throughput, paging latency, access delay etc.) [note: reported by proponent, not the part of TPR requirement] associated with the evaluated power saving technologies, see [IMT-2030 EVAL] for details.

The minimum requirements for Energy efficiency are summarized in Tables below.

[Editor’s note: The following table needs further discussion, including detailed items and the evaluation methodology such as analytical and/or simulation]

Table: The minimum requirements for Network Energy efficiency

[note: Analysis approach for unloaded case. Simulation approach for loaded cases, traffic model e.g. FTP3 as TR38.864. Different load levels for Dense-urban and Rural.]

| [Test environment(s)] ** | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| N.A (by analysis) | empty load | Fully loaded IMT-2030 case | X1% |
| Dense urban-IC | [>=10%] load | Fully loaded IMT-2030 case | X2% |
| Rural-IC | [>=5%] load | Fully loaded IMT-2030 case | X3% |

*empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100

** needed only in case of simulation

Table: The minimum requirements for Device Energy efficiency

[note: Analysis approach for unloaded case. Analysis or simulation for loaded case.]

| [Test environment(s)] ** | Selected load case(s) | The reference case | Relative power saving (%) |
| --- | --- | --- | --- |
| N.A (by analysis) | empty load | Fully loaded IMT-2030 case | A1% |
| [Dense urban-IC] (TBD analysis or simulation) | [>=5%] load or [100Mbps] | fully loaded IMT-2030 case | A2% |

## Sensing-related capabilities

- Detectability (Detection / False Alarm Probability) – Detection probability, PD, is the probability of correctly detecting the presence of the object, and False alarm probability, PFA, is the associated probability that an object is detected given that the object is not present.

- Localization Accuracy - Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value and is the difference between the calculated location and the actual location of an object.

- Velocity Accuracy - Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target.

- Sensing Resolution – Is defined as the minimum difference of the measured value (e.g, range, location) of two target objects, in order to distinguish these two target objects. (Ed note: this may be evaluated by an analytical approach).

## AI-related capabilities

AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications.

The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling related to AI functionalities over the radio interface (between UE and Network). This should include support for one or more of the following for IMT-2030 assisting AI applications i.e. “Network for AI” and/or using AI to improve IMT-2030 performance, i.e. “AI for network”

-	Distributed data processing

- 	Distributed learning

-	AI computing

-	AI model execution

-	AI model inference

-	or other AI capabilities of the RITs/SRITs (to be reported by the proponent)

## Composite requirement

[Editor’s note: Name of this TPR will be discussed further. Currently this is a placeholder.]

This composite requirement is defined as the number of users  that fulfil the required success probability (reliability) of transmitting a layer 2/3 packet within a required [maximum time / latency], which is the time it takes to deliver a data packet [for a given arrival rate] from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Immersive Communication [and Hyper-Reliable Low Latency Communication] usage scenario.

The minimum performance requirement for various test environments is summarized in Table X.

TABLE X

Composite requirement (title_placeholder)

[editor’s note: This table needs further discussions.]

| Test environment | Data rate (Mbits/s) | Latency (ms) | Reliability | Number of satisfied users (/km2) |
| --- | --- | --- | --- | --- |
| Dense Urban - Immersive Communication | [DL = 30Mbps /UL=10Mbps] | [DL=10ms/UL=30ms] | [99%] | [500-1000] devices per km2 |  |
| Other use cases if any, are FFS |  |  |  |  |  |

# Annex 2: analysis and simulation results

## Spectral efficiency

Based on extension of ITU IMT-2020 evaluation methodologies [6], spectral efficiency is simulated for three eMBB test environments of Dense urban, Rural and Indoor. The evaluated carrier frequency is replaced from 4GHz to 7 GHz. The extended MIMO configuration is BS up to 1536 Tx/Rx antenna elements, UE up to 16 Tx/Rx antenna elements. ITU IMT-2020’s configuration is BS up to 256 Tx/Rx antenna elements, UE up to 8 Tx/Rx antenna elements (AE) for 4GHz carrier frequency [6].

The simulation results are in figures and summarized in table below. The total overhead (control channel, reference signal etc.) is counted as 20%. The effect of real channel estimation is considered. Compared with ITU IMT-2020 requirements, 4 to 35 times improvement of spectral efficiency are observed with different extended MIMO configurations at 7 GHz.

Table 8.1: Spectral efficiency of different MIMO configurations@7GHz

| eMBB test environments |  | ITU IMT-2020 requirements of Spectral efficiency (Average/Cell edge, bps/Hz) | Spectral efficiency of different MIMO configurations@7GHz (Average/Cell edge, N x ITU IMT-2020 requirements) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 256x8* | 256x16* | 512x8* | 512x16* |
| DL | Dense urban | 7.8/0.225 | 4.8x/5.6x | 7.2x/9.4x | 6.4x/7.8x | 9x/13.4x |
|  | Rural | 3.3/0.12 | 11.7x/8.4x | 17.7x/14.8x | 16x/12.3x | 22.5x/22.9x |
|  | Indoor | 9/0.3 | 4.1x/6.3x | 6.2x/8.7x | 6.1x/8.9x | 8x/13x |
| UL | Dense urban | 5.4/0.15 | 10.9x/4.2x | 12.4x/5.2x | 13.2x/5.7x | 14.6x/7.4x |
|  | Rural | 1.6/0.05 | 26.7x/8.9x | 29.8x/11.6x | 32.5x/13.6x | 35.4x/17.8x |
|  | Indoor | 6.75/0.21 | 8.3x/16.8x | 10x/21.2x | 11.2x/27x | 12.9x/33.4x |

* for outdoor (Dense urban and Rural)

- BS 256: (M, N, P, Mg, Ng; Mp, Np) = (24,16,2,1,1,8,16), 768 antenna elements

- BS 512 = (24,32,2,1,1,8,32), 1536 antenna elements

For indoor hotspot

- BS 256 = (8,16,2,1,1,8,16), 256 antenna elements

- BS 512 = (8,32,2,1,1,8,32), 512 antenna elements

For outdoor and indoor, UE 8 = (2,2,2,1,1,2,2), UE 16 = (2,4,2,1,1,2,4)

## Composite requirements (for XR use case)

Using the evaluation methodology of 3GPP study on XR (Extended Reality) evaluations for NR in TR38.838, XR capacity is evaluated for eMBB dense urban test environment with similar extended MIMO configurations as above section 7.1 at 7 GHz.

The simulation results are in figures below for DL and UL respectively. It can be seen that, under the simulated cell configurations,

- For downlink, 256x8 MIMO configuration supports about 54 user per cell of ISD=200m (corresponding to 4676 users/km2), with 90% of UEs being satisfied.

- For uplink, 256x8 MIMO configuration supports about 12.5 user per cell of ISD=200m (corresponding to 1082 users/km2), with 90% of UEs being satisfied.

Table 8.2: Traffic model: GOP (Group-Of-Picture) based I/P-Stream traffic model from 3GPP TR38.838

| Application |  | DL I/P-stream | UL pose/control | UL I/P-stream |
| --- | --- | --- | --- | --- |
| Frame per second/Sampling Frequency |  | 60 FPS | 250 Hz | 60 FPS |
| Average data rate |  | R = 30 Mbps α = 2 | 0.2 Mbps | R = 10 Mbps α = 2 |
| Packet delay budget |  | 10ms | 10ms | 30ms |
| Packet success rate |  | 99% | 99% | 99% |
| GOP size |  | K = 8 |  | K = 8 |
| Packet size | Distribution | Truncated Gaussian Distribution | Fixed Size | Truncated Gaussian Distribution |
|  | I-stream Mean | 111112 Bytes | - | 37037 Bytes |
|  | P-stream Mean | 55556 Bytes | - | 18519 Bytes |
|  | STD | 0.105*mean | - | 0.105*mean |
|  | Max | 1.5*mean | - | 1.5*mean |
|  | Min | 0.5*mean | - | 0.5*mean |
| Jitter | Distribution | Truncated Gaussian Distribution |  | Truncated Gaussian Distribution |
|  | Mean | 0 ms |  | 0 ms |
|  | STD | 2 ms |  | 2 ms |
|  | Range | [-4, 4] ms |  | [-4, 4] ms |

## Positioning

To investigate the proper requirement for positioning accuracy, simulations are conducted in two scenarios: InF-SH (Indoor Factory with Sparse clutter and High base station height) and Indoor office (or hotspot as called in ITU). The positioning method of UL-TDOA is used in the simulation with 200MHz SRS bandwidth at carrier frequency around 7GHz.

As results below, the simulation in InF-SH is able to achieve the positioning accuracy of 0.20m@90% CDF for central UE (UE located inside the red box). The result of Indoor office/hotspot is 0.46m@90% CDF due to less LOS BS sites compared with InF-SH scenario.

Accordingly, it is proposed to consider Horizontal accuracy of [20-50cm] @90% for indoor scenario as requirement for positioning accuracy TPR.

Scenario1 InF-SH, ISD=50, BS height=8m                    Scenario2 Indoor office/hotspot, ISD=20, BS height=3m

Central UE

Central UE

| Positioning accuracy (m) |  | 50% CDF | 67% CDF | 80% CDF | 90% CDF |
| --- | --- | --- | --- | --- | --- |
| InF-SH, 7GHz, UL-TDOA, 200MHz | All UE | 0.04 | 0.06 | 0.15 | 0.34 |
|  | Central UE | 0.03 | 0.05 | 0.09 | 0.20 |
| Indoor Hot Spot, 7GHz, UL-TDOA, 200MHz | All UE | 0.06 | 0.18 | 0.43 | 0.95 |
|  | Central UE | 0.04 | 0.09 | 0.17 | 0.46 |

Table: simulation configurations

|  |  | InF-SH | Indoor Hot Spot |
| --- | --- | --- | --- |
| Frequency |  | 6.7GHz | 6.7GHz |
| Bandwidth |  | 200MHz | 200MHz |
| Layout | Room Size (L x W x H) | 300m x 150m x 10m | 120m x 50m x 3m |
|  | ISD | 50m | 20m |
|  | BS number | 18 (6 x 3) | 12 (6 x 2) |
| BS Parameters | BS height | 1.5m | 1.5m |
|  | BS Power | 23dBm | 23dBm |
|  | BS Antenna Number | 1 | 1 |
| UE Parameters | UE height | 1.5m | 1.5m |
|  | UE Power | 23dBm | 23dBm |
|  | UE Antenna Number | 1 | 1 |

## Sensing

Sensing localization accuracy and velocity accuracy are simulated based on the urban grid geometry defined in TR37.885 as below. There are 9 buildings (the green blocks in the figure) with four lanes in between, the lane width is 3.5m. The size of each buildings is 413m (length) x 230m (width) x 20m (height). All BSs are located on the rooftop of the buildings, all the sensing targets (vehicles) are located on the street, and the UEs are located on the sidewalk road. There are totally 18 BS sites, 144 UEs and 27 sensing targets. Inter-site distance (ISD) is 250m. The distance between adjacent UEs is 40m.

The simulations are conducted at the carrier frequency of 7 GHz and 400MHz bandwidth, including 2 cases of BS-UE bistatic LOS and BS-UE bistatic LOS+NLOS. The cooperation among multiple BSs of same or different sites is applied. According to the simulation results, it can be seen that

- For Localization accuracy: take 90% CDF point, “BS-UE bistatic LOS” result is 0.51m, “BS-UE bistatic LOS+NLOS” result is 0.94m

- For Velocity accuracy: take 90% CDF point, “BS-UE bistatic LOS” result is 2.05m/s, “BS-UE bistatic LOS+NLOS” result is 2.95m/s

Layout: Urban grid as TR37.885 ISD=250 m

Localization Accuracy

Velocity accuracy

Table: simulation configurations

| ISD | 250m |
| --- | --- |
| BS number | 18 |
| BS height | 25m |
| UE number | 144 (inter distance 40m) |
| UE height | 1.5m |
| Frequency | 7GHz |
| Bandwidth | 400MHz |
| Antennas TRP | 49dBm |
| Antennas number | 16*32 |
| Target number | 27 ST |
| Target Velocity | 60km/h |

## Energy efficiency

Energy efficiency was introduced as a TPR in IMT-2020. It was defined as qualitative requirement, i.e. “The RIT/SRIT shall have the capability to support a high sleep ratio and long sleep duration.” as in ITU-R report M.2410. To reflect the growing importance of sustainability, it is proposed to define quantitative requirement on energy efficiency TPR for IMT-2030. For the quantitative evaluation of energy efficiency, various methods have been extensively discussed, including sleep ratio in the time domain or relative power saving by applying power model, as well as the evaluated scenarios focusing on unloaded only, or both loaded and unloaded.

We are of the view that “relative power saving of loaded case” is the best way to demonstrate the capability of IMT-2030 on energy efficiency. Below are example evaluations on network and device energy efficiency with the state-of-the-art NR DTX/DRX technology. The 6G energy saving technologies are pending research. Examples below are intended to describe the feasibility of relative power saving with quantitative evaluation.

### Example on network energy efficiency (by simulation approach)

- System level simulation similar as 3GPP TR38.864 “Study on network energy savings for NR”

  - Layout: dense urban and rural, same as Immersive communication

  - Carrier frequency: 4GHz

- Load setting:

  - Load level: add users in each cell until load level >= L% [a load (L)% of a cell is a percentage of resources used for UE specific PDSCH/PUSCH]

  - Traffic model: FTP model 3, Poisson distribution with 200ms mean inter-arrival time of 0.5Mbytes packages, random arrival time within a user and among different users

- Power model: BS Category 2

- Example power saving technology: NR Cell DTX/DRX, (DRX period, on duration) = (1s, 200ms)

- Simulation result

| Test environment | Reference case | Evaluated load level | Relative power saving compared with reference case |
| --- | --- | --- | --- |
| Dense urban | L=100% | L=8% | 73.7% |
| Rural | L=100% | L=6% | 75.2% |

### Example on device energy efficiency (by analysis)

Below is an example evaluation on loaded device energy efficiency with state-of-the-art NR DTX/DRX technology. It is intended to describe the feasibility of “device loaded case”. This example is by analytical approach with fixed MCS. We prefer simulation approach if agreeable, e.g. link level simulation at 50% SNR point, then fading channel, MCS adaptation, HARQ process etc. will be emulated.