3GPP TSG RAN CC#1 for IMT-2030 TPR			RP-25xxxx

Conference call, April 29, 2025

Source:	OPPO

Title:	Proposals on IMT-2030 TPR: Values of legacy TPRs and others

Agenda Item:	xx

Document for:	Discussion

Introduction

In RAN#107 meeting, the IMT-2030 TPR was discussed with summary noted in [1]. In this contribution, our views on Section 1.5 and 2.2 are presented, which will be addressed in the CC#1 for IMT-2030 TPR.

Discussion points on IMT-2030 (6G) TPR

Values of legacy TPRs

Our views on the values of the legacy TPRs are listed in the table below.

| ITU IMT-2030 TPR |  | IMT-2020 Value | Values proposed in this meeting | Moderator proposal | OPPO’s view |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | TBD | Remove the item from IMT-2030 TPR. It is not a requirement from market, but only a theoretical calculation. And this is an excessive marketing which leads to unfeasible user expectation (in IMT-2020, Peak data rate has been 200x of user experienced data rate). If the intention of this item is to justify the CBW requirement, it is OK to add a CBW item, e.g. 200MHz. |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | [2x] of IMT-2020 | Agree with 2x of IMT-2020. |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 2 contributions propose to revisit the evaluation methodology before setting values: - Bandwidth scaling of simulation results for the UL User Experienced Data Rate performance should NOT be used. | DL: TBD UL: Revisit the evaluation methodology before setting values Further discuss applicable deployment scenarios | Agree not to use bandwidth scaling of simulation results. DL: 2x of IMT-2020 UL: 2x of IMT-2020 |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | Prefer 2x of IMT-2020. But agree with 3x of IMT-2020. |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | Prefer 2x of IMT-2020. But agree with 3x of IMT-2020. |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 | [10] | Agree with 10 |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: [1~4] HRLLC: [0.1~1] | TBD | eMBB: 4 URLLC: 1 (No actual justification that IMT-2020 values cannot support HRLLC/IIoT applications in market. Consider the further KPI improvements after the first-wave commercial deployment.) |
|  | Control plane latency (ms) | 20 | [5~20] | TBD | 20 (No justification of new requirement from market.) |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 | [10x] of IMT-2020 | Agree with 10x of IMT-2020. |
| Reliability |  | - 10⁻⁵ | [1~100x] of IMT-2020 | [1-10-5] | 10⁻⁵ (No actual justification that IMT-2020 values cannot support HRLLC/IIoT applications in market. Consider the further KPI improvements after the first-wave commercial deployment.) |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 | Support to add velocity of up to 1000/1200km/h for airplane. No need to improve the spectrum efficiency for the velocity defined in IMT-2020. |
| Mobility interruption time (ms) |  | 0 | 0 ms | [0 ms] | 0 ms |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | 400MHz for system bandwidth (same definition as IMT-2020) | OK with 400MHz |

Others

Our views on the moderator proposals in Section 2.2 of [1] are listed in the table below.

| Moderator proposals | OPPO’s View |
| --- | --- |
| Proposal 2-3: Do not define security, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030. | Prefer to define security and interoperability as qualitative requirements. |
| Proposal 2-4: Sustainability is covered by energy efficiency, do not define separate TPR for sustainability. | Agree not to define it. |
| Proposal 2-5: For coverage, further discuss the following three options: - Option 1: define coverage as a quantitative TPR - Option 2: do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020) - Option 3: define extra connectivity as a qualitative TPR, which was revised from a part of coverage | Prefer Option 2. |
| Proposal 2-6: For resilience, further discuss the following two options: - Option 1: define resilience as a qualitative TPR - Option 2: do not define resilience as a TPR, these can be covered by submission template for IMT-2030. | Prefer Option 2. |

Conclusion

The IMT-2030 TPR should return to the role of guiding technical design of 6G system from market requirement. The proposals in Section 2 are suggested to be considered in CC#1 of IMT-2030 TPR discussion.

References

- RP-250819, Moderator's summary for IMT-2030 TPRs, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.