3GPP TSG RAN CC#1 for IMT-2030 TPR			RP-25xxxx

Conference call, April 29, 2025

Source:	Verizon

Title:	Considerations for IMT-2030 TPR

Agenda Item:	xx

Document for:	Discussion

# Introduction

In RAN#107, the IMT-2030 TPR was discussed and summarized in [1]. In this contribution, we provide our views on two important issues for further discussion of IMT-2030 TPR in CC#1.

# Discussions

## Maximum Achievable vs. Practically user-experienced TPRs

As pointed out by [2], in the IMT-2020 submission for 5G, performance numbers were often chosen to outcompete alternative technologies. This resulted in aggressive theoretical figures that did not always align with what users observed in commercial deployments. As a result, expectation gaps emerged, affecting 5G’s perceived success. For IMT-2030, there is discussion about whether the focus should shift to setting realistic targets that balance ambition with deployment feasibility, thus avoiding a repeat of 5G’s expectation-management challenges.

For the 6G IMT-2030 submission, should our performance targets focus on:

- Option 1: Maximally achievable performance, i.e., theoretical peak numbers calculated based on spectrum efficiency, MIMO rank, protocol overhead, etc., or

- Option 2: Practically achievable performance, i.e., targets that reflect real-world expectations based on typical deployments and user experience.

We still see the value of Option 1, because

- These values can be independently derived from first principles, reflecting the potential of 6G as a technology.

- Practical values in Option 2 depend heavily on operator-specific variables like traffic load, deployment topology, and frequency bands, making standardization of “realistic” numbers difficult, if not risky, i.e., it may result in multiple versions of “achievable” (e.g., 3GPP vs. operator-specific) may need to be explained.

- Philosophically, 3GPP should be driving innovation, not constrained by current (or currently perceived) marketing or deployment limitations.

- And finally, advocating higher TPR sets a high bar for 6G, driving the industry to push technology boundaries and focus on delivering high-quality solutions rather than rushing the process.

On the other hand, we understand the lesson from 5G suggests that over-promising and under-delivering leads to credibility issues. Additional 6G TPR values based on practically achievable goals could better serve both operators and vertical industries, aligning expectations with delivery potential.

Proposal 1: for IMT-2030 TPR submissions,

- Define maximum achievable (not “theoretical”) values as the default presentation for IMT-2030 TPRs to establish technological potential.

- For selected KPIs where the maximum achievable values may significantly diverge from realistic experience, supplement with footnotes or annotations describing typical user-level expectations or e.g., instead of maximum achievale “peak”, 95th/99th-percentile results from simulations of typical or favorable deployment scenarios.

## Maximum Achievable vs. Practically user-experienced TPRs

ITU-R Report M.2412 defines five test environments for evaluating candidate IMT-2020 radio interface technologies (RITs/SRITs) – Indoor Hotspot, Dense Urban, Rural Macro (RMa) for eMBB, Urban Macro (UMa) for mMTC and URLLC. As pointed out by [4], typical suburban deployment in North America, which is somewhat between UMa and RMa with lower user density than UMa but more densely populated with higher structure elevation than RMa is not properly captured by any of the five scenarios We also agree with [4] that the SMa deployment scenario developed in Rel-19 is a good representation of typical suburban deployment in North America.

Proposal 2: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, is included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030 for all envisioned frequency bands.

# Conclusion

The following proposals are to be considered in CC#1 of IMT-2030 TPR discussion.

Proposal 1: for IMT-2030 TPR submissions,

- Define maximum achievable (not “theoretical”) values as the default presentation for IMT-2030 TPRs to establish technological potential.

- For selected KPIs where the maximum achievable values may significantly diverge from realistic experience, supplement with footnotes or annotations describing typical user-level expectations or 95th/99th-percentile results from simulations of typical or favorable deployment scenarios.

Proposal 2: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, is included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030 for all envisioned frequency bands.

# References

- RP-250819, Moderator's summary for IMT-2030 TPRs, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.

- PR-250293, Operator’s view on realistic TPR definition for IMT-2030 / “6G”, Deutsche Telecom et al.

- ITU-R M.2412, Guidelines for evaluation of radio interface technologies for IMT-2020

- RP-250848, Views on 6G Scenarios and Requirements, ATT