3GPP TSG RAN CC#1 for IMT-2030 TPR			                                 RP-xx

Conference call, April 29, 2025

Source:	ZTE Corporation, Sanechips

Title:	Views on RAN study for IMT-2030

Agenda Item:	16.1

Document for:	Discussion and Decision

- Introduction

In RAN#103 meeting, the following RAN plenary work plan was agreed for IMT-2030 aspects [1] where the RAN study includes two phases corresponding to ITU IMT-2030 TPR and 6G general aspects respectively.

RAN plenary work is split into an ITU focused SI (study item) and General RAN SI - IMT-2030 discussion is expected in RAN from 09/24 to 12/24 - RP SI Rel-20 focusing on ITU– IMT-2030: approval 12/24 until 06/25 - RP SI Rel-20 focusing on 6G General: approval 03/25 (after WS), until 06/26

In RAN#106 meeting, the new SID was endorsed for Study on 6G scenarios/use cases and requirements where the first phase on ITU TPR part is included but the second phase on general 6G aspect will be filled in this March RAN plenary meeting.

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - [To fill in additional objectives, on general 6G RAN study, in March’2025]

In this contribution, we elaborate our views on both study phases.

- TPR candidates and values

In ITU-R WP5D#48 in February 2025, input documents including candidate TPR list provided by 3GPP RAN were discussed in SWG radio aspects. A preliminary list of Minimum Technical Performance Requirements (MTPRs) [4] was agreed and sent to SWG Evaluation from SWG radio aspects as shown below:

Preliminary list of Minimum Technical Performance Requirements

(Note: Square brackets indicate further discussion required)

| Minimum Technical Performance Requirement | Method |
| --- | --- |
| AI-related capabilities | (to be determined) |
| Area traffic capacity | (Quantitative) (Note 1) |
| Average Spectral efficiency | (Quantitative) |
| Peak spectral efficiency | (Quantitative) |
| 5th percentile user Spectral efficiency | (Quantitative) |
| Bandwidth | (Quantitative) |
| Connection density | (Quantitative) |
| Control plane latency | (Quantitative) |
| User plane latency | (Quantitative) |
| Energy Efficiency | (Quantitative) |
| Mobility | (Quantitative) |
| Mobility interruption time | (Quantitative) |
| Peak data rate | (Quantitative) |
| User Experienced Data Rate | (Quantitative) |
| Sensing-related capabilities | (Quantitative) |
| Positioning | (Quantitative) |
| Reliability | (Quantitative) (Note 2) |
| [Joint requirement on data rate, latency, reliability and capacity] | To be determined |
| [Coverage] | To be determined |
| [Security] | To be determined |
| [Resilience] | To be determined |
| [Sustainability] | To be determined |
| [Interoperability] | To be determined |

Note 1: The evaluation technique for quantitative requirements is determined by SWG Eval (e.g., analysis, simulation, etc.).

Note 2: The reliability requirement may be replaced by the “Joint Requirement” (if agreed).

On top of the above agreed list from ITU-R WP5D, RAN 107 also agreed to define a composite requirement on data rate, latency, reliability and capacity. Whether to define interoperability, coverage, resilience, security, and a broader sustainability beyond energy efficiency is still under discussion in RAN [10].

Coverage refers to the ability to provide access to communication services for users in a desired service area. This concept does not need to be explicitly defined as a TPR, but can already be reflected by existing communication TPRs. Similar as IMT-2020, there is no need to define TPR for coverage and instead link budget can be provided in submission template.

For security, resilience and interoperability, there are already some descriptions in the IMT-2020 submission template. If needed, more questions can be added to address the new trends in IMT-2030. A TPR would need a clear definition and a clear pass-fail criterion, which we don’t believe these three aspects have.

Proposal 1: Do not define TPR(s) for coverage, security, resilience and interoperability.

As reflected by research literatures [6], there are potentially many technology enablers that can help 6G to achieve higher data rates, lower latency and higher reliability compared to 5G. However, 3GPP 6G day-1 features are still under discussions without formal conclusions. What’s more, the TPR values significantly rely on the test environments (TEs) as well as detailed configurations, and although most people expect that 5G TEs will still be relevant and likely to inherit to 6G, there is yet no formal agreement on that either. Thus, we propose the values for TPRs as follows based on demands. By checking the 3GPP SA1 TR 22.870 “Study on 6G Use Cases and Service Requirements” [12], we can get a clear idea on what use cases 6G needs to be support and what are the demanding TPRs.

Proposal 2: Define the values for the list of TPRs as in the table below to satisfy use cases agreed by SA1.

Target values for TPRs toward IMT-2030

| ITU candidate TPR | Target value of IMT-2030 | Notes | Target value of IMT-2020 | Demanding use cases in IMT-2030 |
| --- | --- | --- | --- | --- |
| Peak data rate | DL: 100, UL: 50 Gbit/s. | 5xIMT-2020 Related to the increase of bandwidth, number of antennas, modulation order, etc | – Downlink peak data rate is 20 Gbit/s. – Uplink peak data rate is 10 Gbit/s. | Holographic communication: '100 Gbps range or beyond' (9.7, 22.870) |
| Peak spectral efficiency | DL: 75, UL: 37.5 bit/s/Hz | 2.5xIMT-2020 (NR can already achieve 1.6x with 256 QAM and 8 MIMO layers. 1024 QAM + 16 layers = 4x) | – Downlink peak spectral efficiency is 30 bit/s/Hz. – Uplink peak spectral efficiency is 15 bit/s/Hz. | See peak data rate |
| User experienced data rate | DL: 500, UL: 250 Mbit/s. | 5x IMT-2020 | – Downlink user experienced data rate is 100 Mbit/s. – Uplink user experienced data rate is 50 Mbit/s. | 8K AR video with compression rate of 25/50 times: DL 960/480 MBps (indoor factory) (11.7, TR 22.870) Immersive gaming: DL 640 Mbps (in a smaller area) (9.1.6, TR 22.870) Synergized gaming enhancement (multi-media via collaboration among UE-network-cloud), UL 100-400 Mbps (9.2.6, 22.870) |
| 5th percentile user spectral efficiency | Indoor Hotspot 0.75(DL), 0.63 (UL) Dense Urban 0.563(DL), 0.45(UL) Rural 0.36(DL), 0.135(UL) bit/s/Hz | 2.5~3x IMT2020 | (bit/s/Hz) Indoor Hotspot – eMBB 0.3(DL) 0.21(UL) Dense Urban – eMBB (NOTE 1) 0.225 0.15 Rural – eMBB 0.12 0.045 | See user experienced data rate |
| Average spectral efficiency | Indoor Hotspot 22.5(DL) 16.875(UL) Dense Urban 19.5(DL) 13.5(UL) Rural 8.25(DL) 4(UL) bit/s/Hz/TRxP | 2.5xIMT-2020 | (bit/s/Hz/TRxP) Indoor Hotspot – eMBB 9 (DL) 6.75(UL) Dense Urban – eMBB (Note 1) 7.8 5.4 Rural – eMBB 3.3 1.6 | See area traffic capacity |
| Area traffic capacity | Downlink 50 Mbit/s/m2 in the Indoor Hotspot-IC | 5x IMT-2020 | Downlink 10 Mbit/s/m2 in the Indoor Hotspot – eMBB | Seamless Immersive Reality in Education: X < 250 Mbit/s/m2 (9.4.6, 22.870) |
| User plane latency | 1ms for IC 0.5ms HRLLC | At least 1/2 x IMT-2020 | 4 ms for eMBB 1 ms for URLLC | 0.1-1ms (ITU-R M.2160) |
| Control plane latency | 10ms consider 5ms | 1/2 or 1/4 x IMT-2020 | 20 ms consider 10ms |  |
| Reliability | 1-10-7 | 1/100 x IMT-2020 | 1-10-5 | Real time digital twins: 1 - 10^(-7) (11.3.6, 22.870) Cooperating Mobile Robots: 1 - 10^(-7) (11.2.6, 22.870) |
| Connection density | 10^8 devices per km2 for Massive | 100x IMT-2020 | 1 000 000 devices per km2 |  |
| Mobility | Add a mobility class for airplanes: 1000 km/h |  | Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Vehicular: 10 km/h to 120 km/h High speed vehicular: 120 km/h to 500 km/h. | ATG communications |
| Mobility interruption time | 0ms | Same as IMT-2020 | 0ms |  |
| Bandwidth | 400 MHz | 4x IMT-2020 | at least 100MHz |  |
| Energy efficiency | TBD |  | / |  |
| Sensing related capability - Detectability (Detection / False Alarm Probability) - Localization accuracy - Velocity accuracy - Sensing resolution | - Detection: 95% - False alarm: 5% - Horizontal accuracy: 2m - Vertical localization accuracy: 5m - Velocity accuracy : 1m/s. | Refer to TS 22.137 sensing service category 2 | / |  |
| Composite requirement on data rate, latency, reliability and capacity | TBD |  | / |  |
| Positioning | Horizontal accuracy: Indoor factory: 0.2m-1m outdoor: 1m-3m Vertical accuracy: Indoor factory: 1m - 3m outdoor: 3m - 5m | Refer to TR 38.855, TR 38.857 | / |  |

It is also worth noting that in the ITU-R, spectrum allocation is a critical mission and the ultimate task and lots of work can be seen as preparatory for spectrum allocation, including the TPRs and evaluations for IMT-2030. In this sense, defining moderately advanced TPR values for IMT-2030 benefit the IMT industry in terms of demonstrating an advanced system performance compared to IMT-2020 and place the IMT industry in a better position to compete for spectrum with other RATs.

Observation 1: Defining moderately advanced TPR values for IMT-2030 helps to place the IMT industry in a better position to compete for spectrum with other RATs.

It can also be noted that 3GPP NR technologies passed the ITU-R IMT-2020 requirements with huge margins for some TPRs, for instance, DL peak data rate can be as high as 171.2 Gbps (in FR2), which is more than 8 times higher than IMT-2020 TPR on DL peak data rate. NR can also achieve 64.6 Gbps (in FR2) UL peak data rate, which is already 6 times higher than IMT-2020 TPR. Thus, even if we propose to set the TPR of peak data rate to be 5 times higher than IMT-2020, it can be seen as a moderately advanced goal as NR can already satisfy this goal.

Observation 2: According to TR 37.910 “Study on self evaluation towards IMT-2020 submission”, many of the proposed IMT-2020 TPR target values are met by 3GPP NR technologies with a huge margin.

During the discussions happening in the ITU-R WP 5D, when discussing the TPR values, members also discuss the related configurations. For instance, to achieve 5 times higher peak data rate, it may imply at least 2 times more bandwidth. To achieve 2.5 times higher peak spectral efficiency, 16 MIMO layers may be supported compared to 8 MIMO layers assumed for IMT-2020. We want to provide some observations on how to correctly interpret the relationship between ITU IMT-2030 TPRs and 3GPP configurations, as well as how a candidate technology can be seen as satisfying IMT-2030 evaluations. From ITU-R M.2412 [11],

8.4 Evaluation configurations Evaluation configurations are defined for the selected test environments. The configuration parameters shall be applied in analytical and simulation assessments of candidate RITs/SRITs. For the cases when there are multiple evaluation configurations under the selected test environment, one of the evaluation configurations under that test environment can be used to test the candidate RITs/SRITs. The technical performance requirement corresponding to that test environment is fulfilled if this requirement is met for one of the evaluation configurations under that specific test environment.

It is clear that from ITU perspective, a technology proposal meets the requirement if any of a supported configuration meets the requirement.

Observation 3: From ITU evaluation perspective, a technology proposal meets the requirement if any of a supported configuration meets the requirement.

As reflected by some previous discussions, the phrase ‘minimum’ TPR by ITU can cause misunderstanding. Some may worry that a ‘minimum’ TPR means that all configurations of a technology must all meet such requirements, while this is certainly not the case.

Observation 4: The ITU ‘minimum’ TPR can be met by using any of the configuration supported by a technology, or, equivalently, the configuration resulting the best performance.

Continuing this discussion, as long as one part of the technology can meet the requirements, the whole technology is judged to pass the evaluation. This means, as long as the best performance of 3GPP 6G meets the IMT-2030 TPRs, the whole 3GPP 6G specifications will be recognized as a valid IMT-2030 technology. Devices and network equipments that operate with any possible configurations will be recognized as part of the 6G system. Take the number of antennas as an example, if the peak data rate is 5 times higher than IMT-2020, which may imply 16 UE Rx antennas at 7 GHz, it means that up to 16 Rx antennas are supported by evaluations (1, 2, 4, 8, 16 are all possible configurations). 3GPP can meet the ITU requirements on peak data rate using 16 Rx configuration at 7 GHz, and it is totally fine if the peak data rate cannot be achieved with 1/2/4/8 Rx as this is how ITU evaluations work and is true for almost all technology proposals.

In short, from ITU evaluation perspective, as long as a part (the best part) of the specifications meet the TPRs, the whole technology proposal meets the IMT-2030 requirements. From commercialization perspective, operators and vendors are free to choose a subset of the specifications / configurations from 3GPP 6G, as all these subsets / configurations are recognized as qualified 6G technologies. From 3GPP perspective, supporting up to 16 antennas at device side for evaluations is an effective way to allow for a higher maximum data rate to be more future proof, as well as making 3GPP terrestrial components more competitive among all the candidate technologies. From both ITU and 3GPP perspectives, the evaluation assumptions do not force vendors or operators to choose any particular implementation, as is clearly stated in the ITU evaluation guidelines,

8.4 Evaluation configurations …… (omitted parts) The configuration parameters (and also the propagation and channel models in Annex 1 of this Report) are solely for the purpose of consistent evaluation of the candidate RITs/SRITs and relate only to specific test environments designed for these evaluations. Therefore, the configuration parameters should not be considered as those that must be used in any deployment of any IMT-2020 system nor should they be taken as the default values for any other or subsequent study in ITU or elsewhere. They do not necessarily themselves constitute any requirements on the implementation of the system.

Thus, supporting more antennas at the device side for the purpose of evaluation has the benefits of being more future proof and demonstrating the advantages of 3GPP terrestrial components, while not forcing implementation choices.

Proposal 3: Consider up to 16 UE Rx for device types such as CPEs at 7 GHz for evaluations, which will be future proof and make 3GPP terrestrial specifications competitive enough for IMT-2030.

On the number of antennas in configurations, we also observe that 8 Rx will likely remain as a configuration under 4 GHz carrier frequency.

Observation 5: According to the ITU discussions, no one proposed to remove Configuration A (4 GHz and 8 UE Rx) from the list of configurations, and thus, 8 UE Rx will likely remain a valid configuration under 4 GHz.

- Summary

Proposal 1: Do not define TPR(s) for coverage, security, resilience and interoperability.

Proposal 2: Define the values for the list of TPRs as in the table below to satisfy use cases agreed by SA1.

| ITU candidate TPR | Target value of IMT-2030 | Notes | Target value of IMT-2020 | Demanding use cases in IMT-2030 |
| --- | --- | --- | --- | --- |
| Peak data rate | DL: 100, UL: 50 Gbit/s. | 5xIMT-2020 Related to the increase of bandwidth, number of antennas, modulation order, etc | – Downlink peak data rate is 20 Gbit/s. – Uplink peak data rate is 10 Gbit/s. | Holographic communication: '100 Gbps range or beyond' (9.7, 22.870) |
| Peak spectral efficiency | DL: 75, UL: 37.5 bit/s/Hz | 2.5xIMT-2020 (NR can already achieve 1.6x with 256 QAM and 8 MIMO layers. 1024 QAM + 16 layers = 4x) | – Downlink peak spectral efficiency is 30 bit/s/Hz. – Uplink peak spectral efficiency is 15 bit/s/Hz. | See peak data rate |
| User experienced data rate | DL: 500, UL: 250 Mbit/s. | 5x IMT-2020 | – Downlink user experienced data rate is 100 Mbit/s. – Uplink user experienced data rate is 50 Mbit/s. | 8K AR video with compression rate of 25/50 times: DL 960/480 MBps (indoor factory) (11.7, TR 22.870) Immersive gaming: DL 640 Mbps (in a smaller area) (9.1.6, TR 22.870) Synergized gaming enhancement (multi-media via collaboration among UE-network-cloud), UL 100-400 Mbps (9.2.6, 22.870) |
| 5th percentile user spectral efficiency | Indoor Hotspot 0.75(DL), 0.63 (UL) Dense Urban 0.563(DL), 0.45(UL) Rural 0.36(DL), 0.135(UL) bit/s/Hz | 2.5~3x IMT2020 | (bit/s/Hz) Indoor Hotspot – eMBB 0.3(DL) 0.21(UL) Dense Urban – eMBB (NOTE 1) 0.225 0.15 Rural – eMBB 0.12 0.045 | See user experienced data rate |
| Average spectral efficiency | Indoor Hotspot 22.5(DL) 16.875(UL) Dense Urban 19.5(DL) 13.5(UL) Rural 8.25(DL) 4(UL) bit/s/Hz/TRxP | 2.5xIMT-2020 | (bit/s/Hz/TRxP) Indoor Hotspot – eMBB 9 (DL) 6.75(UL) Dense Urban – eMBB (Note 1) 7.8 5.4 Rural – eMBB 3.3 1.6 | See area traffic capacity |
| Area traffic capacity | Downlink 50 Mbit/s/m2 in the Indoor Hotspot-IC | 5x IMT-2020 | Downlink 10 Mbit/s/m2 in the Indoor Hotspot – eMBB | Seamless Immersive Reality in Education: X < 250 Mbit/s/m2 (9.4.6, 22.870) |
| User plane latency | 1ms for IC 0.5ms HRLLC | At least 1/2 x IMT-2020 | 4 ms for eMBB 1 ms for URLLC | 0.1-1ms (ITU-R M.2160) |
| Control plane latency | 10ms consider 5ms | 1/2 or 1/4 x IMT-2020 | 20 ms consider 10ms |  |
| Reliability | 1-10-7 | 1/100 x IMT-2020 | 1-10-5 | Real time digital twins: 1 - 10^(-7) (11.3.6, 22.870) Cooperating Mobile Robots: 1 - 10^(-7) (11.2.6, 22.870) |
| Connection density | 10^8 devices per km2 for Massive | 100x IMT-2020 | 1 000 000 devices per km2 |  |
| Mobility | Add a mobility class for airplanes: 1000 km/h |  | Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Vehicular: 10 km/h to 120 km/h High speed vehicular: 120 km/h to 500 km/h. | ATG communications |
| Mobility interruption time | 0ms | Same as IMT-2020 | 0ms |  |
| Bandwidth | 400 MHz | 4x IMT-2020 | at least 100MHz |  |
| Energy efficiency | TBD |  | / |  |
| Sensing related capability - Detectability (Detection / False Alarm Probability) - Localization accuracy - Velocity accuracy - Sensing resolution | - Detection: 95% - False alarm: 5% - Horizontal accuracy: 2m - Vertical localization accuracy: 5m - Velocity accuracy : 1m/s. | Refer to TS 22.137 sensing service category 2 | / |  |
| Composite requirement on data rate, latency, reliability and capacity | TBD |  | / |  |
| Positioning | Horizontal accuracy: Indoor factory: 0.2m-1m outdoor: 1m-3m Vertical accuracy: Indoor factory: 1m - 3m outdoor: 3m - 5m | Refer to TR 38.855, TR 38.857 | / |  |

Observation 1: Defining moderately advanced TPR values for IMT-2030 helps to place the IMT industry in a better position to compete for spectrum with other RATs.

Observation 2: According to TR 37.910 “Study on self evaluation towards IMT-2020 submission”, many of the proposed IMT-2020 TPR target values are met by 3GPP NR technologies with a huge margin.

Observation 3: From ITU evaluation perspective, a technology proposal meets the requirement if any of a supported configuration meets the requirement.

Observation 4: The ITU ‘minimum’ TPR can be met by using any of the configuration supported by a technology, or, equivalently, the configuration resulting the best performance.

Proposal 3: Consider up to 16 UE Rx for device types such as CPEs at 7 GHz for evaluations, which will be future proof and make 3GPP terrestrial specifications competitive enough for IMT-2030.

Observation 5: According to the ITU discussions, no one proposed to remove Configuration A (4 GHz and 8 UE Rx) from the list of configurations, and thus, 8 UE Rx will likely remain a valid configuration under 4 GHz.

- References

- RP-240823 Additional Considerations for 6G Timeline Source: TSG Chairs

- RP-250810 “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- RP-243202	Liaison Statement to External Organizations Minimum requirements related to technical performance for IMT-2030 radio interface(s), source ITU-R WP5D, 3GPP TSG#106, December 2024

- ITU-R WP5D 563 Report on the 48th meeting of Working Party 5D (Geneva, 4-13 February 2025) Chair, WP 5D

- 3GPP RAN, TR 37.910, “Study on self evaluation towards IMT-2020 submission”

- R. Liu, L. Zhang, R. Y.-N. Li, and M. D. Renzo, “The ITU Vision and Framework for 6G: Scenarios, Capabilities, and Enablers,” IEEE Vehicular Technology Magazine, pp. 2–10, 2025.

- R. Liu, Q. Wu, M. Di Renzo, and Y. Yuan, “A path to smart radio environments: An industrial viewpoint on reconfigurable intelligent surfaces,” IEEE Wireless Communications, vol. 29, no. 1, pp. 202–208, 2022.

- R. Liu, J. Dou, P. Li, J. Wu, and Y. Cui, “Simulation and field trial results of reconfigurable intelligent surfaces in 5G networks,” IEEE Access, vol. 10, pp. 122 786–122 795, 2022.

- C. Huang, "Reconfigurable Intelligent Surfaces for Energy Efficiency in Wireless Communication," IEEE Transactions on Wireless Communications, vol. 18, pp. 4157-4170, Aug. 2019.

- RP-250819 Moderator's summary for IMT-2030 TPRs, Moderator (CMCC)

- Report ITU-R M.2412-0 “Guidelines for evaluation of radio interface technologies for IMT-2020”

- 3GPP SA1, TR 22.870, “Study on 6G Use Cases and Service Requirements”

- Appendix: preliminary simulation results

## Composite requirement

For the value of composite requirement for immersive communication, we provide some candidate values shown as follows:

- For test environment of Dense Urban-IC, the DL capacity can be 70Gbits/s/km2.

- For test environment of Urban Macro-IC, the DL capacity is 10G bits/s/km2

  - Traffic model: Video: 45Mbps@60FPS, PER=99%, PDB = 10ms; Haptic: 10 sensors, fixed 100 Byte per sensor, PER = 99.9%, PDB=5ms, pareto distribution arrival

For DL composite requirement in test environment UMa-IC, we provide the following statistics of transmitted XR packet sizes and service time for each XR packet, respectively.

| Figure a) transmitted XR packet sizes | Figure b) service time for each packet |
| --- | --- |

The candidate value of capacity in composite requirement can be calculated by:

(Average transmitted XR packet size / average service time for each packet / A) * UE density,

where average transmitted XR packet size can be obtained in Figure a), average service time for each packet can be obtained in Figure b). UE density is average 5 UE per cell

## Sensing

| Sensing mode | TRP monostatic |
| --- | --- |
| Carrier frequency | 6GHz |
| bandwidth | 100 MHz |
| SCS | 30kHz |
| BS Tx power | 49 dBm |
| BS antenna configuration | (M, N, P, Mg, Ng) = (16, 16, 2, 1, 1), dH = 0.5 λ，dV = 0.5 λ |
| BS height | 35m |
| BS noise figure | 5 dBi |
| Thermal noise | -174dBm/Hz |
| UAV height (sensing target) | 200m |
| UAV velocity | 0-60km/h uniform in horizontal |
| UAV distribution | 5 targets in center sector |
| Number of Drops | 200 |

(a) Velocity

(b) Pos_H_Cluster                          (c) Pos_V_Cluster

Figure: TRP monostatic (coordinated sensing) sensing CDF with (M,N,P,Mg,Ng;Mp,Np)=(16,16,1,1,1,1,1)

Table: TRP monostatic (coordinated sensing) sensing results

| Horizontal localization accuracy | Vertical localization accuracy | Velocity accuracy | Detection probability |
| --- | --- | --- | --- |
| 0.850 | 0.565 | 0.332m/s | 98.0% |