3GPP TSG RAN conference call on IMT-2030 TPRs				RT-25xxxxx

April 29, 2025

Agenda item:	##

Source:	LG Electronics

Title:	LGE views on IMT-2030 TPRs

Document for:	Discussion

RAN#107 discussion on TPR

In RAN#107, the following consensus was reached based on the discussion [1].

Proposal 1: Define positioning accuracy as a quantitative TPR

Proposal 2: Define Sensing-related capabilities as a TPR.

Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection.

Proposal 4: Define energy efficiency as a TPR.

Proposal 5: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL

Proposal 6: For bandwidth TPR, it is proposed to set a target value of 400MHz (same definition as IMT-2020)

- TBD on other TPR values

Proposal 7: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR.

- Applicable usage scenario: immersive communication

Proposed TPRs

LGE’s views on TPRs are summarized in the table below.

| ITU IMT-2030 TPR |  | IMT-2020 Value | LGE view | Comment |
| --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | 1.5~2x of IMT-2020 | Higher number of MIMO streams and higher order modulation are considered as enabling technologies |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | 1.5~2x of IMT-2020 |  |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | 3x of IMT-2020 |  |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | 2x of IMT-2020 |  |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | 2x of IMT-2020 |  |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | 1.5~2x of IMT-2020 |  |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: 4 URLLC: 1 | Open to consider more stringent latency requirements for new traffic types |
|  | Control plane latency (ms) | 20 | 20 | Open to consider more stringent latency requirements |
| Connection density (devices/km²) |  | 10⁶ | 106 /km2 for normal devices 107 /km2 for IoT devices |  |
| Reliability |  | - 10⁻⁵ | 10-5 | Open to consider more stringent reliability requirement for new traffic types |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1.5~2 x IMT-2020 Add spectral efficiency for 1000 Km/h (FFS for SE value) | Need to support high-speed aerial vehicle communication including Air-To-Ground (ATG) communication |
| Mobility interruption time (ms) |  | 0 | Close-to-0 ms | ‘Close-to-0ms’ rather than ‘0ms’ is preferred to avoid mandating the support of concurrent connection of source cell and target cell |
| Bandwidth (MHz) |  | 100 | 400 MHz |  |
| Joint TPR |  |  | Quantitative | Down-select combinations from the KPI set {data rate, latency, reliability, number of satisfied UEs}, and determine relation between the relevant KPIs for the given combination, based on SA1 service requirements for immersive traffic. |
| Positioning |  | NA | Vertical accuracy: <3m Horizontal accuracy <1m | The need of strengthening 3GPP 5G-A based positioning requirements is unclear. |
| Sensing-related capabilities |  | NA | Positioning accuracy horizontal/vertical [0.2-10]m, Velocity accuracy horizontal/vertical [0.1-1.5]m/s, Range resolution [0.375-10]m, Velocity resolution [0.3-5]m, Max sensing service latency [5-60000]ms, Refreshing rate [0.05-600]s, Miss detection rate [0.1-5]%, False alarm rate [2-5]%, Confidence level [95-99]% | Open to down-select target metrics Note: Different TPR values are used depending on the target service scenario, sensing target and service area [Table 6.2-1 of 3GPP TS 22.137]. The value ranges need to be updated based on channel model study progress |
| AI-related capabilities |  | NA | Qualitative | Need further discussion |
| Energy efficiency |  | NA | Quantitative | Need further discussion with evaluation on both network energy efficiency and UE energy efficiency. |
| Security |  | NA | Qualitative, if included. | If included, security protection of MAC CE needs to be considered and Post-Quantum Cryptography to avoid HNDL attack needs to be considered. |
| resilience |  | NA | NA | Can be considered under submission template |
| Coverage |  | NA | NA | No need to define coverage as TPR |
| Sustainability |  | NA | NA | This can be covered by energy efficiency |
| Interoperability |  | NA | NA | Can be considered under submission template |

Reference

[1] RP-250819 Moderator's summary for IMT-2030 TPRs, Moderator (CMCC)