3GPP TSG RAN conference call on IMT-2030 TPRs

April 29, 2025

Agenda Item:	16

Source:	Ericsson

Title:	Ericsson proposals for IMT-2030 TPRs

Document for:	Discussion & approval

## Background

At TSG RAN#107, initial discussions on IMT-2030 TPR was held and progress was made as documented in the Moderator’s summary in RP-250819. The Ericsson input on IMT-2030 TPR is in RP-250082, with additional discussions in RP-250083 and TPR text proposals in RP-250084.

## Values of legacy TPRs

The following is the moderator summary table from RP-250819, with Ericsson comments to several TPR added in a new column. For items without comment, the Ericsson view in RP-250082 remains.

| ITU IMT-2030 TPR |  | IMT-2020 Value | Values proposed in this meeting | Moderator proposal | Ericsson comment |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | TBD |  |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | [2x] of IMT-2020 |  |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 2 contributions propose to revisit the evaluation methodology before setting values: - Bandwidth scaling of simulation results for the UL User Experienced Data Rate performance should NOT be used. | DL: TBD UL: Revisit the evaluation methodology before setting values Further discuss applicable deployment scenarios | Proponents should be encouraged to simulate using the full assumed bandwidth. If bandwidth scaling is used, power-limitation needs to be taken into account. We propose to reduce the number of simultaneously active users from 10 to a lower number, e.g. 2. In addition to a having a requirement for the Dense Urban test environment. A lower requirement for e.g. the Rural test environment can be considered. |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | OK with moderator proposal. |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | OK with moderator proposal. |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 | [10] | Unclear whether the moderator proposal refers to 10x IMT-2020 or 10 Mbit/s/m2. Ericsson’s view of 3x IMT-2020 remains. |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: [1~4] HRLLC: [0.1~1] | TBD | We do not see a radio interface latency of 1ms as limiting end-to-end latency. No need for stricter requirements. |
|  | Control plane latency (ms) | 20 | [5~20] | TBD | We do not see a use case that motivates below 20ms control plane latency. |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 | [10x] of IMT-2020 | OK with moderator proposal. |
| Reliability |  | - 10⁻⁵ | [1~100x] of IMT-2020 | [1-10-5] | OK with moderator proposal. |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 |  |
| Mobility interruption time (ms) |  | 0 | 0 ms | [0 ms] |  |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | 400MHz for system bandwidth (same definition as IMT-2020) | Agreed last plenary. |

## Views on other proposals from the summary

The following proposals were made in RP-250819:

Proposal 2-3: Do not define security, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030.

Proposal 2-4: Sustainability is covered by energy efficiency, do not define separate TPR for sustainability.

Proposal 2-5: For coverage, further discuss the following three options:

- Option 1: define coverage as a quantitative TPR

- Option 2: do not define coverage as a TPR, use link budget evaluation approach (same as IMT-2020)

- Option 3: define extra connectivity as a qualitative TPR, which was revised from a part of coverage

Proposal 2-6: For resilience, further discuss the following two options:

- Option 1: define resilience as a qualitative TPR

- Option 2: do not define resilience as a TPR, these can be covered by submission template for IMT-2030.

Ericsson’s comments on proposal 2-3, 2-4, 2-5, and 2-6

Proposal 2-3: Agree that security and interoperability can be covered by the submission template for IMT-2030.

Proposal 2-4: Agree to have Energy Efficiency as the TPR that is representative for Sustainability.

Proposal 2-5: Support Option 2 for IMT-2030 requirement. For 3GPP internal requirement additional requirement for coverage should be considered, as already described in the 6G RAN Req SID.

Proposal 2-6: Support Option 2. For 3GPP internal requirement consider having requirements on resilience.