CC#1 for IMT-2030 TPR discussion			                                     RT-25nnnn

April 29, 2025, 14:00-16:00 CEST, Microsoft Teams meeting

Agenda Item:	N/A

Source:	AT&T

Title:	Discussion of IMT-2030 TPRs

Document for:	Discussion

# Introduction

Part 1 of the RAN SID on 6G scenarios and requirements has the following objective [1]:

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

Furthermore, to enable 3GPP to contribute to the next ITU-R WP5D meeting, whose submission deadline occurs during 3GPP RAN #108, 3GPP RAN #107 endorsed the following work plan [2]:

- E-mail to circulate input documents and summaries of proposals by using ITU AH reflector - Two conference calls to consolidate the proposals aiming to facilitate agreement in RAN#108 - CC#1 from 14:00 to 16:00 CEST on April 29, 2025 – deadline for contributions on April 24 - CC#2 from 14:00 to 16:00 CEST on June 4, 2025

This contribution presents AT&T’s view as input to conference call #1.

# Discussion of IMT-2030 TPRs

Several companies have argued in favor of maximally achievable TPRs. They are argued to be more easily derived from theory, to reflect aspirational “potential”, to “raise the bar” and to “push technology boundaries”. Even proponents have rightfully observed that maximally achievable TPRs are unconstrained.

AT&T is not convinced that the key to driving innovation lies in aspirational potential. More paramount is closing the gap between practically achievable performance and aspirational potential.

In [3], AT&T expressed views how to unleash continuous innovation and continuous disruption in 6G while also having realistic goals that transcend specifications into meaningful and tangible experiences for users, all with beneficial economics. Among these are a software driven design, a scalable and modular design, and a diligent definition of basic features that are thoroughly tested and certified on Day-1 of 6G.

In response, it was then pointed out by some companies that IMT-2020 has the following note: “These requirements are not intended to restrict the full range of capabilities or performance that candidate RITs/SRITs for IMT-2020 might achieve, nor are they intended to describe how the RITs/SRITs might perform in actual deployments under operating conditions that could be different from those presented in other ITU-R Recommendations and Reports on IMT-2020.”

The overarching goal of the work in ITU-R is to provide minimum technical performance requirements. This is consistent with previous versions of IMT and is what is provided in the ITU-R Recommendation M.2160 “Framework and overall objectives of the future development of IMT for 2030 and beyond” [7]. Specifically, the ITU needs the minimum technical performance requirements (and corresponding evaluation criteria) which show the baseline capabilities of IMT-2030.

Additionally, even if not intended to describe how the RIT/SRIT might perform in actual deployments under operating conditions, at any time, minimum TPRs very well impact design decisions towards the definition of a candidate RIT/SRIT and thereby may very well impact actual deployments under operating conditions. In other words, the statement is only true in one direction: actual deployments under operating conditions may not impact the evaluation of a candidate RIT/SRIT but the requirements for the evaluation of a candidate RIT/SRIT may impact actual deployments under operating conditions.

Hence, proposing maximally achievable TPRs for IMT-2030 goes against what is needed for the ITU-R minimum technical performance requirements. If maximally achievable requirements are to be defined, it would not be within the purview of ITU / IMT-2030 to do this.

Proposal 1: Maximally achievable requirements have no place within the constraints of IMT-2030 as defined by ITU-R WP 5D.

Paramount to the discussion of the capabilities of the usage scenarios for IMT-2030 are also the guidelines for evaluating these capabilities and the corresponding test environments and evaluation configurations/criteria. In ITU M.2412 [4], “guidelines for evaluation of radio interface technologies for IMT-2020,” the evaluation methodology, test environments and evaluation configurations were described. Five test scenarios were chosen, including three outdoor scenarios, - dense urban for eMBB, rural-eMBB, urban Macro-mMTC and URLLC. The deployment parameters of the 3GPP channel models in TR38.901 [5] are used for evaluating the identified scenarios.

The 3GPP channel models are widely used in the industry at large to evaluate various technology components. Their validity and ability to model realistic deployment scenarios is of utmost importance to corroborate the performance of the algorithms used to improve the communication links.

While 3GPP channel models’ parameters are used to model equivalent deployment scenarios in various cities for different operators, often the urban nature of different cities differs across countries and across continents. There is, for example, a big difference in the terrain, urban planning and consequently deployment scenarios between San Francisco and Dallas or New York city, as well as a big difference between San Francisco and Prague or Paris. In the United States, a typical inter-site distance in suburban environments is on average around 1700m. The UMa channel model for urban Macro environments assumes a 500m ISD.

A typical city in the United States, outside of the downtown area, which can be modeled as a sub-urban or sometimes urban environment, is characterized by big residential areas of one or two-story buildings, high vegetation, sometimes rising above 25m, and commercial buildings that are 4 floors in height. These types of deployment scenarios are not well captured by the urban macro UMa scenario, used in the IMT-2020 evaluations, nor the rural macro RMa scenario, used to evaluate eMBB in IMT-2020; yet they constitute more than 70% of the deployment configurations in north America.

Observation 1: Urban Macro and Dense Urban Macro used for evaluating capabilities in IMT-2020 do not include typical urban scenarios in north America

In Rel. 19, an ongoing study item on channel modelling enhancements for 7-24GHz for NR [6], agreed to introduce a new channel model for suburban macro deployments called SMa – suburban macro scenario. The suburban macro scenario captures all the characteristics for a suburban macro deployment, including the inter-site distance, the base station height and the nature of the clutter in these environments.  It is thus very important to include this model in the evaluations of capabilities for IMT-2030, for all the considered frequencies, and all relevant usage scenarios.

Proposal 2: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, are included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030.

Proposal 3: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, are included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030 for all envisioned frequency bands, especially low bands and mid bands (up to 24GHz).

We conclude by briefly mentioning our support for two other proposals related to the IMT-2030 TPRs, namely, the definition of composite requirements, e.g., on data rate, latency, reliability, and area traffic capacity and a departure from bandwidth scaling in the uplink. Unlike the DL, which enjoys a constant power density, the UL is usually power limited and bandwidth scaling does not seem appropriate. Uplink performance is of increasing importance for current and future use cases while at the same time, higher carrier frequencies widen the separation between DL and UL performance. A more realistic assessment of the uplink performance of 6G is therefore paramount.

Proposal 4: Bandwidth scaling is only used in the downlink

Similarly, latency and reliability are of increasing importance, not just for IIoT type of applications, where generally small payloads are assumed to be successfully transmitted with “ultra-low” latency and “ultra-high” reliability, but also for “eMBB like” use cases such as immersive communications. Defining n-tupels of several TPRs that allow for different trade-offs between, e.g., data rate, latency, and reliability are therefore a valued addition to IMT-2030.

Proposal 5: 3GPP to agree/propose composite requirements, e.g., on data rate, latency, reliability, and area traffic capacity

# Conclusion

The following is observed and proposed in this contribution:

Observation 1: Urban Macro and Dense Urban Macro used for evaluating capabilities in ITU-2020 do not include typical urban scenarios in north America

Proposal 1: Maximally achievable requirements have no place within the constraints of IMT-2030 as defined by ITU-R WP 5D.

Proposal 2: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, are included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030.

Proposal 3: Suburban Macro deployments, based on the SMa model introduced and developed in Rel. 19, are included in the test scenarios for evaluating the capabilities of the usage scenarios in IMT-2030 for all envisioned frequency bands, especially low bands and mid bands (up to 24GHz).

Proposal 4: Bandwidth scaling is only used in the downlink

Proposal 5: 3GPP to agree/propose composite requirements, e.g., on data rate, latency, reliability, and area traffic capacity

# References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC/Verizon/NTT DOCOMO, INC./Deutsche Telekom

- RP-250820, Direct Submission to ITU-R from TSG RAN#108 on IMT-2030 TPR values, CMCC/Verizon/NTT DOCOMO, INC./Deutsche Telekom/Novamint

- RP-250848, Views on 6G Scenarios and Requirements, AT&T

- ITU-R M.2412, Guidelines for evaluation of radio interface technologies for IMT-2020

- 3GPP TR38.901, Study on Channel model for frequencies from 0.5 to 100GHz

- RP-234018, New SID: Study on channel modelling Enhancements for 7-24GHz for NR

- ITU-R Recommendation M.2160-0 (11/2023), Framework and overall objectives of the future development of IMT for 2030 and beyond