3GPP TSG RAN CC#1 for IMT-2030 TPR discussion

April 29, 2025

Source:	MITRE

Title:	Views on Technical Performance Requirements (TPRs) of IMT-2030

Agenda Item:	CC#1

Document for:	Discussion

Background

During RAN#107, the following consensus was reached based on initial discussions on IMT-2030 TPR with full summary documented in [1]. The MITRE input to that discussion is in [2]

Values of TPRs

Our views on TPRs and their values following the outcomes of RAN#107 discussions are captured in the table below.

| IMT-2030 TPR |  | IMT-2020 Value | Moderator Summary | MITRE Comments |
| --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | TBD |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [2x] of IMT-2020 | TBD |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | 3x of IMT-2020 | Simulations to use full assumed BW and use at least two different (urban/rural) test environments. |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 |  |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [~3x] of IMT-2020 |  |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 | At least match IMT-2020 requirement of 10 Mbit/s/m2. |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: 4 URLLC: 1 | TBD |
|  | Control plane latency (ms) | 20 | [5-20] | TBD |
| Connection density (devices/km²) |  | 10⁶ | 106 /km2 for normal devices 107 /km2 for IoT devices |  |
| Reliability |  | - 10⁻⁵ | 10-5 | Consider higher reliability for some use cases (industrial, mission-critical, etc.) |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | [1.5~2x] for IMT-2020 Add spectral efficiency for 1000 Km/h (FFS for SE value) | TBD |
| Mobility interruption time (ms) |  | 0 | [0] ms | TBD |
| Bandwidth (MHz) |  | 100 | 400 MHz | 400 MHz system BW as in IMT-2023 |
| Joint TPR |  |  | Quantitative | From SA WG1 service requirements for immersive traffic. |
| Positioning |  | NA | Vertical accuracy: <3m Horizontal accuracy <1m | Consider higher position accuracy than in 5G |
| Sensing-related capabilities |  | NA | Positioning accuracy horizontal/vertical [0.2-10]m, Velocity accuracy horizontal/vertical [0.1-1.5]m/s, Range resolution [0.375-10]m, Velocity resolution [0.3-5]m, Max sensing service latency [5-60000]ms, Refreshing rate [0.05-600]s, Miss detection rate [0.1-5]%, False alarm rate [2-5]%, Confidence level [95-99]% | Define set of capabilities for each service scenario, with [3] as baseline reference |
| AI-related capabilities |  | NA | Qualitative | TBD |
| Energy efficiency |  | NA | Quantitative | TBD |
| Security |  | NA | Qualitative | Consider security in the IMT-2030 submission template. |
| Resilience |  | NA | NA | Consider defining a qualitative TPR to meet ubiquitous coverage needs, e.g., in disaster situation. Additionally consider it part of the IMT-2030 submission template. |
| Coverage |  | NA | NA | TBD |
| Sustainability |  | NA | NA | TBD |
| Interoperability |  | NA | NA | Can be considered under submission template. |

Summary

Proposal 1: Consider qualitative target TPR(s) for coverage, security, resilience and interoperability either as qualitative values or as part of the IMT-2030 submission template

Proposal 2: TPRs in the table above aim at satisfying the use cases defined by SA WG1.

Proposal 3: Security to be included in the IMT-2030 submission templates addressing preservation of confidentiality and integrity of data and identities part of IMT-2030, quantum-safe, and timely protection.

Proposal 4: Resilience definition according to Proposal 2-3 in [1]. The definition qualitative definition to be based on current understanding of this TRP in ITU-R WP 5D of supporting continuous communication and rapid restauration if discontinued e.g., in disasters.

References

[1]	RP-250819 Moderator's summary for IMT-2030 TPRs, Moderator (CMCC)

[2]	RP-250461 Views on IMT-2030 Performance Requirements for RAN's 6G Study

[3]	3GPP TS 22.137 Integrated Sensing and Communication - Table 6.2-1