3GPP TSG RAN CC#1	for IMT-2030 TPR

Conference Call, Apr., 2025

Source:	Xiaomi

Title:	Xiaomi Views on IMT-2030 TPRs for CC1

Document for:	Discussion and Decision

- Introduction

In RAN#107, the following workplan for 6G TPR discussion was approved [1]:

To complete the work on TPRs in RAN#108, it is proposed that the work will progress between RAN#107 and RAN#108 by: - E-mail to circulate input documents and summaries of proposals by using ITU AH reflector - Two conference calls to consolidate the proposals aiming to facilitate agreement in RAN#108 - CC#1 from 14:00 to 16:00 CEST on April 29, 2025 – deadline for contributions on April 24 - CC#2 from 14:00 to 16:00 CEST on June 4, 2025

Building on our RAN#107 contribution [2], this submission offers further input regarding 6G TPR.

- Value of legacy TPRs

In [3], the value of legacy TPRs were proposed by the moderator. Our views on Moderator proposal are summarized in the table below:

Table 1: Views on values of legacy TPRs

| ITU IMT-2030 TPR |  | IMT-2020 Value | Moderator proposal | Xiaomi Comments |
| --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | TBD | [3-4x] IMT-2020 |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [2x] of IMT-2020 | Agree |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: TBD UL: Revisit the evaluation methodology before setting values Further discuss applicable deployment scenarios | For DL, bandwidth scaling can still be used, and the TPR value can be set as [3-4x] IMT-2020; For DL, we support to use the actual bandwidth in simulation to directly calculate the 5% user experience data rate; |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [3x] of IMT-2020 | We prefer to set the value as [2x] of IMT-2020, but we can accept moderator proposal |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [3x] of IMT-2020 | [2x] of IMT-2020 is preferred, but can accept moderator proposal |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [10] | Fine with moderator proposal |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | TBD | IC: 1ms HRLLC: 0.5ms |
|  | Control plane latency (ms) | 20 | TBD | 10 |
| Connection density (devices/km²) |  | 10⁶ | [10x] of IMT-2020 | Fine with moderator proposal |
| Reliability |  | - 10⁻⁵ | [1-10-5] | Fine with moderator proposal |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 | Support to add new velocity class |
| Mobility interruption time (ms) |  | 0 | [0 ms] | Fine with moderator proposal |
| Bandwidth (MHz) |  | 100 | 400MHz for system bandwidth (same definition as IMT-2020) |  |

- Other TPRs

Our views on other TPR candidates are summarized in the table below:

Table 2: Views on other TPRs

| Candidate TPRs | Comments |
| --- | --- |
| Sensing-related capabilities | Quantitative TPR items for sensing related capabilities should be defined, including Missed detection probability and False alarm probability, Sensing accuracy, and Sensing resolution |
| energy efficiency | a TPR for energy efficiency which can be quantitively evaluated |
| Security | Prefer to not define security, interoperability as IMT-2030 TPRs |
| Sustainability | Agree that it is covered by energy efficiency, there is no need to define separate TPR for sustainability |
| coverage | Prefer to use link budget evaluation approach |
| resilience | Prefer to not define resilience as IMT-2030 TPRs |

- Conclusion:

In this contribution, we summarized our views and comments on 6G TPRs in Table 1 and Table 2.

- References

- RP-250820, Direct Submission to ITU-R from TSG RAN#108 on IMT-2030 TPR values, RAN#107, Mar., 2025.

- RP-250354, Discussion on 6G SI and TPR, RAN#107, Mar., 2025.

- RP-250819, Moderator's summary for IMT-2030 TPRs, Mar. 2025.