CC#2 for IMT-2030 TPR discussion			                       RT-250027

June 4, 2025, 14:00-16:00 CEST

Source:	Moderator (CMCC)

Title:	Moderator's summary for IMT-2030 TPRs in CC#2

- Background

In RAN#107, 3GPP TSG RAN discussed the IMT-2030 TPRs and the following proposals in RP-250819 were agreed:

Proposal 1: Define positioning accuracy as a quantitative TPR Proposal 2: Define Sensing-related capabilities as a TPR. Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection. Proposal 4: Define energy efficiency as a TPR. Proposal 5: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL Proposal 6: For bandwidth TPR, it is proposed to set a target value of 400MHz (same definition as IMT-2020) - TBD on other TPR values Proposal 7: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR. - Applicable usage scenario: immersive communication

In order to submit contributions to ITU-R WP5D before the submission deadline on June 12, two conference calls are planned to progress between RAN#107 and RAN#108 in RP-250820.

  - CC#1 from 14:00 to 16:00 CEST on April 29, 2025 – deadline for contributions on April 24

  - CC#2 from 14:00 to 16:00 CEST on June 4, 2025

- Values of legacy TPRs

| ITU IMT-2030 TPR |  | IMT-2020 Value | Proposed Values | Agreement in CC#2 | Moderator proposal |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value |  |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 |  |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 | For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL - Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. | Note: depending on SE |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered. | Indoor hotspot-IC/Dense urban-IC: TBD on the values Assuming 7GHz with: - BS: up to 1024 elements - UE: up to 6Rx or 8Rx, up to 4Tx or 8Tx or Assuming 4GHz with: - BS: up to 256 elements - UE: up to 6Rx or 8Rx, up to 4Tx or 8Tx Rural-IC: TBD |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | Indoor hotspot-IC/Dense urban-IC: TBD on the values Assuming 7GHz with: - BS: up to 1024 elements - UE: up to 6Rx or 8Rx, up to 4Tx or 8Tx or Assuming 4GHz with: - BS: up to 256 elements - UE: up to 6Rx or 8Rx, up to 4Tx or 8Tx Rural-IC: TBD |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 |  | Depending on average spectral efficiency and bandwidth assumption |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: [1~4] HRLLC: [0.1~1] |  | IC: [4ms] or [2ms] HRLLC: [1ms] or [0.5ms] |
|  | Control plane latency (ms) | 20 | [5~20] |  | [20ms] or [10ms] |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 |  | [1~100x] of IMT-2020 for mMTC |
| Reliability |  | 1- 10⁻⁵ | [1~100x] of IMT-2020 |  | [1-10-5] with same IMT-2020 assumptions [1-10-6~1-10-7] with larger latency assumptions |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h |  | Indoor Hotspot-IC: [1.5x] of IMT-2020 @10 km/h Dense Urban- IC: [1.5x] of IMT-2020 @30 km/h Rural-IC: [1~1.5x] of IMT-2020 @120 km/h [1~1.5x] of IMT-2020 @500 km/h Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. TBD on whether to include other velocities, e.g. 30km/h, 60km/h |
| Mobility interruption time (ms) |  | 0 | 0 ms |  | Alternative 1: Not to define a requirement Alternative 2: [0ms] under certain conditions |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | RAN#107 agreement: set a target value of 400MHz (same definition as IMT-2020) (same definition as IMT-2020) |  |

- Other items in [ ] in ITU

2.1 Resilience

Previous moderator proposal: Proposal 2-6: For resilience, further discuss the following two options: - Option 1: define resilience as a qualitative TPR - Option 2: do not define resilience as a TPR, these can be covered by submission template for IMT-2030.

Definition texts on Resilience for discussion during RAN Plenary #108 (RP-251234, SoftBank)

Resilience refers to capabilities of the networks and systems to continue operating correctly during and after a natural disturbance, such as the loss of primary source of power, etc.

The RIT/SRIT should support functionalities over the radio interface that enable continuous operation or rapid temporary restoration during and after natural disasters. These functionalities may include, but are not limited to, support for connecting to one or more of the following network infrastructures within the terrestrial component of IMT, which are temporarily operated to enhance connectivity during these situations:

- 	HIBS

- 	Advanced relays and repeaters

- 	Base station onboard a vehicle, airship and/or vessel

- 	or other network infrastructures within the terrestrial component of IMT that are robust and/or adaptable to physical damage and functional disruptions of ground facilities caused by disasters (to be reported by the proponent).

- Conclusion

The agreements in CC#2 for IMT-2030 TPRs are captured in the 5th column of the table highlighted in green.

| ITU IMT-2030 TPR |  | IMT-2020 Value | Proposed Values | Agreement in CC#1 | Agreement in CC#2 |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value |  |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 |  |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 | For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL - Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. | Note: depending on SE |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered. | Indoor hotspot-IC/Dense urban-IC: TBD on the values Assuming 7GHz with: - BS: up to 1024 elements - UE: up to 6Rx or 8Rx, up to 4Tx or 8Tx or Assuming 4GHz with: - BS: up to 256 elements - UE: up to 6Rx or 8Rx, up to 4Tx or 8Tx Rural-IC: TBD |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | Indoor hotspot-IC/Dense urban-IC: TBD on the values Assuming 7GHz with: - BS: up to 1024 elements - UE: up to 6Rx or 8Rx, up to 4Tx or 8Tx or Assuming 4GHz with: - BS: up to 256 elements - UE: up to 6Rx or 8Rx, up to 4Tx or 8Tx Rural-IC: TBD |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 |  | Note: Depending on average spectral efficiency and bandwidth assumption |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: [1~4] HRLLC: [0.1~1] |  | IC: [4ms] or [2ms] HRLLC: [1ms] or [0.5ms] |
|  | Control plane latency (ms) | 20 | [5~20] |  | [20ms] or [10ms] |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 |  | [1~100x] of IMT-2020 for mMTC |
| Reliability |  | 1- 10⁻⁵ | [1~100x] of IMT-2020 |  | [1-10-5] with same IMT-2020 assumptions [1-10-6~1-10-7] with larger latency assumptions |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h |  | Indoor Hotspot-IC: [1.5x] of IMT-2020 @10 km/h Dense Urban- IC: [1.5x] of IMT-2020 @30 km/h Rural-IC: [1~1.5x] of IMT-2020 @120 km/h [1~1.5x] of IMT-2020 @500 km/h Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. TBD on whether to include other velocities, e.g. 30km/h, 60km/h |
| Mobility interruption time (ms) |  | 0 | 0 ms |  | Alternative 1: Not to define a requirement Alternative 2: [0ms] under certain conditions |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | RAN#107 agreement: set a target value of 400MHz (same definition as IMT-2020) (same definition as IMT-2020) |  |

# Reference

- RP-250868	Considerations for ITU IMT-2030 TPRs	KT Corp.

- RP-250974	Discussion on 6G Scenarios and Requirements	OPPO

- RP-251037	Samsung views on IMT-2030 TPRs	Samsung

- RP-251039	On IMT-2030 Technical Performance Requirements	Apple

- RP-251181	Ubiquitous Positioning for IMT-2030 TPRs	Airbus

- RP-251216	Discussion on IMT-2030 technical performance requirements (TPR)	vivo

- RP-251234	Proposals on Technical Performance Requirements (TPRs) of IMT-2030	SoftBank Corp.

- RP-251264	Target Values for IMT-2030 Technical Performance Requirements	Nokia

- RP-251309	Input on IMT2030 TPRs	MediaTek Inc.

- RP-251366	Consideration on TPRs relating to ITU-R IMT-2030	Huawei, HiSilicon

- RP-251416	Discussions on minimum technical performance requirements for IMT-2030 radio interface(s)	NTT DOCOMO, INC.

- RP-251420	Views on IMT-2030 TPR	Qualcomm

- RP-251472	Views on IMT-2030 Technical Performance Requirements (TPRs)	China Telecom

- RP-251497	Proposals for IMT-2030 TPR	SK Telecom

- RP-251556	Discussion on minimum TPR for IMT-2030	Xiaomi

- RP-251601	Considerations on Composite Requirement for IMT2030 TPRs	China Unicom

- RP-251624	Views on IMT-2030 TPR	CATT

- RP-251663	Technical Performance Requirements (TPR) for IMT-2030	Ericsson

- RP-251666	Views on the study of IMT-2030 requirements	CEWiT,Tejas Networks,IITM,IITK

- RP-251701	AT&T Views on minimum TPRs for IMT-2030	AT&T

- RP-251738	Views on IMT-2030 TPRs	ZTE Corporation, Sanechips

- RT-250005  Proposal on IMT-2030 TPRs_Huawei, Huawei

- RT-250006  OPPO proposals for IMT-2030 TPR CC#1, OPPO

- RT-250007  T-Mobile proposals for IMT-2030 TPR CC#1, T-Mobile USA

- RT-250008  Verizon proposals for IMT-2030 TPR CC1, Verizon

- RT-250009  ZTE Views on RAN study for IMT-2030 CC 1, ZTE

- RT-250010  Proposal on IMT-2030 TPR CC#1-vivo, vivo

- RT-250011  3GPP TSG RAN CC#1_TPR for IMT-2030_SB_v2, Softbank

- RT-250012  Samsung views on IMT-2030 TPRs, Samsung

- RT-250013  Discussions on minimum technical performance requirements for IMT-2030 radio interface(s), NTT DOCOMO

- RT-250014  Proposals on IMT-2030 TPR, SK Telecom

- RT-250015  LGE views on IMT-2030 TPRs, LG Electronics

- RT-250016  Views on IMT 2030 TPRs, Mediatek

- RT-250017  Views on the study of IMT-2030 requirements, Indian Institute of Technology (Madras), CEWiT, IIT Kanpur, Tejas Networks, Indian Institute of Technology (Hyderabad), WiSig Networks

- RT-250018  Ericsson proposals for IMT-2030 TPR, Ericsson

- Rt-250019  IMT-2030 TPRs – Nokia Views, Nokia

- RT-250020  AT&T input to CC on IMT-2030 TPR discussion, AT&T

- RT-250021  MITRE views on IMT-2023 TPRs for CC#1, MITRE

- RT-250022  Xiaomi Views on IMT-2030 TPRs for CC1, Xiaomi

- RT-250023  Operator’s view on realistic TPR definition for IMT-2030 / “6G”, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM

- RP-250070	View on IMT-2030 Technical Performance Requirements (TPRs)	Samsung

- RP-250082	Technical Performance Requirements (TPR) for IMT-2030	Ericsson

- RP-250083	Discussion of new TPR for IMT-2030	Ericsson

- RP-250084	TR 38.8xx/9xx text proposal for IMT-2030 TPR	Ericsson

- RP-250114	Proposals on Technical Performance Requirements (TPRs) of IMT-2030	SoftBank Corp.

- RP-250145	Target Values for IMT-2030 Technical Performance Requirements	Nokia

- RP-250153	Views on the study of IMT-2030 requirements	Indian Institute of Tech (M), CEWiT, IIT Kanpur, Tejas Networks, Indian Institute of Tech (H), WiSig Networks

- RP-250172	Discussion on 6G Scenarios and Requirements	OPPO

- RP-250182	Views on Candidate Set of TPRs	Spreadtrum, UNISOC

- RP-250189	View on RAN Study on 6G Scenarios and Requirements	SK Telecom

- RP-250210	Discussion on IMT-2030 technical performance requirements (TPR)	vivo

- RP-250263	Discussion on IMT-2030 TPRs	CMCC

- RP-250293	Operator’s view on realistic TPR definition for IMT-2030 / “6G”	Deutsche Telekom, Orange, Telefonica, Telenor, Spark NZ, TeliaCompany

- RP-250298	Views on IMT-2030 TPR	Qualcomm

- RP-250354	Discussion on 6G SI and IMT-2030 TPR	Xiaomi

- RP-250365	Discussions on minimum technical performance requirements for IMT-2030 radio interface(s)	NTT DOCOMO, INC.

- RP-250388	Views on IMT-2030 Technical Performance Requirements (TPRs)	China Telecom

- RP-250431	Further considerations on IMT-2030 TPRs	Huawei, HiSilicon

- RP-250434	On IMT-2030 Technical Performance Requirements	Apple Computer Trading Co. Ltd

- RP-250461	Views on IMT-2030 Performance Requirements for RAN's 6G Study	MITRE Corporation

- RP-250484	IMT2030 TPR considerations	MediaTek Inc.

- RP-250512	Consideration and proposals on IMT-2030 TPRs	CATT

- RP-250567	Views on RAN study for IMT-2030	ZTE Corporation, Sanechips

- RP-250571	Considerations on candidate IMT2030 TPRs for sustainability	China Unicom

- RP‑250646  Views on 6G Scenarios and Requirements    AT&T

# Annex 1: Summary of proposed TPR values

| ITU IMT-2030 TPR |  | IMT-2020 Value | Comments | Assumptions |
| --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | - MTK: [1.5x] IMT-2020 (also fine to remove TPR) - Nokia: NOT to define a requirement: - Option 1: leaving the value for peak data rate for the technology proponent to provide. - Option 2: removing the TPR for Peak data rate. - OPPO: Prefer Alt. 1 given in CC#1. If the intention of this item is to justify the CBW requirement, it is OK to add a CBW item, e.g. 200MHz. If most of companies support Alt. 2, suggest same value as IMT-2030. - AT&T: Alt. 1 or Alt. 2 with [1x] of IMT-2020 - Apple: Prefer Alt. 1 given in CC#1 - Verizon: Define maximum achievable (not “theoretical”) values - Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: Same as IMT-2020 - SKT: Same as or more than IMT-2020 - NTT DOCOMO, Samsung: Prefer Alt. 2 given in CC#1. [1.5x] IMT-2020 - KT: [1.5x] IMT-2020 for non-mmWave, [1x] is acceptable for mmWave - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 22 Gbps for an aggregated bandwidth of 400 MHz and 45 Gbps for an aggregated bandwidth of 800 MHz. The proponent shall report the bandwidth used in the calculation. - LGE: [1.5-2x] IMT-2020 - Ericsson: [2.5x] IMT-2020 - CATT: [1-2.5x] IMT-2020, Alt. 1 is also OK - Xiaomi: prefer Alt. 1 in CC#1 - CMCC: [3-5x] IMT-2020 - Vivo: [4x] IMT-2020 - Qualcomm, Spreadtrum, UNISOC, ZTE, China Telecom (a reasonable value is needed): [5x] IMT-2020 - Huawei: [5-10x] IMT-2020 | - MTK: - 4GHz and 7GHz, with practical bandwidths, not 5MHz FDD and TDD - DL: 1024QAM, 8 layers; UL: 256QAM, 4 layers; Indicate “Assumes ‘FWA’ device type” - NTT DOCOMO: [1.5x] is achieved assuming 1024QAM / 256QAM, 8 layers for DL / UL and bandwidth of 500 MHz. - Qualcomm: DL/UL Layers: Up to 16/8; Mod: Up to 1024QAM - China Telecom: The peak spectral efficiency has consensus with 1.5-2.5 times higher than IMT-2020. Based on the RAN#107 agreement, the target value of bandwidth is 400MHz - CATT: Keeping the same bandwidth applied to IMT-2020 evaluation |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | - SKT: Same as or more than IMT-2020 - Samsung: [1.5x] IMT-2020 - KT, MTK, Nokia: [1.5x] IMT-2020 - Apple: [1.5x] IMT-2020 - LGE, CMCC, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: [1.5-2x] IMT-2020 - China Telecom: [1.5-2.5x] IMT-2020 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: [1-1.5x] IMT-2020 - Ericsson: [1.7x] IMT-2020 - NTT DOCOMO, Vivo: [2x] IMT-2020 - Xiaomi: [1.5x] IMT-2020 - OPPO: [2x] IMT-2020 for DL, [1.5x] IMT-2020 for UL - Huawei: [2-3x] IMT-2020 - Qualcomm, ZTE: [2.5x] IMT-2020 - CATT: Without layer increase: [1.5x]; With layer increase: 3.75x(DL)/3x(UL) of IMT-2020 TPR, depending on consensus to UE antenna - AT&T: [1.5x] IMT-2020 | - MTK: - 4GHz and 7GHz, with practical bandwidths, not 5MHz FDD and TDD - DL: 1024QAM, 8 layers; UL: 256QAM, 4 layers; Indicate “Assumes ‘FWA’ device type” - Samsung: Assuming 1024 QAM (~25% over 256 QAM), 8/4 layers for DL/UL, overhead reduction compared to IMT 2020 (~30% to 20%). - Nokia: [1.5x] can be achieved in DL with 256-QAM and 8-layer MIMO, and in the UL with 256-QAM and 4-layer MIMO. - Apple: For 700 MHz, 4 GHz and 7 GHz the number of UE antenna elements is up to 8 Tx /Rx. - Vivo: assume an antenna configuration to enable eight spatial layers (streams) in both downlink and uplink. Higher modulation (e.g. 1024QAM, 4096 QAM) for downlink is possible. - NTT DOCOMO: Assuming 1024QAM / 256QAM and 8 layers for DL / UL - Qualcomm: DL/UL Layers: Up to 16/8; Mod: Up to 1024QAM - ZTE: NR can already achieve 1.6x with 256 QAM and 8 MIMO layers. 1024 QAM + 16 layers = 4x |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | - SKT: Same as or more than IMT-2020 - CMCC: [1.5-3x] IMT-2020 (DL) - NTT DOCOMO: [1.5x] IMT-2020 - KT, OPPO: [2x] IMT-2020 - MTK: [2.8x] IMT-2020 for DL-7GHz; TBD for UL - CATT: [2-3x] IMT-2020 (DL) - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: - For IC: DL: [1-1.5x] IMT-2020; UL: [1.5-2x] IMT-2020 - For UC: DL: >=2 Mbit/s, UL: >=2 Mbit/s - Bandwidth scaling can be applied in scenarios where the bands being aggregated are in the same frequency range. - LGE, China Telecom: [3x] IMT-2020 - Xiaomi: TBD - Spreadtrum, UNISOC: [3x] IMT-2020 (DL) - vivo : [4-6] IMT-2020 (DL) - ZTE: [5x] IMT-2020 - Huawei: DL: [5-10x] IMT-2020; UL: simulation with actual bandwidth, i.e. no “bandwidth scaling” - Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: - DL: [0.5-5x] IMT-2020 - UL: [0.04-4x] IMT-2020 - Apple: - DL: [3x] IMT-2020 - UL: [4x] IMT-2020 - Qualcomm: - DL: [5x] IMT-2020 - UL: TBD - Samsung: - DL: [4x] IMT-2020 - UL: TBD. Set target values after discussion on UL evaluation configurations. - propose ‘5 th percentile user data rate’ as a new title to distinguish it from IMT 2020 user experienced data rate | - MTK: - 4GHz and 7GHz. Dense Urban only - 7GHz: DL 256 Tx/6Rx, UL 4Tx/256Rx - 4GHz: DL 64Tx/4Rx, UL 2Tx, 64Rx - Vivo: 7GHz, BS: 4x/8x/12x of 128 elements, UE: 4RX and 2/4TX - Ericsson: We are open to to reduce the number of simultaneously active users from 10 to a lower number, e.g. 2. - NTT DOCOMO: Assuming up to 256QAM, 4 layers for DL / UL and bandwidth of 400 MHz - Qualcomm: For Dense Urban. See Antenna assumptions below Evaluation Freq; UE (Tx/Rx); BS (Tx/Rx). 700 MHz; Up to 4/4; Up to 64/64. 4 GHz; Up to 8/8; Up to 256/256. 7HGz; Up to 8/16; Up to 1024/1024. 30 GHz; Up to 32/32; Up to 2048/2048. - Xiaomi: 7GHz carrier frequency is assumed for 6G system. At each TRxP, each antenna port corresponds with 4 antenna elements in macro layer of dense urban and 1 antenna element in indoor hotspot, respectively; At each UE, 4 antenna elements driven by 4 TXRUs are assumed. 1024 antenna elements in macro layer of dense urban or 32 antenna elements in indoor hotspot. - Nokia: For the DL, very large BW was assumed in IMT-2020 (e.g. up to 700 MHz in TDD). This is not practical on non-mm bands. Consider limiting the max BW allowed. |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: - For IC: DL: [1.5x] IMT-2020; UL: [1.5-2x] IMT-2020 - For UC: DL: >0.1 bps/Hz, UL: >0.1 bps/Hz - Apple: [1.5x] IMT-2020 - SKT, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: [1.5-2x] IMT-2020 - China Telecom: [1.5-3x] IMT-2020 - KT, LGE, Nokia, Xiaomi: [2x] IMT-2020 - OPPO: prefer [2x] IMT-2020, but agree with [3x] - vivo, Ericsson (OK with [3x]), CMCC: [2-3x] IMT-2020 - Samsung: [3x] for DL, [2x] for UL - ZTE: [2.5-3x] IMT-2020 - Qualcomm, NTT DOCOMO: [3x] IMT-2020 - Huawei: [3-5x] IMT-2020 - CATT: - DL: [2-3x] IMT-2020 - UL: [1.5-2x] IMT-2020 - MTK: - 7GHz: DL: [2.8x]; UL: [2x] - 4GHz: DL: [1.5x]; UL: [1.3x] | - MTK: - 4GHz and 7GHz only. Dense Urban and Indoor Hotspot only. TBD whether to add e.g.Rural 700MHz and associated targets - 7GHz: DL 256Tx/6Rx, UL 4Tx/256Rx. 4GHz: DL 64Tx/4Rx, UL 2Tx/64Rx. Normalize achieved value with DL/UL radio resource ratio. Assumed UE Tx PC3, open to consider PC2. - HW: 7GHz, BS up to 1536 Tx/Rx antenna elements, UE up to 16 Tx/Rx antenna elements; TE: Dense urban, Rural and Indoor - CMCC: 7GHz, 64 TXRUs with 128/256/512 antenna elements and 128 TXRUs with 512/1024 antenna elements - Samsung: Assuming a larger number of antenna elements than those used for IMT 2020 - OPPO: Antenna assumption for 3x of IMT-2020 in DL: Up to CSI-RS 256T4R@7GHz - CATT: DL: to reach 3x, at 4GHz, max 256 antenna; at 7GHz, max 1024 antenna, 8 antenna at UE; UL:1.5x(assuming 4Tx UE antenna ) ~ 2x(assuming 8Tx UE antenna) of IMT-2020 target - NTT DOCOMO: - Assuming up to 256QAM and 4 layers for DL / UL - Indoor Hotspot: 7GHz, 30GHz; Dense Urban: 4 GHz, 7GHz; Rural: 700 MHz, 4 GHz - Qualcomm: Antenna assumptions are same as user experienced data rate |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks - For IC: DL: [1.5x] IMT-2020; UL: [1.5-2x] IMT-2020 - For UC: DL: >=2 bps/Hz, UL: >=1 bps/Hz - NTT DOCOMO, Apple: [1.5x] IMT-2020 - AT&T, SKT, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: [1.5-2x] IMT-2020 - China Telecom: [1.5-3x] IMT-2020 - KT, Xiaomi, LGE, Nokia: [2x] IMT-2020 - OPPO: [2x] IMT-2020, but can agree with [3x] - vivo, Ericsson (OK with [3x]), CMCC: [2-3x] IMT-2020 - ZTE: [2.5x] IMT-2020 - Qualcomm: [3x] IMT-2020 - Samsung: [3x] IMT-2020. Assuming a larger number of antenna elements than those used for IMT 2020. - Huawei: [3-5x] IMT-2020 - CATT: - DL: [2-3x] IMT-2020 - UL: [1.5-2x] IMT-2020 - MTK: - 7GHz: [2.8x] both DL and UL - 4GHz: DL: [1.5x]; UL: [1.3x] | - MTK: - 4GHz and 7GHz only. Dense Urban and Indoor Hotspot only. TBD whether to add e.g.Rural 700MHz and associated targets - 7GHz: DL 256Tx/6Rx, UL 4Tx/256Rx. 4GHz: DL 64Tx/4Rx, UL 2Tx/64Rx. Normalize achieved value with DL/UL radio resource ratio - NTT DOCOMO: Assuming up to 256QAM and 4 layers for DL / UL - Qualcomm: Antenna assumptions are same as user experienced data rate |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | - MITRE: at least same as IMT-2020 - SKT: Same as or more than IMT-2020 - Apple, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: Same as IMT-2020 - CMCC: [1.5-3x] IMT-2020 - LGE: [1.5-2x] IMT-2020 - OPPO: prefer [3x] IMT-2020, but agree with [5x] - CATT, IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: [2-3x] IMT-2020 - Ericsson, Qualcomm: [3x] IMT-2020 - Xiaomi: [4x] IMT-2020 - Samsung: [3-5x] IMT-2020 - Vivo: [4-6x] IMT-2020 - China Telecom, KT, Nokia, ZTE: [5x] IMT-2020 - Huawei: [5-10x] IMT-2020 - NTT DOCOMO: - [1.5x] IMT-2020 @ Indoor Hotspot - IC - 0.78 Mbps/m2 @ Dense Urban – IC - MTK: - DL: [2.8x] IMT-2020 (for InH) - 7GHz - UL: [2.8x] IMT-2020 (for InH) - 7GHz - Open to add Dense Urban | - MTK: - 4GHz and 7GHz only. Indoor Hotspot. Open to add Dense Urban. - 7GHz: DL 256 Tx/6Rx, UL 4Tx/256Rx - 4GHz: DL 64Tx/4Rx, UL 2Tx, 64Rx - NTT DOCOMO: Assuming up to 256QAM and 4 layers. For Indoor Hotspot-IC, 12 TRxPs and bandwidth of 400 MHz are assumed. Dense Urban-IC also can be considered. Target value is calculated based on the achievable maximum average spectral efficiency assuming bandwidth of 400 MHz and ISD = 200 m. - Qualcomm: For In-Hotspot, derived from Avg SE. - Xiaomi: Assuming that the same TRxP density as 5G, 2 times of improvements on average spectral efficiency, and 2 times increase on channel bandwidth, totally 4 times improvements on area traffic capacity can be expected. - CATT: Indoor hotspot assuming 12TRP only to define value - Samsung: Assuming enhanced average spectral efficiency and wider bandwidth. |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | - AT&T, Qualcomm, KT, LGE, Nokia, OPPO, Samsung, Ericsson, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: Same as IMT-2020 - SKT: Same as or more than IMT-2020 - NTT DOCOMO, CMCC: - IC: 2ms - HRLLC: 0.5ms - China Telecom: - HRLLC: 0.1-0.5ms - Apple: - IC: 1ms - HRLLC: 1ms - Xiaomi, ZTE: - IC: 1ms - HRLLC: 0.5ms - Huawei: - IC: 1ms - HRLLC: 0.5-1ms - Vivo: - IC: 1-2ms - HRLLC: 0.5-1ms - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: - IC: 2-4 ms - HRLLC: 0.5-1ms - MTK: - IC: Unclear that we need an “eMBB” target - HRLLC: 1ms - CATT: IC: 2ms; No strong view to HRLLC while 0.5ms is challenging - Spreadtrum, UNISOC - IC: 2-4 ms - HRLLC: 0.5 ms | - MTK: - 4GHz and 7GHz TDD, with option to combine with 700MHz in uplink - SBFD and UL/DL band decoupling can be considered. - eMBB: If included, depends on assumptions vs HRLLC - NTT DOCOMO: For user plane, SCSs of 15 / 30 kHz for IC and 60 / 120 kHz for HRLLC are assumed. |
|  | Control plane latency (ms) | 20 | - AT&T, KT, MTK, LGE, Ericsson, OPPO: Same as IMT-2020 - SKT: Same as or more than IMT-2020 - Qualcomm, Apple: 15ms - Xiaomi: 5-10ms - Spreadtrum, UNISOC, NTT DOCOMO, CMCC, China Telecom, Huawei, CATT: 10ms - Samsung: 10ms considering 2 step RACH in Rel 16 or potential enhancements such as RACH-less approach for IMT 2030 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 10-20ms - Vivo, 10-15ms - Nokia: 15ms - ZTE: 10ms, consider 5ms | - MTK: - 4GHz and 7GHz TDD, with 700MHz option for UL - SBFD and UL/DL decoupling can be considered. - NTT DOCOMO: For control plane, enhanced procedure may be required (e.g., 2-step RACH) |
| Connection density (devices/km²) |  | 10⁶ | - MTK, Samsung, Ericsson (OK with [10x]), Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM: Same as IMT-2020 - SKT: Same as or more than IMT-2020 - AT&T, KT, OPPO, Nokia, Qualcomm, Apple, CATT: [10x] IMT-2020 - LGE: [1x] for normal devices; [10x] for IoT devices - Xiaomi, NTT DOCOMO: [1-10x] IMT-2020 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: [5-10x] IMT-2020 - Vivo, Huawei, China Telecom: [10-100x] IMT-2020 - ZTE: [100x] IMT-2020 - CMCC: 108-109 connections per km2 per day | - NTT DOCOMO: Assuming bandwidth of 180 kHz, SCS of 15 kHz and packet size of 32 bytes within packet delay of 10s. Assumption of bandwidth should be discussed. |
| Reliability |  | 1 - 10⁻⁵ | - AT&T, Qualcomm (Or 10x, with latency >1ms), KT, MTK, LGE, OPPO, Samsung, Ericsson, Nokia, vivo, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM, Xiaomi: 1-10−5 - SKT: Same as or more than IMT-2020 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 1-10-5 ~ 1-10-6 - Huawei, CMCC, China Telecom: 1-10-6 ~ 1-10-7 - Apple: 1-10−6 - ZTE, NTT DOCOMO: 1-10-7 - MITRE: Consider higher reliability for some use cases (industrial, mission-critical, etc.) | - NTT DOCOMO: Assuming packet size of 32 bytes, required maximum time of 1 ms and SU-MIMO with rank 1. Assumptions of packet size and required maximum time should be discussed. |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | - Mobility class: - AT&T, Qualcomm, KT, SKT, Samsung, MTK, NTT DOCOMO, Nokia, Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM, Apple: same as IMT-2020 - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: Divide Vehicular into two categories 10-30/60 km/h for Low Speed Vehicular and 30/60-120 km/h for Medium Speed Vehicular - ZTE, LGE, OPPO, CMCC, Xiaomi, China Telecom propose to introduce a new class of mobility (up to 1000/1200 km/h) for ATG. - Spectral efficiency: - AT&T, Samsung, OPPO, MTK: Same as IMT-2020 - Vivo: [1.5x] IMT-2020 - LGE: [1.5-2x] IMT-2020 - NTT DOCOMO: [2x] IMT-2020 - Qualcomm, Nokia: the associated data rates should be increased - Vivo suggest to add the practical Urban mobility deployment scenario as a test environment to define the mobility classes and minimum requirement. | - NTT DOCOMO: Assuming up to 256QAM and 2 layers. - Qualcomm: Focus on terrestrial UEs. |
| Mobility interruption time (ms) |  | 0 | - AT&T, Qualcomm, KT, NTT DOCOMO, OPPO, Samsung, CMCC, Xiaomi, China Telecom, Huawei, Apple, CATT, ZTE, vivo, IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 0ms - SKT: Same as or more than IMT-2020 - LGE: Close-to-0 ms (to avoid mandating the support of concurrent connection of source cell and target cell) - MTK: 15ms - Nokia: a low value (TBD). The requirement does not necessarily need to be “0”. | - MTK: If 0ms is targeted, then it shall only apply for special cases such as mTRP or inter site spectrum aggregation. |
| Bandwidth (MHz) |  | 100 | - China Telecom, MTK, Ericsson, CMCC, Huawei, Qualcomm, KT, SKT, MITRE, NTT DOCOMO, OPPO, Samsung, Nokia, vivo, China Telecom, Apple, ZTE: 400MHz - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: 200MHz | - IIT (Madras), CEWiT, IIT Kanpur, Tejas Networks, IIT (Hyderabad), WiSig Networks: Indian regulator is unlikely to give 400 MHz per operator in 7GHz band - Qualcomm: up to 2 GHz (including freqs > 6GHz) |

# Annex 2: Summary of evaluation configurations

| [RP-251037 Samsung] |  |
| --- | --- |
| [RP-251039 Apple] | Proposal 3: for 700 MHz, 4 GHz and 7 GHz the number of UE antenna elements is up to 8 Tx /Rx. |
| [RP-251663 Ericsson] | Add a frequency band in the range ~7GHz to the existing set of IMT-2020 frequency bands. For the frequency band in the range ~7GHz, use up to at least four times more antennas elements than used for 4 GHz for IMT-2020, i.e. up to at least 1024 antenna elements. Consider using more antenna elements also for 30 GHz, at least the same number as for 7 GHz. |
| [RP-251738 ZTE] | - Proposal 3: Consider up to 16 UE Rx for device types such as CPEs at 7 GHz for evaluations, which will be future proof and make 3GPP terrestrial specifications competitive enough for IMT-2030. Observation 5: According to the ITU discussions, no one proposed to remove Configuration A (4 GHz and 8 UE Rx) from the list of configurations, and thus, 8 UE Rx will likely remain a valid configuration under 4 GHz. |
| [RP-251217 vivo] | - Proposal 1. The following configuration assumptions could be considered for 6G scenarios: - Carrier frequency: additional spectrum of 7 GHz. - Aggregated system bandwidth: up to 2 GHz (DL+UL) in 7GHz spectrum. - UE antenna elements: up to 8 Tx and Rx antenna elements. |
| [RP-251216 vivo] | Table 2 Simulation assumptions for spectral efficiency 5G; 6G. Scenario; Dense Urban (ISD=200m); Dense Urban (ISD=200m). Carrier Frequency; 4GHz; 7GHz. Simulation band; 20MHz; 20MHz. SCS; 30KHz; 30KHz. Duplex; DDDSU; DDDSU. gNB (M,N,P,Mg,Ng; Mp,Np); 32P128E: (8,8,2,1,1;2,8); 128P512E: (16,16,2,1,1;4,16) 256P1024E:(32,16,2,1,1;8,16) 512P1536E:(24,32,2,1,1;8,32). UE (M,N,P,Mg,Ng; Mp,Np); 2T(1,1,2,1,1;1,1) 4T/4R (1,2,2,1,1;1,2); 2T(1,1,2,1,1;1,1) 4T/4R (1,2,2,1,1;1,2). DL CSI; SRS-based precoding, non-codebook; SRS-based precoding, non-codebook. UL CSI; Non-codebook; Non-codebook. DL MIMO mode; MU-MIMO; MU-MIMO. DL DMRS port number; 12; 24. UL MIMO mode; SU-MIMO; SU-MIMO. Traffic model; Full buffer; Full buffer. |