3GPP TSG RAN Meeting #100	RP-230905

Taipei, June 12-14, 2023

Agenda Item:	9.3.1.7

Source:	Futurewei

Title:	Discussion on Rel-18 RedCap

Document for:	Discussion and Decision

# Introduction

For the Rel-18 RedCap work item [1], RAN1 first focused on baseband complexity reduction and then started detailed discussions for a peak data rate reduction (PR1) variant after RAN#99 [2]. During the following RAN1 discussions for this variant, concerns were raised that the data rate (and complexity) of the PR1 variant can be higher than the lowest complexity Rel-17 RedCap device. This contribution examines this complexity/data rate issue and provides a proposal for how to resolve this issue.

# Discussion

RAN1 is discussing two variants for Rel-18 complexity reduction: variant 1 (BW3/PR3+PR1) and variant 2 (PR1). With variant 1, complexity reduction and data rate reduction are first achieved by using fewer resource blocks within a 20 MHz BW (BW3/PR3). An add-on peak rate reduction (+PR1) further lowers the data rate. Variant 2 can achieve a complexity reduction by lowering the peak data rate (PR1) of a Rel-17 RedCap device.

## Peak data rate reduction

For peak data rate reduction, the parameters, specifically the product w = vLayers·Qm·f, where the number of layers vlayers{1,2}, the modulation order Qm{1,2,4,8} and the scale factor f{0.4, 0.75, 0.8, 1}, are chosen so that the data rate computation according to [38.306, clause 4.1.2]

becomes lower than the current constraint:

For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 4.

In a nutshell, the value “4” is lowered in Rel-18. This intent was captured in the WID. Due to differences in the number of available PRBs in the above formula, the new PR1-only variant will have a value about four times lower than the original BW3/PR3+PR1 variant for a similar minimum target data rate of 10Mbps.

For discussion purposes, the above formula can be simplified for downlink 15 kHz SCS transmissions and can be expressed as a function of bandwidth (number of PRBs NPRBBW) and w = vLayers·Qm·f,

DL data rate=0.133757∙w∙NPRBBW

For Rel-17 RedCap and variant 2, NPRBBW=106 while for variant 1, NPRBBW=25.

The following figure shows the data rate as a function of w for both variants.

Fig. 1. DL data rate as a function of the product of the number of layers, modulation order, and scale factor for 15 kHz SCS.

The figure shows the 10 Mbps target for Rel-18, the lowest data rate for Rel-17 RedCap (corresponding to w=4) as well as the trendlines for variant 1 and variant 2. Individual points of w for one and two layers for each variant are shown. From Fig. 1 we observe:

- Even with 256QAM and 2 layers, the data rate for variant 1 does not exceed the minimum Rel-17 data rate.

- The data rate for variant 2 (PR1 variant) can be same or larger than the minimum Rel-17 data rate.

Observation 1. For the PR1 variant, the data rate can be higher than the minimum Rel-17 data rate.

With the RAN1#113 agreement on the values of w for each variant (10 Mbps target), the data rates compared to the Rel-17 RedCap minimum data rate are shown in the following table.

Table 1. Minimum DL data rates for two Rel-18 variants compared to Rel-17 RedCap (15 kHz SCS

| R17 w=4 | Variant 1: R18 Baseband and peak rate reduction as add-on, w =3.2 | Variant 2: R18 Peak data rate reduction standalone, w=0.75 |
| --- | --- | --- |
| 56.7 Mbps | 10.7 Mbps | 10.6 Mbps |

The observation shows the difficulty of achieving exactly 10 Mbps. While the data rates are higher than 10 Mbps, the data rates are within the spirit of the work item justification of “The supported peak data rate for Rel-18 RedCap targets to 10Mbps”.

## Interpreting the RAN#99 agreement

Companies in RAN1#112-bis-e and RAN1#113 had different understandings of the RAN#99 agreement

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps

Based on the WID and past agreements, many companies understood that both variants had the same minimum data rate. Other companies understood that both variants supported a fixed data rate.

This difference in understanding is significant. With the understanding of “minimum”: (1) the existing formulas do not change and (2) the support of higher scale factors, 256QAM and/or 2-layer MIMO allows UEs to support higher data rates if so desired (depending on the values of w selected). In contrast, with the understanding of “fixed”: (1) the existing formulas change and (2) the support of 256QAM and/or 2-layer MIMO will not change the data rates. To complete the work item a common understanding of the RAN agreements (or a new compromise) is necessary.

Observation 2. A common understanding of the RAN agreement (or a new compromise) is needed to complete the work item.

## Moving forward

The work item first focused on complexity reduction, which did not have any issue with regards to Rel-18 data rates overlapping with Rel-17 data rates, even when 2-layer MIMO and 256QAM are supported. However, once variant 2 was introduced, the data rate (and complexity) for variant 2 could be higher than Rel-17 RedCap. Given the concerns about the allowable data rates for variant 2 and the different understanding on the RAN agreement, further RAN guidance is needed. Several options are listed below for discussion. Except for opt. 1, there is an assumption of a minimum target data rate of 10 Mbps being supported:

Opt. 1: Data rate is strictly 10 Mbps.

Opt. 2: A minimum of 10 Mbps is supported

Opt. 3: Limit features. In Rel-17 RedCap, DL 256QAM is optional and both 1 and 2-layer devices are supported. If DL 256QAM were excluded from Rel-18, the range of w is 3.2 to 12 (corresponding to 40.12 Mbps) for variant 1 while the range is 0.75 to 12 (corresponding to 170 Mbps) for variant 2. If 2 layers were excluded from Rel-18, the range of w is 3.2 to 8 (corresponding to 26.8 Mbps) for variant 1 while the range is 0.75 to 8 (corresponding to 113.4 Mbps for variant 2). If both were not supported, the range for variant 1 is 3.2 to 6 (corresponding to 20.1 Mbps) and 0.75 to 6 (85 Mbps) for variant 2.

Opt. 4: Limit scale factors. The supported scale factor can be limited to a subset of f{0.4, 0.75, 0.8, 1}, perhaps in combination with 2-layer MIMO and/or 256 QAM. For example, 1 may be disallowed.

Opt. 5: Limit the range of w. Both a minimum and a maximum value of w can be specified. For variant 1, if the upper bound of w were 6, the data rate would be 20.1 Mbps. Supporting both 256QAM and 2-layer MIMO would not be possible for variant 1. For variant 2, if the upper bound of w were 2, the data rate would be 28.4 Mbps. This value means that 256QAM may not be supported.

Opt. 6: Specify a maximum data rate. For example, if the maximum data rate were 20 Mbps, RAN1 would determine the corresponding value of w. For variant 1, 3.2  w  6, the data rate is between 10 and 20 Mbps. For variant 2, 0.75  w  1.5 produces a maximum data rate of 21.3 Mbps. It is up to the UE to select what features to support.

Opt. 7: Specify limits for variant 2 only.

In comparing the options, as Opt. 1 and Opt. 2 had split support in RAN1, so it is unlikely that either option can get full support in RAN. Opt. 3 still has possibly higher data rates for variant 2. Further, it is undesirable to disallow 2-layer UEs as some Rel-17 architectures may be reused for Rel-18 devices. Opt. 4 may prevent some higher data rates but may also exclude some useful combinations of lower data rates. Opt. 5 and Opt. 6 are somewhat equivalent. For Opt. 5, a criterion is needed to determine the range, while for Opt. 6 a range is determined based on a maximum data rate criterion. Opt. 7 is attractive as the original variant did not have any data rate issues, so any fix could be applied only to the new second variant.

We understand the views behind the companies on each side and would be willing to consider compromises. The simplest compromise would be Opt. 2 for the existing variant and Opt. 1 for the new variant, which is acceptable as the variants still behave the same during initial access. However, other compromises that do not prohibit MIMO (such as Opt. 6) could also be acceptable.

Observation 3. By default, Rel-18 RedCap continues to support either 1- or 2-layer MIMO just as in Rel-17.

Proposal: As a compromise, the PR1-only variant has a fixed target data rate of 10Mbps, while the original BW3/PR3+PR1 variant has a minimum target data rate of 10Mbps.

# Conclusion

We observe and propose the following:

Observation 1. For the PR1 variant, the data rate can be higher than the minimum Rel-17 data rate.

Observation 2. A common understanding of the RAN agreement (or a new compromise) is needed to complete the work item.

Observation 3. By default, Rel-18 RedCap continues to support either 1- or 2-layer MIMO just as in Rel-17.

Proposal: As a compromise, the PR1-only variant has a fixed target data rate of 10Mbps, while the original BW3/PR3+PR1 variant has a minimum target data rate of 10Mbps.

# References

RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”, Ericsson, RAN#98, Dec. 12-16, 2022

RP-230778, “Proposal for PR1 in eRedcap”, CMCC, RAN#99, Mar. 20-23, 2023

R1-2303938, “RAN1 agreements for Rel-18 NR RedCap”, Ericsson, RAN1#112bis, Apr. 17-26, 2023

RP-230176, “Views on work item scope for Rel-18 RedCap UEs”, FUTUREWEI, RAN#99, Mar. 20-23, 2023

# Appendices

## Appendix 1

From Rel-18 WID [1]

### 4.1	Objective of Core part WI

The objective is to specify support for the following enhancements:

Power saving/energy efficiency enhancements

- Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4]

  - Note that this objective requires SA2, CT1 and CT4 involvement

Complexity/cost reduction

- Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4]

  - UE BB bandwidth reduction

    - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL

    - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth.

    - Support additional separate early indication(s) [RAN1, RAN2]

  - UE peak data rate reduction

    - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction

    - The relaxed constraint is, e.g., 1 (instead of 4).

    - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap.

  - Both 15 kHz SCS and 30 kHz SCS are supported.

  - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction.

  - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified.

Notes:

- The work defined as part of this WI is not to overlap with LPWA use cases.

- Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured.

- This WI considers all applicable duplex modes unless otherwise specified.

Check in RAN#99 regarding:

- Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

## Appendix 2

From the RAN#99 agreement [2]

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1