3GPP TSG-RAN WG4 Meeting #110											R4-2403878

# Athens, GR, 26 Feb – 01 Mar, 2024

Title: 	WF on 2Rx XR UE requirements

Agenda Item:	6.4

Source: 	Moderator (Apple), Nokia, Meta Ireland

Document for:	Approval

Agreement:

- Two Rx antenna port XR UE: A non-(e)RedCap XR UE that is equipped with only two Rx  antenna ports in frequency band(s) where 4 Rx antenna ports are required. The UE is intended to be worn on human head. When in use, is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

Agreement:

- XR		eXtended Reality

Agreement:

- On conducted receiver sensitivity, consider two options

  - 0.5dB tightening compared to the existing 2Rx UE conducted REFSENS, which is considered feasible by some UE vendors

  - 2Rx XR UE meets the 4Rx handheld UE conducted REFSENS

- On OTA performance:

  - It is agreed to specify OTA TRS requirements per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4Rx XR, considering the performance degradation value for 2Rx based on 4Rx measurement campaign.